Project specific RAM Plan Elektra 2.x ÖBB

ELEKTRA ÖBB

| APPROVAL |  |  |
| --- | --- | --- |
|  | Name and Role/Job Title | Esignature |
| Written by | Florian Moser / RAM-AM |  |
| Verified by | Christian Janneschitz / QAM |  |
| Approved by | Siavash Vaziri / SEM |  |

| LOG OF CHANGES |  |  |  |
| --- | --- | --- | --- |
| Revision | Date | Author | Change Description |
| 01 PD 01 | 2025-07-22 | Florian Moser | First proposal |
| 01 RL | 2025-08-04 | Florian Moser | See section 1.4.2 |

Contents

1.	Scope	6

1.1	Purpose	6

1.2	Intended audience	6

1.3	Document Management	6

1.4	Change History	6

1.4.1	Edition 01 PD 01	6

1.4.2	Edition 01 RL	6

2.	References and Terms	7

2.1	Referenced Documents	7

2.2	Standards and Procedures	7

2.3	Terms, Abbreviations and Acronyms	8

3.	Description of the system	10

3.1	System overview	10

3.1.1	SEA integration into ELEKTRA 2.x	11

3.2	High availability architecture	11

3.3	Hardware components	13

4.	RAM Requirements	14

4.1	Reliability Requirements	14

4.2	Availability Requirements	15

4.3	Maintainability Requirements	15

5.	RAM Plan	16

5.1	Methods and Tools used	16

5.2	RAM detailed activities, analyses and documents	16

5.2.1	Reliability prediction reports	16

5.2.2	Reliability Analysis	16

5.2.3	Maintainability	16

6.	RAM Deliverables	17

6.1	List of RAM deliverable Documents	17

6.2	RAM Requirements	17

6.3	Periodical RAM activities report	17

List of Tables

Table 1: Referenced Documents	7

Table 2: Standards and Procedures	7

Table 3 : Terms, Abbreviations and Acronyms	9

Table 4: RAM requirements	17

List of Figures

Figure 1: System architecture of ELEKTRA 2.x	10

Figure 2: System architecture for high availability	12

Figure 3: Availability concept and related terms	14

# Scope

## Purpose

This document presents the Reliability, Availability and Maintainability Plan (RAM) of the ELEKTRA 2.x system for ÖBB.

The RAM plan defines the reliability, availability and maintainability objectives, tasks to be performed and documents to be produced and delivered at various phases of the project’s lifecycle, to address the functional and technical requirements specified in the respective feature lists and requirements documentation.

For information about organisation, work packages, milestones and application of the Management System refer to [PMP_EL2.x].

This RAM Plan covers everything up to BL 29.40, see also [PSEMP_EL_BL29_40].

## Intended audience

The intended audience of the project specific RAM Plan are the following functions:

- Project Design Authority (PDA)

- Project Manager (PM)

- Solution Engineering Manager (SEM)

- RAM Assurance Manager (RAM-AM)

- Safety Assurance Manager (SAM)

- Quality Assurance Manager (QAM)

- Validator (VAL)

- Verifier (VER)

- Project Security Manager (PSM)

## Document Management

The management of this document will be performed according to the Document Management Rules [DMR] and the Document List and Review Matrix [DLRM].

## Change History

### Edition 01 PD 01

- First proposal.

### Edition 01 RL

- Review comments considered.

# References and Terms

## Referenced Documents

| Id | Reference |
| --- | --- |
| [ADD_EL2.x] | Elektra Evolution, Architectural Design Document - System Architecture; 3BU 15000 0400 PEAPA, Ed.25 |
| [RAM_Plan_Generic] | RAM Plan Generic; 3BU 15000 1712 DUAPA, Ed.06 |
| [EL_GLOSS] | Glossary: Definitions, Acronyms and Abbreviations - ELEKTRA 2.x System 3BU 15000 0081 DCAPA, Ed. 06 |
| [PMP_EL2.x] | Project Management Plan, ELEKTRA Evolution, Technical Project 3BU 15000 0101 DPAPA, Ed. 07 |
| [SD_EL2.x] | Systembeschreibung; ESTW; ELEKTRA 2.x ÖBB; 3BU 15000 0250 EDAPC, Ed. 17 |
| [PSEMP_EL_BL29_40] | PMP-SEMP, ELEKTRA OBB BL29.40; 3BU 15000 2444 DPAPA, Ed.01PD01 |
| [SRS_EL2.x] | ELEKTRA Evolution - System Requirements Specification - ELEKTRA 2.x; 3BU 15000 0300 DTAPA, Ed.38 |
| [VDD_EL_HW] | Version Delivery Description, ELEKTRA Hardware; 3BU 15000 5225 ELAPA, Ed.39 |

Table 1: Referenced Documents

## Standards and Procedures

| Id | Reference |
| --- | --- |
| [Alcarel] | Reliability Standard Failure Rate Factor; 1AA 00210 0001 BCZZA |
| [Birolini] | Reliability Engineering. Theory and Practice; ISBN: 354040287X, 6th Ed |
| [DLRM] | Document List and Review Matrix for engineering projects; 83530224-DOC-AUT-EN-009 |
| [DMR] | Document Management Rules; 83530061-DOC-AUT-EN-013 |
| [EN 50126-1] | EN 50126-1:2017 Railway applications – The Specification and Demonstration of Reliability, Availability, Maintainability and Safety (RAMS), Part 1: Generic RAMS process. |
| [IEC60050(192)] | IEC 60050-192:2015; International Electrotechnical Vocabulary (IEV) - Part 192: Dependability |
| [MIL_HDBK217F] | US MIL-HDBK 217F:1991, Reliability prediction for electronic equipment |

Table 2: Standards and Procedures

## Terms, Abbreviations and Acronyms

| Term | Meaning |
| --- | --- |
| Availability | Probability that an item is in a state to perform a required function under given conditions at a given instant of time, assuming that the required external resources are provided. Error! Reference source not found. A qualitative definition, focused on ability rather than on probability is also usual. The steady-state value of the point availability can be expressed in general by . Error! Reference source not found. |
| CC | Central Controller |
| DGP | Diagnostic Processor |
| DPR | Dependability Requirement |
| EC | Element Controller |
| EC-MUX | Element Controller Multiplexer |
| Failure rate | Limit for of the probability that an item will fail in the time interval , given that the item was new at and did not fail in the interval , divided by . Error! Reference source not found. |
| Fault tree | A logic diagram showing which fault modes of subitems or external events, or combinations thereof, result in a given fault mode of the item. Error! Reference source not found. |
| FiT | Failures in Time (unit: ) |
| Ground Benign (GB) | Nonmobile, temperature and humidity controlled environments readily accessible to maintenance; includes laboratory instruments and test equipment, medical electronic equipment, business and scientific computer complexes, and missiles and support equipment in ground silos. Error! Reference source not found. |
| Ground Fixed (GF) | Moderately controlled environments such as installation in permanent racks with adequate cooling air and possible installation in unheated buildings; includes permanent installations of air traffic control radars and communication facilities [MIL_HDBK217F]. |
| GB 40 | Ground Benign 40˚ C |
| GF 40 | Ground Fixed 40˚ C |
| IC | Interface Controller |
| LAN | Local Area Network |
| LDC | Long Distance Communication |
| Error! Reference source not found. | The Military Handbook is published by the US Department of Defense, and provides mathematical models for electrical, electronic and electro-mechanical components. Each model calculates the component failure rate for given combinations of environmental conditions, component value and type, electrical stress conditions and manufactured quality level |
| MMI | Man Machine Interface |
| MTBF | Mean Time Between Failures - The expectation of the operating time between failures. Error! Reference source not found. is usually reserved for items with constant failure rate . In this case, is the expected value (mean) of the exponentially distributed item’s failure-free operating time. In particular , where is the given, fixed cumulative operating time and the total number of failures (failed items) during . Error! Reference source not found. |
| MTTR | Mean Time To Restore - The expectation of the time to restoration. Error! Reference source not found. is the expected value (mean) of an item’s repair time. It is obtained from the distribution function of the repair time as. For reliability or availability calculation a constant repair rate (i.e. exponentially distributed repair times with) can often be used to get approximate results. Error! Reference source not found. |
| RAM | Reliability, Availability and Maintainability |
| RaSTA | Rail Safe Transport Application |
| RIC | Remote Interface Controller |
| SCU | Serial Communication Unit |
| SEA | Sichere Elektronische Außenelementansteuerung |
| WP | WorkPlace |

Table 3 : Terms, Abbreviations and Acronyms

Additional Elektra 2.x specific abbreviations are listed in [EL_GLOSS].

# Description of the system

## System overview

ELEKTRA 2.x is a modular and scalable electronic interlocking system. It integrates advanced computing platforms, standardized communication protocols, and robust safety and security mechanisms to ensure reliable and efficient control of railway infrastructure. The system structure of the technical system is shown in Figure 1 below, taken from [ADD_EL2.x].

Figure 1: System architecture of ELEKTRA 2.x

The system architecture includes:

- Man-Machine Interface (MMI): Operates on Linux-based PCs, with the EBO2 graphical interface providing intuitive user interaction and support for multiple operator workstations.

- Central and Element Controllers (CCA/CCB, ECA/ECB): These core components manage interlocking logic and field element control. Built on the TAS Platform, they support redundancy and are interconnected via Ethernet.

- Long Distance Communication (LDC): Facilitates secure communication with remote systems using standardized protocols and redundant configurations.

- Diagnosis Processor (DGP): Centralized monitoring and diagnostic unit that ensures system transparency and supports maintenance activities.

- Interface Controllers (ICA/ICB): Interface with physical field elements using fieldbus and microcontroller technologies.

- SEA/SEC/NIC Modules: Handle outdoor element control via CAN bus, offering diverse and safe operation in field environments.

### SEA integration into ELEKTRA 2.x

The SEA (Sichere Elektronische Außenelementansteuerung) represents a modern and modular approach to controlling outdoor railway elements within the ELEKTRA 2.x system architecture. SEA decentralizes the element logic by relocating it directly to the signal mast via the Trackside Control Unit (TCU). See also section 7.4 of [SD_EL2.x]. The EC-MUX acts as a protocol converter between the central control logic and the TCUs, while the Outdoor Element Controller (OEC) within each TCU executes the element-specific logic. Communication between EC-MUX and OEC is handled via the SAHARA protocol, ensuring safe and redundant data transmission.

The EC-MUX (Element Controller Multiplexer) operation on MEN-D6 or MEN-D10 platforms, is a key component in the SEA architecture of ELEKTRA 2.x. It serves as a protocol converter between the central control logic (CC) and the decentralized Trackside Control Units (TCUs).

## High availability architecture

To ensure continuous operation in critical environments, the ELEKTRA 2.x system incorporates a high availability architecture with multiple layers of redundancy and fault tolerance. The design supports rapid recovery from hardware or communication failures without interrupting operational functionality.

Figure 2 below, taken from [ADD_EL2.x] gives an overview of all redundancy concepts used in ELEKTRA 2.x.

Figure 2: System architecture for high availability

| Abbreviations | CCA’, CCB’ | active application |
| --- | --- | --- |
|  | CCA”, CCB” | standby application |
|  | ECA’, ECB’ | primary application |
|  | ECA”, ECB” | secondary application |

The following outlines the redundancy mechanisms implemented across key components: (a detailed description can be found in [ADD_EL2.x] and [SD_EL2.x])

- Man-Machine Interface (MMI): work-place / cold standby redundancy

Work-place redundancy is used on the MMI layer, i.e. more than one MMI component can be used in parallel. In case of a faulty MMI component the user can move to another MMI work-place. Alternatively, one MMI component can have two redundancies like in today's ELEKTRA system. But only one redundancy is running, while the other is turned off. When using EBO2 both redundancies are running.

- Central Controller (CC – CCA/CCB): warm standby redundancy

The redundancies of the CC layer are in a warm standby mode: the operating system is running, HW checks are performed regularly, the application code and the databases are placed into the main memory of the computers. The applications are ready to boot if the active redundancy has to be shut down. This leads to operational consequences (signals fall to stop, routs must be cancelled and set again by the operator).

- Element Controller (EC – ECA/ECB): hot standby redundancy

In a hot-standby configuration, the secondary unit operates in parallel with the primary, continuously synchronized and ready to take over instantly (without operational consequences) in case of failure.

- Interface Controllers (IC – ICA/ICB): non-redundant

Each IC serves a backpanel and connects to ECs via a redundant fieldbus. A failure in an IC affects only the connected interface boards (up to 4 per IC).

- Long Distance Communication (LDC): hot standby redundancy

Same as EC – hot standby

- Diagnosis Processor (DPG): non-redundant

Operates as a single unit. It is isolated from safety-critical functions but provides essential diagnostic data.

## Hardware components

Hardware components of the ELEKTRA 2.x system are listed in the Version Delivery Description, ELEKTRA Hardware, [VDD_EL_HW].

- Central Controller (CC): TAS Platform-based industrial computers (e.g., D6, D10).

- Element Controller (EC): TAS Platform hardware, same as CC.

- Interface Controllers (IC): Microcontroller-based units, see [VDD_EL_HW].

- Long Distance Communication (LDC): TAS Platform-based, using the same hardware as CC/EC

- Man-Machine Interface (MMI): Commercial off-the-shelf (COTS) components.

# RAM Requirements

The reliability, availability and maintainability requirements have to be identified and referenced within the Dependability Analysis. This Dependability Analysis should also provide a proper apportionment and refinement of these requirements according to the system design. A general description of all RAM aspects can be found in [RAM_Plan_Generic].

## Reliability Requirements

Concerning reliability, RAM activities have to focus on the following goals:

- Determination of all possible failures of the system with focus to random hardware faults. Analysis of their consequences with respect to reliability, availability and maintainability (MTTR) and calculation of the associated failure rates.

- Determination of the impact of failures on the functionality of the system.

- The key parameters for describing RAM features of a system, on the basis of [EN 50126-1] are:

    - Mean Up Time MUT, resp. Mean Time Between Failure MTBF,

    - Mean time to Restore MTTR and the

    - Availability A, resp. Unavailability (U = 1-A).

Figure 3, obtained from [EN 50126-1], illustrates key parameters of the availability concept.

Figure 3: Availability concept and related terms

The MUT is the mean lifetime of a system between two failures. MTTR gives the time required from failure to restart operation.

## Availability Requirements

In order to show an appropriate behaviour of the system and sub-systems with respect to availability, a

system analysis shall be carried out to determine availability figures of components and sub-systems. The result shall be part of the Reliability Analysis.  It shall be pointed out, that the availability figures depend on the proper installation, maintenance and operation conditions of the system and sub-systems as stated in the installation and maintenance manuals.

## Maintainability Requirements

Concerning maintainability, RAM activities have to focus on the following goals:

- Identification and implementation of diagnostic measures to minimize system downtime

    - Detect fault alarms that have to be recorded during operation of the system.

    - Prepare appropriate counter measures in case of failures.

- Determine the time necessary for failure diagnosis and recovery of the availability of the system in case of system or component failures.

- Identification of necessary maintenance actions including time required to conduct them.

All requirements concerning maintenance activities need to be summarized to plan necessary actions. Those actions subsume inspection, preventive and corrective maintenance, repair and spare part logistic.

# RAM Plan

## Methods and Tools used

The reliability calculation will be done according to [EN 50126-1] and the [MIL_HDBK217F] based Alcatel Reliability Standard [Alcarel] and according to the environmental conditions Ground Benign (GB) for indoor components and Ground Fixed (GF) for outdoor components.

## RAM detailed activities, analyses and documents

Within this section, documents and activities necessary for defining and analysing reliability, availability and maintainability requirements are described.

### Reliability prediction reports

The reliability prediction report presents reliability figures of electronic components on PBA level, based on the calculation methods described in section 5.1.

### Reliability Analysis

The Reliability Analysis is a key document, with focus on HW relevant aspects, describing reliability, availability and maintainability features.

Reliability aspects of SW have to be assured by following the rules of [EN 50126-1]. An analysis of SW reliability is not part of the Reliability Analysis where only permanent random HW faults and system failures will be analysed.

Failure rates for indoor components will be calculated

To be able to make reliable statements and to demonstrate that the required RAM goals will be obtained, the following information about the hardware and the purpose of the system has to be collected:

    - what kind of sub-systems / components will be used,

    - quantity of each sub-system / component,

    - Failure rates of all PBAs and devices used,

    - MTBF and MTTR for each sub-system / component.

The Reliability Analysis shall point out measures, how availability can be reached according to the mission goals.

### Maintainability

To ensure a proper installation, operation and maintenance of the system, corresponding installation and maintenance documents have to be provided. MTTR and related terms have to be checked with respect to fulfilment of information stated in these documents.

# RAM Deliverables

## List of RAM deliverable Documents

This section summarizes the documents which are foreseen during RAM activities:

- RAM Plan (this document),

- Reliability Prediction Reports,

- Reliability Analysis (required).

Note: RAM documentation must be available according to the Document List and Review Matrix for engineering projects [DLRM].

## RAM Requirements

The RAM related customer requirements need to be checked, clarified if necessary and appropriately refined during the different development phases. If additional RAM related aspects are to be considered in order to fulfill RAM related customer requirements, they should comply with corresponding standards [EN 50126-1].

In the following, RAM related requirements, taken from [SRS_EL2.x] are listed:

| Identifier | Titel |
| --- | --- |
| DPR:0201 (TRT285) | System reliability (System down time < 3 min) |
| DPR:0202 (TRT286) | System reliability (System down time < 8 h) |
| DPR:0203 (TRT287) | System availability |
| DPR:5207 (TRT833) | Serial connection ELEKTRA <-> AzLM - No Availability Restrictions |

Table 4: RAM requirements

## Periodical RAM activities report

The RAM Plan can be updated if necessary, in each phase of the product life cycle, described in [EN 50126-1].

Possible reasons are:

- Review comments of this document,

- Hardware changes,

- Change of standards and procedures.

Each change of the RAM Plan results in a new edition. At each new edition, the reasons for change will be documented in section 1.4.

END OF DOCUMENT