ADD System Architecture

ELEKTRA 2.x

System

| APPROVAL |  |  |
| --- | --- | --- |
|  | Name and Role/Job Title | Esignature |
| Written by | ARC/DEV Matthias Westermeier |  |
| Verified by | QAM Janneschitz Christian |  |
| Approved by | PDA Erwin Glaser |  |

| LOG OF CHANGES |  |  |  |
| --- | --- | --- | --- |
| Revision | Date | Author | Change Description |
| 01 | 1998-11-25 | AUTVA/RTSS PAM SA B. Appel AUTVA/RTPS PAM EP N. König | First release |
| 20 | 2015-03-19 | Thales Austria / TAE ARC J. Strau | See chapter 1.4.20 |
| 21 | 2016-01-29 | Thales Austria / TAE DES F. Kuslics, R. Kun | See chapter 1.4.21 |
| 22 | 2016-03-18 | Thales Austria / TAE DES F. Kuslics, R. Kun | See chapter 1.4.22 |
| 23 | 2021-12-16 | Thales Austria/TRS ARC Karl Padourek | See chapter 1.4.23 |
| 24 | 2022-02-02 | Thales Austria/TRS ARC Karl Padourek | See chapter 1.4.24 |
| 25 | 2024-10-21 | Klaus Botschen | See chapter 1.4.25 |
| 26 | 2025-11-10 | Matthias Westermeier | See chapter 1.4.26 |

Contents

1.	Introduction	9

1.1	Safety relevant Design Decisions	9

1.2	Purpose	9

1.3	Intended Audience	9

1.4	Evolution of the Architectural Design Document	9

1.4.1	ED 01	9

1.4.2	ED 02	10

1.4.3	ED 03	10

1.4.4	ED 04	10

1.4.5	ED 05	10

1.4.6	ED 06	10

1.4.7	ED 07	10

1.4.8	ED 08	10

1.4.9	ED 09	11

1.4.10	ED 10	11

1.4.11	ED 11	11

1.4.12	ED 12	11

1.4.13	ED 13	11

1.4.14	ED 14	11

1.4.15	ED 15	11

1.4.16	ED 16	11

1.4.17	ED 17	12

1.4.18	ED 18	12

1.4.19	ED 19	12

1.4.20	ED 20	12

1.4.21	ED 21	12

1.4.22	ED 22	12

1.4.23	ED 23	13

1.4.24	ED 24	15

1.4.25	ED 25	15

1.4.26	ED 26	16

1.5	References and Terms	17

1.5.1	Referenced Documents	17

1.5.2	Standards and Procedures	22

1.5.3	Terms, Abbreviations and Acronyms	22

2.	System Overview	23

2.1	Objectives for the system architecture	24

2.2	Differences to current ELEKTRA 1.x system	24

2.3	Scalability	25

2.4	Availability	26

2.4.1	High availability	27

2.4.2	Standard availability	29

2.5	Safety aspects	29

2.6	Security aspects	30

2.7	Interoperability with ELEKTRA 1.x components	30

2.8	Replacement of MMI by EBO2	32

3.	System Context	33

4.	System Design	34

4.1	Design Method	34

4.2	Decomposition Description	34

4.2.1	Architecture of an ELEKTRA 2.x component	34

4.2.2	Connectivity between ELEKTRA 2.x components	35

4.2.3	Connectivity with non ELEKTRA 2.x components	36

4.2.4	Connectivity with ELEKTRA 1.x components	37

5.	Description of the Architectural Design Components	39

5.1	Release Unit CAR	39

5.1.1	ELEKTRA Personality (EP) - Extension of CAR for Communication	39

5.1.2	XCEL: Binding to ELEKTRA 1.x	40

5.1.3	CAR on TAS Platform	41

5.1.4	CARIX: CAR on LINUX	43

5.1.5	XEP: EP running without CAR	43

5.1.6	LWATCH: Get diagnosis from CS about connection status	44

5.2	Migration of technical functionality	45

5.2.1	General functionality	45

5.2.2	Optical Status Display	47

5.3	Warm-standby redundancy concept for CC	48

5.4	Hot-standby redundancy concept for EC	49

5.5	Integration of CCA and CCB on one HW computer	50

5.6	Integration of ECA and ECB on one HW computer	51

5.7	Remove VCB for MMI	51

5.8	Long Distance Communication (LDC)	52

5.9	Additional components	53

5.10	Timing Requirements	53

5.11	Functions needed to check the system	54

5.12	LAN components and handling of failures	55

5.12.1	Ethernet addresses used within ELEKTRA LAN	56

5.13	Redundant and non-redundant MMI	57

5.14	End-to-End Checks	58

5.15	Detection of high system load (“Hochlasterkennung”)	59

5.16	Usage of methodsafe EBO2 instead of methodsafe MMI	60

5.17	Methodsafe MMI	60

5.18	Supervision of reactivity	61

5.19	Operating System TAS Platform (TAS-PLF)	62

5.19.1	Use TAS Platform including its hardware	62

5.19.2	Hardware Tests during Startup and Operation	67

5.19.3	Skip memory test after reboot (Quickstart – only TAS-PLF 1.x)	69

5.19.4	Scheduling Policy and Priority used by ELEKTRA	71

5.19.5	Device mapping	73

5.20	Operating System Linux (Distribution defined by TAS-PLF)	76

5.21	Outdoor-Element controlling with EEC (SEA/SEC/NIC)	76

5.22	Use Voting to detect inconsistencies between channel A and channel B	77

5.23	Use OCS SBJ to provide RaSTA protocol for external communication	78

5.24	Use DGP/SCWS for external Log Collector and NTP server connectivity	79

5.25	Use of TCDs for establishing connection between local ELEKTRA 2.x and remote ECs.	79

6.	Taking account of SRACs and SecRACs of underlying systems in the design of ELEKTRA	80

6.1	SRACs TAS-PLF	80

6.1.1	SRACs TAS-PLF CORE-SW	80

6.1.2	SRACs TAS-PLF CORE-HW	98

6.2	SRACs OCS	128

6.2.1	SRACs OCS System 2.x	128

6.2.2	SRACs OCS-EL / OCS-SB	139

6.3	Conditions CAR	148

6.3.1	Identification of the DOORS databases	148

6.3.2	Implementation of the conditions	149

6.3.3	Template for DOORS export Conditions CAR	156

6.4	SecRACs TAS Platform	156

7.	Index of SRACs and Requirements	157

7.1	Index of SRACs	157

7.1.1	Index of SRACs TAS-PLF CORE-SW	157

7.1.2	Index of SRACs TAS-PLF CORE-HW	159

7.1.3	Index of SRACs OCS System 2.x	162

7.1.4	Index of SRACs OCS-EL / OCS-SB	163

7.1.5	Index of Conditions CAR	164

7.1.6	Macro to extract bookmarks defined in the document	165

7.2	Index of Requirements	168

List of Tables

Table 1 : Referenced Documents ELEKTRA and EBO2	19

Table 2 : Referenced Documents CAR	19

Table 3 : Referenced Documents TAS-PLF	20

Table 4 : Referenced Documents OCS	21

Table 5 : Referenced Documents EEC (SEA/SEC)	21

Table 6 : Standards and Procedures	22

Table 7: Selected variants for Project Areas	24

Table 8: Assumptions for number of elements for different scalability levels	25

Table 9: Scalability numbers of ELEKTRA 2.x components	26

Table 10: Number of FE components for different stations	26

Table 11: Interoperability with ELEKTRA 2.x components	31

Table 12: Interoperability with remote systems Austria and Hungary	32

Table 13: Interoperability with remote systems Switzerland	32

Table 14: Threat/Category relationship defined in Table B.2 of [EN 50159]	46

Table 15: Threats/Defences matrix defined in Table 1 of [EN 50159]	46

Table 16: ELEKTRA internal network interfaces – definition for address	56

Table 17: ELEKTRA internal network – definition for subnet	57

Table 18: ELEKTRA internal network – definition for hostnumber	57

Table 19: HW boards of TAS-PLF to be used by ELEKTRA applications	67

Table 20: ELEKTRA scheduling policy and priority	73

Table 21: DOORS EoC module identification of TAS-PLF CORE-SW	80

Table 22: DOORS SRAC module identification of TAS-PLF CORE-SW	81

Table 23: DOORS EoC module identification of TAS-PLF CORE-HW	98

Table 24: DOORS SRAC module identification of TAS-PLF CORE-HW	99

Table 25: DOORS EoC module identification of OCS-EL / OCS-SB	139

Table 26: DOORS SRAC module identification of OCS-EL / OCS-SB	139

Table 27: DOORS EoC module identification of CAR_EP	148

Table 28: DOORS SRAC module identification of CAR_EP	149

List of Figures

Figure 1: System architecture of ELEKTRA 2.x	23

Figure 2: System architecture for high availability	27

Figure 3: System architecture for standard availability	29

Figure 4: ELEKTRA 2.x RCU and DGP with ELEKTRA 1.x	31

Figure 5: Software architecture of an ELEKTRA 2.x Component for a chill application	34

Figure 6: Software architecture of an ELEKTRA 2.x Component for a non-chill application	35

Figure 7: Local communication	36

Figure 8: Communication via ELEKTRA Network	37

Figure 9: Connectivity to current ELEKTRA 1.x components	38

Figure 10: Connectivity to current ELEKTRA 1.x components using a converter	38

# Introduction

## Safety relevant Design Decisions

For all functions, which require SIL4 according to [EN 50128], appropriate methods have to be applied. These methods are defined in chapter 5.4.1 of Safety Plan, ELEKTRA 2.x, [EL_SP].

## Purpose

This ‘Architectural Design Document (ADD)’ is a top-level architecture specification of hardware and software components of the subsystem ‘System Architecture (SA)’ of the ELEKTRA 2.x system. The following chapters give an overview of the proposed internal structures and workings. The aspects described in the following chapters have been widely discussed with members of the technical project ELEKTRA Evolution and the ELEKTRA product management during the ‘Technical Feasibility Study ELEKTRA Evolution’ [EL04].

This document shows how the ELEKTRA 2.x system will be structured in order to satisfy the requirements stated in ‘ELEKTRA Evolution - System Requirements Specification - ELEKTRA 2.x’ [EL03], and the security related requirements for the ELEKTRA 2.x system stated in System Requirements Specification Security Elektra 2.x [SecSRS].

Some system aspects (e.g. „How many field elements can be processed by one EC component“) depend on the characteristics of the target hardware.

## Intended Audience

The intended audience for this ADD is:

Product Design Authority (PDA)

System Engineering Manager ELEKTRA Generic System (SEM)

System Engineering Manager ELEKTRA International (SEM)

System Engineering Manager ELEKTRA Switzerland (SEM)

System Engineering Manager ELEKTRA Austria (SEM)

Software Developer (DES) of ELEKTRA 2.x, ELEKTRA 2.x MMI, EEC

Safety Assurance Manager (SAM)

Speciality Engineer – RAM Assurance Manager (RAM)

Quality Assurance Manager (QAM)

Security Assurance Manager (SecAM)

Project Security Manager (PSM)

## Evolution of the Architectural Design Document

### ED 01

First edition

### ED 02

Update of chapter 2.3;

Update of referenced requirements in chapter 5

### ED 03

add end-to-end-checks;

update requirement identifiers

### ED 04

add redundant MMI (new chapter 5.13);

EP is part of CAR (chapter 5.1 – 5.4 have been changed to 5.1.1 – 5.1.4);

DPR: 0216, DPR: 0228, DPR: 0229 und SAR: 1214 added

### ED 05

New requirements added:
FUR: 1106: chapter 5.13
FUR: 1107: chapter 5.12, 5.13
SAR: 1101, SAR: 1102: chapter 5.3

### ED 06

New requirements added:
FUR: 3801: chapter 5.11
FUR: 3802: chapter 5.11
FUR: 3803: chapter 5.11
FUR: 3804: chapter 5.13
FUR: 3805: chapter 5.13
FUR: 3806: chapter 5.13

Add editions in chapter “references” (especially for SRS).

Chapter 2: Consider new LAN-requirements and those requirements which are were added in edition 06.

### ED 07

Correction:
chapter 5.14.1.1: LDI removed from list of exceptions

New requirements added:
new chapter 5.15

New document reference [24]

### ED 08

New requirements added:
SAR: 2501 (TRT659)

SAR: 2502 (TRT660)

Extension:
chapter 5.14: Extended by station-specific identifier

### ED 09

The reasons for this edition are the development of EBO2 and ELEKTRA2 for Hungary.

Corrected Table 6: 1.x MMI only for development phase

Chapter 4.2.1: new figure 6

Chapter 4.2.3: added “figure 10” to last figure

Chapter 5.14.5: description updated

EBO2 added: Chapter 2, 2.7 and 5.16, document reference [25]

XEP added: Chapter 5.1.5

### ED 10

The reason for this edition is the addition of requirements for ELEKTRA CH.

New Chapter 5.17

New Chapter “Index of Requirements”

### ED 11

The reason for this edition is the addition of new requirements.

New Chapter 5.18

### ED 12

The reason for this edition is a new customer (Elektra Bulgaria, NRIC).
Modification in chapter 2 and chapter 5.2

Requirement number increased (full requirement representation).

### ED 13

Corresponding [3] the requirement DPR: 0328 (TRT7) is deleted in chapter 2.4.1, 5.3, 5.4 and 7.

### ED 14

New references [32] – [36].

New chapter 5.19 including new requirements SAR:0404 and SAR:0405.

Chapter 5.2 splitted into 2 subchapters.

Replaced OBB, SBB, MAV by Austria, Switzerland, Hungary.

### ED 15

Extended chapter 5.2.2 by prioritisation for usage of LEDs.

### ED 16

Extended chapter 2 and chapter 2.3 by SEA (‘Sichere Elektronische Außenelementansteuerung’)

New chapter 5.20‚ Outdoor-Element controlling with SEA

### ED 17

Chapter 1.5: Corrected outdated references.

Chapter 5.14.1.1: Adjusted the description of how timeout signal may be excluded from end-to-end checks.

Chapter 5.14.1.2: Improved description.

Chapter 5.14.1.3: Corrected the used crc32 library.

Chapter 5.14.2: Adjusted the file paths to reflect the reorganization of the directory structure.

Chapter 5.14.3: Split the existing chapter into three sections, one of which describes the new application-specific details of the end-to-end checks.

Chapter 5.14.5: Added the processing of timeout signals as an exception to the country-independent functionality of the end-to-end checks.

### ED 18

Chapter 5.18: Added new requirements (CC supervises PC (FUR:4403 – FUR:4406, SAR: 4411), see also CQD ELSYS-344).

Chapter 2.3: Changed from four to eight Element-Controller (FUR: 0804).

Chapter 5.19.1: Online test: No shutdown when loop counter of Disk-Test do not increase after Disk-Error.

Chapter 1.5: Editions corrected

### ED 19

Chapter 5.19.2: quickstart only supported by TAS-PLF 1.10 and 1.11.

New chapter 5.12.1.

Chapter 5.20: added reference to SSDD SEC Schweiz.

Chapter 5.1.3, 5.1.4: added necessary number of CAR running on one computer.

Chapter 5.1.2: XCEL is part of EP.

### ED 20

Chapter 5.14 modified because of feature OBB28.10:FL:99 “Anlagenkennung in internen CHILL-Signalen”. Description of functionality previously covered by SAR:2502 only is now extended to SAR:2502, SAR:2504, SAR:2505, and SAR:2506 (covering the same scope of functionality as before).

### ED 21

Chapters 1.5, 5.18, 5.19.1, 5.19.2 are modified because of CQD ELSYS-447 (Aktivierung der Reaktivitätsüberwachung in der Elektra mit Plf 2.x).

### ED 22

Chapter 5.14.1.1 is modified because the “End-to-End Checks” is extended for the signals of reactivity processes.

The chapter 5.18 is modified, because the requirements FUR:4430 and FUR_4431 are modified as well as FUR 4436 is introduced.

### ED 23

#### ED 23 PD 01

Cover Page and Title Page replaced by actual version of Title Page.

Editions and Change History: table reduced to first edition, actual edition and predecessor of actual edition.

Chapter 1.2, 1.3, 1,4: removed reference to outdated document [10]

Chapter 1.3: updated audience

Chapter 1.4: renamed to History to use it as replacement of chapter 8.

Chapter 1.5.1:

Changed order of documents to be able to group them by theme and use group related reference id

removed unused reference documents

replaced reference documents not available in DWF by available ones

replaced reference documents not containing the necessary information by documents containing the necessary information

editions updated

added additional reference documents (e.g. documents referenced by TAS-PLF and OCS SACs)

Chapter 1.5.2:

Changed ids

removed unused reference documents

added additional reference documents

Chapters 1.6 and 1.7 merged to 1.6

Chapter 2.4: Undo the erronous merge and move the requirement back to chapter 2.3

Chapter 5.2.1: added SACs to Purpose and Function; added additional description after the table.

Chapter 5.12.1: table description added for Network interface, Subnet and hostNumber.

Chapter 5.18: added SACs to Purpose.

Chapter 5.19:

extend the chapter to TAS-PLF in general; therefore add new subchapter 5.19.1

former chapter 5.19.1 is now 5.19.2; added SACs to Purpose, Function and Processing.

former chapter 5.19.2 is now 5.19.3; added SACs to Purpose

New chapter 5.19.4 – replaces EL-SYS-14-060_MM_EMV-test-SEC-Signal_140507.doc “Meeting Minutes, Festlegung der Elektra Prozess Prioritäten mit Plf 2.x”.

- Former chapter 5.20 is now 5.21

- New chapter 5.20 for Linux

- Existing chapter 6 “Feasibility and Resource Estimates” removed, as it is outdated.

- New chapter 6 “Taking account of SACs of underlaying systems in the design of ELEKTRA” (solves PCR CELSYS-488).

- Chapter 7: now extended to include also an index of SACs.

#### ED 23 PD 02

Changes according to review report EL-SYS-20-159_RevRep_3BU_15000_0400_PEAPA_23PD01 ED 01.

Chapter 1.5.1: Lists converted into tables.

Chapter 1.5.1: changed title of [EL22] to be consistent to DWF.

Chapter 1.5.1: added new documents [FS_MS_EBO2], [EL_GEN_FUNC], [EL_LAN_DIAG], [EL_SSS_SBB], [EEC_SDD_NIC_HU], [EEC_SSDD_EL_DCA]; removed [EEC_SSDD_GEN], as it is identical to [SE50].

Chapter 1.5.1: updated reference documents to latest edition available on 2020-11-12.

Chapter 1.5.2: Updated [EN 50128] to [EN 50128:2011/prAA:2019]

Chapter 5.12: remove requirement DPR:0224, as it is referenced in chapter 5.19.2; add requirement FUR:6205 (solves PCR CELSYS-635).

Chapter 5.16: head line extended, additional reference document added.

Chapter 5.19.1: add requirement GCR:0823 (solves PCR CELSYS-635).

Chapter 5.22: new chapter to provide an overview about the different voters in the different ELEKTRA products.

#### ED 23 PD 03

Changes according to review report EL-SYS-20-159_RevRep_3BU_15000_0400_PEAPA_23PD01 ED 02.

Chapter 1.5.1: split into subchapters; referenced documents sorted according to their ID.

#### ED 23 PD 04

Chapter 1.5.2: EN50128 updated to the released amendments.

Chapter 5.1.2: list for usage of XCEL updated.

Chapter 5.14: subchapters with detailed description moved to “DDD Generic Functions” [EL_GEN_FUNC], as this DDD contains another part of the detailed description (solves CELSYS-756).

Chapter 5.19.5: new chapter describing device mapping

Chapter 5.23: new chapter for OCS SBJ and RaSTA.

#### ED 23 PD 05

Thales Logo and document template updated.

Chapter 1.5.1: renamed some document IDs; updated document editions; new documents [EL_EBO2_EL1PC], [EL_SYAD_EL1_CH], [EL_MIX_EL1_RAS], [SACDEF].

Chapter 1.5.2: updated EN 50128.

Chapter 5.1.2: removed obsolete condition [CAR:APPL_COND:02] and related description.

Chapter 5.14: add reference to [EL_GEN_FUNC].

Chapter 2.6: updated Figure 5 to latest design decision; updated Table 12 to be consistent to Figure 5.

Chapter 6.1: updated SRACs to TAS-PLF 2.4.2.

Chapter 6.2: updated SRACs to OCS 2.1.9 and OCS non-red 2.1.9.

Chapter 6.3: updated conditions to CAR 5.1.2.

Chapter 7.1: updated tables to actual SRACs and conditions; sorted tables according to column id.

#### ED 23 RL

Changes according to review report EL-SYS-21-184.

### ED 24

#### ED 24 PD 01

Chapter 1.5.2: changed id [EN 50128:2011] to [EN 50128]; changed id [EN 50159:2010] to [EN 50159]; added “A1:2020” to EN 50159.

Chapter 5.14: replaced text reference by bookmark reference for EN 50159.

Chapter 5.19.1: replaced text reference by bookmark reference for EN 50159; added clarification for model 3 taskset.

Chapter 6.1.1: update reference to EN 50159 according to new id; update DOORS module version.

#### ED 24 RL

Changes according to review report EL-SYS-22-032.

### ED 25

#### ED 25 PD 01

Chapter 1.2 Purpose: Add reference to SecSRS.

Chapter 1.5.1.1: Add document references to SecSRS, SecMP, SecSSDD, and SecEoC.

Chapter 1.5.1.3: Add document reference to Security Handbook TAS PLF.

Chapter 1.5.2: Add document reference to IEC-62443.

New chapter 2.6 Security aspects.

Chapter 4.2.2 Connectivity between ELEKTRA 2.x components: Add security related connections.

Chapter 4.2.3 Connectivity with non ELEKTRA 2.x components: Add security related connections.

New chapter 5.24 Use DGP/SCWS for external Log Collector and NTP server connectivity.

Chapter 6: Mention SecRACs in the headline.

New chapter 6.4 SecRACs .

Chapter 7.2: Add security requirements to index.

#### ED 25 PD 02

- Update to HITACHI template, update “Thales RBC” to “Hitachi RBC”.

- Chapter 5.19.2, paragraph for Processing: CORE-2.x-SW:SAC:0173: For GenSys >= 2.19.10, partition 4 is used for the Security Configuration Overlay.

#### ED 25 RL

- Chapter 6.4: TAS PLF SecRACs are handled in Evidence of Compliance [SecEoC].

- Removal of deleted requirements:

  - FUR:0101 (TRT12) add FUR:0102 (TRT13) from chapter 2.7 (combination ELEKTRA 1.x MMI (VCA,VCB) with ELEKTRA 2.x).

  - FUR:0129 (TRT28), FUR:0130 (TRT29) from chapter 5.7.

  - FUR:0373 (TRT75) interoperability with PSU from chapter 2.7 and chapter 5.9.

  - SAR:0351 (TRT177) Inversion of signals chapter 2.5 and chapter 5.1.1.

  - DPR:0208 (TRT292) LAN network management support chapter 5.12.

### ED 26

#### ED 26

- Chapter 5.25: Use of TCDs for establishing connection between local ELEKTRA 2.x and remote ECs.

## References and Terms

### Referenced Documents

#### Referenced Documents ELEKTRA and EBO2

| Id | Title | Document Number | Ed./Ver. |
| --- | --- | --- | --- |
| [EL_AZLM_OCSSB] | ADD IF Elektra-AzLM over OCS-SB | 3BU 15000 5138 DSAPC | Ed. 06 |
| [EL_DDD_FBI] | DDD Field Bus Interface - FBI; ELEKTRA 2.x | 3BU 15000 0675 DSAPA | Ed. 06 |
| [EL_EBO2_EL1PC] | Report, EBO2 an Elektra 1 PC | EL-OBB-06-176 | Ed. 01 PD 01 |
| [EL_FS_MS_EBO2] | Funktionelle Spezifikation, Methodensichere Einheitliche Bedienoberfläche EBO V2 | 3BU 15000 0211 DSAPC | Ed. 06 |
| [EL_GEN_FUNC] | Detailed Design Document Generic Functions, ELEKTRA System | 3BU 15000 0440 PEAPA | Ed. 11 |
| [EL_GLOSS] | Glossary: Definitions, Acronyms and Abbreviations - ELEKTRA 2.x System | 3BU 15000 0081 DCAPA | Ed. 06 |
| [EL_LAN_DIAG] | Detailed Design Document, LAN-Diagnose, Elektra2 | 3BU 15000 4316 PEAPC | Ed. 04 |
| [EL_LIM_INT] | Beschreibung, System Limits, ELEKTRA International | 3BU 15000 9021 EDAPA | Ed. 14 |
| [EL_LIM_OBB] | Beschreibung, Systemgrenzen ESTW ELEKTRA 2.x OeBB | 3BU 15000 0203 EDAPC | Ed. 12 |
| [EL_LIM_SBB] | Konfiguration, SSDD Statische und dynamische Systemgrenzen ELEKTRA Schweiz | 3BU 15000 3435 DSAPC | Ed. 06_02 |
| [EL_MIX_EL1_RAS] | System Design Mixed Architecture, ELEKTRA1 with X25 RaSTA | 3BU 15000 2385 DSAPA | Ed. 03 |
| [EL_RBC_OCS] | ADD Interface IXL - RBC over OCS | 3BU 15000 3019 DSAPC | Ed. 15 |
| [EL_SC] | Sicherheitskonzept ELEKTRA 2.x, All countries | 3BU 15000 0622 DUAPC | Ed. 04 |
| [EL_SP] | Safety Plan, ELEKTRA 2.x | 3BU 15000 0602 DUAPA | Ed. 06 |
| [EL_SSS_SBB] | System Requirements Specification, Sbb, Elektra Evolution | 3BU 15000 0309 DTAPA | Ed. 42_01 |
| [EL_SYAD_EL1_CH] | Test_Specification, SYSTEM ADMINISTRATION; ELEKTRA 1.x Schweiz | 3BU 15000 3124 QTAPA | Ed. 01 |
| [EL_SYNC] | Synchronisierung CC-Ebene, Funktionelle Spezifikation | 3BU 20100 AAAA DSAPC | Ed. 01B |
| [EL_SYS_ADM] | Architectural_Design_Document, System Administration, ELEKTRA 2.x | 3BU 15000 5197 PEAPA | Ed. 06 |
| [EL_VP_SSCA] | Verification Plan - ELEKTRA 2.x - Static Source Code Analysis | 3BU 15000 0653 DUAPA | Ed. 07 |
| [EL02] | Product Strategy Plan of ELEKTRA | 3BU 15000 2500 DBAPA | Ed. 20A |
| [EL03] | ELEKTRA Evolution - System Requirements Specification - ELEKTRA 2.x | 3BU 15000 0300 DTAPA | Ed. 38 |
| [EL04] | Technical Feasibility Study ELEKTRA Evolution | 3BU 15000 8000 UPAPA | Ed. 01 |
| [EL05] | Technical Feasibility Study MMI ELEKTRA Evolution | 3BU 15000 8001 UPAPA | Ed. 01 |
| [EL06] | Technical Feasibility Study Field Element Controlling ELEKTRA Evolution | 3BU 15000 8002 UPAPA | Ed. 01 |
| [EL07] | Architectural Design Document ELEKTRA EVOLUTION Field Element Controlling | 3BU 15000 0402 PEAPA | Ed. 11 |
| [EL08] | Architectural Design Document ELEKTRA EVOLUTION Man Machine Interface | 3BU 15000 0401 PEAPA | Ed. 10 |
| [EL09] | ELEKTRA SBB, Systembeschreibung | 3BU 00101 ABAA EDAPC | Ed. 16 |
| [EL10] | Project Management Plan, ELEKTRA Evolution, Technical Project | 3BU 15000 0101 DPAPA | Ed. 07 |
| [EL15] | Review report of feasibility studies of SA, MI, FE 10.07.98 | 980710_Questioning_rr.doc | 10.07.98 |
| [EL17] | ELEKTRA ÖBB, Systembeschreibung | 3BU 10000 0002 EDAPC | Ed. 06 |
| [EL18] | Funktionelle_Spezifikation, TCD - Transmission Controller Double Channel | 3BU 00426 ABAA DSAPA | Ed. 08 |
| [EL24] | Architectural Design Document ELEKTRA 2.x Kommunikation über Ethernet | 3BU 15000 0409 PEAPC | Ed. 02 |
| [EL25] | Detailed Design Document ELEKTRA 2.x System Safety-Mechanismus zur Beherrschung von Systemverzögerungen | 3BU 15000 4088 PEAPC | Ed. 01 |
| [EL26] | Architectural Design Document ELEKTRA ÖBB EBO2 | 3BU 15000 4430 EBAPA | Ed. 11 |
| [EL27] | Verfahrenssicherheitsnachweis Kommunikation über Ethernet | 3BU 15000 0662 DUAPC | Ed. 03 |
| [EL29] | Architectural Design Document Methodsafe MMI, ELEKTRA 2.x Schweiz | 3BU 15000 4116 DSAPC | Ed. 13 |
| [EL30] | Detailed Design Document ELEKTRA 2.x System Mechanismus zur Beherrschung von Reaktionsverzögerungen | 3BU 15000 4970 PEAPC | Ed. 06 |
| [EL31] | System Description Interlocking System LockTrack 6131 (ELEKTRA) International | 3BU 15000 9010 EDAPA | Ed. 12A |
| [EL34] | Verification Report, Quickstart Pufferzeiten D2/D6 | EL-SYS-09-077 | Ed. 01 |
| [EL42] | System/Segment Design Document, System Subsystem Design Description, Remote Control New | 3BU 15000 9059 DSAPA | Ed. 04 |
| [GTS_Glossary] | GTS Glossary | 3BU 15000 8008 WDAPQ | Ed. 04 |
| [PVGEN_SAC] | Sicherheitsrelevante Anwendungsbedingungen PV GEN 2.19.x | 3BU 15000 5308 DUAPC | Ed. 03 |
| [SecEoC] | Evidence of Compliance IEC 62443-4-2 Elektra 2.x and SCWS | 3BU 15000 5314 DUAPA | Ed. 04 |
| [SecMP] | Security Management Plan Elektra 2.x | 3BU 15000 5300 DPAPA | Ed. 03 |
| [SecSRS] | System Requirements Specification Security Elektra 2.x Generic | 3BU 15000 5310 DTAPA | Ed. 04 |
| [SecSSDD] | SSDD Security Elektra 2.x | 3BU 15000 5311 DSAPA | Ed. 04 |

Table 1 : Referenced Documents ELEKTRA and EBO2

#### Referenced Documents CAR

| Id | Title | Document Number | Ed./Ver. |
| --- | --- | --- | --- |
| [CA_ADD_CAR] | Detailed_Design_Document, DDD CAR | 3BU 15000 5193 PEAPA | Ed. 03 |
| [CA_EOF_CAR] | Funktionsnachweis, ELEKTRA 2.x, Evidence of Functionality, CAR/EP | 3BU 15000 5191 DUAPA | Ed. 06 |
| [CA_PG_CAR] | Programmers Guide, CAR Programmers Guide | 3BU 15000 5163 PWAPA | Ed. 07 |
| [CA_PORT_CAR] | Architectural_ Design_Document, Porting Description to TAS Platform and Linux | 3BU 15000 0407 PEAPA | Ed. 03 |

Table 2 : Referenced Documents CAR

#### Referenced Documents TAS-PLF

| Id | Title | Document Number | Ed./Ver. |
| --- | --- | --- | --- |
| [COMP] | Compatibility Matrix SW/HW - TAS Platform – Deliverables | 3CR 00055 9637 DFAFA | Ed. 42 |
| [CONFREF] | Configuration Reference Manual - TAS Platform Core SW - 2.x | 3CR 00055 5142 PCAPA | Ed. 14 |
| [GDE DIAG] | User Guide - TAS Platform Core SW - 2.x - Diagnosis Mechanisms Tutorial | 3CR 00055 5144 PCAPA | Ed. 15 |
| [GDE PRG] | Programmers Guide - TAS Platform Core SW - 2.x | 3CR 00055 5141 PCAPA | Ed. 14 |
| [HB SEC] | Security Handbook - TAS Platform Core SW - 2.x | 3CR 00055 6793 PCAPA | Ed. 05 |
| [RPT VAL ADA] | Validation Report Ada - TAS Platform Core SW - 2.4.1 - Ada Compiler 7.3.1 | 3CR 00055 7444 QZAPA | Ed. 01 |
| [RPT VAL GCC] | Validation Report C - TAS Platform Core SW - 2.4.0 - C Compiler gcc 7.3.0 3CR 00055 7060 QZAPA, Ed. 03 Validation Report C++ - TAS Platform Core SW - 2.4.0 - C++ Compiler g++ 7.3.0 | 3CR 00055 7061 QZAPA | Ed. 04 |
| [RPT VER GCC] | Verification Report C - TAS Platform Core SW - 2.4.0 - C Compiler gcc 7.3.0 3CR 00055 7063 QZAPA, Ed. 01 Verification Report C++ - TAS Platform Core SW - 2.4.0 - C++ Compiler g++ 7.3.0 | 3CR 00055 7064 QZAPA | Ed. 02 |
| [SACDEF] | SRAC Definition and Handling (see German repository: livelink) | 83551754-DDQ-DEU-EN | - |
| [SECOVL] | MEMO Security Configuration Handling TAS Platform 2.4.1 | 3CR 00055 7812 UEAPA | Ed. 02 |
| [SETPG] | Setup Guide TAS Platform Core SW - 2.x | 3CR 00055 5145 PCAPA | Ed. 14 |
| [TP_Core2x_SACs] | Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform Core - 2.x | 3CR 00055 5070 DUAFQ | Ed. 23 |
| [TP_PLF1_OLT] | Software Design Description - TAS Platform - Startup Tests and OLT Server | 3CR 00055 1113 PEAPA | Ed. 12 |
| [TP_PLF1_QS_OLT] | Software Design Description, TAS Platform Core, Quickstart OLT (only TAS Platform 1.x)= | 3CR 00055 1135 PEAPA | Ed. 04 |
| [TP_PLF1_UG_OLT] | User Guide, TAS Platform Core, OLT Quickstart Feature (only TAS Platform 1.x) | 3CR 00055 0165 PCAPA | Ed. 04 |
| [TP_PLF2_OLT] | Detail Design Document, TAS Platform Core SW – 2.x, Startup Tests and OLT Server | 3CR 00055 5568 DSAPA | Ed. 13 |
| [TP13] | Product Description, Top Level Architecture - TAS Platform - System Level | 3CR 00055 0101 DEZZA | Ed. 09 |
| [TP14] | Product Description, Top Level Architecture - TAS Platform Core - 2.x - System Level | 3CR 00055 5030 DEAPA | Ed. 02 |

Table 3 : Referenced Documents TAS-PLF

#### Referenced Documents OCS

| Id | Title | Document Number | Ed./Ver. |
| --- | --- | --- | --- |
| [TP_OCS_EL] | Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform OCS - 2.x - OCS EL | 3CR 00055 5768 DUAPQ | Ed. 05 |
| [TP_OCS_EoC_NR] | Evidence of Compliance - TAS Platform OCS - non-red vs SAC | 3CR 00055 5772 DUAPA | Ed. 06 |
| [TP_OCS_SB] | Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform OCS - 2.x - OCS SB | 3CR 00055 5769 DUAPQ | Ed. 05 |
| [TP_OCS_Sys] | Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform OCS - 2.x - System & OCS-RED | 3CR 00055 5329 DUAFQ | Ed. 08 |
| [UG OCS API 2.x] | API Users Guide - TAS Platform OCS - 2.x - One-Channel-Safe | 3CR 00055 5235 PCAFA | Ed. 05 |
| [UG OCS CONF 2.x] | User Guide - TAS Platform OCS - 2.x - Configuration Reference Manual | 3CR 00055 5236 PCAFA | Ed. 06 |
| [UG_OCSNR_API_2.x] | API Users Guide - TAS Platform OCS - 2.x - non-red | 3CR 00055 5786 PCAPA | Ed. 02 |
| [UG_OCSNR_CONF_2.x] | User Guide - TAS Platform OCS - 2.x - non-red - Configuration Reference Manual | 3CR 00055 5787 PCAPA | Ed. 04 |

Table 4 : Referenced Documents OCS

#### Referenced Documents EEC (SEA/SEC)

| Id | Title | Document Number | Ed./Ver. |
| --- | --- | --- | --- |
| [EEC_CFG_OV] | Configuration Overview: EEC, SEC/SEA, NIC, ICU | 3BU 61400 0119 QMAPA | Ed. 10 |
| [EEC_SDD_NIC_HU] | SDD NIC-HU - Neighbor Interface Controller Hungary | 3BU 62300 1202 PEAPA | Ed. 09 |
| [EEC_SSDD_EL_DCA] | SSDD, ELEKTRA 2.x with DCA elements | 3BU 15000 9160 DSAPA | Ed. 06 |
| [SE_ADD_SEA] | Architectural Design Document SEA, ELEKTRA 2.x ÖBB | 3BU 61400 0004 DSAPA | Ed. 06 |
| [SE_ADD_SW_GEN] | SSDD SEA SW GEN | 3BU 61400 0119 DSAPA | Ed. 24 |
| [SE_DDD_IntegrationSEA] | Detailed Design Document, Integration SEA, ELEKTRA ÖBB | 3BU 15000 2146 PEAPC | Ed. 10 |
| [SE_SSDD_SEC_CH] | System Subsystem Design Description SEC Software Schweiz | 3BU 62050 0210 DSAPC | Ed. 12 |

Table 5 : Referenced Documents EEC (SEA/SEC)

### Standards and Procedures

Remark: The following table contains only references that are needed for the design of ELEKTRA. References used only within citations of SRACs and conditions are not included in the table.

| Id | Reference |
| --- | --- |
| [EN 50128] | EN 50128:2011 + A1:2020 + A2:2020 – Railway application – Communication, signalling and processing systems – Software for railway control and protection systems |
| [EN 50159] | EN 50159:2010 + A1:2020 – Railway applications – Communication, signalling and processing: Safety-related communication in transmission systems |
| [DDQS] | Document Management Rules 83530061-DOC-AUT-EN, Rev. 011 |
| [SpecTTP] | Specification of the Basic TTP/C Protocol, 1.12.1996, Institut für Technische Informatik, TU Wien, and Documentation from ESPRIT TTA Project |
| [IEC62443-4-1] | ISA/IEC 62443 Security for industrial automation and control systems IEC 62443-4-1:2018: Secure product development life-cycle requirements |
| [IEC62443-4-2] | ISA/IEC 62443 Security for industrial automation and control systems IEC 62443-4-2:2019: Technical security requirements for IACS components |

Table 6 : Standards and Procedures

### Terms, Abbreviations and Acronyms

See [EL_GLOSS] and GTS Glossary [GTS_Glossary].

# System Overview

ELEKTRA 2.x, as a product of the technical project ELEKTRA Evolution, is an electronic interlocking system adapted to the operational requirements of a national railway. The system structure of the technical system is shown in Figure 1.

Figure 1: System architecture of ELEKTRA 2.x

The Man Machine Interface (MMI) is based on commercially available personal computers running LINUX operating system. MMI can be integrated with Microsoft Windows based applications on an IBO server (‘Integrierte Bedienoberfläche’). In a first step the subsystem VCA is used as MMI-application. In a second step VCA is replaced by a new Java-application called EBO2 for Austria and Hungary.

The Central Controller subsystem (CCA, CCB), the Element Controller subsystem (ECA, ECB), the Long Distance Communication subsystem (LDA, LDB) and the Diagnosis component (DGP) are based on TAS Platform (see [TP13], [TP14]). All these components are connected via Ethernet. Remote diagnosis concept (retrieve of field diagnosis data via LDC and X.25) is preserved.

The Interface Controllers for the Field Elements (ICA, ICB) are based on micro-controller and fieldbus technology.

SEA (‘Sichere Elektronische Außenelementansteuerung’) is an alternative to conventional outdoor element controlling using an EC connected over the fieldbus to the ICs. The main architectural change is, that the EC element processes (OEC’s, Outdoor Element Controller) are allocated in the outdoor system. EC-MUX is the Element Controller/Multiplexer component of SEA, which acts as a transparent replacement of a conventional EC – that means, the interfaces to the CC and DGP are left unchanged (except for a day/night status indication), so all concepts applicable to an EC are applicable to an EC-MUX too (see 5.21).

Peripheral components (e.g. printer, disc) can be integrated in the system architecture. Connectivity to ELEKTRA components (e.g. via Ethernet or serial/parallel interface) depends on the type of components to be chosen by HW.

As a result of the Go/Nogo decisions (Review report of feasibility studies of SA, MI, FE [EL15]) based on the feasibility studies ([EL04], [EL05], [EL06]) the following realisation variants have been selected:

| PA | Variant | Description |
| --- | --- | --- |
| SA | SA-1 | System Architecture with non-redundant CC subsystem (special case of SA-3), optional integration of A and B channel into one computer |
| SA | SA-2 | System Architecture with warm standby redundancy for CC subsystem (containing SA-1 as special case), optional integration of A and B channel into one computer |
| FE | FE-3 | Redundant, diverse Element Controllers with non-redundant, diverse Interface Controllers, communication based on TTP, backwards compatible to interface boards of ELEKTRA 1.x and selectively optimised new ELEKTRA 2.x interface boards |
| FE | SEA | Diverse Outdoor Element Controllers (OEC), Outdoor Interfaces (OIF) and indoor multiplexer (EC-MUX), communication between OECs und OIFs via CAN-bus |
| MI | MI-3 | Non-safe, non-diverse MMI based on LINUX PC (special case of MI-2) |
| MI | MI-2 | Method-safe, non-diverse MMI (containing MI-3 as special case) |

Table 7: Selected variants for Project Areas

The chosen variants concerning the system architecture (SA-1, SA-2) are described in more details in this document. For the variants concerning FE resp. MMI refer to the corresponding Architectural Design Documents.

The following chapters 2.1 – 2.5 describe ELEKTRA using MMI with VCA-application. EBO2 is first mentioned in chapter 2.7.

## Objectives for the system architecture

The commercial, technical, marketing and operational aspects for the system architecture are described in [EL02].

## Differences to current ELEKTRA 1.x system

From the system architecture point of view, these are the differences to the current ELEKTRA 1.x system:

- use of method safe MMI instead of safe MMI

- CC redundancy:

ELEKTRA 1.x: Voting 2s2*(2a3) – at first redundancy voting (a, due to availability reasons), then diversity voting (s, due to safety reasons)

ELEKTRA 2.x: Voting 1a2*(2s2) – at first diversity voting (s, due to safety reasons), then redundancy voting (a, due to availability reasons)

- integration of CCA and CCB on one hardware computer (use of different hardware computers must be possible as well)

- integration of ECA and ECB on one hardware computer

- EC redundancy:

ELEKTRA 1.x and ELEKTRA 2.x: primary/secondary handling:

Only messages of primary EC is accepted by CC

Primary ECA and primary ECB have to run on different computers (if possible)

- FE: use of fieldbus technology instead of parallel input/output cabling, use of micro-controller technology to control the interface boards

- use of TAS platform as common software platform and communication system as a continuation of AEOS and CEL system

- use of commercial standard components, communication media and protocols instead of proprietary solutions as far as possible

## Scalability

The system ELEKTRA 2.x can be used for small stations (up to 15 points), medium stations (15 - 70 points) and large stations (more than 70 points).

For a “BOS-Zentrale” (Austria) many MMIs and LDC are needed, but no ECs.

From the system architecture point of view no restrictions have been identified.

For the different scalability levels the following numbers of trackside elements are assumed (Example for Switzerland):

| Element Type | small |  | medium |  | large |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | IF boards | backp. | IF boards | backp. | IF boards | backp. |
| Points | 12 | 4 | 48 | 16 | 96 | 32 |
| main signals | 16 | 8 | 64 | 32 | 128 | 64 |
| shunting signals | 15 | 5 | 60 | 20 | 120 | 40 |
| track sections | 24 | 4 | 96 | 16 | 192 | 32 |
| Total |  | 21 |  | 84 |  | 168 |

Table 8: Assumptions for number of elements for different scalability levels

For system ELEKTRA2.x ÖBB with SEA the system limits are described in “Beschreibung, Systemgrenzen ESTW ELEKTRA 2.x OeBB” [EL_LIM_OBB] and in chapter System limits in “Architectural Design Document SEA, ELEKTRA 2.x ÖBB” [SE_ADD_SEA].

For system ELEKTRA2.x Switzerland the system limits are described in “Konfiguration, SSDD Statische und dynamische Systemgrenzen ELEKTRA Schweiz” [EL_LIM_SBB].

For system ELEKTRA2.x International the system limits are described in “Beschreibung, System Limits, ELEKTRA International” [EL_LIM_INT].

One backpanel is normally served by one IC component (except usage of some ELEKTRA 1.x interfaces). One EC can handle up to 4 segments (FUR:0803 (TRT86):	EC must handle up to 4 fieldbus subsystems), one segment consists of up to 21 IC components (see Table 9). From the architectural point of view one EC component for a small and medium station and two EC components for a large station are needed. Due to performance and availability constraints a configuration, in which less elements are served by one EC component, might be required (see Table 10).

| max. number of workplaces (*) | (***) |
| --- | --- |
| max. number of EC components | (***) |
| target number of segments for one EC component | 4 |
| max. number of IC components on one segment (**) | 21 |
| max. number of IC components in total (**) | EC * 84 |

Table 9: Scalability numbers of ELEKTRA 2.x components

(*) 	10 workplaces are defined in [EL09], 40 workplaces can be defined with the new remote control concept (INT:FL:FEA:0237 Multiple EBO2 CC Connections [EL09]).

(**) 	numbers are based on TTA technology.

(***) 	unlimited from the system architecture point of view, but also depends on requirements concerning hardware, data preparation and application software. It must be possible to handle eight Element Controllers (ECs) (FUR:0804 (TRT279):		Handling of eight Element Controllers).

FUR:1107 (TRT507):	It must be possible to use components with only one redundancy. The second redundancy is not realized and therefore, must not lead to an error message.

|  | small | medium | large | maximum |
| --- | --- | --- | --- | --- |
| Number of ICs | 21 | 84 | 168 | 336 |
| Min. number of segments | 1 | 4 | 8 | 16 |
| Min. number of EC components (non redundant) | 1 | 1 | 2 | 4 |

Table 10: Number of FE components for different stations

When putting the system into operation, it must be possible to boot the system with as less redundancy as possible.

OAR:1004 (TRT232):	Putting into operation with CC only

OAR:1005 (TRT243):	Putting into operation without redundancy

OAR:1016 (TRT280):	Putting into operation without LAN-redundancy

OAR:1017 (TRT281):	Putting into operation without fieldbus-redundancy

OAR:1018 (TRT282):	Putting into operation without LDC-redundancy

FUR:1106 (TRT506):	It must be possible to mark a VC to be used only temporarily. If this VC is missing, no error has to be reported.

## Availability

With the product ELEKTRA 2.x the high end as well as the low end of the market should be covered. Therefore the system has to meet high availability requirements and standard availability requirements.

Transparency of the redundancy degree has to be guaranteed for the operational functionality (GCR:0304 (TRT112):	System configuration transparency for operational functionality,
 GCR:0305 (TRT113):	System configuration handling of technical functionality).

FUR:1107 (TRT507):	It must be possible to use non-redundant components.

An identification concept for spare components has to be defined. It has to fulfil the following requirements (FUR:0363 (TRT66):	Identification concept for spare hardware computers):

- exchange of components easily by the customer

- mechanism used on standard hardware component

- independent of the application running on the hardware component to be replaced

Solutions: BCD Switches for coding one byte on LPT-port. This byte is read at startup time and determines by the use of a numbering scheme, which applications have to run on the computer hardware.

DPR:0224 (TRT316): Flash card failure: A failure concerning the flash card shall not lead to a shutdown.

ELEKTRA SBB also has to provide selective safety reactions: “FUR:6403 (TRT1228): Voting im CC – Auslösung selektiver Sicherheitsreaktionen“ and further requirements contained in “System Requirements Specification, Sbb, Elektra Evolution” [EL_SSS_SBB].

### High availability

High availability is required for train lines and stations with heavy traffic, i.e. main lines, in countries with high standards of time table accuracy. Figure 2 shows the different redundancy concepts of MMI, CC and EC components. Due to simplicity reasons only one EC component (including the corresponding IC components) is shown in the figure.

Figure 2: System architecture for high availability

| Abbreviations | CCA’, CCB’ | active application |
| --- | --- | --- |
|  | CCA”, CCB” | standby application |
|  | ECA’, ECB’ | primary application |
|  | ECA”, ECB” | secondary application |

active, standby, primary, secondary are dynamic states of an application and can therefore change during operation.

In Figure 2 MMIs have 2 redundancies. But it is also possible to use only 1 redundancy for all MMIs.

FUR:1106 (TRT506):	It must be possible to mark a VC to be used only temporarily. If this VC is missing, no error has to be reported.

Work-place redundancy is used on the MMI layer, i.e. more than one MMI component can be used in parallel. In case of a faulty MMI component the user can move to another MMI work-place.

Alternatively one MMI component can have two redundancies like in today's ELEKTRA system. But only one redundancy is running, while the other is turned off. When using EBO2 (ELEKTRA OBB and ELEKTRA International) both redundancies are running.

ELEKTRA International increases the number of available MMIs with the new remote control concept (INT:FL:FEA:0237 Multiple EBO2 CC Connections [EL09]).

The redundancies of the CC layer are in a warm standby mode: the operating system is running, HW checks are performed regularly, the application code and the databases are placed into the main memory of the computers. The applications are ready to boot if the active redundancy has to be shut down.

SAR:1101 (TRT505):	If CC is configured as redundant, CMOS data have to be deleted after booting. This has to be done to prevent to boot with too old data. Upon entering warm standby mode each component has to copy CMOS data from the active component. A standby CCA copies from the active CCA, a stand-by CCB copies from the active CCB. In addition, CMOS are deleted, if they are found to be corrupt. CMOS data of active components are deleted if there is an inconsistency between the active CCA and the active CCB.

The redundancies of the EC layer are in a hot standby mode (including the primary/secondary voting concept) as it is done in today’s ELEKTRA system.

The functionality of ECA and ECB is integrated on one HW computer, the primary redundancies of ECA and ECB (i.e. ECA’, ECB’) have to be located on two different redundant HW computers (SAR:1201 (TRT189):	Safe switch-over,
 DPR:0316 (TRT6):	Redundancy concept for FE subsystem).

IC components are realised non redundantly, i.e. a faulty IC component affects the corresponding IF boards (up to 4).

The DGP component (not shown in the figure) is non-redundant as in the current ELEKTRA 1.x system.

The redundancy concept for LDC components (not shown in the figure for simplicity reasons) is the same as in ELEKTRA 1.x.

A redundant Ethernet is used for the communication between different ELEKTRA 2.x components

FUR:3801 (TRT517):	The failure of a LAN connection (Ethernet connection) must lead to the error “Anlagenfehler”; a corresponding error message has to be printed on the error printer (“Störungsdrucker”). Additional LAN-errors are neither indicated to the operator nor printed. The failure of a LAN connection (LAN error) is defined as the interruption of the connection between two computers (even if the redundant connection is still living). There is no differentiation which network component causes the interruption.

FUR:3802 (TRT518):	It must be possible to enquire the state of every single LAN connection (“OK”/”not OK”) on the diagnosis system (WebRide).

FUR:3803 (TRT519):	The diagnosis of LAN connections must include only projected computers and the currently active MMI-PCs.

FUR:3804 (TRT520):	The failure of a LAN connection to a redundant MMI-PC (with application VC) which is switched off must not lead to AFE/ANL.

FUR:3805 (TRT521):	A redundant MMI-PC which is switched off has to be shown on the diagnosis system and the corresponding LAN connections have to be indicated as “not OK”.

FUR:3806 (TRT522):	A VC for temporary use is always ignored in the LAN diagnosis system. LAN errors will therefore not lead to the error “Anlagenfehler” and no error message is printed on the error printer (“Störungsdrucker”).

A redundant fieldbus is used for the connection between an EC component and the corresponding IC components. Four fieldbus segments can be connected on one EC component.

The redundancy degree of the components is separated from the redundancy degree of the connections, i.e. redundant components could also communicate via a non-redundant Ethernet.

### Standard availability

For the low end market a system architecture which serves standard availability requirements is provided (see Figure 3). This system architecture is a special case of the highly available system architecture. No HW redundancy is used at all. A single failure on the CC component leads to a system shut-down (FUR:1107 (TRT507):	It must be possible to use non-redundant components).

Figure 3: System architecture for standard availability

## Safety aspects

Software diversity is preserved (SAR:0807 (TRT184):	Preservation of ELEKTRA's two diverse channel approach).

The system architecture of ELEKTRA 2.x defines that two software channels are executed on one hardware computer. This definition requires the following additional safety mechanisms:

- Method-safe MMI

- Avoidance of common-mode faults in case two diverse software-channels perform safety-related functions on one hardware computer (SAR:1204 (TRT192):	Two channels on one hardware). This can be covered with inversion of channel data or with additional end-to-end communication checks.

- Disjunct memory space is required if two software channels are executed on one hardware computer (SAR:1205 (TRT193):	Memory-space).

- Safe switch over to a standby component (applicable for all redundant components) without losing data in both channels (SAR:1201 (TRT189):	Safe switch-over, SAR:1101 (TRT505):	Delete CMOS data – double redundant).

- Safety mechanisms in FE

## Security aspects

Starting with GenSys 2.19.10, security related aspects have been analysed for ELEKTRA 2.x in accordance with the IEC 62443 series of standards Part 4-2, [IEC62443-4-2], resulting in security related requirements written down in the [SecSRS]. The Security Management Plan [SecMP] describes the procedural measures which ensure compliance with the IEC 62443 series of standards, particularly Part 4-1 Secure product development lifecycle requirements, [IEC62443-4-1]. Architectural implementation of the security requirements is described in the SSDD Security Elektra 2.x [SecSSDD]. In short, the implementation comprises:

- Disabled root login (ELSEC:SRS:REQ:0008), and a new admin account (ELSEC:SRS:REQ:0003) with a station specific password which is randomly generated during flashcard image creation (ELSEC:SRS:REQ:0004). Login is restricted (ELSEC:SRS:REQ:0012) to the console, and for secure shell (SSH) to the LAN Labor (chapter 5.12.1), a specific network interface which is not connected on operational systems.

- System hardening with the aspects of resource limitations for the admin account (ELSEC:SRS:REQ:0009), and reducing accessibility of sensitive files.

- Syslog configuration that collects security related auditable events (ELSEC:SRS:REQ:0016) from all ELEKTRA 2.x computing components and is able to forward them to an external log server (ELSEC:SRS:REQ:0017).

- Host firewall (ELSEC:SRS:REQ:0014) configuration that denies and logs (ELSEC:SRS:REQ:0015) all network packets that are not specifically permitted in a security configuration which is generated by data preparation.

- An option to synchronize the hardware clock to an external NTP time source (ELSEC:SRS:REQ:0023).

## Interoperability with ELEKTRA 1.x components

The interoperability between ELEKTRA 1.x components and ELEKTRA 2.x components is shown in Table 11.

To be able to provide the communication using the RaSTA protocol for ELEKTRA 1.x for customer OBB the communication between ELEKTRA 1.x CCA/CCB and ELEKTRA 2.x RCU is required.

Figure 4: ELEKTRA 2.x RCU and DGP with ELEKTRA 1.x

All other combinations are only considered for migration steps within the ELEKTRA 2.x development process, i.e. in order to test the different ELEKTRA 2.x subsystems (e.g. DGP, CC) independently.

|  | 1.x MMI | 1.x CC | 1.x PC | 1.x SCU | 1.x DGP |
| --- | --- | --- | --- | --- | --- |
| ELEKTRA 2.x RCU | Not considered (no direct interaction) | Must be supported | Not considered (no direct interaction) | Must be supported | not considered |
| ELEKTRA 2.x DGP | Must be supported | Must be supported | Must be supported | Must be supported | --- |
| ELEKTRA 2.x other components | development phase (*) | development phase (*) | development phase (*) | development phase (*) | not considered |

Table 11: Interoperability with ELEKTRA 2.x components

(*) Note, that the combination used in the development phase does not necessarily provide all functionality used for field operation (e.g. no provision for redundancy handling).

The line for DGP in Table 11 is required for ELEKTRA 1.x with RaSTA protocol.

Currently used interface boards of ELEKTRA 1.x are served by EC components.

Interoperability with remote components and systems:

Interoperability with current remote components and systems is performed via LDC components (see Table 12 and Table 13). The combinations to be considered will not be defined within the technical project ELEKTRA Evolution.

|  | 1.x RIC |
| --- | --- |
| ELEKTRA 2.x component | must be supported |

Table 12: Interoperability with remote systems Austria and Hungary

|  | remote EC | electronic block |
| --- | --- | --- |
| ELEKTRA 2.x component | must be supported | must be supported |

Table 13: Interoperability with remote systems Switzerland

## Replacement of MMI by EBO2

To fulfil the new requirements of Austria a fully graphical Men-Machine-Interface called EBO2 is needed. EBO2 replaces the MMI of ELEKTRA 2.x. For details see Architectural Design Document ELEKTRA ÖBB EBO2 [EL26].

A difference is the redundancy handling of EBO2:

FUR:4101 (TRT683):	The failure of a LAN connection to a redundant MMI-PC (with application EBO2) which is switched off must lead to AFE/ANL.

To provide a fully graphical Men-Machine-Interface for ELEKTRA International EBO2 is used too. ELEKTRA International also requires to connect an EBO2 to more than one CC with the new remote control concept (INT:FL:FEA:0237 Multiple EBO2 CC Connections [EL09]).

# System Context

Interfaces of the ELEKTRA 2.x system are described in [EL09], [EL17], [EL31].

# System Design

## Design Method

The design of ELEKTRA 2.x system architecture is a redesign of ELEKTRA 1.x system architecture taken into account the objectives described in [EL10].

## Decomposition Description

### Architecture of an ELEKTRA 2.x component

Figure 5 shows the software architecture of an ELEKTRA 2.x component.

Figure 5: Software architecture of an ELEKTRA 2.x Component for a chill application

System software provides the necessary functionality for communication and running CHILL application. TAS Platform provides the standard communication services (see [TP13], [TP14])

(FUR:0337 (TRT52):	Use of TAS Platform (AEOS 4.x, AEOS 5.x),

GCR:0812 (TRT124):	Using TAS Platform computers for EC).

EP is the tailored version of the communication of TAS platform for ELEKTRA applications. XCEL functionality defines the CEL functionality on top of the TAS platform. EP and XCEL are part of CAR.

(FUR:0339 (TRT54):	XCEL on top of TAS Platform (AEOS 4.x, AEOS 5.x)).

This functionality is needed, if communication between current ELEKTRA 1.x components and ELEKTRA 2.x components is required. The CAR system describes the run-time environment for one or more ELEKTRA CHILL applications (see [CA_ADD_CAR], [CA_PORT_CAR]).

(FUR:0338 (TRT53):	CAR on top of TAS Platform (AEOS 4.x, AEOS 5.x)).

The technical functionality as a part of the application software deals with system aspects and network management aspects of the ELEKTRA system, whereas the operational functionality provides the specific railway interlocking services for the user („Bedienebene“, „Zentralebene“, „Peripherieebene“ and „Leittechnikschnittstelle“) (see [EL09]).

Figure 6: Software architecture of an ELEKTRA 2.x Component for a non-chill application

### Connectivity between ELEKTRA 2.x components

Standard communication protocols and communication mechanisms are used between the ELEKTRA 2.x components. With the use of the TAS platform a POSIX API is provided (see [TP13], [TP14]). The physical layer and the communication protocol are hidden for the overlaying software layers (EP, technical functionality, operational functionality).

The communication services for the operational and technical functionality (i.e. CHILL signal, routing) are preserved:

FUR:0341 (TRT55):	Services of TAS Platform (AEOS 4.x, AEOS 5.x),

FUR:0309 (TRT46):	Routing mechanism of ELEKTRA Classic,

FUR:0306 (TRT44):	Signal sequence,

FUR:0307 (TRT45):	Number of signal priorities.

Communication between ELEKTRA 2.x components is performed via the ELEKTRA Network with following characteristics (see Figure 7):

- communication media and protocol: UDP over Ethernet, X.25 (via LDC component)

- communication data: RMT data between ELEKTRA 2.x components

- routing functionality as in ELEKTRA 1.x.

Security related communication between ELEKTRA 2.x components (RFC5424 via TCP for syslog messages, NTPv4 via UDP for NTP) is done on the same ELEKTRA Network where safety critical communication is performed (ELSEC:SRS:REQ:0018). In order to mitigate for security related network traffic affecting safety related network traffic, syslog-ng is configured to limit the number of messages per second in accordance with the TAS PLF Security Handbook [HB SEC].

In the special case of the connection of remote EC components via TCD the TCD-TCD network is physically separated from the ELEKTRA network. The firewall is active on each component.

The network connectivity between ELEKTRA 2.x components is restricted with a host firewall implemented with netfilter, configured according to the “deny all, allow by exception” principle (ELSEC:SRS:REQ:0014).

Figure 7: Local communication

Long distance communication will be added in a next version of this document

### Connectivity with non ELEKTRA 2.x components

Communication between ELEKTRA 2.x components and non ELEKTRA 2.x components is performed via the ELEKTRA Network with following characteristics (see Figure 8):

- communication media and protocol: UDP over Ethernet, X.25 (via LDC component)

- communication data: raw data between ELEKTRA 2.x components and non ELEKTRA 2.x components

- routing functionality as in ELEKTRA 1.x

Figure 8: Communication via ELEKTRA Network

At the beginning of ELEKTRA 2.x it was planned to get a new ESM that is directly connected to the redundant ELEKTRA Network.

Actually, there exists no “non ELEKTRA 2.x component” that is connected to the redundant ELEKTRA Network.

Security related communication between ELEKTRA 2.x components and non ELEKTRA 2.x components (external NTP time source, external Log Collector) is using networks other than the redundant ELEKTRA Network. The security related communication comprises syslog messages (RFC5424 via TCP or UDP, CEF via UDP) and NTPv4 messages (via UDP). Only the ELEKTRA 2.x components DGP and SCWS are configured to handle security related communication via external networks. All communication to external components is restricted with firewall settings.

Non-security and non-safety related communication to non-ELEKTRA 2.x components comprises the simulation and test tool framework of EPS, and commissioning support equipment like console terminal. Communication is restricted with host firewall settings on all ELEKTRY 2.x components to the lab ethernet interface port, which is not connected during normal operation (ELSEC:SRS:REQ:0012 and ELSEC:SRS:REQ:0014). Protocols used are SSH via TCP, NFS via TCP/UDP, and proprietary protocols.

### Connectivity with ELEKTRA 1.x components

In order to communicate with current ELEKTRA 1.x components VNI signals on top of a VOTRICS protocol have to be exchanged. This functionality is provided by XCEL in the EP context.

Figure 9: Connectivity to current ELEKTRA 1.x components

A new type of application – “CONVERTER” - is necessary to connect ELEKTRA 1.x PCs with ELEKTRA 2.x CCs. This application has the duty to route CHILL signals from LAN connections to serial HDLC connections (FUR:1576 (TRT221):	New application "CONVERTER" needed to combine CLASSIC PCs with Evolution CCs).

Figure 10: Connectivity to current ELEKTRA 1.x components using a converter

# Description of the Architectural Design Components

## Release Unit CAR

### ELEKTRA Personality (EP) - Extension of CAR for Communication

| Type: | CAR I/O HANDLER for Communication of Chill Application |
| --- | --- |
| Purpose: | FUR:0306 (TRT44): Signal sequence FUR:0307 (TRT45): Number of signal priorities FUR:0309 (TRT46): Routing mechanism of ELEKTRA Classic FUR:0332 (TRT49): Periodical supervision of the connections between computers FUR:0345 (TRT58): Configuration of routing FUR:0354 (TRT61): Routecopy mechanism FUR:0355 (TRT62): Signal arrival sequence FUR:0357 (TRT63): Signal length GCR:0347 (TRT115): System configuration transparency for runtime behaviour of applications GCR:0348 (TRT116): Time increments PRR:0322 (TRT165): Signal throughput of the communication system MDR:0311 (TRT128): Level of diagnosis data |
| Function: | - Binding between ELEKTRA CHILL signals to TAS Platform message queues - Routing of ELEKTRA CHILL signals within ELEKTRA net - Multicasting CHILL signals - Generation of the synchronous time tick - Interface to the ELEKTRA technical functionality and DGP (ELEKTRA component and connection up/down information) - extension of CAR for communication of CHILL applications |
| Subordinates: | XCEL, XEP |
| Dependencies: | - ELEKTRA NET topology stable - CAR on TAS Platform available - TAS Platform available - ETHERNET links available |
| Interfaces: | - ELEKTRA and TAS Platform configuration files, see [TP13], [TP14] - CAR HANDLER interface, see [CA_PORT_CAR], [CA_PG_CAR] - TAS Platform API, see [TP13], [TP14] |
| Resources: | - memory: 2 user space processes (about 1 MB) - Message Queue Buffers scalable Estimation: with 35 ELEKTRA hosts, message length is 150 bytes and buffer size 100, the Message Queue buffers require about 2 MB. For speeding up communication more memory will be needed. |
| References: | ELEKTRA 1.x documentation |
| Processing: | The EP handles the incoming and outgoing message stream of CHILL signals between CAR applications. On the outgoing side the signal is sent into a TAS Platform Message Queue which connects two ELEKTRA applications residing on either one or more HW boards. The routing functionality is derived by reading an ELEKTRA CHILL instance configuration and by defining the concrete TAS Platform configuration file. On the incoming side messages received from TAS Platform message queues are sent to the CAR component RMT. If a duplication of a CHILL signal to additional CHILL instances is required, the EP component replicates this signal either on incoming or on outgoing side. For CC applications the EP incoming part, generates time ticks for synchronous time delivery to them. |
| Data: | EP generates a static Routing Table (smaller than 10 KB) at boot time |

### XCEL: Binding to ELEKTRA 1.x

| Type: | Subcomponent of EP |
| --- | --- |
| Purpose: | FUR:0339 (TRT54): XCEL on top of TAS Platform (AEOS 4.x, AEOS 5.x) PRR:0322 (TRT165): Signal throughput of the communication system GCR:0347 (TRT115): System configuration transparency for run-time behaviour of applications TIR:0308 (TRT209): Protocols to supported for communication with ELEKTRA Classic applications FUR:1575 (TRT220): Binding of Evolution CC to CLASSIC PC required FUR:0120 (TRT21): EP (ELEKTRA Personality) on operating system 'Linux' |
| Function: | - Connects ELEKTRA 1.x components to ELEKTRA 2.x - Add additional routing information to CHILL signals (VNI) - Binding to HDLC serial lines providing the CEL protocol - Handling of identify byte - Running on TAS Platform and Linux |
| Subordinates: | none |
| Dependencies: | - EP available - serial HDLC links available |
| Interfaces: | - ELEKTRA and TAS Platform configuration files, see [TP13], [TP14] - TAS Platform API, see [TP13], [TP14] |
| Resources: | none, except code size (<100 KB) |
| References: | ELEKTRA 1.x documentation TAS Platform XCEL documentation |
| Processing: | XCEL is a term for EP providing communication via serial links to ELEKTRA 1.x components. XCEL adds the VNI (VOTRICS Net Index) to CHILL signals which are sent to ELEKTRA 1.x components. The VNI values are part of the EP routing configuration. The configuration also defines the mapping of a specific serial device to the ELEKTRA 1.x component connected to it. The XCEL functions are part of EP. XCEL is required because of the following reasons: - Allow mixing of ELEKTRA2 and ELEKTRA1 components during the migration from ELEKTRA1 to ELEKTRA2 (relevant for development only). - PC-SUBIT of ELEKTRA1: Allow to run TPC (test application) on ELEKTRA2 hardware. - Proof of concept: Connect ELEKTRA 1.x PCs to ELEKTRA 2.x CC (see [EL_EBO2_EL1PC]). - Connect ELEKTRA 2.x DGP to ELEKTRA 1.x CC for System Administration Test ELEKTRA1 (see [EL_SYAD_EL1_CH]). - Provide the usage of RaSTA protocol to ELEKTRA 1.x OBB by connecting an ELEKTRA 2.x RCU to ELEKTRA 1.x CC (see chapter 5.23 “Use OCS SBJ to provide RaSTA protocol for external communication”). |
| Data: | - |

### CAR on TAS Platform

| Type: | Runtime System for ELEKTRA CHILL Applications on TAS Platform |
| --- | --- |
| Purpose: | FUR:0330 (TRT47): Scheduling strategy for ELEKTRA applications FUR:0331 (TRT48): Number of ELEKTRA applications served by CAR FUR:0338 (TRT53): CAR on top of TAS Platform (AEOS 4.x, AEOS 5.x) FUR:0357 (TRT63): Signal length GCR:0303 (TRT111): System configuration transparency concerning database access GCR:0347 (TRT115): System configuration transparency for run-time behaviour of applications PRR:0322 (TRT165): Signal throughput of the communication system FUR:0814 (TRT89): Execution of A/B-Software on the same computer [CAR:APPL_COND:01]: No Safety Functions [CAR:FMEA:RULE:01]: CHILL Exceptions [CAR:FMEA:RULE:10]: DBS Integrity [CAR:FMEA:RULE:11]: Error Buffer |
| Function: | - Provide CAR runtime system on TAS Platform with same functionality as for ELEKTRA 1.x without recovery - Support more than one CHILL application on one board (only TAS-PLF 1.x) |
| Subordinates: | RMT, EBS, DBS, CAFE, DBG, CAR offline tooling |
| Dependencies: | - TAS Platform - CHILL build environment |
| Interfaces: | TAS Platform API |
| Resources: | - Depends on Application requirements Typical memory amount: RMT: 5 MB, DBS: 2 to 8 MB, EBS: 0,5 MB |
| References: | CAR, see [CA_ADD_CAR] |
| Processing: | RMT is responsible to schedule CHILL instances and to provide the CHILL communication mechanisms of one CHILL application. For communication to external (not application local) instances RMT interfaces with EP. DBS provides the ELEKTRA runtime database. EBS serves the CAR error buffer. TAS Platform 1.x: only one CAR is running on a computer. This CAR handles all CHILL applications running on this computer. TAS Platform 2.x: CAR handles only one CHILL application. Each CHILL application running on a computer has its own CAR. [CAR:APPL_COND:01]: ELEKTRA2 needs CAR because of the following reasons (on TAS-PLF as well as on Linux): Provide runtime environment for CHILL programs Provide the functions of CAR of ELEKTRA1 Provide a database Provide a default function for unhandled exceptions Provide stack supervision of CHILL processes ELEKTRA2 needs EP because of the following reasons: Provide communication with CHILL signals by using the communication system of TAS-PLF cs-remote Replacement of VOTRICS of ELEKTRA1 ELEKTRA does not require any safety functions from CAR/EP. [CAR:FMEA:RULE:01]: CAR provides the procedure “set_exit_routine”. This procedure has one parameter: the pointer to a procedure that is called in case an unhandled exception occures. By providing an ELEKTRA specific procedure to “set_exit_routine” ELEKTRA can overwrite the default procedure of CAR. The ELEKTRA specific procedure must not return, but call either enter_halt or car_exit to terminate the calling application. [CAR:FMEA:RULE:10]: ELEKTRA will use the following types of relations provided by the database and DBS: ROM: This type will be used for station specific data. These data will be generated by data preparation tools of the customer project. All ROM relations will be secured by CRC provided by DBS. CMOS: This type will be used for dynamic data that have to be stored in a reset stable way. All CMOS relations will be secured by CRC provided by DBS. After the restart of an application a consistency check is executed between channel A and channel B. Details will be described in [EL_SYS_ADM]. RAM: This type will used for dynamic data that are relevant only during operation. A RAM relation may be secured by CRC provided by DBS. An ELEKTRA application can choose, if it wants to have a CRC for a specific RAM relation. ELEKTRA uses two diverse channels and voting to detect inconsistencies between the to channels. A failing write operation is one of many reasons that can lead to an inconsistency between the two channels. [CAR:FMEA:RULE:11]: CAR provides an error buffer. ELEKTRA will use this buffer configured as ring buffer. This buffer contains diagnosis information generated by an ELEKTRA application. ELEKTRA will ensure that the error buffer is stored in the reset stable memory of the computer. This shall ensure that the error buffer is available after a new start of CAR and the ELEKTRA application. ELEKTRA shall use the error buffer only for diagnosis purposes. |
| Data: | - |

### CARIX: CAR on LINUX

| Type: | Runtime system for ELEKTRA CHILL Applications on LINUX |
| --- | --- |
| Purpose: | FUR:0117 (TRT18): Environment (pthreads, timeouts, shared-memory, etc.) for CAR-handler on 'Linux' FUR:0118 (TRT19): CAR on operating system 'Linux' FUR:0357 (TRT63): Signal length PRR:0322 (TRT165): Signal throughput of the communication system GCR:0347 (TRT115): System configuration transparency for run-time behaviour of applications |
| Function: | - Provide CAR functionality on LINUX (similar to CAR on TAS Platform) |
| Subordinates: | similar to CAR on TAS Platform |
| Dependencies: | LINUX |
| Interfaces: | - LINUX API - CS (= Platform Component for message transport) available on LINUX |
| Resources: | - Similar to CAR on TAS Platform (see chapter 5.1.3 “CAR on TAS Platform”) |
| References: | CAR on TAS Platform TAS Platform |
| Processing: | Similar to CAR on TAS Platform. Each CHILL application running on a computer has its own CAR. |
| Data: | - |

### XEP: EP running without CAR

| Type: | Subcomponent of EP |
| --- | --- |
| Purpose: | FUR:0846 (TRT667): XEP (EP running without CAR system) required for LINUX FUR:0847 (TRT668): XEP (EP running without CAR system) required for AEOS 4.x and AEOS 5.x FUR:0848 (TRT669): XEP has to provide TCP/IP connections |
| Function: | - XEP is required to support application without running a CAR system providing the EP functionality. - Additional to twofold redundant LAN the XEP shall provide TCP/IP connections in parallel |
| Subordinates: | none |
| Dependencies: | - XEP available, EP available - ETHERNET links available |
| Interfaces: | - ELEKTRA and TAS Platform configuration files, see [TP13], [TP14] - TAS Platform API, see [TP13], [TP14] |
| Resources: | none, except code size (<100 KB) |
| References: | TAS Platform CAR documentation CEL documentation |
| Processing: | XEP provides communication between CHILL applications and application without CAR. |
| Data: | - |

### LWATCH: Get diagnosis from CS about connection status

| Type: | Subcomponent of EP |
| --- | --- |
| Purpose: | Forward state change of the remote message queues from CS to all EPs/XEPs running on a computer. [CAR:FMEA:RULE:12]: LWATCH |
| Function: | - ELEKTRA needs the status of the physical connections of the redundant internal LAN to be able to provide diagnosis information. - [CAR:FMEA:RULE::12]: The supervision of connections by EP (LWATCH) must not be used for safety critical function. It is intended for diagnosis only. |
| Subordinates: | none |
| Dependencies: | - XEP available, EP available - ETHERNET links available |
| Interfaces: | EP resp. XEP CHILL signal interface |
| Resources: | None |
| References: | CAR, see [CA_ADD_CAR] |
| Processing: | CS provides the status of the connection via remote message queues on a logical level and on a physical level. To be able to forward this information to all ELEKTRA applications executing on the same computer a distribution process LWATCH is necessary. LWATCH will distribute the status information to the different EPs and XEPs executing on the same computer. ELEKTRA shall use the messages containing information about the status of the connections for diagnosis purposes only. |
| Data: | - |

## Migration of technical functionality

### General functionality

| Type: | Technical functionality used in all ELEKTRA 2.x components (CC, EC, LDC, DGP, RCU) |
| --- | --- |
| Purpose: | GCR:0305 (TRT113): System configuration handling of technical functionality GCR:0359 (TRT119): Source code basis for technical functionality MDR:0311 (TRT128): Level of diagnosis data GCR:0352 (TRT118): Scalability of application resources OAR:1008 (TRT246): Use of computers with wrong SW MDR:0213 (TRT297): Ethernet link failure MDR:1002 (TRT230): Recognition of defective components MDR:1007 (TRT245): Easy failure recognition SAR:0353 (TRT178): Availability of printer-buffer data SAR:0402 (TRT238): same dongle-identifier SAR:0403 (TRT241): CHILL-Signals FUR:0363 (TRT66): Identification concept for spare hardware computers SAR:1211 (TRT276): Identification of links SAR:1212 (TRT277): Identification of installation [CORE-2.x-SW:SAC:0026]: Limitation of TAS Platform with regard to security [CORE-2.x-SW:SAC:0172]: QoS Priority usage [CORE-2.x-SW:SAC:0114]: Neighbourhood and version checks [CORE-2.x-SW:SAC:0018]: Check of TaskSets. |
| Function: | Technical functionality consists of following functionality: - boot phases - supervision and diagnosis of ELEKTRA components - version controlling (see [EL_SYS_ADM] chapter 3.4.6 “Version Control”) - configuration - redundancy handling of ELEKTRA components - general inquiry (“Generalabfrage”) Migration and adaptation issues are: - range of nodes supervised by a designated component (CCA) - reaction to individual faults - monitoring of diagnostics from the communication system - life cycles (see [EL_SYS_ADM] chapter 3.4.5 “Periodical Supervision of Connections” + chapter 3.4.13 “Detection of Life Cycle between Busexe respectively FBI and EC”) - first application process - high load situations (Timedrift Supervision, see chapter 5.15 “Detection of high system load (“Hochlasterkennung”)” and [EL_SYS_ADM] chapter 3.4.12 “Detection of high system load, regulation of application load”) - supervision of reactivity (Reactivity Supervision, see chapter 5.18 Supervision of reactivity and [EL_SYS_ADM] chapter 3.4.17 for TAS-PLF 1.x and 3.4.18 for TAS-PLF 2.x) - Scalability of application resources - diagnosis for defective components - end to end checks of chill signals (see chapter 5.14 “End-to-End Checks” and [EL_SYS_ADM] chapter 3.4.11 “Detection of end_to_end_checks”) - identification of computers (see chapter 5.12.1 “Ethernet addresses used within ELEKTRA LAN”) Definition of ELEKTRA 2.x system limits is available in [EL_LIM_INT], [EL_LIM_OBB] and [EL_LIM_SBB]. |
| Subordinates: | - Warm-standby redundancy handling for CC components (see chapter 5.3 “Warm-standby redundancy concept for CC”) - Hot standby redundancy handling for EC components (see chapter 5.4 “Hot-standby redundancy concept for EC”) |
| Dependencies: | Current technical functionality stored in ClearCase environment |
| Interfaces: | - interface to operational functionality unchanged - interface to EP - interface to DGP adapted - ELEKTRA Network configuration |
| Resources: | same as current technical functionality |
| References: | see [EL09], [EL17] , [EL31] |
| Processing: | - |
| Data: | - |

[EN 50159] defines different categories of networks based on threats (see 14). [EN 50159] also defines necessary defences against the threats (see Table 15).

Table 14: Threat/Category relationship defined in Table B.2 of [EN 50159]

| Category | Repetition | Deletion | Insertion | Re-sequence | Corruption | Delay | Masquerade |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cat. 1 | + | + | + | + | ++ | + | - |
| Cat. 2 | ++ | ++ | ++ | + | ++ | ++ | - |
| Cat. 3 | ++ | ++ | ++ | ++ | ++ | ++ | ++ |
| Key - Threat can be neglected. + Threat exists, but rare; weak countermeasures sufficient. ++ Threat exists; strong countermeasures required. NOTE This matrix of threats is only a guide – analysis will always be necessary to determine whether countermeasures are required and to what degree. Each threat will be dependent on network type, application and configuration. |  |  |  |  |  |  |  |

Table 15: Threats/Defences matrix defined in Table 1 of [EN 50159]

| Threats | Defences |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sequence number | Time stamp | Time- out | Source and destination identifiers | Feed-back message | Identification procedure | Safety code | Cryptographic techniques |
| Repetition | X | X |  |  |  |  |  |  |
| Deletion | X |  |  |  |  |  |  |  |
| Insertion | X |  |  | X (a) | X (b) | X (b) |  |  |
| Re-sequence | X | X |  |  |  |  |  |  |
| Corruption |  |  |  |  |  |  | X (c) | X |
| Delay |  | X | X |  |  |  |  |  |
| Masquerade |  |  |  |  | X (b) | X (b) |  | X (c) |
| (a) Only applicable for source identifier. Will only detect insertion from invalid source. If unique identifiers cannot be determined because of unknown users, a cryptographic technique shall be used, see 7.3.8. (b) Application dependent. (c) See 7.4.3 and Clause C.2. |  |  |  |  |  |  |  |  |

According to these tables the internal network of ELEKTRA will be category 1. The communication of ELEKTRA originally had no defense against delay of the network. The introduction of reactivity supervision (see chapter 5.18) is a measure to detect a delay in parts of the communication (not all communication paths are covered by reactivity supervision).

### Optical Status Display

| Type: | Display for status information. |
| --- | --- |
| Purpose: | MIR:0317 (TRT152): Hex-display information |
| Function: | Substitution for hex-display information of ELEKTRA 1 has to be provided for maintenance and diagnosis reasons. For MEN-D2 computers two LEDs are available for application defined display. For MEN-D6 computers two LEDs are available for application defined display. For MEN-D10 computers four LEDs are available for application defined display. For PCs no hardware is available. Additionally the hex-display information is sent to DGP to display it on the terminal. |
| Subordinates: | One LED is assigned to one application. Therefore two applications can display their status (hex-display information). The LEDs are controlled directly by the application without an interface provided by CAR. The access routines are part of the application. They check, if a display is available on the actual computer. These routines have to be changed, if the hardware changes. |
| Dependencies: | Driver available |
| Interfaces: | MEN-D2/MEN-D6/MEN-D10 computer: see TAS-PLF documentation [TP13], [TP14] |
| Resources: | MEN-D2/MEN-D6/MEN-D10 computer: LED0 and LED1 |
| References: | MEN-D2/MEN-D6/MEN-D10 computer: see TAS-PLF documentation [TP13], [TP14] |
| Processing: | The generated hex-display information is the same as in ELEKTRA 1. This information is sent to DGP by all applications. This information can be displayed on the DGP terminal on request. Additionally the LEDs of the MEN-D2/MEN-D6/MEN-D10 computer are used: LED0 is used by TCD, IAU, CCA, ECA, RCA, DGP LED1 is used by TCD, IAU, CCB, ECB, RCB, SCU Initially a LED is dark. It is turned on, when the application starts. As long as the application is working, the LED is blinking. There are following blink frequencies: - 2 sec on, 2 sec off: application is standby/secondary - 4 sec on, 4 sec off: application is active/primary If the LED is permanently turned on or off during operation, the application does not work correctly any more. TCD and IAU are using the LEDs in a different way: LED0: switched on for 2s if status of a logical connection is shown on LED1. LED0 (TCD): shows the state of a connection. After a state has been shown, LED0 is switched off for 1s and no state is shown on LED1. After all logical connections have been shown, a break of 4s is done. LED0 (IAU): shows the state of channel A and B. LED1 (TCD): during setup of a connection the following states are shown: (1) 8x on-off (cycle 125ms):state down (2) 2x on-off (cycle 500ms): state connect (3) in state "configuration finished" LED1 (IAU): (1) 8x on-off (cycle 125ms):state ELEKTRA-down/RBC-down – application is running and waits for connection to CC (2) 2x on-off (cycle 500ms): state ELEKTRA-up/RBC-down – connection to CC is completely setup – wait for connection to RBC (3) on (2s): ELEKTRA-up/RBC-up – connection to RBC is completely setup – the FSS – route state is sent periodically to RBC. If more than 2 applications are running on the same computer, only 2 applications are allowed to use the LEDs. Therefore a prioritisation is necessary. The following list defined the prioritisation in descending order: - TCD (including IAU) - CCA, CCB - RCA, RCB - ECA, ECB - DGP, SCU |
| Data: | Devices: LED0 - /dev/hexd2 , LED1 - /dev/hexd1 Writing 0 to device means ‘turn off’, 1 means ‘turn on’. |

## Warm-standby redundancy concept for CC

| Type: | Redundancy concept for CC components, software module within technical functionality and EP |
| --- | --- |
| Purpose: | DPR:0314 (TRT5): Standby concept for CC applications PRR:0320 (TRT163): Standby switch over time SAR:1201 (TRT189): Safe switch-over SAR:0315 (TRT175): Availability of CMOS data SAR:0353 (TRT178): Availability of printer-buffer data SAR:1101 (TRT505): Delete CMOS data - double redundant SAR:1102 (TRT512): Delete CMOS data - single redundant FUR:1703 (TRT224): Independence of different standby redundancy mechanisms |
| Function: | - supervision protocol between redundant CCs - data synchronisation between redundant components (CMOS data, printerbuffer data) - strategy for keeping or deleting CMOS data dependent on CC-redundancy. - definition of warm-standby mode - fault detection and switch over to standby component - diagnosis information for connected components and DGP Concept has to be applied for CCA and CCB executed on the same HW computers and on different HW computers |
| Subordinates: | - |
| Dependencies: | - |
| Interfaces: | - diagnosis interface to connected components - diagnosis interface to DGP |
| Resources: | - reliable communication services between redundant components (based on redundant Ethernet) - identification mechanism for redundant components |
| References: | [EL_SYS_ADM] |
| Processing: | concept definition is explicitly considered in the PMP |
| Data: | - |

## Hot-standby redundancy concept for EC

| Type: | Redundancy concept for EC component, software module within technical functionality and EP of CC component and EC component |
| --- | --- |
| Purpose: | SAR:1201 (TRT189): Safe switch-over PRR:0321 (TRT164): EC update time (“Generalabfrage”) DPR:0316 (TRT6): Redundancy concept for FE subsystem DPR:0810 (TRT8): two-fold redundant EC FUR:1703 (TRT224): Independence of different standby redundancy mechanisms DPR:0223 (TRT315): No automatic switch over of EC SAR:1214 (TRT314): EC-Switch-Over |
| Function: | - migration of current hot-standby redundancy concept - primary/secondary voting mechanisms - fault detection and switch over to standby component - diagnosis information for connected components and DGP Redundancy concept also used for LDC components - No automatic switch over of EC |
| Subordinates: | - |
| Dependencies: | - “Überwachungsbaugruppe” - reliable communication services between redundant components (based on redundant Ethernet) - identification mechanism for redundant components |
| Interfaces: | - diagnosis interface to connected components - diagnosis interface to DGP |
| Resources: | - |
| References: | - |
| Processing: | Primary/secondary voting mechanism (in ELEKTRA1 performed by VOTRICS) are realised within EP, this requires a “Generalabfrage” afterwards In case of switch over no loss of data (in two channels) is accepted. Therefore the primaries have to be located on different hardware boards (even the two channels are executed on one hardware board. This requires a mechanism during the startup/restart phase of a EC application. |
| Data: | - |

## Integration of CCA and CCB on one HW computer

| Type: | Software module within technical functionality and EP in the CC component |
| --- | --- |
| Purpose: | GCR:0301 (TRT110): Scalability of CC SAR:1204 (TRT192): Two channels on one hardware SAR:1205 (TRT193): Memory-space GCR:0347 (TRT115): System configuration transparency for run-time behaviour of applications GCR:0303 (TRT111): System configuration transparency concerning database access GCR:0304 (TRT112): System configuration transparency for operational functionality FUR:0330 (TRT47): Scheduling strategy for ELEKTRA applications FUR:0331 (TRT48): Number of ELEKTRA applications served by CAR FUR:0346 (TRT59): Exit and restart of applications on one computer GCR:0349 (TRT117): System configuration transparent for time increments. FUR:0350 (TRT60): Execution time supervision DPR:0222 (TRT313): Only one CC-processing board per redundancy |
| Function: | Integration of CCA and CCB on one HW computer: - Implementation of the scheduling strategy - CAR extension: - handling of more than one application - stop/restart of application - multiple database handling CAR extensions also necessary for integration of ECA and ECB on one HW computer (FUR:0814 (TRT89): Execution of A/B-Software on the same computer) Only one CC-processing board per redundancy |
| Subordinates: | see also chapter 5.1.3 |
| Dependencies: | - definition of application constraints derived from the evaluation of the time-window test - Using TAS Platform instead of VOTRICS has an influence on the signal arrival sequence. The effect on the CCA/CCB application and synchronisation has to be evaluated even if the two channels are executed on different hardware boards. |
| Interfaces: | no influence on operational functionality |
| Resources: | more than 64 MB are necessary FUR:0333 (TRT50): Memory to be supported FUR:0334 (TRT51): Non volatile RAM |
| References: | - Evaluation results derived from time-window test - see [EL_SYNC] |
| Processing: | CAR is changed to handle more than one application on one computer. Each application has its own resources: dispatcher, queues, database, time, EP. |
| Data: | - |

## Integration of ECA and ECB on one HW computer

Described in Architectural Design Document ELEKTRA EVOLUTION Field Element Controlling [EL07].

## Remove VCB for MMI

Described in Architectural Design Document ELEKTRA EVOLUTION Man Machine Interface [EL08].

| Type: | Software module in MMI |
| --- | --- |
| Purpose: | FUR:0127 (TRT26): VCB removal FUR:0128 (TRT27): Cancel lifecycle telegrams between VCA and VCB |
| Function: | - Remove VCB in MMI with minimal changing of CC application Note: safety principle of the 2nd divers channel is replaced by a safe method – see chapter 5.17 Methodsafe MMI and chapter 5.16 Usage of methodsafe EBO2 instead of methodsafe MMI. |
| Subordinates: | - |
| Dependencies: | - |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | Communication direction VC to CC: use dummy VCB processes to generate signals of VCB no change of CC needed. Communication direction CC to VC: configuration of EP is used to suppress signals to VCB. |
| Data: | - |

## Long Distance Communication (LDC)

| Type: | Long distance communication (LDC) component |
| --- | --- |
| Purpose: | FUR:1304 (TRT106): Integration of LDC functions on EC FUR:0370 (TRT72): Communication interface between ELEKTRA systems FUR:0367 (TRT69): Number of external communication lines to be supported FUR:0368 (TRT70): Integration of RCU and SCU FUR:1105 (TRT284): More than one LDC computer |
| Function: | - Functionality covered with ELEKTRA TCD component - safe, two channel long distance communication [EN 50159] - same redundancy concept used as for EC components - integration/separation of LDC and EC on one hardware board - The communication interface between ELEKTRA systems must not consist of proprietary data (RMT Header, VNI Index, Layer 4). This is necessary for transferring data between ELEKTRA 2.x systems and ELEKTRA 1.x systems. Design of LDC depends on the specific ELEKTRA applications to be served. LDC is realised by TCD, IAU and by RCU/SCU. TCD is used by ELEKTRA Switzerland for - Electronic Block between 2 ELEKTRAs (or any system that implements the same protocol) - Interface between local CC/DGP and remote ECs in case of communication via category 2 network IAU is used by ELEKTRA Switzerland for - Interface to RBC Alstom RCU/SCU are used by ELEKTRA Switzerland for - Interface to ILTIS - Interface to RBC HITACHI RCU/SCU are used by ELEKTRA Austria and ELEKTRA International for - Remote control of other interlocking systems - Block communication between electronic interlocking systems - Interface to RIC - Interface to Grauli - Interface to RZÜ (Aramis) RCU/SCU are used by ELEKTRA International additionally for - Interface to AKF (for customer Gysev) - Interface to Köfe (used instead of RZÜ) |
| Subordinates: | - |
| Dependencies: | - TAS Platform available - long distance communication protocol (X.25) |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | see TCD documentation [EL18] |
| Processing: | - |
| Data: | - |

## Additional components

| Type: | Hardware components |
| --- | --- |
| Purpose: | FUR:0372 (TRT74): Printer replacement by electronic storage TVR:1102 (TRT234): ELEKTRA Labor PC FUR:1103 (TRT235): DGP running on Linux PC FUR:1104 (TRT236): DGP on MEN-D2 needs video controller FUR:0364 (TRT67): Integration of DGP and RDU FUR:0369 (TRT71): RIC functionality MIR:0136 (TRT145): Signal lamp (AST) replacement |
| Function: | - Printer replacement has to be replaced be electronic storage medium - ELEKTRA labor PC: it must be possible to run all applications (except X25) on one Linux PC - Integration of DGP and RDU on the same computer hardware - DGP on Linux has to be supported - DGP on MEN-D2 with video controller and monitor has to be supported as replacement of today’s terminal - RIC functionality (Austria, Hungary) has to be supported - NIC has to be supported (Austria, Hungary) as replacement for RIC - Today's signal lamp "AST" of the control desk (Austria, Hungary) must be replaced by an adequate function. |
| Subordinates: | - |
| Dependencies: | - TAS Platform available |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | - |
| Data: | - |

## Timing Requirements

| Type: | Requirements for Timing |
| --- | --- |
| Purpose: | FUR:1303 (TRT105): Maximum Detection time of SW subsystem failure SAR:1202 (TRT190): Loss of information SAR:1207 (TRT195): Disclosure time SAR:1208 (TRT196): Restrictive actions during safety-reaction SAR:1209 (TRT197): Non-restrictive actions during safety-reaction PRR:0323 (TRT166): Command reaction time PRR:0324 (TRT167): High load situations DPR:0201 (TRT285): System reliability (System down time < 3 min) DPR:0202 (TRT286): System reliability (System down time < 8 h) DPR:0203 (TRT287): System availability SAR:0335 (TRT176): Error reaction time |
| Function: | - The time between the complete failure of a SW subsystem (e.g. an application, the communication system etc.) and its detection (and appropriate reaction to it) has not to exceed 1 sec. - If information is lost in both channels, a safety-reaction has to be performed. - The system has to be designed in a way, that a disclosure time of random faults of 48 hours is sufficient. - In case of a safety-reaction all restrictive actions (“ausführungssichere Kommandos”) to reach the safe ground-state have to be performed within 1 sec. - In case of a safety-reaction a non-restrictive action (“hemmungssichere Kommandos”) must not be performed and this state has to be retained until full service is reached. |
| Subordinates: | - |
| Dependencies: | - TAS Platform available - long distance communication protocol (X.25) |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | - |
| Data: | - |

## Functions needed to check the system

| Type: | Functions to check the system |
| --- | --- |
| Purpose: | MDR:1012 (TRT250): Read-out of statuses of the outdoor equipment MDR:1019 (TRT283): Read-out of statuses of elements within the station topology MDR:1011 (TRT249): Change (replacement) of printed boards MDR:1010 (TRT248): Manual switch between redundancies OAR:1013 (TRT251): Simulation of outdoor components PRR:0602 (TRT228): Definition of configuration and scaling MDR:1003 (TRT231): "Signalwegverfolgung" MDR:1009 (TRT247): Data transmission on the lines DPR:0216 (TRT304): Redundant fans-wheels DPR:0228 (TRT321): Independence of external systems in case of faults DPR:0229 (TRT322): Independence of ELEKTRA-units in case of faults FUR:3801 (TRT517): LAN-failure FUR:3802 (TRT518): LAN diagnosis – WebRide FUR:3803 (TRT519): LAN diagnosis |
| Function: | - Read-out of statuses of the outdoor equipment of EC and station topology of CC - Change (replacement) of printed boards - Manual switch between redundancies - Simulation of outdoor components - Definition of configuration and scaling - The fans must be monitored and the failure must be indicated to the operator - Invalid messages must be reported and indicated to the operator. |
| Subordinates: | - |
| Dependencies: | - |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | - |
| Data: | - |

## LAN components and handling of failures

| Type: | Hardware |
| --- | --- |
| Purpose: | DPR:0204 (TRT288): LAN jabber protection DPR:0205 (TRT289): LAN excessive collision protection DPR:0206 (TRT290): LAN environment DPR:0207 (TRT291): LAN replacing of devices DPR:0209 (TRT293): LAN default mode of devices DPR:0212 (TRT296): CC-Ethernet link failure DPR:0401 (TRT218): Redundant LAN DPR:1001 (TRT229: Failure of a component connected to the bus FUR:0210 (TRT294): LAN distances > 100m FUR:0211 (TRT295): Hub distance extender module FUR:1107 (TRT507): non redundant components SAR:0214 (TRT298): Galvanic separation of Ethernet-ports on TAS-Platform computer FUR:6205 (TRT1088): Remote Control: decentralized architecture - maximal number of subnet |
| Function: | - Hardware failures should not lead to a complete system reset. - speed and distance of LAN |
| Subordinates: | - |
| Dependencies: | - |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | The internal communication of ELEKTRA2 uses a redundant LAN (instead of serial communication in ELEKTRA1). The redundant LAN is handled by “cs remote” (the communication system of TAS-PLF). “cs remote” provides diagnosis messages for the availability or unavailability of the individual LAN redundancies. The configuration of “cs remote” determines if the LAN is redundant or not redundant. For ELEKTRA International the maximum number of possible subnets within ELEKTRA LAN in the supervised area shall be 40. The Ethernet addresses used within the ELEKTRA internal LAN are defined according chapter 5.12.1. |
| Data: | - |

### Ethernet addresses used within ELEKTRA LAN

Up to 4 network interfaces have to be configured for ELEKTRA internal use:

- 2 interfaces for the redundant LAN

- 1 interface for connection to a gateway in the HITACHI lab

- 1 interface for connection EBO2 - MOVUS

ELEKTRA uses the following definitions (IPv4):

- IP address range: 192.168.0.0 - 192.168.255.255

- Netmask: 255.255.255.0

| Network Interface | Definition for address |
| --- | --- |
| LAN Labor | 192.168.<subnetLabor>.<hostNumber> |
| LAN A ELEKTRA internal | 192.168.<subnetLanA>.<hostNumber> |
| LAN B ELEKTRA internal | 192.168.<subnetLanB>.<hostnumber> |
| MOVUS ELEKTRA internal | 192.168.<subnetMovus>.<hostNumber> |

Table 16: ELEKTRA internal network interfaces – definition for address

The definition for the different subnets and for hostNumber is contained in the following tables.

To allow more different networks the definition of subnet has been changed for ELEKTRA using TAS-PLF 2.x. The number of subnets defined for TAS-PLF 2.x is required by [EL42].

| Subnet | Range TAS-PLF 1.x | Range TAS-PLF 2.x |
| --- | --- | --- |
| <subnetLabor> | 0 – 9 | 0 – 39 |
| <subnetLanA> | 10 – 19 | 40 – 79 |
| <subnetLanB> | 20 – 29 | 80 – 119 |
| <subnetMovus> | 30 – 39 | 120 – 159 |
| UNUSED | 40 – 255 | 160 – 255 |

Table 17: ELEKTRA internal network – definition for subnet

| hostNumber | usage |
| --- | --- |
| 0 | Reserved for network |
| 1 | Gateway (only LAN Labor) |
| 2 – 99 | ELEKTRA hosts |
| 100 – 254 | UNUSED |
| 255 | Reserved for network |

Table 18: ELEKTRA internal network – definition for hostnumber

## Redundant and non-redundant MMI

| Type: | Software and hardware function |
| --- | --- |
| Purpose: | FUR:1106 (TRT506): VC for temporary use FUR:1107 (TRT507): non redundant components FUR:3001 (TRT484): Operator workplace with redundancy FUR:3004 (TRT487): Switch over, if one PC fails FUR:3005 (TRT488): Mouse, keyboard, monitors not duplicated FUR:3006 (TRT489): change of defect mouse, keyboard, monitor SAR:3001 (TRT490): Only active PC is allowed to run FUR:3804 (TRT520): LAN diagnosis – switched-off redundant MMI-PC FUR:3805 (TRT521): LAN diagnosis - display for switched-off redundant MMI-PC FUR:3806 (TRT522): LAN diagnosis - VC for temporary use |
| Function: | - Redundant MMI like redundant VC in ELEKTRA 1.x - Non-redundant MMI |
| Subordinates: | - |
| Dependencies: | - Hardware for switching over mouse, keyboard, monitor, loud speaker and power supply is needed in case of redundant MMI |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | - CC has to handle 2 redundancies of one MMI. Only one redundancy is running, while the other one is turned off. - CC has to handle a non-redundant MMI. - CC has to handle a MMI, that is only available for temporary use. |
| Data: | There must be a configuration to define the actual number of redundancies for one MMI. |

## End-to-End Checks

| Type: | Software function |
| --- | --- |
| Purpose: | SAR:2501 (TRT659): Safety mechanism for sending CHILL-Signals based on ELEKTRA LAN communication between different applications SAR:2502 (TRT660): Safety mechanism for sending CHILL-Signals - Station-ID SAR:2504 (TRT1071): CHILL – Signals – Station-ID, safety reaction SAR:2505 (TRT1070): CHILL – Signals – Station-ID unique SAR:2506 (TRT1068): CHILL – Signals – Station-ID activation FUR:4107 (TRT1100): CAR - Callbacks at sending/receiving of message [CAR:FMEA:RULE:02]: Source and Destination Check |
| Function: | - SAR:2501 (TRT659): All sent signals (the inverted and the not inverted) must be protected with a modified CRC-Code on application level. The CRC is specified in [EL_GEN_FUNC]. - SAR:2502 (TRT660): The sender of chill signals shall add a Station-ID to the RMT data packet. The receiver of chill signals shall check the correctness of the received Station-ID. - SAR:2504 (TRT1071): CHILL signals received from different "processor" board containing a wrong Station-ID shall result in a safety reaction i.e. shutdown/reboot. - SAR:2505 (TRT1070): The Station-ID(s) used by ELEKTRA LAN(s) must be unique in the entire network. Adding the Station-ID to the RMT data packet and checking of it shall be implemented by a diverse mechanism. - SAR:2506 (TRT1068): Processing of the Station-ID is activated by providing the dedicated environment variable "STATION_CHECK". - FUR:4107 (TRT1100): CAR must call the configured callback functions of the application when sending or receiving a message. - [CAR:FMEA:RULE::02]: The application must check the message source and safety code for all safety critical messages. - [CAR:FMEA:RULE::03]: The application must use two messages for all messages that lead to a less safe state. At least one of the messages must reveal if it has not been encoded by the application via call-back (e.g. inversion). There is no need that the messages themselves are different, but 2 messages are needed logically. Including of Station-ID in messages enables the receivers of the messages to verify that messages are intended for them (if this information is not present in the content of the message data). This fulfills the requirement of the uniqueness of the identifiers for entities in the entire transmission system (according to [EN 50159]). Definition: Station-ID: Bitstring provided in file station_check.inc located in the station directory of the file system and is used by end-to-end check. |
| Subordinates: | - |
| Dependencies: | CAR |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | The purpose of the end-to-end checks is to secure the data of Chill signals during their transmission in order to protect them from possible common mode failures in non-diverse software layers (Elektra personality, operating system, communication system) (SAR:0406 (TRT311): Safe communication over Ethernet). The technical basis for implementing the end-to-end checks is a callback mechanism for all Chill signal send system calls (send, send_signal, put_internal_message_queue, put_external_message_queue, put_queue_internal, put_queue_external, start_signal_timeout) and a separate callback mechanism for all Chill signal receive system calls (receive, receive_signal) in CAR. The procedures to be called by these callback mechanisms have to be provided by the application and must be declared ("installed") during the start-up of the application by using the CAR system calls send_signal_callback and receive_signal_callback, respectively. These procedures will be described in more details in the following sections. The callback mechanism ensures that the declared procedure is called at an appropriate step during the send or receive system call. The declared procedure is therefore executed in the application context, which is exactly the purpose of end-to-end checks. CAR must use the callback functions provided by the application whenever a signal is sent or received (FUR:4107 (TRT1100)). For details for communication via Ethernet see Architectural Design Document ELEKTRA 2.x Kommunikation über Ethernet [EL24]. [CAR:FMEA:RULE::03]: ELEKTRA will use signals with different signal code for the application channels. Both signals will be protected according to the mechanism described above. |
| Data: | - |

For a detailed description see [EL_GEN_FUNC].

## Detection of high system load (“Hochlasterkennung”)

| Type: | Software function |
| --- | --- |
| Purpose: | FUR:1929 (TRT662): Detection of high system load – measurement of time drift SAR:1901 (TRT663): Detection of high system load – safety mechanism SAR:1902 (TRT664): Detection of high system load has to be done in a safe way |
| Function: | - The time drift between time of CC application and uptime of the OS is calculated. This is done by CCA and CCB separately. - If the calculated time drift is greater than a configurable value, a shutdown has to be done |
| Subordinates: | - |
| Dependencies: | - |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | - CCA and CCB calculate the time drift every time the update time signal is received. For details see [EL25] |
| Data: | There must be a configuration to define the upper limit for the time drift of CCA and CCB separately. |

## Usage of methodsafe EBO2 instead of methodsafe MMI

Described in Funktionelle Spezifikation, Methodensichere Einheitliche Bedienoberfläche EBO V2 [EL_FS_MS_EBO2] and Architectural Design Document ELEKTRA ÖBB EBO2 [EL26].

| Type: | Software and hardware function |
| --- | --- |
| Purpose: | FUR:4101 (TRT683): LAN diagnosis – switched-off redundant MMI-PC |
| Function: | - Redundant EBO2 like redundant VC in ELEKTRA 1.x (EBO2 uses hot standby redundancy). - EBO2 must support method safe input and display for the user of EBO2. |
| Subordinates: | - |
| Dependencies: | - Hardware for switching over mouse, keyboard, monitor, loud speaker and power supply is needed in case of redundant MMI |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | - CC has to handle 2 redundancies of one EBO2. Both redundancies are running. - CC has to handle a non-redundant EBO2. |
| Data: | There must be a configuration to define the actual number of redundancies for one EBO2. |

## Methodsafe MMI

Described in Architectural Design Document Methodsafe MMI, ELEKTRA 2.x Schweiz [EL29].

| Type: | Software and hardware function |
| --- | --- |
| Purpose: | SAR:3025 (LRT296): Safety degree of the computer for indication data FUR:3026 (LRT297): Life cycle between safe computer and MMI SAR:3027 (LRT298): reaction of safe computer, if life cycle of MMI is missing SAR:3028 (LRT299): reaction of MMI, if life cycle of safe computer is missing SAR:3031 (LRT302): Corruption of the data during the transmission between safe computer and MMI. SAR:3032 (LRT303): independence of safe computer and MMI. |
| Function: | - CC and RCU send display data to MMI. These computers (CC and RCU) have to be realized with SIL4 according to [EN 50128]. - For life cycle the concept of ELEKTRA1.x is used. - If the life cycle is missing in CC or RCU, the concept of ELEKTRA1.x is used for reaction. - If the life cycle is missing in MMI(VCA), the concept of ELEKTRA1.x is used for reaction. - For transmission of data between CC and MMI the end-to-end-checks of chapter 5.14 have to be used. - The computers for CC and the MMI application must have independent hardware and independent software. |
| Subordinates: | - |
| Dependencies: | - Application VCA running on Linux |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | - |
| Data: | - |

## Supervision of reactivity

| Type: | Software function |
| --- | --- |
| Purpose: | FUR:4400 (TRT721): Monitoring of reactivity of ELEKTRA system components - detection of max. age of data FUR:4401 (TRT722): Monitoring of reactivity of ELEKTRA system components - configurable max. age of data FUR:4403 (TRT964): Monitoring of reactivity of the system components - determination of response time FUR:4404 (TRT971): Monitoring of reactivity of the system components - congestion reaction FUR:4405 (TRT970): Monitoring of reactivity of the system components - return to normal operation FUR:4406 (TRT969): Monitoring of reactivity of the system components - threshold values SAR:4411 (TRT963): Monitoring of reactivity of ELEKTRA system components - SIL-4 FUR:4420 (TRT1106): Monitoring of reactivity of the system components – congestion reaction and diagnosis FUR:4421 (TRT1107): Monitoring of reactivity of the system components – return to normal operation and diagnosis FUR:4422 (TRT1110): Monitoring of reactivity of the EC by the CC - usage of EC-FlowControl |
|  | FUR:4423 (TRT1109): Monitoring of reactivity of the CC by the RCU - usage of RCU-FlowControl FUR:4430 (TRT1118): Monitoring of reactivity of the CC by the RCU - the CC sends to RCU the longest response time of the monitored ECs FUR:4431 (TRT1119): Monitoring of reactivity of the CC by the RCU - supervision of 2 time limits FUR:4436 (TRT1127): Monitoring of reactivity of the RCU by the CC - usage of RCU-FlowControl [CORE-2.x-SW:SAC:0172]: QoS Priority usage |
| Function: | - Each ELEKTRA system component must be able to monitor the age of data received by any other ELEKTRA system component, to detect the exceeding of an upper time limit of the age of data and to perform a safety reaction depending on the specific application of the monitoring component. The implementation will be limited in the following way: 1. supervising components: RCA, RCB, ECA, ECB, TCD-A, TCD-B supervised components: CCA, CCB 2. supervising components: CCA, CCB supervised components: ECA, ECB, RCA, RCB, TCD-A, TCD-B - The functionality "monitoring of the reactivity of ELEKTRA system components" has to be realized according to SIL4 of [EN 50128]. |
| Subordinates: | - |
| Dependencies: | - |
| Interfaces: | - |
| Resources: | ELEKTRA 2.x computer |
| References: | - |
| Processing: | - For details see [EL30] |
| Data: | The upper limit of the age of data must be configurable in the monitoring component for each component that has to be monitored. |

## Operating System TAS Platform (TAS-PLF)

### Use TAS Platform including its hardware

| Type: | Use TAS Platform as operating system running on hardware released by TAS Platform |
| --- | --- |
| Purpose: | FUR:0333 (TRT50): Memory to be supported FUR:0843 (TRT446): Up to 288 MB memory required FUR:0334 (TRT51): Non volatile RAM FUR:0337 (TRT52): Use of TAS Platform FUR:0341 (TRT55): Services of TAS Platform FUR:0343 (TRT56): Location of application code and system code FUR:0344 (TRT57): Configurable serial interface FUR:0345 (TRT58): Configuration of routing FUR:0362 (TRT65): Scalability of communication media GCR:0361 (TRT120): Highly available communication media FUR:0363 (TRT66): Identification concept for spare hardware computers FUR:0367 (TRT69): Number of external communication lines to be supported FUR:0821 (TRT90): TAS Platform must provide appropriate fieldbus interface GCR:0823 (TRT1090): ELEKTRA IXL using TAS Platform 2.x on MEN-D10, MEN-D6 and MEN-D2 computers FUR:0825 (TRT92): Replacement for the plant ID plug FUR:0829 (TRT299): TAS Platform must provide an access to a PC-MIP module for parallel I/O FUR:0830 (TRT302): Fixed numbering and allocation scheme for communication links FUR:1574 (TRT219): Twofold redundant LAN connection FUR:1577 (TRT317): OCS protocol FUR:1701 (TRT222): Identification of redundancies FUR:4200 (TRT1330): Board Serial Number D6/D10 MIR:0318 (TRT153): System reset [CORE-2.x-SW:SAC:0008]: Choice of a safe programming model. [CORE-2.x-SW:SAC:0196]: Check for validated hardware [CORE-2.x-SW:SAC:0112]: boot configuration [CORE-2.x-SW:SAC:0182]: Non safe diagnosis data [CORE-2.x-SW:SAC:0184]: No impact of diagnosis on safety critical operation [CORE-2.x-SW:SAC:0197]: Safe digital I/O [CORE-2.x-SW:SAC:0183]: No impact of diagnosis on real time properties [CORE-2.x-HW:SAC:0020]: Supported hardware configurations [CORE-2.x-HW:SAC:0170]: Refresh of 128 MB CF memory cards [CORE-2.x-HW:SAC:0173]: Refresh of programmed memory cards [CORE-2.x-HW:SAC:0198]: No virtualization for safety relevant operation |
| Function: | TAS Platform has to support at least 288 MB RAM and at least 128 MB ROM (Flashcard). TAS Platform has to support up to 288 MB RAM. 20 MB non volatile RAM has to be supported. TAS Platform (AEOS 4.x, AEOS 5.x) is the software platform to be used for the ELEKTRA components. TAS Platform (AEOS 4.x, AEOS 5.x) has to provide communication and fault tolerance services. Application recovery services are not needed. Application code and system code located on the same or on two different flashcards have to be supported. A configurable serial interface has to be provided by TAS Platform (used e.g. for DCF77 or GPS receiver). The routing of logical communication channels must be static off-line configurable with files. Communication media must be able to be used in redundant and non redundant configuration. A highly available communication media has to be supported for ELEKTRA Evolution system architecture. This is achieved with the use of redundant communication paths. An identification concept for spare hardware computers has to be provided in order to fulfill the following requirements: easy exchange by customer, independent of the application running on the hardware component to be replaced. LDC component has to support external communication lines. The number of used lines must be configurable between 1 and 20. The EC will be a TAS Platform computer. For that reason, the TAS Platform has to provide the appropriate fieldbus interface (fieldbus driver), e.g., TTP or CAN. It must be possible to identify the railway station and to provide this information to other subsystems (e.g. CC). The ELEKTRA Plant-ID setting is realised with a rear adapter board for the MEN D2/D6/D10 computer board. This setting is realised with 12 BCD switches. In order to read these switches the PC-MIP module '15P013' from MEN will be used on the MEN D2/D6/D10 computer board. The handling of this PC-MIP module must be provided by the TAS Platform. There must be a fixed numbering and allocation scheme for the communication links which are provided by the TAS Platform. The relevant links are all external links which are realised as PCI-devices, e.g., PCI-MIP-Modules (Ethernet, parallel I/O) and M-Modules (TTP, and HDLC). Also the Ethernet-Links on the LINUX-PC for the realisation of the redundant LAN (redundant CS) must be considered. A 2-fold redundant LAN is necessary to connect the ELEKTRA boards to increase the availability to the communication. Communication between ELEKTRA and another product which is based on TAS Platform (e.g. AzLM) by the OCS protocol. MEN hardware must include a mechanism (supported by TAS platform) to identify redundancies by assigning a small integer number starting with 1 to each redundant component. Assignment must be unique for each subsystem. Each individual computer board of the types D6 or D10 shall be identified by a serial number. It must be possible by analyzing the DGP data to assign the individual board serial number to each used board in the system. Manual reset and self-reset of the whole system resp. for each computer has to be provided. The following applications will use TAS-PLF as their operating system: DGP, CCA, CCB, RCA, RCB, SCU, ECA, ECB, TCD, IAU, EC-MUX, SEA, SEC, NIC. [CORE-2.x-SW:SAC:0008]: ELEKTRA will not use the recovery mechanism of TAS-PLF and therefore is not restricted to model 1 TaskSets. ELEKTRA (including CAR) will use model 3 TaskSets. The following default values shall be used for the taskset configuration: .min_red DEFAULT = 1; .max_red DEFAULT = 1; .ce_location DEFAULT = "0"; .sched_policy DEFAULT = "sched_posix"; .priority DEFAULT = 17; Model 3 must only be fulfilled for those parts of ELEKTRA (including CAR) that are known to CS by configuration. These parts are EP and LWATCH. All other parts are “OLT supervised Posix processes”. [CORE-2.x-SW:SAC:0112]: The boot level in NVRAM will be set to 2 before delivery. The boot level configured in /etc/inittab (default value 1) will not be changed by ELEKTRA. A so called “delete flashcard” (DE: “Lösch-Flashcard”) will be provided for the customer to set the boot level in NVRAM to 2. This special flashcard uses boot level 4 to execute the specific actions. Boot level 3 will be defined identically to boot level 2. Boot level 5 will be defined like boot level 2, but no application will be started automatically. This boot mode can be activated only manually for debugging purposes in the lab. Boot level S (single user mode, default value of boot level in /etc/inittab) is set by TAS-PLF in case it detects a severe error (e.g. one of the tests of OLT detects an error). This mode does not start the application. It does also not start the network communication. [CORE-2.x-SW:SAC:0182], [CORE-2.x-SW:SAC:0184]: The non safe diagnosis data of TAS-PLF will be used by ELEKTRA for diagnosis purposes only. There will be only one exception: The directory tree /var/diag/olt/* must be used for checking the correct working of startup tests and runtime tests. Diagnosis functionality will not be used for safety critical operation. [CORE-2.x-SW:SAC:0197]: All core system hardware boards provide a number of general purpose I/O pins (GPIO). When used correctly, those pins can be used to drive safety critical equipment. GPIO is not used by ELEKTRA. ELEKTRA will use the input provided by the devices “/dev/id” and “/dev/id_long” for getting the host number and for getting the plant-id. Reaction in case of an error in host number: invalid number: no application is defined for this number; this will cause a reboot of the boards. wrong number: number exist; either no application is defined for this number in the actual installation or the number will be used twice; this will cause a reboot of one or both boards. Reaction in case of an error in plant-id: check against defined number will fail and will cause a reboot of the board. COM 2 interface: ELEKTRA also provides the usage of the COM2 interface to read in the disturbance of the fans of the computer cabinet. This information is used for diagnosis purposes only. Ethernet interface: An Ethernet interface is used for ELEKTRA internal communication as well as for communication to external communication partners. An error can be detected if the used application protocol has defined measures to detect a transmission error. TTP interface: A TTP interface is used for the communication between an EC and its ICs. An error can be detected if the used application protocol has defined measures to detect a transmission error. HDLC synchronous: A synchronous HDLC interface is used for the connection to a PAD to provide communication over a serial X.25 connection. An error can be detected if the used application protocol has defined measures to detect a transmission error. HDLC asynchronous: A asynchronous HDLC interface is used for to communication to ESM, to a radio clock, to Webride and to MDS. An error can be detected if the used application protocol has defined measures to detect a transmission error. [CORE-2.x-SW:SAC:0183]: A diagnosis interfaces (serial interface or Ethernet interface) will only be used by DGP in field mode. DGP will execute on an independent board. Therefore it will not directly influence the realtime behaviour of the SIL-4 functionality of ELEKTRA. |
| Subordinates: | Hardware supported by TAS Platform: MEN-D2, MEN-D6, MEN-D10, SCC, SC33 |
| Dependencies: | — |
| Interfaces: | — |
| Resources: | Depends on how application uses TAS Platform |
| References: | See [TP13], [TP14] |
| Processing: | TAS Platform is responsible or the following functions: Scheduling of CAR, EP and ELEKTRA applications. Communication over remote message queues provided by CS-remote. Permanent check of the hardware to detect failures (see also chapter 5.19.2). |
| Data: | Configuration files will define: The network address of the hardware. The ELEKTRA applications that have to be started on the hardware. The necessary remote message queues. |

[CORE-2.x-SW:SAC:0196], [CORE-2.x-HW:SAC:0020], [CORE-2.x-HW:SAC:0198]:

The following table shows the validated hardware that will be used by ELEKTRA. “EC/EC-MUX” means that both applications are running on the same hardware.

|  | MEN-D2 | MEN-D6 | MEN-D10 | SCC | SC33 | EPCM |
| --- | --- | --- | --- | --- | --- | --- |
| CCA, CCB | X | X | X | -- | -- | -- |
| DGP | x | x | x | -- | -- | -- |
| ECA, ECB | X | X | X | -- | -- | -- |
| RCA, RCB | X | X | X | -- | -- | -- |
| SCU | X | X | X | -- | -- | -- |
| TCD | X | X | X | -- | -- | -- |
| IAU | -- | X | X | -- | -- | -- |
| EC-MUX | -- | X | X | -- | -- | -- |
| EC/EC-MUX | -- | -- | X | -- | -- | -- |
| OEC | -- | -- | -- | X | X | X |

Table 19: HW boards of TAS-PLF to be used by ELEKTRA applications

TAS Platform Core Software shall directly run on this hardware.

Virtualisation is restricted to the test environment used by developers of ELEKTRA and to dedicated tests in the lab.

[CORE-2.x-HW:SAC:0170], [CORE-2.x-HW:SAC:0173]:

Programmed memory cards shall be refreshed in regular intervals. The refresh utility of TAS-PLF will be used by ELEKTRA and ECC. The first refresh cycle will be after 8h and afterwards a refresh cycle will occur every 365 days.

### Hardware Tests during Startup and Operation

| Type: | Tests of hardware during startup and operation of TAS-PLF |
| --- | --- |
| Purpose: | DPR:0224 (TRT316): Flash card failure SAR:0404 (TRT242): Inversion-function of CPU SAR:0405 (TRT269): Clock-Test SAR:1213 (TRT278): Control of selfchecks [CORE-2.x-SW:SAC:0097]: Neighbourhood check for startup-test [CORE-2.x-SW:SAC:0098]: Safety reaction associated to delayed success of on-line tests [CORE-2.x-SW:SAC:0194]: Restricted settings in field mode [CORE-2.x-SW:SAC:0192]: Non-“elf” text segments shall be tested by the application for data integrity [CORE-2.x-SW:SAC:0177]: Check of safety relevant data during start-up [CORE-2.x-SW:SAC:0092]: Limitations while using permanent storage. [CORE-2.x-SW:SAC:0173]: MD5 Files for protected partitions or image files to be produced off-line. |
| Function: | During startup of TAS-PLF the startup tests have to be executed. During operation the runtime tests of TAS-PLF have to be executed. Every application has to control if the selfchecks are performed in time. Each executable generated by the ELEKTRA build environment shall include the offline generated MD5 signature. |
| Subordinates: | The tests must be turned on. |
| Dependencies: | The hardware tests are part of TAS-PLF |
| Interfaces: | /proc/selfcheck contains the output of the tests (valid for TAS-PLF 1.x) /var/diag/olt/status (valid for TAS-PLF 2.x) |
| Resources: | — |
| References: | See [TP_PLF1_OLT] (TAS-PLF 1.x) or [TP_PLF2_OLT] (TAS-PLF 2.x) |
| Processing: | First the startup tests are executed during booting of TAS-PLF. If they succeed, the runtime tests are started. The results of these tests have to be read by the application to set the necessary reaction: - NOINFO: ok, has to be replaced by FINISHED within the first cycle, otherwise shutdown - RUNNING: ok, no reaction (TAS-PLF 2.x only) - FINISHED: ok, no reaction - DELAYED nnn: nnn is the delay in seconds; ok, no reaction (TAS-PLF 1.x only) - DELAYED_DONE: ok, no reaction (TAS-PLF 1.x only) - ERROR: leads to shutdown in field version, except for disk test - DISABLED: leads to shutdown in field version - TERMINATED: leads to shutdown in field version (TAS-PLF 1.x only) - UNTESTED: leads to shutdown in field version (TAS-PLF 1.x only) If the number of loops is not increased within a period of 24 hours, this leads to shutdown in field version (except Disk-Error occurred). Each individual test must be finished within the cycle time. The cycle time of the runtime tests has the following value: - TAS-PLF 1.x: 24 h (default of TAS-PLF) - TAS-PLF 2.1.3x (PV GEN 2.17.x): 24 h (default of TAS-PLF) - TAS-PLF 2.4.x (PV GEN 2.19.x): 20 h (reduced by ELEKTRA) A further test compares the system clock and the clock chip. A failure concerning the flash card shall not lead to an immediate shutdown. After the next shutdown, the restart of application shall be prevented. Details about the handling of a failure of the flash card are described in chapter 2.3.1 of [EL_GEN_FUNC]. Additionally the application checks the checksums of the ROM and CMOS relations of the application database. A failure leads to a shutdown. The same behaviour is valid for RAM relations that have a checksum configured. The integrity of the “elf” text segments is also checked by the runtime tests. The test provides a list of all “elf” files checked. The list shall be sent to DGP periodically to be able to see which files are part of the test. [CORE-2.x-SW:SAC:0194]: The following settings of TAS-PLF shall not be used by ELEKTRA in field mode: - LAB_MODE (OLT, CS/FT and CS/NMR) - NO_STARTUP_TESTS. (OLT, CS/FT and CS/NMR) - NO_ONLINE_TESTS (OLT and CS/NMR) The default value 'no' must be set for those settings. [CORE-2.x-SW:SAC:0192]: Executables built by the GNUCHILL compiler or C compiler will include the MD5 signature provided by TAS-PLF. This is ensured by the build environment of ELEKTRA. A list of checked files is provided by the runtime tests. This list is sent periodically to DGP as diagnosis messages. [CORE-2.x-SW:SAC:0177]: Configuration data files will be protected by MD5 checksum. The checksum will generated offline. In case configuration data have to modified at the target, the old checksum will be checked by the program that modifies the data. The same program will also generate the MD5 checksum for the modified version of the data. The protection of the CAR database is described in chapter 5.1.3 “CAR on TAS Platform” (see explanation of [CAR:FMEA:RULE:10] for details). TCD resp IAU will not use MD5 checksums for their configuration data files. TCD resp. IAU will execute plausibility checks of the configuration data (e.g. value must be within defined range). Otherwise invalid data will be detected by the communication partner. [CORE-2.x-SW:SAC:0092], [CORE-2.x-SW:SAC:0173]: An ELEKTRA flashcard will have four partitions: Partition 1 will contain TAS-PLF; mounted readonly; protected by OLT using MD5 sum. Partition 2 will contain the checksum files for partition 1 and 3; mounted readonly. Partition 3 will contain ELEKTRA executables, configuration files and database files; protected by OLT using MD5 sum. Partition 4 will provide space for reset stable syslog; mounted writeable. Valid only for GenSys >= 2.19.10: Partition 4 contains the Security Configuration Overlay, which consistency is verified by TAS PLF scripts before integration into the overlay file system during system startup (see MEMO Security Configuration Handling TAS Platform 2.4.1, [SECOVL]). MD5 files for protected partitions – partition 1, 3 and (for GenSys >= 2.19.10) partition 4 – will be generated off-line as part of the ELEKTRA build process. ELEKTRA will also use the RAM disk provided by TAS-PLF. Some of the configuration files for CS and EP will be copied from partition 3 to the RAM-disk and modified according to the actual network definitions. These modified files will also be protected by a MD5 checksum, which will be checked by cs when reading the files. |
| Data: | /proc/selfcheck (for TAS-PLF 1.x) and /var/diag/olt/status (for TAS-PLF 2.x) contain the result of the TAS-PLF tests. The records of ROM- and CMOS-relations of the database have checksums. |

### Skip memory test after reboot (Quickstart – only TAS-PLF 1.x)

| Type: | Tests of hardware during startup of TAS-PLF |
| --- | --- |
| Purpose: | PRR:0319 (TRT162): Boot time [CORE-2.x-SW:SAC:0097]: Neighbourhood check for startup-test [CORE-2.x-SW:SAC:0098]: Safety reaction associated to delayed success of on-line tests |
| Function: | Boot of the system has to be performed within 180 seconds. Boot time is the time interval between system reset and operation (AFG, WIN) The Quickstart feature provided with TAS Platform 1.10 is an optional feature that allows to significantly reduce the duration of the system startup. This is mainly accomplished by an incremental progression of the DRAM memory test. Each time a memory slice has been successfully tested the OLT sets a secured progress marker in a non-volatile memory location. When the system is powered down and restarted the most recent memory slice is retested at once and after the application has been re-established, the memory test continues to test the next slice in background testing mode. Depending of the power down time the quick start library calculates if “a new full start is necessary” or not. In the Elektra System the Quick Start function is only needed for System Reboots or short power down situations (SW replacement). Therefore the Elektra System uses for saving the relevant data for calculating “new full start necessary” in a buffered memory location (/mem). The buffering of this memory location is limited to max. 48h (see [EL34]). After this time the stored data (additionally protected by an 16Bit CRC) isn’t valid and a Reboot with full start is initiated. The Elektra System uses for checking “new full start necessary” the real time clock. The real time clock is also buffered and not guaranteed running in a power down phase. After a power down phase without buffering of the real time clock, the real time clock is set by the platform to the default value 1.1.1970. Therefore after booting with quick start, it is checked if the actual time is younger than the stored time. If this happens the real time clock is not correct and a full start is initiated. With the above mentioned measures a quick start without a following full start is only possible for a down time < 48h and this time is extremely shorter than the allowed down time calculated by the quick start library (>14d, see [EL34]), it is also a full start initiated if the real time clock increases, therefore the SAC:PLF:137 hasn’t any effect for the Elektra System. |
| Subordinates: | — |
| Dependencies: | — |
| Interfaces: | TAS Platform application: The application can get the status of the OLT by reading the “/proc/selfcheck/” entries. TAS Platform <-- application The application can initiate a full startup test the next time the system is powered up without instant reboot by writing the token “lock” to “/proc/qsctrl”. |
| Resources: | — |
| References: | [TP_PLF1_UG_OLT] Chapter “User library”, [TP_PLF1_QS_OLT] |
| Processing: |  |
| Data: | /sysmnt/sbin/boot.env Environment variable OLT_QUICKSTART=yes |

This function is only supported by TAS Platform 1.10 and TAS Platform 1.11.

### Scheduling Policy and Priority used by ELEKTRA

This chapter is based on EL-SYS-14-060_MM_EMV-test-SEC-Signal_140507.doc “Meeting Minutes, Festlegung der Elektra Prozess Prioritäten mit Plf 2.x”.

It provides definition to fulfill [CORE-2.x-SW:SAC:0188] Priority of application threads.

#### FIFO versus RR

SCHED_FIFO (first-in-first-out) and SCHED_RR (round-robin) are the realtime scheduling classes in Linux. The difference is that SCHED_FIFO threads are not pre-empted by other threads with the same priority but run as long as they want to. SCHED_RR threads will only run for a certain timeslice until pre-empted by other SCHED_RR threads with the same priority.

Non-realtime threads are marked with SCHED_OTHER and are scheduled after SCHED_FIFO and SCHED_RR threads. Within the scheduling classes (SCHED_FIFO, SCHED_RR and SCHED_OTHER) threads are scheduled according to their priority.

In TAS-PLF 1.x the timeslice of scheduling policy RR (“Round Robin”) has a maximum length of 100ms.

In TAS-PLF 2.x the length of the timeslice of scheduling policy RR is not known.

Definition: The application processes of ELEKTRA must use scheduling policy RR. By using RR no supervision processes of TAS-PLF will be influenced.

RR has advantages in case of problems of an ELEKTRA application: if one application channel blocks the other channel still is able to execute.

#### Usage in ELEKTRA

For the application the following priorities are allowed:

RR or FIFO: priority 11 to 69 (range defined in pthread_priority.h). This range is defined by TAS-PLF and may be changed from one version to another version. Therefore only relative ranges shall be used based on min value and max value.

At the time writing this chapter TAS-PLF provides an available range of min+58. The calculated value must be checked against the max value of TAS-PLF. When exceeding this value a diagnosis must be generated followed by an exit.

Explanation “Plf 2.x Schedule Class”:

TS … Scheduling Other

FF ... First in First out

RR … Round Robin

Explanation “Set / Inherited”:

S … Set

I … Inherited

I  S … must be changed from Inherited to Set

| Process Application Handler | Thread | Set / Inherited | Plf 2.x Scheduling Policy | Plf 2.x Priority | Plf 1.x Scheduling Policy | Plf1.x Priority |
| --- | --- | --- | --- | --- | --- | --- |
| User Interface | Several serial |  | TS | - |  |  |
| cs |  |  | FF | - |  |  |
| inetd |  |  | TS |  |  |  |
| syslog-ng, logwriter |  |  | TS |  |  |  |
| Start bash |  | S | RR | Min + 5 |  |  |
| olt_sup |  | I | RR | Min + 5 |  |  |
| lwatch | 1 thread per application, all with same priority | I | RR | Min + 5 |  |  |
| car | Main | I | RR | Min + 6 |  |  |
| car | Switch TTY thread | I | RR | Min + 6 |  |  |
| car | Timeout thread Interface | S | RR | Min + 30 |  |  |
| car | Timeout thread | S | RR | Min + 30 |  |  |
| busexe | Main | S | RR | Min + 21 |  |  |
|  | Scan thread per TTP segment, all with same priority | S | RR | Min + 20 |  |  |
|  | Write tread per TTP segment all with same priority | S | RR | Min + 20 |  |  |
| checkappl |  | S | RR | Min + 2 |  |  |
| onermt | Process per application (bash) | I | RR | Min + 5 |  |  |
| EP pca(b)-iq_2_1 | Main thread | I | RR | Min + 5 |  |  |
| EP pca(b)-iq_2_1 | Queue threads | S | FF | Min + 6 |  |  |
| olt_hdl_2_1 | Handler application | I | RR | Min + 5 |  |  |
| pcb_fbi_2_1 | Application thread | I | RR | Min + 5 |  |  |
| pcb_fbi_2_1 | Scan thread | S | RR | Min + 18 |  |  |
| pca_appl_2_1 | Application (cca, pca/b, …) | I | RR | Min + 5 |  |  |
| CCB |  | S | RR | Min + 4 |  |  |
| OCS |  | S | RR | Min + 15 |  |  |
| CMOS Server |  | I | RR | Min + 5 |  |  |
| CMOS Client |  | I | RR | Min + 5 |  |  |
| Further Handlers |  | I | RR | Min + 5 |  |  |
| TCD/tcm | tcd_tcm_main | S | RR | Min + 21 | RR | 150 |
|  | tcd_tcm_hexd | S | RR | Min + 16 | RR | 128+10 |
|  | tcd_tcm_sclk | S | RR | Min + 17 | RR | 128+11 |
|  | tcd_tcm_proc | S | RR | Min + 6 | RR | 128+0 |
|  | tcd_tcm_mutex | S | RR | Min + 15 | RR | 128+9 |
|  | tcd_tcm_idle | S | RR | Min + 18 | RR | 128+12 |
|  | tcd_tcm_load | S | RR | Min + 19 | RR | 128+13 |
|  | tcd_tcm_syslog | S | RR | Min + 6 | RR | 128+0 |
| TCD/tei | tcd_tei_main | S | RR | Min + 21 | RR | 150 |
|  | tcd_tei_el_recv | S | RR | Min + 7 | RR | 128+1 |
|  | tcd_tei_el_diag | S | RR | Min + 7 | RR | 128+1 |
|  | tcd_tei_el_lc | S | RR | Min + 11 | RR | 128+5 |
|  | tcd_tei_tcc_recv | S | RR | Min + 7 | RR | 128+1 |
| TCD/tcc | tcd_tcc_main | S | RR | Min + 21 | RR | 150 |
|  | tcd_tcc_ctrl | S | RR | Min + 10 | RR | 128+4 |
|  | tcd_tcc_user_recv | S | RR | Min + 10 | RR | 128+4 |
|  | tcd_tcc_user_send | S | RR | Min + 10 | RR | 128+4 |
|  | tcd_tcc_line_recv | S | RR | Min + 9 | RR | 128+3 |
|  | tcd_tcc_line_send | S | RR | Min + 9 | RR | 128+3 |
|  | tcd_tcc_line_lc | S | RR | Min + 11 | RR | 128+5 |
| TCD/iau/iaa | tcd_iaa_main | S | RR | Min + 21 | RR | 150 |
|  | tcd_iaa_il_recv | S | RR | Min + 7 | RR | 128+1 |
|  | tcd_iaa_il_diag | S | RR | Min + 7 | RR | 128+1 |
|  | tcd_iaa_rbc_lstat | S | RR | Min + 7 | RR | 128+1 |
|  | tcd_iaa_rbc_timer | S | RR | Min + 11 | RR | 128+5 |
|  | tcd_iaa_rbc_recv | S | RR | Min + 8 | RR | 128+2 |
|  | tcd_iaa_rbc_send | S | RR | Min + 8 | RR | 128+2 |
| TCD/iau/iab | tcd_iab_main | S | RR | Min + 21 | RR | 150 |
|  | tcd_iab_il_recv | S | RR | Min + 7 | RR | 128+1 |
|  | tcd_iab_il_diag | S | RR | Min + 7 | RR | 128+1 |
|  | tcd_iab_rbc_lstat | S | RR | Min + 7 | RR | 128+1 |
|  | tcd_iab_rbc_recv | S | RR | Min + 8 | RR | 128+2 |
|  | tcd_iab_rbc_send | S | RR | Min + 8 | RR | 128+2 |
| TCD/iau/mux | tcd_mux_main | S | RR | Min + 8 | RR | 130 |
|  | alecom_ale_mux | S | RR | Min + 8 | RR | 128+2 |
|  | alecom_ale_ping | S | RR | Min | RR | 128-64 |
|  | alecom_line_send | S | RR | Min + 8 | RR | 128+2 |
|  | alecom_line_send | S | RR | Min + 8 | RR | 128+2 |
|  | alecom_line_recv | S | RR | Min + 8 | RR | 128+2 |
|  | alecom_line_recv | S | RR | Min + 8 | RR | 128+2 |
|  | reader_thread | S | RR | Min + 8 | RR | 130 |
|  | ers_reader_thr | I S | RR | Min + 6 | RR | 128 |
|  | to_thread | S | RR | Min + 10 | RR | 132 |

Table 20: ELEKTRA scheduling policy and priority

Remark to inetd:

The process inetd is a demon that is used for many services (e.g. telnet, ftp, rdate). If ELEKTRA keeps the CPU busy, inetd can possibly not be scheduled because of its low scheduling class TS. This scheduling class is intended by TAS-PLF and may not be changed by the application. This can lead to the unavailability of the services serviced by inetd.

### Device mapping

TAS-PLF provides a device mapper for serial and Ethernet devices. This device mapper ensures stable device names for the application even if the operating system changes the device names because of a broken device.

The following tables show the mapping based on their position on the related HW board.

Asynchronous serial devices:

| TAS-PLF name, position on board | Name based on mapping |
| --- | --- |
| ttyT6 | aserg |
| ttyT7 | aserh |
| ttyT4 | asere |
| ttyT5 | aserf |
| ttyT2 | aserc |
| ttyT3 | aserd |
| ttyT0 | asera |
| ttyT1 | aserb |

Synchronous serial devices:

| TAS-PLF name, position on board | Name based on mapping |
| --- | --- |
| sync6 | sserg |
| sync7 | sserh |
| sync4 | ssere |
| sync5 | sserf |
| sync2 | sserc |
| sync3 | sserd |
| sync0 | ssera |
| sync1 | sserb |

TTP devices:

| TAS-PLF name, position on board | Name based on mapping |
| --- | --- |
| fbd3 | ttp3 |
| fbd2 | ttp2 |
| fbd1 | ttp1 |
| fbd0 | ttp0 |

CAN devices:

| TAS-PLF name, position on board | Name based on mapping |
| --- | --- |
| rcan3 | --- |
| rcan2 | can0 |
| rcan1 | --- |
| rcan0 | --- |

Ethernet devices D2 motherboard:

| Numbered position on board on front | Name based on mapping | Position on board not on front | Name based on mapping |
| --- | --- | --- | --- |
| 1 | lan2 | 4 | eth3 |
| 2 | lan1 |  |  |
| 3 | eth0 |  |  |

Ethernet devices D2 additional board (Quad Ethernet board):

| Numbered position on board on front | Name based on mapping | Position on board on front | Name based on mapping |
| --- | --- | --- | --- |
| 1 | lan12 |  |  |
| 2 | lan13 | 5 | --- |
| 3 | lan14 | 6 | --- |
| 4 | lan15 |  |  |

Ethernet devices D6 motherboard, D10 motherboard, QEMU:

| Numbered position on board on front | Name based on mapping |
| --- | --- |
| 2 | lan2 |
| 3 | lan12 |
| 0 | eth0 |
| 1 | Lan1 |
| 4 | lan13 |
| 5 | lan14 |

## Operating System Linux (Distribution defined by TAS-PLF)

| Type: | Use the Linux distribution as operating system that is supported by TAS Platform |
| --- | --- |
| Purpose: | FUR:0119 (TRT20): CS (Communication System) on operating system 'Linux' FUR:0123 (TRT22): Multi-threading support FUR:0124 (TRT23): Shared-memory access FUR:0125 (TRT24): Support of timer interrupts FUR:0126 (TRT25): TCP/IP and Ethernet support GCR:0162 (TRT107): Use Intel-based Linux-PC for ELEKTRA MMI |
| Function: | CS (with all functions like within TAS platform) must be available for the operating system 'Linux'. The chosen operating system 'Linux' must support multi-threading. The chosen operating system 'Linux' must support shared-memory access. The chosen operating system 'Linux' must support (system-)timer interrupts. The chosen operating system 'Linux' must support TCP/IP over Ethernet. ELEKTRA MMI must be an Intel-based personal computer with the operating system 'Linux'. The following applications will use Linux as their operating system: VCA (ELEKTRA SBB), EBO2 (ELEKTRA ÖBB and ELEKTRA INTERNATIONAL). |
| Subordinates: | Hardware supported by Linux |
| Dependencies: | — |
| Interfaces: | — |
| Resources: | Depends on how application uses Linux |
| References: | - |
| Processing: | Linux is responsible or the following functions: Scheduling of CAR, EP and ELEKTRA applications. Communication over remote message queues provides by CS-remote. |
| Data: | Configuration files will define: The network address of the hardware. The ELEKTRA applications that have to be started on the hardware. The necessary remote message queues. |

## Outdoor-Element controlling with EEC (SEA/SEC/NIC)

The outdoor-element controlling is described in Architectural Design Document SEA, ELEKTRA 2.x ÖBB [SE_ADD_SEA] and System Subsystem Design Description SEC Software Schweiz [SE_SSDD_SEC_CH].

The EC element processes, allocated in the outdoor system, receive signal aspect commands via an indoor multiplexer (called EC-MUX) from the CC and send their responses back the same way. Thus, the translation from a signal aspect to a set of light point state vector is done within the outdoor system.

The interface between the CC and the conventional EC respective the EC-MUX is the same, except the reporting of the Day/Night-Status where the CC must send updates to every EC-MUX.

Also, the interface to the DGP has to be extended, because the diagnostic information, SEA has to report, is different from that of EC. These changes have to be implemented country specific (for OBB see Detailed Design Document, Integration SEA, ELEKTRA ÖBB [SE_DDD_IntegrationSEA]).

The communication to the dedicated outdoor elements (light points, train protection elements like INDUSI, etc.) is done using a CAN bus. The CAN bus is connected to the OEC (Outdoor Element Controller) and to the OIFs (Outdoor Interfaces). These can be interfaces on an interface hardware board (e.g. for INDUSI, Balise Driver or even interfaces controlling conventional light points or IOs) or elements on the mast connected directly over the CAN bus (e.g. iLED). Each OIF is equipped with two CPUs for two channels running diverse software. The OEC sends commands in terms of “light on”, “light off”, and so on, which are interpreted by the channels and applied to the hardware in a safe manner (featuring kind of a “master / slave” principle by using a mutual supervision principle of both channels).

A further variant of EEC is NIC. NIC shall work as replacement of existing RIC. Therefore, NIC has to provide an interface to the following external systems:

- Block system Sbl 60A (Austria and Hungary)

- Block system ZG62 (Austria and Hungary)

- Block system MAV (Hungary)

- Agreement (DE: “Zustimmung”; Austria)

The first version is NIC-HU. It provides the interface to block system MAV. NIC-HU is described in “SSDD, ELEKTRA 2.x with DCA elements” [EEC_SSDD_EL_DCA] and “SDD NIC-HU - Neighbor Interface Controller Hungary” [EEC_SDD_NIC_HU].

The different types of EEC are described by Configuration Overview: EEC, SEC/SEA, NIC, ICU [EEC_CFG_OV].

## Use Voting to detect inconsistencies between channel A and channel B

Voting is used to detect an inconsistency between a CHILL signal received from channel A and a CHILL signal received from channel B for the same command or the same state. If the receiver also consists of a channel A and a channel B, voting is done independently in both channels.

Examples for possible reasons for an inconsistency:

- HW error on the interface to an outdoor element that affects only one channel.

- Design or implementation error in one SW channel.

- Delay in communication that affects only one channel.

ELEKTRA has the following types of voters:

Event triggered voter (“ereignisgesteuerter Voter”):	
This voter type ensures, that a signal is coming from channel A and B.	
Reaction in case a signal is received only from one channel: it either is ignored (diagnosis message) or leads to a safety reaction.

State triggered voter (“zustandsgesteuerter Voter“):	
This voter type stores states of an element for channel A and B and generates a resulting state depending on the design definition; if an inconsistency lasts longer than the timeout defined by the design a predefined resulting state is generated (normally KAZ – “keine aktuellen Zustandsdaten” no actual state data).

List of voters needed in ELEKTRA:

- Each application layer has at least one input voter:

- CC:

- Command-Voter (for signals from MMI/EBO2 and RCU): event triggered voter.

- Message-Voter (signals from EC and RCU): state triggered voter.

- Voter for handling the 2 redundancies of one channel of a LDI connection (signals from TCD ELEKTRA Switzerland to CCA or CCB): event triggered voter.

- MMI/EBO2:

- Display-Voter (signals from CC and RCU): state triggered voter.

- RCU:

- Elektra-Voter (signals from CC): state triggered voter.

- Voter for 2-channel connection to external partner for incoming messages (signals from external to RCU): event triggered voter.

- EC:

- Instruction-Voter (signals from CC): event triggered voter.

- CCA and CCB also have output voter:

- Display-Comparator (signals to VC and RCU): state triggered voter, detection of inconsistency leads to a safety reaction.

- Instruction/Message-Handler (signals to PC and RCU): event triggered voter, detection of inconsistency leads to a safety reaction.

## Use OCS SBJ to provide RaSTA protocol for external communication

| Type: | Extend ELEKTRA by protocol RaSTA |
| --- | --- |
| Purpose: | FUR:1578 (TRT1433): TAS-PLF OCS SBJ communication framework PRR:4906 (TRT1436): External RCU communication by using TAS-PLF OCS SBJ communication framework TIR:6700 (TRT1435): Hybrid System - ELEKTRA 2 components on ELEKTRA 1 |
| Function: | - Use OCS SBJ in RCU to implement the RaSTA protocol for ELEKTRA OBB. - RaSTA protocol is used to replace the serial X.25 communication. - To be able to use RaSTA protocol in ELEKTRA1 a hybrid version of ELEKTRA1 is necessary: replace ELEKTRA 1.x RCU by ELEKTRA 2.x RCU (HW and SW). Additionally ELEKTRA 1.x DGP is replaced by ELEKTRA 2.x DGP (HW and SW). |
| Subordinates: | None |
| Dependencies: | - XCEL available (see chapter 5.1.2 “XCEL: Binding to ELEKTRA 1.x”) - Usage of serial HDLC links by XCEL available |
| Interfaces: | - VOTRICS interface - SCU ELEKTRA1 interface |
| Resources: | None |
| References: | ELEKTRA architecture: see [EL_MIX_EL1_RAS] |
| Processing: | RaSTA needs configuration file to setup communication. A special configuration for CS/EP is necessary for hybrid version of ELEKTRA1 for RCU and DGP. This new configuration replaces the part of the VOTRICS configuration for RCU and DGP. The additional option “vni” must be used to connect an ELEKTRA2 route definition to the according ELEKTRA1 route definition. The communication between the ELEKTRA2 components RCU and DGP uses the redundant LAN of ELEKTRA2. The communication RCU-SCU, RCU-CCA/CCB and DGP-CCA/CCB must use serial links for communication. The serial links used are part of the configuration. |
| Data: | - RaSTA configuration - Route configuration |

## Use DGP/SCWS for external Log Collector and NTP server connectivity

In accordance with [CORE-2.x-SW:SAC:0026], the external log server and NTP time source shall be connected to the DGP board for ELEKTRA 2.x (ELSEC:SRS:REQ:0018 and ELSEC:SRS:REQ:0024), and not connected to the ELEKTRA internal network.

SCWS functionality uses only one HW board for all required applications (CC, RCU and DGP), the connectivity to the external log server and NTP time source shall be configured to use a separate LAN interface (one of LAN 12, 13, 14, 15) which is not used for safety critical communication.

## Use of TCDs for establishing connection between local CC/DGP and remote ECs.

| Type: | Extend ELEKTRA 2.x SBB by category 2 connections for remote EC |
| --- | --- |
| Purpose: | FUR:6110 (1655) Verbindung CC - EC über TCD: Anzahl der Verbindungen je "local" TCD FUR:6111 (1656) Verbindung CC - EC über TCD: Anzahl der Verbindungen je "remote" TCD FUR:6112 (1657) Verbindung CC - EC über TCD: Gesamtanzahl der TCD-Verbindungen FUR:6113 (1658) Verbindung CC - EC über TCD: LAN-Diagnose |
| Function: | - Use category 2 connections to connect remote ECs to the local ELEKTRA 2.x system via local/remote TCD-TCD setup. |
| Subordinates: | - |
| Dependencies: | - ELEKTRA 2.x station system has to be D10 |
| Interfaces: | - ELEKTRA 2.x D10 TCD |
| Resources: | - |
| References: | System Requirements Specification, SBB, Elektra Evolution, Chapter 4.1.13 VERBINDUNG EC - CC ÜBER TCD (61.2) [EL_SSS_SBB] |
| Processing: | TCD and CS/EP-Routing needs configuration files to setup communication. In the TCD configuration every remote EC(s) connection has to be setup accordingly. The current generic CS/EP configuration has to be replaced with a unique configuration tailored to the using station and its local/remote TCDs/ECs infrastructure. The TCD-TCD connection is using just a single logical TCD connection per TCD. The limitations are: - Every local TCD has a max. of 4 connections (remote TCD and/or LDI). - Every remote TCD has a max. of 4 EC and a max. of 3 ECMUX connections. - Every interlocking has a max. of 5 LDI and 4 remote EC connections. |
| Data: | - TCD configuration - CS/EP-Route configuration |

# Taking account of SRACs and SecRACs of underlying systems in the design of ELEKTRA

## SRACs TAS-PLF

The SRACs of TAS-PLF are described in “Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform Core - 2.x” [TP_Core2x_SACs]. The following sub-chapters are based on a DOORS export of the according SRAC database.

### SRACs TAS-PLF CORE-SW

#### Identification of the DOORS databases

| DOORS/RMF Version: | Client 9.7.0.0 – Server 9.7.0.0 / RMF 6.1.0.9 |
| --- | --- |
| DOORS/RMF Module Name: | ELEKTRA PV GEN_PLF SW_EoC |
| DOORS/RMF Module Version (Baseline): | 1.2 |
| DOORS/RMF Module Path: | /SAC-DB AT/Generic Components/Thales-AT VIE/ |
| DOORS/RMF Export View: | SAE WEXP Export View Extended |
| DOORS/RMF Current Filter: | ((WEXP Bookmark is not empty) OR (((NOT (Column 'SAC Status' contains 'obsolete' (case sensitive))) AND (NOT (Column 'SAC Status' contains 'deleted' (case sensitive)))) AND ((((Column 'System Architecture' contains 'non redundant ELEKTRA' (case sensitive)) OR (Column 'System Architecture' contains 'non redundant SB' (case sensitive))) OR (Column 'System Architecture' contains 'non redundant' (case sensitive))) OR (Column 'System Architecture' contains 'red_and_non-red' (case sensitive))))) |

Table 21: DOORS EoC module identification of TAS-PLF CORE-SW

| DOORS/RMF Version: | Client 9.7.0.0 – Server 9.7.0.0 / RMF 6.1.0.9 |
| --- | --- |
| DOORS/RMF Module Name: | TAS-PLF_Core_2.x_System_SACs |
| DOORS/RMF Module Version (Baseline): | 8.0 (PLF 2.4.2) |
| DOORS/RMF Module Path: | /SAC-DB AT/Generic Components/TS-DE Stuttgart/ |
| DOORS/RMF Export View: | SAE WEXP Export View Extended |
| DOORS/RMF Current Filter: | ((((((system architecure includes non redundant SB) OR (system architecure includes non redundant ELEKTRA)) OR (system architecure includes non redundant)) OR (system architecure includes red_and_non-red)) AND (TA SAC Status != deleted)) AND (TA SAC Status != obsolete)) AND (IE PUID is not empty) |

Table 22: DOORS SRAC module identification of TAS-PLF CORE-SW

#### Implementation of the conditions

| [CORE-2.x-SW:SAC:0008] | Choice of a safe programming model. |
| --- | --- |
| SAC-Text: The application has to choose the appropriate programming model. For safety critical TaskSets, programming model 1 shall be used. If model 3 TaskSets are used, the application shall provide a full safety concept. |  |
| Status: active Architecture: red_and_non-red | Comment: Detailed information is given by 'Programmers Guide - TAS Platform Core SW - 2.x, 3CR 00055 5141 PCAPA'. |
| Implementation in ELEKTRA: | ELEKTRA will use model 3 TaskSets – see chapter 5.19.1 "Use TAS Platform including its hardware". |

| [CORE-2.x-SW:SAC:0151] | Compliance to restrictions of Programmers Guide |
| --- | --- |
| SAC-Text: The programmers guide (3CR 00055 5141 PCAPA) defines a list of allowed API calls per Model (1 or 3) in the tables “POSIX System Calls - Model compliance” and “Non POSIX System Calls – Model compliance”. These tables shall be observed. |  |
| Status: active Architecture: red_and_non-red | Comment: None. |
| Implementation in ELEKTRA: | The tables shall be handled in the same way as coding rules. The restrictions shall be checked in the verification step "Static Source Code Analysis" (Verification Plan - ELEKTRA 2.x - Static Source Code Analysis [EL_VP_SSCA]). |

| [CORE-2.x-SW:SAC:0026] | Limitation of TAS Platform with regard to security |
| --- | --- |
| SAC-Text: The TAS PLF safety case covers the safe communication within category 1 and 2 networks according to [EN 50159] Note that category 1 or 2 Networks can be coupled by secure transport mechanism as described in [HB SEC]. TAS PLF supports applications to implement several security properties, which have to be adjusted according to the specific application’s and customer’s security needs. |  |
| Status: active Architecture: red_and_non-red | Comment: Refer to [EN 50159] for the definition of closed and open networks used in this SAC. See the TAS Platform security handbook [HB SEC] regarding features and advices. Be sure that the security properties do not corrupt safety and availability properties. E.g. a strong encryption will enhance security but has an impact to processor load and therefore to real-time properties which might violate [CORE-2.x-SW:SAC:0180]. |
| Implementation in ELEKTRA: | Network for ELEKTRA internal communication: this network will fulfill only category 1 according to [EN 50159] (see chapter Limitation of TAS Platform with regard to security "General functionality"). Without further measures this restrictions also applies to "remote control new" of ELEKTRA International, as the communication EBO2 – CC uses the ELEKTRA internal communication. Network for communication of ELEKTRA with an external communication partner: The interface specification or the design document must explicitely define the category according to [EN 50159]. External communication partners are: • All communication partners connected to DGP via Ethernet (e.g. WebRide, new WebRide of HITACHI Switzerland including ESM replacement, ...) • All communication partners connected to RCU/SCU via Ethernet (e.g. connection to another ELEKTRA, …) • All communication partners connected to TCD or IAU via Ethernet External communication partners are not connected to the ELEKTRA internal network. They are connected to additional Ethernet interfaces available on the HW boards of DGP, SCU, TCD and IAU. |

| [CORE-2.x-SW:SAC:0172] | QoS Priority usage |
| --- | --- |
| SAC-Text: Application development teams shall define the priority level for all network communication based on policies for usage of priority levels on IP level. All CS and other SIL-4 communication shall use highest priority. Except for SIL-4 communication, all other elements connected to the network, shall use lower priority. The underlying network infrastructure must be capable of handling QoS. |  |
| Status: active Architecture: red_and_non-red | Comment: Although QoS should provide higher reliability for transmission, there is no guarantee that Basic Integrity or NON-SIL components (e.g. pre-existing components like the OSS service FTP) on the same network do not send packets with high priority that may have influence to the available bandwidth. Applications have to prove that the bandwidth influence of the component will not decrease availability of the whole system. Communication via the network is treated as “grey-Channel”. SIL-4 Modules (like CS/FT, CS/NMR and OCS) must issue a safety reaction in case of communication disturbance. |
| Implementation in ELEKTRA: | ELEKTRA will neither use nor define QoS for its internal network. ELEKTRA will use end-to-end supervision of the communication instead to ensure that the internal network behaves as expected: • Life cycle (see chapter 5.2.1 "General functionality" and [EL_SYS_ADM] chapter 3.4.5 "Periodical Supervision of Connections" + chapter 3.4.13 "Detection of Life Cycle between Busexe respectively FBI and EC") • High system load (see chapter 5.15 "Detection of high system load (“Hochlasterkennung”)" and [EL_SYS_ADM] chapter 3.4.12 "Detection of high system load, regulation of application load") • Reactivity supervision (see chapter 5.18 "Supervision of reactivity" and [EL_SYS_ADM] chapter 3.4.17 "Monitoring of reactivity of ELEKTRA system components – TAS-PLF 1.x" + chapter 3.4.18 "Monitoring of reactivity of ELEKTRA system components – TAS-PLF 2.x"). • Voting of incoming information in case this information must be provided by a channel A and a channel B signal. The voting shall ensure that the information contained in both signals is identical (see chapter 5.22 "Use Voting to detect inconsistencies between channel A and channel B" to get a short overview). If one of these supervisions detetcts a slow or missing communication, an appropriate reaction is defined. The definition for scheduling strategy and priority within ELEKTRA is defined in chapter 5.21 "Scheduling Policy and Priority used by ELEKTRA" (see also [CORE-2.x-SW:SAC:0188]). |

| [CORE-2.x-SW:SAC:0096] | Absence of tolerance to human errors |
| --- | --- |
| SAC-Text: In the field, maintenance professionals shall not interact with the services provided by TAS PLF Core unless evidence is given that human errors during those interactions cannot lead to a hazard. Especially, but not only, human-made parameters shall not feed the TAS PLF Core API, unless evidence is given that no human error can lead to a hazard. Similarly configurations, settings and files shall not be altered. |  |
| Status: active Architecture: red_and_non-red | Comment: Evidence might be achieved by encapsulating interactions with humans in a dedicated layer with a precisely specified protocol. This protocol shall be defined in such a way, that hazardous influence is excluded - independently of human errors. |
| Implementation in ELEKTRA: | ELEKTRA will not provide an interface for the customer to get access to TAS-PLF Core services. Further restrictions will be described in maintenance documents. These restrictions shall include the following ones: • It is forbidden to connect anything to the COM1 interface of an ELEKTRA computer. • It is forbidden to connect an additional device to the ELEKTRA internal LAN. |

| [CORE-2.x-SW:SAC:0157] | Avoiding erroneous combinations of TAS Platform Core versions and configuration data |
| --- | --- |
| SAC-Text: The application, TAS Platform Hardware, Software and configuration must be compatible to each other. This can be ensured by validating the combinations. The compatibility matrix is the reference for the compatibility of TAS Platform components. |  |
| Status: active Architecture: red_and_non-red | Comment: See Compatibility Matrix 3CR 00055 9637 DFAFA. |
| Implementation in ELEKTRA: | ELEKTRA will use the templates provided by TAS-PLF for configuration data. ELEKTRA will choose SW HW combinations according to the compatibilty matrix of TAS-PLF [COMP]. The compatibility between TAS-PLF and ELEKTRA will be ensured by configuration management of ELEKTRA. |

| [CORE-2.x-SW:SAC:0114] | Neighbourhood and version checks |
| --- | --- |
| SAC-Text: Neighbours shall check each other for matching software types and versions (Application and TAS Platform) Neighbours are the CEs within a CN or CG, and CNs or CGs that communicate with each other. Care must be taken to detect following faults: a) Upgrade of a CG, CN or CE with a wrong software (type) b) Upgrade of a CG, CN or CE with a wrong software (version) |  |
| Status: active Architecture: red_and_non-red | Comment: About a) and b): Upgrading the majority of the CEs in a complete system (2oo3, 2x2oo2) with flash cards containing wrong software leads to an operating CN or CG with a different software or software version, which can only be detected by the neighbour CN or CG. About a): Upgrading a CE with the flash-card of a different product type leads to its exclusion from the CN because, at CE boot, the TAS Platform layer exchanges its version as well as CRC signature of the text segments of all model 1 TaskSets. b) wrong CE is covered by TAS Platform for model 1 TaskSets, as it votes at point X the CRC signature of their text segments (if .notextcrc is set to 0). For non-model 1 TaskSets provided by the application, a vote of the system version might help. |
| Implementation in ELEKTRA: | The version check within ELEKTRA includes the TAS-PLF version implicitly, because the TAS-PLF version is part of the ELEKTRA baseline. ELEKTRA SW will not implement an explicit check to ensure the usage of the correct TAS-PLF. The compatibility between TAS-PLF and ELEKTRA will be ensured by configuration management of ELEKTRA through the definition of the ELEKTRA baseline. The same strategy will also be used for CAR and EP (resp. XEP). ELEKTRA2 will reuse the version check implemented already in ELEKTRA1. This version check ensures the compatibility between the different applications and the compatibility to the station specific data (see [EL_SYS_ADM] chapter 3.4.6 "Version Control"). |

| [CORE-2.x-SW:SAC:0196] | Check for validated hardware |
| --- | --- |
| SAC-Text: During startup the hardware (e.g. CPU frequency and number of active cores) has to be checked directly or indirectly for evidence that the hardware match the configuration it was validated. If the match fails, an appropriate safety reaction has to be set. |  |
| Status: active Architecture: non redundant | Comment: This SAC has become necessary, because there are now board variants that are software configuration dependent. Currently, for CIQ3, CIQ6, CU3, D10 and SC33, an operation with one or more cores is technically possible, for CIQ3, the core frequency can be different. In a redundant architecture, TAS Platform ensures via CS-FT or CS-NMR that all CEs in the CN are matching (equal). In a non-redundant architecture, the application has to take over this task. |
| Implementation in ELEKTRA: | ELEKTRA will use validated HW (for details see chapter 5.19.1 "Use TAS Platform including its hardware" Table 19). The restrictions concerning the mix of different HW boards will be defined by the baseline or by the product management. |

| [CORE-2.x-SW:SAC:0191] | Data download via network |
| --- | --- |
| SAC-Text: Data download via network is allowed under following preconditions: 1. Data integrity must be ensured (e.g. using MD5 checks). 2. Consistency (correct version, all necessary files available,...) must be ensured. The user shall prepare and apply a concept for the download. This concept shall define procedures and provide safety measures and prove that those are correct and sufficient to ensure the integrity and consistency of the data. |  |
| Status: active Architecture: red_and_non-red | Comment: This SAC replaces CORE-2.x-SW:SAC:190 and applies to data download in general. This is: 1. booting via network without on-board mass storage device. 2. booting from mass storage device and loading additional data or software from network. These can be config files, application executable code and data, operating system. 3. updating the content or part of the content of the local mass storage via network. Since the network download can directly affect availability, the download concept shall also include availability measures and protection against erroneous use of this feature. OLT Disktest: If a mass storage device is equipped, the OLT disk test is enabled and tests the disk but not the data downloaded from network. Note, that a permanent check of the data on the server (like the disk test on CF cards) is not safety relevant, if all safety aspects (data integrity and consistency) are checked at boot time. To detect data corruption on the server earlier, a permanent check on the server is recommended. Using the TAS Platform Maintenance/Download service (MNT 2.x) covers all these safety aspects. Please note that according to [EN 50128], only validated configurations (hardware, executable and configuration data) are allowed for safety relevant operation when using the “Exchangeable OS Partition” feature (FL: PLF-2.x:FL:824) introduced with TAS Platform 2.4.0. |
| Implementation in ELEKTRA: | ELEKTRA will not provide download via network. Exception: EEC will use the download mechanism provided by TAS-PLF (see [SE_ADD_SW_GEN]). |

| [CORE-2.x-SW:SAC:0158] | Handling components with different Software Integrity Levels. |
| --- | --- |
| SAC-Text: The requirement regarding components of different software safety integrity levels according to EN50128 shall be fulfilled (see 7.3.4.9 in EN50128:2011) “Where the software consists of components of different software safety integrity levels then all of the software components shall be treated as belonging to the highest software safety integrity level unless there is evidence of independence between the higher software safety integrity level components and the lower software safety integrity level components. This evidence shall be recorded in the Software Architecture Specification.” |  |
| Status: active Architecture: red_and_non-red | Comment: One possible solution is to separate components with different safety integrity levels into different processes This provides a level of independence between pieces of software. Other measures may be developed by applications. Influence of software on TAS Platform, if encapsulated in a TaskSet or POSIX process, is covered by the TAS Platform’s architecture (memory protection). Please note that this SAC especially (but not only) applies to diagnostic, monitoring or other non safety relevant software components, e.g. SNMP, NTP. |
| Implementation in ELEKTRA: | ELEKTRA will use different processes for the different applications. Channel A and channel B of an application like CC will run in different processes. The communication channels A and B of TCD resp. IAU will run in different processes too. There is no complete separation of functions/components with different safety integrity levels: • Diagnosis component DGP is separated from the other components • CCA contains SIL4 functions as well as basic integrity functions. For basic integtriy functions the same qualitative measures are applied as for SIL 4 functions. |

| [CORE-2.x-SW:SAC:0188] | Priority of application threads |
| --- | --- |
| SAC-Text: Application threads shall be executed with a priority >= AEOS_THREAD_USER_MIN_PRIO and priority <= AEOS_THREAD_USER_MAX_PRIO Threads with a priority below AEOS_THREAD_USER_MIN_PRIO have no influence on the system's realtime behavior (e.g syslogd, OLT). |  |
| Status: active Architecture: red_and_non-red | Comment: See also [GDE PRG] |
| Implementation in ELEKTRA: | The definition for scheduling strategy and priority within ELEKTRA is defined in chapter 5.19.4 "Scheduling Policy and Priority used by ELEKTRA" (see also [CORE-2.x-SW:SAC:0172]). |

| [CORE-2.x-SW:SAC:0085] | Safety concept for the non-redundant TAS Platform configuration |
| --- | --- |
| SAC-Text: In case of using the 1oo1 TAS PLF architecture the application shall provide and apply a safety concept enabling the usage of the non-redundant TAS Platform configuration for a safety critical application. This safety concept paper shall document the methods to achieve the required Safety Integrity Level (SIL). For instance, as for the non-redundant configurations CS is started with a non-redundant functionality instead of FT or NMR. CS can be started either -SIM or REMOTE for non-redundant operation. The safety concepts associated to FT or NMR cannot be reused: - fault detection and reaction by FM, including the insurance that a single fault causes no hazard, - hazard rate calculations in the Fault Tree Analysis - limitation of incarnations with the help of Q Values, - recovery. For the TAS Platform boards D10 and SC33, special care regarding the RAM-Test must be taken, since the test strategy and the safety argumentation is different for those boards, see [IVS_RAMTEST]. |  |
| Status: active Architecture: non redundant | Comment: To fulfill the safety concept for non-redundant applications, software diversity can be used. |
| Implementation in ELEKTRA: | ELEKTRA provides the document "Sicherheitskonzept ELEKTRA 2.x, All countries" [EL_SC]. |

| [CORE-2.x-SW:SAC:0079] | Compiler restrictions |
| --- | --- |
| SAC-Text: The TAS Platform provides a validated and assessed C and C++ compiler and an ADA compiler. The usage restrictions of the compilers are written in the compiler validation reports (see [RPT VAL GCC], [RPT VER GCC], [RPT VAL ADA]). Additionally, the TAS Platform provides coding rules for C and C++ which are based on the validation. Regarding coding rules for ADA, see the ADA Validation report ([RPT VAL ADA]). The restrictions and the coding rules are mandatory for application projects that use the validated TAS Platform compiler. An evidence how to observe the restrictions shall be provided by the application project. Programmers shall not rely on non documented behavior of compilers or other software components or tools. |  |
| Status: active Architecture: red_and_non-red | Comment: None. |
| Implementation in ELEKTRA: | ELEKTRA will use the C compiler. This will be ensured by the build environment of ELEKTRA. ELEKTRA will also use the C coding rules of TAS-PLF for C code. ELEKTRA will use the GNUCHILL compiler, the PAMELA pre-compiler. Coding rules for CHILL and PAMELA will be provided based on the restrictions of the tools. The coding rules must be part of the verification step "Static Source Code Analysis" (Verification Plan - ELEKTRA 2.x - Static Source Code Analysis [EL_VP_SSCA]). |

| [CORE-2.x-SW:SAC:0097] | Neighbourhood check for startup-test |
| --- | --- |
| SAC-Text: At start-up, each local application shall poll the /var/diag/olt/status file for evidence that each test has been run successfully since boot. Thus before entering the operational state it shall be checked by a neighbour application, that the global startup test state provided by /var/diag/olt/status has been set to “FINISHED”. If the state “FINISHED” was not reached the affected CE shall be shutdown or rebooted. |  |
| Status: active Architecture: non redundant | Comment: For redundant architecture, this SAC is covered by the Feature "OLT Supervision" implemented in FT starting with TAS Platform 2.1.3 and NMR. For non-redundant architecture a mechanism to cover this SAC has to be defined in the safety concept. |
| Implementation in ELEKTRA: | ELEKTRA will use 2 different implementations for checking the correct result of the startup test. These 2 implementations will run in different processes on the affected CE; they will be started at different points in time. If the state "FINISHED" will not be reached the affected CE will be rebooted. Further details see chapter 5.19.2 and chapter 5.19.3. |

| [CORE-2.x-SW:SAC:0098] | Safety reaction associated to delayed success of on-line tests |
| --- | --- |
| SAC-Text: During operation, the /var/diag/olt/status file of every CE has to be checked directly or indirectly for evidence that the global cycle counter “Cycle” has been incremented at least once within the OLT cycle time (OLT_CYCLETIME, maximum 24 hours). If the counter was not incremented during the OLT cycle time, an appropriate safety reaction has to be set |  |
| Status: active Architecture: non redundant | Comment: The check may be provided directly on the mentioned file or indirectly via providing the information to i.e. a communication partner. For redundant architecture, this SAC is covered by the Feature "OLT Supervision" implemented in FT starting with TAS Platform 2.1.3 and NMR. For non-redundant architecture, this SAC still apply. |
| Implementation in ELEKTRA: | ELEKTRA will use 2 different implementations for checking the correct result of the runtime test. These 2 implementations will run in different processes on the affected CE; they will be started at different points in time. One process will check the results every 3 hours and ensure that each individual runtime test has been executed at least once within 24 hours; the other process will check the results every 20 hours 30 minutes. If one of these processes detects a missing progress of one of the tests, a safety reaction will be set (see chapter 5.19.2 "Hardware Tests during Startup and Operation" and 5.19.3 "Skip memory test after reboot (Quickstart – only TAS-PLF 1.x)": • Disk test: set a marker to indicate the detected error of the disk test. The next safety reaction of the application will find this marker and therefore enforce a reboot of the CE. In case the startup test still detects the flashcard error it will switch the CE to single user mode and thereby inhibit the start of the application. • All other test: initiate a reboot of the CE in case the online test did not set its panic reaction. Both processes also check that the status of each individual runtime test indicates if it is still running. Otherwise the same safety reaction is set as in case of a missing progress. |

| [CORE-2.x-SW:SAC:0194] | Restricted settings in field mode |
| --- | --- |
| SAC-Text: TAS PLF provides global settings which are not allowed to be used in the field: - LAB_MODE (OLT, CS/FT and CS/NMR) - NO_STARTUP_TESTS. (OLT, CS/FT and CS/NMR) - NO_ONLINE_TESTS (OLT and CS/NMR) For field usage, the default value 'no' must be set for those settings. |  |
| Status: active Architecture: red_and_non-red | Comment: The default value ensures field mode setting. Refer to Configuration Reference Manual - TAS Platform Core SW - 2.x, 3CR 00055 5142 PCAPA' and ‘Setup Guide TAS Platform Core SW - 2.x 3CR 00055 5145 PCAPA for further information. |
| Implementation in ELEKTRA: | ELEKTRA will use the default value 'no' for these global settings (see chapter 5.19.2 "Hardware Tests during Startup and Operation"). Otherwise the checks for the correct values of the startup tests will fail and will cause a safety reaction. |

| [CORE-2.x-SW:SAC:0192] | Non-“elf” text segments shall be tested by the application for data integrity |
| --- | --- |
| SAC-Text: Text or constant segments of safety relevant TaskSets which are not built as “elf” file including the offline generated MD5 signature have to be checked by the application for data integrity at startup (before usage). |  |
| Status: active Architecture: red_and_non-red | Comment: According to CORE-2.x-SW:SAC:073, this data has to be checked every FDT in memory. The OLT text segment tests only check text segments including the MD5 signature, all other files generated with a different file format are not checked. Starting with TAS PLF Core 2.1.2 this SAC was introduced to allow execution of java *.jar files, which are ignored by OLT and therefore do not lead to a safety reaction when faulty. C and C++ files compiled by the TAS Platform compiler to elf-files still include the MD5 signature within the image itself and are handled by the OLT text segment test. The structure of all other files (e.g. Java byte code files, or other data files) loaded into process memory are not known to OLT and therefore cannot be checked by OLT. |
| Implementation in ELEKTRA: | Executables built by the GNUCHILL compiler or C compiler will include the MD5 signature provided by TAS-PLF (see chapter 5.19.2 "Hardware Tests during Startup and Operation"). J4S provides a similar test that is used by the java application of EEC. |

| [CORE-2.x-SW:SAC:0073] | Periodic check of Safety relevant data |
| --- | --- |
| SAC-Text: The integrity of safety critical data shall be checked at start-up and every fault detection time. For a redundant architecture, this means that all pieces of dynamic safety critical data shall be voted between the independent replicas at least once within 24 hours. For non-redundant architecture, the integrity has to be checked in another way. |  |
| Status: active Architecture: red_and_non-red | Comment: Static safety critical data, which are defined as 'const' are included in the textsegment test. OLT cycle time is fixed configured to 24 hours. For the 2oo2, 2oo3 and Nx2oo2 architectures the TAS PLF voting mechanism (send data into a voted message queue) can be used. In case of 2oo3 (CS/FT) the replica is excluded, in case of 2oo2 (CS/FT) and Nx2oo2 (CS/NMR) the CN is brought into safe state. See also CORE-2.x-SW:SAC:177 for the check at startup. To fullfil this SAC, redundant applications can use the CS/FT and CS/NMR mechanism to compare memory regions (sobjects) between the CEs. |
| Implementation in ELEKTRA: | The code will be protected by MD5 signature and checked by textsegment test (see [CORE-2.x-SW:SAC:0192]). Permanent HW faults causing changes of ELEKTRA RAM data will detected by DRAM test. This is true for MEN-D2 and MEN-D6; for MEN-D10 another argumentation has to be provided. Transient errors of RAM data will be detected by usage of these data. In case of rarely used outdoor elements the detection can last up to one year. The protection of the CAR database is described in chapter 5.1.3 "CAR on TAS Platform" (see explanation of [CAR:FMEA:RULE:10] for details). HW faults will cause different results in the two channels of ELEKTRA because of the diversity in implementation and diversity in data representation. Additional measures for TCD/IAU: each TDC resp. IAU channel will be executed as individual process. One process will handle the messages of ELEKTRA’s channel A; another process will handle the messages of ELEKTRA’s channel B. TCD resp. IAU will add data to protect the transportation of the message to the receiver. This protection will be done without storing or processing the message. TCD: The comparison and error detection of the received data will be done by the neighbour of TCD. IAU: The data will be secured and processed in a divers manner and they will be voted periodically. |

| [CORE-2.x-SW:SAC:0177] | Check of safety relevant data during start-up |
| --- | --- |
| SAC-Text: Static safety relevant data shall be declared as 'const' where possible to ensure that these data are checked by the text segment OLT - both at start-up and every fault detection time. For static safety critical data, which cannot be declared as 'const' - for instance configuration data - one of the following measures shall be implemented by the applications, after reading from the flash card: a.) the application shall implement a safety code (for instance MD5) check with an offline generated safety code after reading the data. Any failing check shall trigger a safety reaction. or b.) the application shall perform two diverse read functions to the flash memory and compare the diverse read data AND syntactic and semantic checks shall be performed for these data by the application. Any failing check shall trigger a safety reaction. |  |
| Status: active Architecture: red_and_non-red | Comment: See also CORE-2.x-SW:SAC:073 for periodic checks. Since PLF Core 2.0.1 libcdb provides an MD5 check feature that fulfills this requirement. |
| Implementation in ELEKTRA: | Configuration data will be protected and checked (see chapter 5.19.2 "Hardware Tests during Startup and Operation"). |

| [CORE-2.x-SW:SAC:0018] | Check of TaskSets. |
| --- | --- |
| SAC-Text: After start-up, the application has to check whether all required TaskSets and tasks are running and whether the configured message queues have been created successfully. Checking the return values of the process, thread and message queue creation functions is necessary, additionally the application has to judge if all TaskSets are ready for operation. If one item is missing or in a wrong state a safety reaction shall be triggered. |  |
| Status: active Architecture: red_and_non-red | Comment: CS checks if all tasks of a TaskSet are up and running. |
| Implementation in ELEKTRA: | ELEKTRA will use end-to-end supervision on application layer using life cycles. A missing queue or a missing TaskSet will be detected by a missing life cycle (see chapter 5.2.1 "General functionality" and [EL_SYS_ADM] chapter 3.4.5 "Periodical Supervision of Connections". |

| [CORE-2.x-SW:SAC:0180] | Adherence to realtime constraints |
| --- | --- |
| SAC-Text: The applications shall meet their realtime constraints concerning the reaction time to other components (PLF or not PLF) according to their needs. Safety related reaction times shall be tested under all load conditions. In the case of a violation of the realtime behavior a safety reaction has to be performed. |  |
| Status: active Architecture: red_and_non-red | Comment: Note that realtime supervision cannot be implemented by Model 1 TaskSets using the CLOCK_SYNCHRONIZED or the CLOCK_SYNCMONO. For the definition and priorization of TaskSets the application shall observe the following TAS PLF Core behaviour: - POSIX timers are designed not to expire before the specified timeout. - The timeout event may be delivered later if: a.) a higher priority process is running or runnable or b.) a SCHED_FIFO process of the same priority is already running when the timeout expires. In case of multiple application on one CN, the supervision methods of MAPS can be enabled to help that the applications do not unintentionally interfear with each other. Special care has to be taken that diagnostic actions or logging information do not affect the required realtime behaviour although our timing supervision like TaskSet Watchdog, Message ageing or timeliness supervision (see the chapter “Realtime supervision” in the TAS Platform Page: 45 Safety Application Conditions - TAS Platform Core - 2.x 3CR 00055 5070 DUAFQ document) Validation has to ensure that all diagnostic actions or logging information enabled during safety relevant operation does not stress the system over the limits. |
| Implementation in ELEKTRA: | ELEKTRA will supervise its realtime behaviour with the following mechanisms: • Reactivity Supervision (see chapter 5.18 "Supervision of reactivity") • Timedrift Supervision (see chapter 5.15 "Detection of high system load (“Hochlasterkennung”)") • Voting checks the realtime behaviour implicitly (see chapter 5.22 "Use Voting to detect inconsistencies between channel A and channel B") |

| [CORE-2.x-SW:SAC:0115] | Only relative timers are allowed for use in safety relevant applications |
| --- | --- |
| SAC-Text: Safety related applications shall only use relative timeouts. |  |
| Status: active Architecture: red_and_non-red | Comment: Relative timeouts are built based on a clock, that is independent of realtime clocks and so they are not influenced from setting the realtime clock by external time sources. |
| Implementation in ELEKTRA: | ELEKTRA will use the following mechanisms for measuring the time: • Time provided by CAR API • Mode TIME and related function provided by GNUCHILL • CLOCK_MONOTONIC CLOCK_REALTIME may only be used for diagnosis purposes. The usage of CLOCK_REALTIME for measuring time is forbidden in ELEKTRA, as this time is allowed to be changed by the user of ELEKTRA. In case of using C functions based on real-time only (e.g. sem_timedwait) additional measures must be defined in the design of the related functionality. |

| [CORE-2.x-SW:SAC:0109] | Restrictions about the single user mode |
| --- | --- |
| SAC-Text: In case the single-user mode is entered, the following restrictions shall be observed: - the use of the console is forbidden with on-line machines. - a defect board shall only be repaired at the factory. - a board shall not be reinserted in a network / put back to operations, unless evidence is given, that the board is free of faults. |  |
| Status: active Architecture: red_and_non-red | Comment: None. |
| Implementation in ELEKTRA: | The usage of the console is forbidden for field usage in ELEKTRA. The maintenance manuals of ELEKTRA will describe the necessary actions that have to be taken in case a board does not become fully operational. |

| [CORE-2.x-SW:SAC:0092] | Limitations while using permanent storage. |
| --- | --- |
| SAC-Text: Model 1 TaskSets shall be influenced by permanent data or executables stored on protected partitions or protected image files only. The information derived from dynamically writable permanent storage shall not influence their safety critical decisions unless a safety concept is provided. For writable partitions or image files on flash memory, data can be cached. Caution shall be taken to ensure that data read really comes from the permanent medium. |  |
| Status: active Architecture: red_and_non-red | Comment: In case of file access after point X the application has to follow the rules for recovery, e.g. a file open() after point X is not recovered. |
| Implementation in ELEKTRA: | ELEKTRA will use model 3 TaskSets. ELEKTRA will use flashcard and RAM disk as permanent storage (see chapter 5.19.2 "Hardware Tests during Startup and Operation"). |

| [CORE-2.x-SW:SAC:0112] | boot configuration |
| --- | --- |
| SAC-Text: The boot level in the NVRAM (set via /usr/bin/nvtool) shall be configured to operative. The boot level configured in /etc/inittab (initdefault) shall not be configured to operative. In general, scripts (including boot and shutdown scripts) are under responsibility of the application. |  |
| Status: active Architecture: red_and_non-red | Comment: For maintenance actions, the boot level in NVRAM can be set to maintenance. The boot level in /etc/inittab is only used if a fault in NVRAM has occurred. |
| Implementation in ELEKTRA: | The boot levels supported by ELEKTRA are defined in chapter 5.19.1 "Use TAS Platform including its hardware". |

| [CORE-2.x-SW:SAC:0123] | Tracing tools allowed for lab usage only |
| --- | --- |
| SAC-Text: The OS tracing and debugging tools shall not be used in field. This includes ftrace, strace, debug-kernel and gdb. The CS trace mechanism is developed for field operation. |  |
| Status: active Architecture: red_and_non-red | Comment: TAS Platform 2.1.4 introduces the CS trace feature which was developed according to SIL4 and is allowed to be used in the field. Please note that CORE-2.x-SW:SAC:180 has to be applied to all features including the chosen tracing feature. The syslog mechanism is allowed for field operation but no safety relevant decisions may be derived from the data |
| Implementation in ELEKTRA: | The tracing and debugging tools will not be used in field. |

| [CORE-2.x-SW:SAC:0173] | MD5 Files for protected partitions or image files to be produced off-line. |
| --- | --- |
| SAC-Text: MD5 Files for protected partitions or image files to be produced off-line on the validated development environment. |  |
| Status: active Architecture: red_and_non-red | Comment: Using maintenance services for downloading software upgrades forces an off-line production of MD5 files. When using the TAS Platform data partitioning concept based on Image files (introduced with TAS Platform 2.4.0) the safety relevant image files shall be treated like safety relevant, protected disk partitions in the conventional disk partition concept. The tool "disksign" (online md5 calculation for a partition) for a specific partition can be used if there are other measures that safety relevant data is checked via offline generated safety codes. |
| Implementation in ELEKTRA: | The following flashcard partitions will be protected with MD5 using off-line generated MD5 files: • Partition containing TAS-PLF • Partition containing the files of ELEKTRA The MD5 checksum is checked periodically (see chapter 5.19.2 "Hardware Tests during Startup and Operation"). |

| [CORE-2.x-SW:SAC:0182] | Non safe diagnosis data |
| --- | --- |
| SAC-Text: It is not permitted to use the following non-safe diagnosis data in safety relevant deduction: - Files /var/diag/*, /sys/class/net/eth0/*, /sys/class/hwmon/hwmon0/* - commands sensors, bmc - Syslog |  |
| Status: active Architecture: red_and_non-red | Comment: None. |
| Implementation in ELEKTRA: | The usage of non safe diagnosis data is defined in chapter 5.19.1 "Use TAS Platform including its hardware". |

| [CORE-2.x-SW:SAC:0184] | No impact of diagnosis on safety critical operation |
| --- | --- |
| SAC-Text: The diagnosis functionality shall be suitable only for outputting data and inputting non-safety-critical instructions to ensure that the safety critical operation cannot be affected. |  |
| Status: active Architecture: red_and_non-red | Comment: Please note, that the term “diagnosis functionality” applies to hardware and software-interfaces (e.g. SNMP) that were not developed according to SIL 4 as well as remote diagnosis capabilities like the “stream trace data” feature. |
| Implementation in ELEKTRA: | The usage of diagnosis functionality is defined in chapter 5.19.1 "Use TAS Platform including its hardware". |

| [CORE-2.x-SW:SAC:0197] | Safe digital I/O |
| --- | --- |
| SAC-Text: To ensure safety when using digital I/O, following hardware properties have to be applied: • at least 2 independent hardware channels (2oo2,2oo3 or similar construction) • for output, a hardware voter with diverse implementation (relays) to vote the output shall be used. • for input, at least 2 diverse input pins on different hardware channels shall be read and voted by software. (e.g. twisted pins on relays to detect systematic hardware faults.) • unambiguous identification of the hardware channels to be sure that correct I/O memory and I/O pin is used (mapped) • No software other than the desired SIL level should be used for setting or reading the I/O pins. Because the drivers delivered with TAS Platform software are classified Basic Integrity, application software shall access the pins at a very low level without driver. The recommended procedure is to map the physical address of the I/O port into process memory and access it directly with CPU memory read and write commands (read/write via pointer in C). The hardware address of the I/O pin (the address mapping) has to be set up during startup time, and treated as safety-critical data according to the respective TAS Platform SACs for safety critical static data. Measures have to be provided to be sure that a wrong address or mapping is not hazardous because the memory mapping can use Basic Integrity components. Based on the concept provided above, the application has to develop an appropriate safety concept for handling and checking I/Os in their system. If the application does not stick to this procedure, an own safety concept for safe-I/O has to be developed. |  |
| Status: active Architecture: red_and_non-red | Comment: From the platform point of view, there is no standard test to ensure that the I/O pin itself (hardware driver etc.) is working correctly. Therefore the safety relevant mission time of the hardware has to be applied when calculating hazard rates, or TAS-Platform external methods have to be used to test the I/O pins. For non-red a similar concept has to be applied. |
| Implementation in ELEKTRA: | All core system hardware boards provide a number of general purpose I/O pins (GPIO). When used correctly, those pins can be used to drive safety critical equipment. GPIO is not used by ELEKTRA. Additional information is provided in chapter 5.19.1 "Use TAS Platform including its hardware" in the section written for this SRAC. |

| [CORE-2.x-SW:SAC:0183] | No impact of diagnosis on real time properties |
| --- | --- |
| SAC-Text: The use (data transfer) of diagnosis interfaces (serial interface and external Ethernet interface if used as diagnosis interface) shall not impact the real-time properties of the system. |  |
| Status: active Architecture: red_and_non-red | Comment: This SAC covers also the serial to USB converted interface of the newer Core System hardware. |
| Implementation in ELEKTRA: | The impact of diagnosis is described in chapter 5.19.1 "Use TAS Platform including its hardware" in the section written for this SRAC. |

#### Template for DOORS export SRACs TAS-PLF CORE-SW

| <<SAC PUID including Bookmark>> | <<SAC Synopsis including Bookmark>> |
| --- | --- |
| SAC-Text: <<SAC Description>> |  |
| Status: <<SAC Status>> Architecture: <<System Architecture>> | Comment: <<SAC Comments>> |
| Implementation in ELEKTRA: | <<Object Text including DocRef>> |

### SRACs TAS-PLF CORE-HW

#### Identification of the DOORS databases

| DOORS/RMF Version: | Client 9.7.0.0 – Server 9.7.0.0 / RMF 6.1.0.9 |
| --- | --- |
| DOORS/RMF Module Name: | ELEKTRA PV GEN_PLF HW_EoC |
| DOORS/RMF Module Version (Baseline): | 1.1 |
| DOORS/RMF Module Path: | /SAC-DB AT/Generic Components/Thales-AT VIE/ |
| DOORS/RMF Export View: | SAE WEXP Export View Extended |
| DOORS/RMF Current Filter: | ((WEXP Bookmark is not empty) OR ((((NOT (Column 'SAC Status' contains 'deleted' (case sensitive))) AND (NOT (Column 'SAC Status' contains 'obsolete' (case sensitive)))) AND ((((((Column 'Class' contains 'SC33' (case sensitive)) OR (Column 'Class' contains 'SCC' (case sensitive))) OR (Column 'Class' contains 'EPCM' (case sensitive))) OR (Column 'Class' contains 'D10' (case sensitive))) OR (Column 'Class' contains 'D6' (case sensitive))) OR (Column 'Class' contains 'D2' (case sensitive)))) AND ((((Column 'System Architecture' contains 'red_and_non-red' (case sensitive)) OR (Column 'System Architecture' contains 'non redundant ELEKTRA' (case sensitive))) OR (Column 'System Architecture' contains 'non redundant SB' (case sensitive))) OR (Column 'System Architecture' contains 'non redundant' (case sensitive))))) |

Table 23: DOORS EoC module identification of TAS-PLF CORE-HW

| DOORS/RMF Version: | Client 9.7.0.0 – Server 9.7.0.0 / RMF 6.1.0.9 |
| --- | --- |
| DOORS/RMF Module Name: | TAS-PLF_Core_2.x_HW_SACs |
| DOORS/RMF Module Version (Baseline): | 9.0 (PLF 2.4.2) |
| DOORS/RMF Module Path: | /SAC-DB AT/Generic Components/TS-DE Stuttgart/ |
| DOORS/RMF Export View: | SAC_WEXP_Export_View |
| DOORS/RMF Current Filter: | ((((((system architecure includes non redundant SB) OR (system architecure includes non redundant ELEKTRA)) OR (system architecure includes non redundant)) OR (system architecure includes red_and_non-red)) AND (TA SAC Status != deleted)) AND (TA SAC Status != obsolete)) AND (IE PUID is not empty) |

Table 24: DOORS SRAC module identification of TAS-PLF CORE-HW

#### Implementation of the conditions

| [CORE-2.x-HW:SAC:0020] | Supported hardware configurations |
| --- | --- |
| SAC-Text: Only Core System hardware configurations defined in the respective TAS PLF Core 2.x Release Notes shall be used. To ensure the TAS-Platform safety case, hardware parts not provided by the Core System and used in the final system must not violate the safety concept and the applicable standards. |  |
| Status: active Architecture: red_and_non-red | Comment: The board firmware is part of the hardware configuration. CIQ3, EPCM, MCC, SCC, CU3, SC33: Power supply, backplane and sub-rack are not a part of the Core System and therefore not considered in the safety case. For the D10, the power supply is delivered by TAS Platform. The backplane and subrack are not yet available at the time of delivery of TAS Platform 2.4.2. For an overview of supported hardware configurations per TAS Platform version see also the Compatibility Matrix 3CR 00055 9637 DFAFA. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | ELEKTRA will use supported hardware configurations (for details see chapter 5.19.1 "Use TAS Platform including its hardware" Table 19; see also [CORE-2.x-SW:SAC:0196] and [CORE-2.x-SW:SAC:0157]). |

| [CORE-2.x-HW:SAC:0198] | No virtualization for safety relevant operation |
| --- | --- |
| SAC-Text: The TAS Platform Core Software shall run directly on the hardware configurations defined in the respective TAS PLF Core 2.x Release Notes. No intermediate layer (virtualisation) in any way is allowed since this might have impact that was not considered in the safety case. To ensure the TAS-Platform safety case, hardware parts not provided by the Core System and used in the final system must not violate the safety concept and the applicable standards. |  |
| Status: active Architecture: red_and_non-red | Comment: EN50128 requires that the software must be used on the hardware it was validated in the way it was validated. This means that an intermediate virtualisation layer is not allowed as long it is not validated and included in the safety case. For an overview of supported real or virtual hardware configurations per TAS Platform version see also the Compatibility Matrix 3CR 00055 9637 DFAFA Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | In context of ELEKTRA TAS Platform Core Software shall directly run on the hardware configuration defined for ELEKTRA (for details see chapter 5.19.1 "Use TAS Platform including its hardware" Table 19; see also [CORE-2.x-SW:SAC:0196] and [CORE-2.x-SW:SAC:0157]). |

| [CORE-2.x-HW:SAC:0011] | Hardware installation (D2, D6 and CIQ6 only) |
| --- | --- |
| SAC-Text: The following Core Systems have to be installed and used according to the respective documents: - D2: Core System D2 Guideline Application, 3CR 30911 0013 GAAFA - D6: Core System D6 Manual, 3JA 80001 AAAA GAAFA - CIQ6: Core System CIQ6 Manual: 7HA 10077 AAAA 108EN An upgrade of a Core System D2 to a Core System D6 has to be carried out according to the Upgrading Instruction 3JA 80001 AAAA RJAFA. An upgrade of a Core System D6 to a Core System CIQ6 has to be carried out according to the Upgrading Instruction 7HA 10077 AAAA 583EN |  |
| Status: active Architecture: red_and_non-red | Comment: No manuals exist for all other Core Systems, since they are not complete systems but contain just the CPU boards Complete systems like D2, D6 and CIQ6 contain fan, sub-rack, power supply in addition to the CPU board. For the D10, the power supply is delivered by TAS Platform. The backplane and subrack are not yet available at the time of delivery of TAS Platform 2.4.2. Class: CIQ6 D2 D6 |
| Implementation in ELEKTRA: | Cabinets for ELEKTRA and EEC will be designed according these documents and will be tested in a qualification test.ELEKTRA will use its own cabinets to fulfill the needs of ELEKTRA. Each cabinet type has to pass a qualification test. The result of the qualification test is available in a released test report. |

| [CORE-2.x-HW:SAC:0017] | The BIOS / U-BOOT software shall remain unchanged |
| --- | --- |
| SAC-Text: The BIOS / U-Boot and it's settings shall not be changed. |  |
| Status: active Architecture: red_and_non-red | Comment: EPCM: With this condition a setting of the jumpers JP1 and JP2 on the EPCM board is not allowed, as with the jumpers the BIOS setting is changed. The jumpers are only provided for debug purposes in the laboratory. Note that for the newer hardware boards (D10, SC33), the BIOS is protected by a hardware based trust mechanism. Class: CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | The BIOS will not be changed by ELEKTRA and EEC. |

| [CORE-2.x-HW:SAC:0172] | Download to (creation of) loading medium |
| --- | --- |
| SAC-Text: The user shall prepare a concept for the download of data into the loading medium (e.g. memory card, flash memory or on network). This concept shall define and prove that the chosen procedures and the provided safety measures against corruption of the software are correct and sufficient. For a download during safety-relevant operation an additional safety concept shall be prepared. |  |
| Status: active Architecture: red_and_non-red | Comment: Using the TAS Platform Maintenance/Download service (MNT 2.x) can help fulfilling this SAC. Please note that according to [EN50128], only validated configurations (hardware, executable and configuration data) are allowed for safety relevant operation when using the “OS-Partition upgrade” feature introduced with TAS Platform 2.4.0. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | ELEKTRA does not use download. Download used in EEC is described in "SSDD SEA SW GEN" [SE_ADD_SW_GEN]. |

| [CORE-2.x-HW:SAC:0008] | Components without fault detection measures |
| --- | --- |
| SAC-Text: For the following components TAS PLF Core 2.x does not provide fault detection measures: D2: - RTC and SRAM of RTC (both parts are buffered by an on board battery) - SRAM (buffering via external battery voltage interface possible) D6: - SRAM, Real Time Clock (RTC) of the Southbridge (both parts are buffered for a short time by an on board gold cap) EPCM: Real Time Clock (RTC) or the RTC CMOS SRAM of the Tolapai (buffering via external battery voltage interface possible, short time buffering by an on board gold cap) MCC and SCC: - Real Time Clock (RTC) - the Microcontroller MPC5121e internal SRAM CIQ6 and CIQ3: - Real Time Clock (RTC) CU3: - FPGA including bridges between CPU and FPGA part of Altera Cyclone V SoC - Real Time Clock (RTC) - TPM SC33 and D10 - Real Time Clock (RTC) - TPM Non-volatile memory (e.g.MRAM or FERAM) for all boards where applicable. In case of using any one of these components the application shall ensure the functional safety of the system. |  |
| Status: active Architecture: red_and_non-red | Comment: In case that applications use the on board battery buffering on D2, the battery shall be changed at least after 7 years. The battery is leakage protected. Note that the TAS Platform Q-Value makes use of the non-volatile memory. The fault detection measure of the Q-Value is implemented by TAS Platform by adding a safety code. To be sure that the application does not use the hardware components mentioned above, the application’s source code to access the component can be searched: e.g. for the RTC, a search for the /dev/rtc, /proc/driver/rtc or /sys/class/rtc can be performed. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | ELEKTRA and EEC will not need buffering of SRAM and RTC. |

| [CORE-2.x-HW:SAC:0014] | Usage of external interfaces |
| --- | --- |
| SAC-Text: If the application uses any external interface (Ethernet, CAN, MVB or Profibus if applicable), then the application shall ensure the channel separation and the absence of influence. The application shall also verify that the decoupling properties (500 V DC insulation) provided by TAS PLF Core 2.x are sufficient. A safe data transmission via these interfaces shall be compliant to the standard EN 50159. |  |
| Status: active Architecture: red_and_non-red | Comment: EPCM: A potential free voltage supply (5V CAN, GND CAN) is necessary, if insulation (500 VDC) is required on the CAN interface. The PCI-104 (EPCM) interface are internal interfaces to the interface boards (MVB and Profibus). EPCM: 1000 V DC on front Ethernet MCC: - Front Ethernet: 1000 V DC (700 V rms) - CAN 1700 V DC (1200 V rms) SCC: - Ethernet (front): 1000 V DC - CAN (rear): 500 V DC - CAN (on-board connector): 1700V DC (1200 V rms) CIQ6: - Ethernet 1190 VDC CIQ3: - Insulation (CAN and Ethernet): 1150 VDC / 815 Vrms CU3: - Insulation (Ethernet and I/O Pins) : 630 V AC (890 V DC) D10: - Insulation on Ethernet SC33: - Insulation in ETH (front/back), - CAN (back), - GPIO on front via daughterboard (AF77) Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | ELEKTRA internal communication via redundant LAN will be protected by end-to-end checks (see chapter 5.14 "End-to-End Checks"). ELEKTRA communication via HDLC or CAN will be protected as defined by the respective protocol used for communication. ELEKTRA communication via TTP bus will protected by CRCs. Channel A and channel B will use different CRCs. The protection will be implemented as end-to-end protection (see [EL_DDD_FBI]). The fulfillment by the HW of ELEKTRA will be shown by qualification tests. |

| [CORE-2.x-HW:SAC:0037] | Usage of the diagnosis interface on front for application specific purposes |
| --- | --- |
| SAC-Text: D2, D6, EPCM: The serial interface, COM1 on front, is defined as diagnosis interface. If this interface is used for any other application specific purposes, then the application shall ensure the functional safety of the system. Thereby it has to be considered, that if the COM1 port is used as operational I/O port, then this port shall be tested against conducted disturbances too. Other Core System hardware implementing a serial to USB converted interface on the front: The serial to USB converted interface on front is defined as diagnosis interface. If this interface is used for any other application specific purposes, then the application shall ensure the functional safety of the system. Thereby it has to be considered, that if this interface is used as operational I/O port, then this port shall be tested against conducted disturbances too. |  |
| Status: active Architecture: red_and_non-red | Comment: D2, D6, EPCM: The COM1 is defined as non operational I/O port (diagnosis). Therefore no tests against conducted disturbances have been carried out. Other Core System hardware implementing a serial to USB converted interface on the front: The serial to USB converted interface (as well as the serial interface on the rear connector) is defined as non operational I/O port (diagnosis). Therefore no tests against conducted disturbances have been carried out. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | ELEKTRA and EEC will not use the serial interface – COM1 on front – in field. |

| [CORE-2.x-HW:SAC:0009] | Additional interfaces without fault detection measures |
| --- | --- |
| SAC-Text: For the following interfaces TAS PLF Core 2.x does not provide fault detection measures: D2: - Keyboard/mouse interface - External battery voltage interface - PCMCIA interface for the connection of additional application specific hardware - Infrared interface - ISA bus - PC MIP interfaces (slots A and E) on the CPU board - PC MIP interfaces (slots E and F) on the Quad Ethernet board - JTAG connector on the Quad Ethernet board - All unused slots on the backplane (only three slots are connected: Slot 1 (CPU (master) and rear I/O (rear)), slot 2 (Quad Ethernet Controller) and the power supply slot.) - Following interfaces on the transition adapter (CT02): PC MIP, IDE1, IDE2, floppy disk, COM1, COM2, GPIO, USB, utility - All unused CPCI connections (the CPCI interface is used for the connection of the CPU with the transition adapter and the connection of the CPU with the Quad Ethernet Controller and the power supply. All other CPCI connections (including the hot bus, IPMB and PXI trigger lines) are not used). - ID-Interface: transition adapter + hex-switch (SW1, SW2) D6: - Universal Serial Bus (USB) - All unused slots on the backplane (only two slots are connected: Slot 1 (CPU (master) and rear I/O (rear)) and the power supply slot.) - Following interfaces on the rear adapter (CT04-01): COM1, COM2, USB and the additional switches (SW3-SW14) - Following interfaces on the transition adapter (CT02): PC MIP, IDE1, IDE2, floppy disk, COM1, COM2, GPIO, USB, utility - All unused CPCI connections (the CPCI interface is used for the connection of the CPU with the rear adapter (transition adapter) and the connection of the CPU with the power supply. All other CPCI connections (including the hot bus, IPMB and PXI trigger lines) are not used). - ID-Interface: transition adapter + hex-switch (SW1, SW2) or rear adapter (SW1, SW2) EPCM: - Universal Serial Bus (USB) on the internal connector or on the rear connector - Following connections on the rear connector: serial interface COM1, COM2, the 8-bit I/O bus, the external reset input, the external interrupt input or the PowerOK output - ID-Interface: 8-bit ID interface MCC: - Serial I2C interfaces on the rear connector - Serial RS232 interface on the rear connector - Battery voltage input for the RTC on the rear connector - Remote reset input (opto-coupler isolated) on the rear connector - Reset output (open drain) on the rear connector SCC: - I2C on the front - Program plug detection on the front - I2C interface at the rear connector - RS232 interface at the rear connector - Battery voltage input for the RTC at the rear connector - Remote reset input at the rear connector - POWEROK output at the rear connector - CAN interface at the on-board connector - GPIO interfaces (5 and 7) at the rear connector - ID-Interface: 8-bit ID interface CIQ6: - on-board I2C interface on the CIQ6 board - USB interface on the front (including over-current fault signal) - 56 bit I/O interfaces at the rear - battery voltage input for the RTC at the rear - CPCI interface (64 bit) at the rear - switch off/on input (CPCI_BD_SEL#) at the rear - remote reset input (CPCI_C_PBRST#) at the rear - serial interface at the rear - USB interface at the rear (including over-current fault signal) - additional slots on the backplane - additional power supply connectors on the backplane - additional power supply spring clips on the backplane - ID interface / ID bits (8-bit) CIQ3: - USB mass storage interface on-board - USB interface on the front - I2C interface on the front (for program plug including plug sense, plug drive and write protect) - I2C interface at the rear - RS232 interface at the rear - SPI interface at the rear - remote reset input at the rear - reset output (PowerOK) at the rear - battery voltage input for the RTC at the rear - 8-Bit ID interface CU3: - USB type A interfaces on the front - USB serial console via USB-B interface - RS-485 interface on the front - I/O interfaces at the rear - I2C interfaces at the rear - battery voltage input for the RTC at the rear In case of using any one of these interfaces the application shall ensure the functional safety of the system, in particular the independence of the computing channels shall not be cancelled by the equipment connected to these interfaces. |  |
| Status: active Architecture: red_and_non-red | Comment: The ID interface can be used to define the TAS PLF CE identification. For the CE identification functionality TAS PLF Core provides the measures to avoid hazards due to failures on the ID interface. On the Boards D2, D6 & EPCM, COM1 is wired to the front and to the back connector. Do not use the front and the back connection for COM1 simultaneously to prevent damage on the board. You can use COM2 on the rear connector independently of COM1. CIQ6: On the rear adapter CT04 48 bit I/O are connected to the switches SW3 - SW14. The additional 2 bit I/O (GPIO 4 and GPIO5) of the CIQ6 board are routed to the PCMIP connector, on the rear adapter which is not assembled. On the rear adapter CT02 the 50 bit I/O are connected to PC MIP connectors and are therefore not intended for use. At the Core System CIQ6 only two slots of the backplane are equipped. These are slot 1 (CPU (master) and rear I/O (rear)) and the power supply slot. All other slots and additional power supply connectors/spring clips are not used. The ID bits are configured via the hex switches (SW1 and SW2) on the rear adapter. Class: CIQ3 CIQ6 CU3 D2 D6 EPCM MCC SCC |
| Implementation in ELEKTRA: | ELEKTRA will use only the following interfaces from the list on D2 and D6: • ID interface: This interface will be used to define the TAS-PLF CE identification. • COM2 interface: This interface will be used for the fan supervision in case no EC is necessary for the station. The supervision is no real communication as the interface will be used only to read in the actual status of the fan supervision. • CPCI interface: This interface will be used to define the TAS-PLF CE identification. the connection to the D201/D203 carrier boards equipped with HDLC or TTP modules - D2 does also use the Quad Ethernet board over the CPCI, but this board is already approved by TAS Plf Core. |

| [CORE-2.x-HW:SAC:0199] | Additional interfaces without fault detection measures (D10, SC33) |
| --- | --- |
| SAC-Text: For the following interfaces TAS PLF Core 2.x does not provide fault detection measures: D10, SC33: - USB type A interfaces on the front - USB serial console via USB-B interface - I/O and serial interfaces at the rear D10: - 56bit I/O (= ID), - RS232 rear, - USB rear, - CPCI rear SC33: - battery voltage input for the RTC at the rear - rear: - ISA-like 8bit, SPI, I2C, CAN, 8bit ID, reset input and output, RS232, USB - front: - RS485, I2C, GPIO (option AF77) In case of using any one of these interfaces the application shall ensure the functional safety of the system, in particular the independence of the computing channels shall not be cancelled by the equipment connected to these interfaces. |  |
| Status: active Architecture: red_and_non-red | Comment: None. Class: D10 SC33 |
| Implementation in ELEKTRA: | ELEKTRA will use only the following interfaces from the list on D10: • I/O interface for ID interface: This interface will be used to define the TAS-PLF CE identification. • one serial interface: This interface will be used for the fan supervision in case no EC is necessary for the station. The supervision is no real communication as the interface will be used only to read in the actual status of the fan supervision. • CPCI interface: This interface will be used to define the TAS-PLF CE identification. the connection to the D203 carrier board equipped with HDLC or TTP modules and the power supply. |

| [CORE-2.x-HW:SAC:0010] | Additional functions without fault detection measures |
| --- | --- |
| SAC-Text: For the following functions TAS PLF Core 2.x does not provide fault detection measures: D2: - Floating point and MMX instructions - DMA (Direct Memory Access) - APIC (Advanced Programmable Interrupt Controller) - AGP (Accelerated Graphics Port) - Plug and play front end (ISA bus controller) - Standby voltage supervision D6: - Floating point and MMX instructions - DMA (Direct Memory Access) EPCM: Additional Tolapai features: - Floating point and MMX instructions - APIC (Advanced Programmable Interrupt Controller - DMA (Direct Memory Access) - EDMA (Enhanced Direct Memory Access) - HPET (High Precision Event Timers) - IEEE-1588 (timestamp unit) - permanent throttling - SMI (System Management Interrupt) - WDT (Watchdog Timer) MCC and SCC: Additional MPC5121e features: - Auxiliary Execution Engine(AXE) - Clock Frequency Measurement (CFM) - Direct Memory Access (DMA) - Floating point unit - IIM/Fusebox - Integrated Programmable Interrupt Controller (IPIC) - MBX graphics controller - Real Time Clock (RTC) - Power Management Control (PMC)/Power management (Sleep Modes) - Temperature sensor - Watchdog Timer (WDT) CIQ6 and CIQ3: Additional T1022 features: - Floating point unit - power management (core/device power management modes, software controlled dynamic frequency management), - Event Processing Unit (EPU) and Nexus Port Controller (NPC) - Multicore Programmable Interrupt Controller (MPIC) - Peripheral Access Management Unit (PAMU) - Security Monitor - Additional CPLD features: - board variant Additional I350 features: - thermal throttling CIQ3 Additionally: Ethernet Phy features: - power management Supervision Controller (SVC) features: - HW watchdog CU3: - floating point unit - power management - Board Management Controller (BMC) features: HW watchdog Additional ARM Cortex-A9 MPCore features: - Instructions: NEON, Thumb, Thumb2, MMX, and all other CPU features not tested (see 3CR 00055 6888 DUAPA Safety Concept - TAS Platform Core SW - 2.x - OLT for ARMv7-A architecture) In case of using any one of these additional functions the application shall ensure the functional safety of the system. |  |
| Status: active Architecture: red_and_non-red | Comment: MCC and SCC: The MPC5121e internal temperature sensor for the signals TEMP and T125 is disabled and not intended for use, as the sensor is too inaccurate. The MPC5121e internal Watchdog Timer (WDT) is disabled and not intended for use, as an MPC5121e external (clock independent) Watchdog Timer is available. The MPC5121e internal Real Time Clock (RTC) is not intended for use, as an MPC5121e external (clock independent) Real Time Clock is available. Power management is activated and used by TAS PLF. TAS PLF uses the Doze mode only. All other modes are not used by TAS PLF and not intended for use by the application. AXE, IMM, Fusebox and MBX are disabled and not intended for use. CIQ6, CIQ3: Regarding Interrupts see also Hardware Item Description of the respective board. If using such functions, either directly or indirectly via libraries (pre-existing or new) the application has to ensure that a malfunction has no impact on safety relevant operation. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | ELEKTRA will not use any of the mentioned functions for SIL4 functions. Floating point may be used for basic integrity functions like statistical calculations. |

| [CORE-2.x-HW:SAC:0200] | Additional functions without fault detection measures (D10, SC33) |
| --- | --- |
| SAC-Text: For the following functions TAS PLF Core 2.x does not provide fault detection measures: D10, SC33: - floating point unit - power management - Board Management Controller (BMC) features: HW watchdog - Additional Intel Apollo lake features that are not tested (see 3CR 00055 7116 BEAPA Safety Concept - TAS Platform Core SW - 2.x - OLT for Intel Atom processor E3900 series) In case of using any one of these additional functions the application shall ensure the functional safety of the system. |  |
| Status: active Architecture: red_and_non-red | Comment: If using such functions, either directly or indirectly via libraries (pre-existing or new) the application has to ensure that a malfunction has no impact on safety relevant operation. Class: D10 SC33 |
| Implementation in ELEKTRA: | ELEKTRA will not use any of the mentioned functions for SIL4 functions. Floating point may be used for basic integrity functions like statistical calculations. |

| [CORE-2.x-HW:SAC:0028] | Insulation coordination (D2 and D6 only) |
| --- | --- |
| SAC-Text: In case of using the Core Systems D2 and D6, the application shall ensure, that the in the Insulation Coordination specified classes, circuits and insulations are suitable and sufficient. Especially the insulation of the power supplies and the external Ethernet interfaces have to be considered. |  |
| Status: active Architecture: red_and_non-red | Comment: See Insulation Coordination 3CR 05500 AAAA QSAFC (D2) and 3JA 80001 AAAA BDAFQ (D6) Note that this SRAC is not applicable to the Core System HW CIQ6, D10 since the respective manuals of the core systems are a pre-requisite for a safe operation anyway as described in [SACDEF]. Class: D2 D6 |
| Implementation in ELEKTRA: | Cabinets for ELEKTRA and EEC will be designed according these documents and will be tested in a qualification test. |

| [CORE-2.x-HW:SAC:0174] | Power supply concept |
| --- | --- |
| SAC-Text: The computing elements shall be supplied with own (separate) voltages, to ensure the independence among the channels. Safe potential separation for both ground and voltages shall be ensured via the transformers of the power supplies. The value of the dielectric strength between the primary and the secondary side depends on the use of the system and shall be defined and proven by the application. |  |
| Status: active Architecture: red_and_non-red | Comment: The power supply is not a part of the Core System Hardware except D2, D6 and CIQ6. The power supply shall be considered in the safety case of the application. The fulfilment of the SAC regarding the separation for both ground and voltages can be verified by review of the architecture and the insulation coordination of the application. Class: CIQ3 CU3 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | Cabinets for ELEKTRA and EEC will be designed according this SRAC and will be tested in a qualification test. |

| [CORE-2.x-HW:SAC:0030] | Common mode influence of the power supplies |
| --- | --- |
| SAC-Text: A common mode influence of the power supplies via primary-side cross coupling shall be excluded by appropriate measures. |  |
| Status: active Architecture: red_and_non-red | Comment: This can be realized e.g. by use of filters and fuses. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | Cabinets for ELEKTRA and EEC will be designed according this SRAC and will be tested in a qualification test. |

| [CORE-2.x-HW:SAC:0029] | Extension of the power supply |
| --- | --- |
| SAC-Text: The user shall verify, that the decoupling properties of the power supplies are sufficient. The user is also responsible to ensure the functional safety of the system, especially the independence of items, if the - +5 V or +3.3 V (D2, D6 and CIQ6) - +5 V (EPCM, SC33) - +4,5 V ... 28 V (MCC) - +4,75 V ... 28 V (SCC) - +5 V ... 24 V (CIQ3) - 24V, 5V (CU3) - 3.3 V, 5 V, 12 V (D10) power supply is used for more than to supply one independent channel or if additional application-specific voltages (e.g. +12 V) are provided and used. |  |
| Status: active Architecture: red_and_non-red | Comment: Possible additional application-specific voltages: D2, D6 and CIQ6: +12 V and - 12 V Insulation of power supplies: Primary against secondary side: 2,500 VAC / 3,500 VDC Secondary side against shield: 500 VAC / 700 VDC Primary side against shield: 2,500 VAC / 3,500 VDC EPCM: +12 V and - 12 V MCC, SCC, D10, SC33: No additional application specific voltage inputs available The additional application specific voltages are not supervised, except the +12V on the D6. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | Cabinets for ELEKTRA and EEC will be designed according this SRAC and will be tested in a qualification test. |

| [CORE-2.x-HW:SAC:0021] | 230 V (AC) power supply 3CR 30604 ADAA (D6 only) |
| --- | --- |
| SAC-Text: To guarantee a failure free operation in case of using the Core System D6, additional measures on the primary side of the power supply 3CR 30604 ADAA are necessary, to ensure that the on the primary side of the power supply coupled energy-rich transients generate no disturbances on the secondary side (5 V DC), which exceed the following values: Symmetrical: Umax = 100 mV (Deviation of the supply voltage 5 V DC by readjustment in the power supply) Unsymmetrical: Ûmax = 3 V (Residual impulse secondary side conductor against case) |  |
| Status: active Architecture: red_and_non-red | Comment: This can be realized by the connection of appropriate filters. This SAC is a specific result of the Core System D6 qualification. Class: D6 |
| Implementation in ELEKTRA: | Cabinets for ELEKTRA and EEC will be designed according this SRAC and will be tested in a qualification test. |

| [CORE-2.x-HW:SAC:0015] | Protection against unauthorized access |
| --- | --- |
| SAC-Text: The system shall be protected against unauthorized access (e.g. by a lockable room, cabinet or casing). |  |
| Status: active Architecture: red_and_non-red | Comment: Note that this SAC is an additional hint to enforce at least the basic hardware security measures. Therefore this SAC is implicitly applicable for all hardware systems, and is no longer explicitly mentioned for new hardware (e.g. CIQ3). It will be removed in future TAS Platform versions. See also [SACDEF]. Class: CIQ6 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | The protection against unauthorized access shall be ensured by the customer. |

| [CORE-2.x-HW:SAC:0042] | Non safe diagnosis |
| --- | --- |
| SAC-Text: It is not permitted to use the non safe diagnosis data to deduce safety relevant activities. This applies to data - stored in the EEPROM for factory data - displayed by the LEDs or alphanumeric display - sent via the serial to USB converted interface or the external Ethernet interface (if used as diagnosis interface), - send via the interface between SVC or BMC and the CPU. |  |
| Status: active Architecture: red_and_non-red | Comment: MCC, SCC, CIQ3, CIQ6, CU3, D10, SC33 Diagnosis data: - reset reason provided by SVC/BMC - factory lifetime data stored in the EEPROM - signals displayed by the LEDs Diagnosis interfaces: - interface between SVC/BMC and CPU - serial to USB converted interface - external Ethernet interface (if used as diagnosis interface) CIQ6: Insulation (serial to USB converted interface): 1190 VDC CIQ3: Insulation (serial to USB converted interface): 860 VDC CU3: Insulation 630 V AC / 890 V DC (against the core and against each other) Console insulation provided on D6, MCC, SCC, CIQ6, CIQ3, CU3, D10, SC33 Other Core System Hardware Boards according to their documentation. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | ELEKTRA and EEC will not use these data for safety relevant decisions. LEDs and reset reason will be used for diagnosis purposes. |

| [CORE-2.x-HW:SAC:0025] | Suitably trained personnel |
| --- | --- |
| SAC-Text: Only suitably trained personnel shall carry out works like e.g. installation, commissioning and maintenance (e.g. exchange of defect boards) of a TAS PLF Core 2.x system. Only instructors with the required technical competence, professional experience or professional training shall carry out the training. The training shall deal with a description of the states of the system, diagnosis, fault search and handling, safety application conditions and the protection of persons. |  |
| Status: active Architecture: red_and_non-red | Comment: Note that this SAC is an additional hint to the basic safety measures of [EN 50126]. Therefore this SAC is implicitly applicable for all hardware systems, and is no longer explicitly mentioned for new hardware (e.g. CIQ3). It will be removed in future TAS Platform versions. See also [SACDEF]. Class: CIQ6 D2 D6 EPCM MCC SCC |
| Implementation in ELEKTRA: | The correct training must be ensured by HITACHI Austria; it is not part of the ELEKTRA development. The correct handling of the HW will be described in maintenance manuals. |

| [CORE-2.x-HW:SAC:0022] | Personal safety |
| --- | --- |
| SAC-Text: The TAS PLF Core HW requires special care about protection of persons, as there are dangerous voltages available. Therefore measures (e.g. safety tools, insulation pads, measures to switch off the power supplies, etc.) have to be taken to protect the personnel working with the system. Thereby specific national rules and standards of the intended field of application shall be considered. |  |
| Status: active Architecture: red_and_non-red | Comment: Example for a national standard: VDE 0100 in Germany. Note that this SAC is an additional hint to the basic safety measures of [EN 50126]. Therefore this SAC is implicitly applicable for all hardware systems, and is no longer explicitly mentioned for new hardware (e.g. CIQ3). It will be removed in future TAS Platform versions. See also [SACDEF]. Class: CIQ6 D2 D6 EPCM MCC SCC |
| Implementation in ELEKTRA: | The necessary measures will be defined on product level; the measures are not part of the development. |

| [CORE-2.x-HW:SAC:0018] | No replacement under voltage |
| --- | --- |
| SAC-Text: The Core System D2, D6, EPCM, CU3 components shall not be removed or plugged in under voltage. |  |
| Status: active Architecture: red_and_non-red | Comment: The CPU board D6 provides a hot swap function, which must be activated by the hot swap switch on the front, if the board has to be removed under voltage. The hot swap function controls also plugging of the board under voltage. The other components of the Core System D6, like e.g. the power supply, do not provide such a feature. The MCC, SCC, CIQ6, CIQ3. D10 and SC33 boards can be hot plugged. Class: CU3 D2 D6 EPCM |
| Implementation in ELEKTRA: | The correct handling of the HW will be described in maintenance manuals. |

| [CORE-2.x-HW:SAC:0023] | Repair or firmware update of components only at manufacturer |
| --- | --- |
| SAC-Text: TAS PLF Core HW components shall be repaired or updated (firmware) only by the manufacturer and not by the customer. Faulty or defect components shall be returned to the manufacturer. Only complete components have to be replaced. |  |
| Status: active Architecture: red_and_non-red | Comment: This SAC implicitly restricts the usage of the programming interfaces for the firmware. Class: CIQ6 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | This restriction will be part of the repair process for product ELEKTRA. |

| [CORE-2.x-HW:SAC:0201] | Environmental conditions |
| --- | --- |
| SAC-Text: The environmental conditions as defined per TAS Platform Core System in the respecitve documents are to be observed. |  |
| Status: active Architecture: red_and_non-red | Comment: The environmental conditions are defined in the following documents: Core System Hardware Document containing environmental conditions CIQ6 Manual Core System CIQ6 7HA 10077 AAAA 108EN CIQ3 User Manual CIQ3 Hardware Unit 7HA 10143 AAAA 108EN CU3 Version Delivery Description, CU3, OTE3 3BU 80812 4845 QEAPA D10 Version Delivery Description - MEN-D10 (User manual -> technical data) 3BY 09015 0017 FMAPA (Planned) SC33 Version Delivery Description - MEN-SC33 (Planned) Class: CIQ3 CIQ6 CU3 D10 SC33 |
| Implementation in ELEKTRA: | The environmental conditions shall be defined by product ELEKTRA. This definition shall be observed by the customer. |

| [CORE-2.x-HW:SAC:0012] | Environmental conditions (Core System D2) |
| --- | --- |
| SAC-Text: The specified location for the Core System D2 is a cabinet in a not-air-conditioned building according to [EN 50125-3] with the following environmental conditions: Type Conditions Ambient temperature, operation 0° - +60°C Ambient temperature, storage -25°C - +70°C Humidity 5..95% 0.55 g/m3 to 25 g/m3 No condensation Shock 3M1 [EN 60721-3-3] Vibration 3M1 [EN 60721-3-3] ESD /EMC [EN 50121-1], [EN 50121-4] [EN 61000-6-2], [EN 61000-6-4] # (sub-rack) IP20 [EN 60529] Pollution degree (boards) PD1 [EN 50124-1] Altitude Class A1, 1400 m Wind, rain, etc. shall not be considered. The environmental conditions shall be kept by the application. The application has to prove, if the intended environment (location, interior of the cabinet) agrees with the specified environmental conditions of the Core System D2. Regarding condensation and pollution constructive measures on equipment level (cabinet) have to be taken by the application to keep the required condition. If the conditions under which the Core System D2 is used are aggravated (e.g. due to dust, smoke, frost, condensation, etc.), the user shall specify such conditions and prove their ineffectiveness. Remark: The compliance to the humidity, ESD and EMC conditions has to be proven by the application. |  |
| Status: active Architecture: red_and_non-red | Comment: None. Class: D2 |
| Implementation in ELEKTRA: | The environmental conditions shall be defined by product ELEKTRA. This definition shall be observed by the customer. |

| [CORE-2.x-HW:SAC:0033] | Environmental conditions (Core System D6) |
| --- | --- |
| SAC-Text: The specified location for the Core System D6 is a cabinet in a not-air-conditioned building according to [EN 50125-3] with the following environmental conditions. Type Conditions Ambient temperature, operation 0° - +60°C Ambient temperature, storage -25°C - +70°C Humidity 5..95% 0.55 g/m3 to 25 g/m3 No condensation Shock 3M1 [EN 60721-3-3] Vibration 3M1 [EN 60721-3-3] ESD /EMC [EN 50121-1], [EN 50121-4] [EN 61000-6-2], [EN 61000-6-4] Protection (sub-rack) IP20 [EN 60529] Pollution degree (boards) PD1 [EN 50124-1] Altitude Class A1, 1400 m Wind, rain, etc. shall not be considered. The environmental conditions shall be kept by the application. The application has to prove, if the intended environment (location, interior of the cabinet) agrees with the specified environmental conditions of the Core System D6. Regarding condensation and pollution constructive measures on equipment level (cabinet) have to be taken by the application to keep the required condition. If the conditions under which the Core System D6 is used are aggravated (e.g. due to dust, smoke, frost, condensation, etc.), the user shall specify such conditions and prove their ineffectiveness. |  |
| Status: active Architecture: red_and_non-red | Comment: None. Class: D6 |
| Implementation in ELEKTRA: | The environmental conditions shall be defined by product ELEKTRA. This definition shall be observed by the customer. |

| [CORE-2.x-HW:SAC:0039] | Environmental conditions (Core System EPCM) |
| --- | --- |
| SAC-Text: The Core System EPCM is designed for on-board units and wayside applications (vital equipment outside 3 m zone) and can be operated under the following environmental conditions: Type Conditions Ambient temperature, operation EPCM board -40°C - +58°C 2) (normal air convection) 3JA 80320 AA**, 3JA 80330 AA** EPCM board with PCI-104 board -40°C - +58°C 2) (normal air convection) 3JA 80321 AA**, 3JA 80331 AA**, 3JA 80323 AA**, 3JA 80333 AA** EPCM board with additional heat sink -40°C - +60°C 1) (normal air convection) 3JA 80325 AA**, 3JA 80335 AA** EPCM board -40°C - +70°C (airflow > 1 m/s) 3JA 80320 AA**, 3JA 80330 AA** EPCM board with PCI-104 board -40°C - +70°C (airflow > 1 m/s) 3JA 80321 AA**, 3JA 80331 AA**, 3JA 80323 AA**, 3JA 80333 AA** EPCM board with additional heat sink -40°C - +70°C (airflow > 1 m/s) 3JA 80325 AA**, 3JA 80335 AA** Ambient temperature, storage -40°C - +85°C (all board variants) Humidity 95 % [EN 50125-1] No condensation Shock Class B, category 1 [EN 61373] Vibration Class B, category 1 [EN 61373] ESD / EMC [EN 50121-3-2] and [EN 50121-4] 3) Pollution degree (boards) PD1 [EN 50124-1] Altitude 3000 m The application has to consider, that the surface temperature of the EPCM boards rises, around 1K per 1000m, due to the decreasing cooling ability of air. Wind, rain, etc. shall not be considered. The environmental conditions shall be kept by the application. The qualification has been carried out with reference set-ups . A new qualification statement is necessary, if the application-specific configuration deviates from the reference set-ups. For this statement additional qualification tests may be necessary. Regarding condensation, ESD/EMC and pollution constructive measures on equipment level (sub-rack, rack, etc.) have to be taken by the application to keep the required conditions. If the conditions under which the Core System EPCM is used are aggravated (e.g. due to dust, smoke, frost, condensation, etc.), the user shall specify such conditions and prove their ineffectiveness. Remarks: 1) At the upper temperature value of 60°C during qualification test the EPCM Core system was 2°C below its switch off limit. 2) Conclusion by analogy. 3) The CAN bus has been assumed as operational I/O port with cable lengths < 3 m. Therefore no tests against conducted disturbances have been carried out at this port. |  |
| Status: active Architecture: red_and_non-red | Comment: None. Class: EPCM |
| Implementation in ELEKTRA: | ELEKTRA does not use EPCM. EEC uses EPCM. The environmental conditions shall be defined as part of the development of the EEC HW. |

| [CORE-2.x-HW:SAC:0048] | Environmental conditions (SCC) |
| --- | --- |
| SAC-Text: The Core System SCC is designed for the use in buildings without climatic control (more than 3 m from the track), trackside in a cubicle (1 to 3 m from the track) as well as on-board and can be operated under the following environmental conditions: Type Conditions Ambient temperature (board), operation -40 °C ... +85 °C Ambient temperature (board), storage -40 °C ... +85 °C Humidity 7HA 10057 AA** 5..95 %, no condensation 7HA 10058 AA** (with coating) 5..100 %, condensation Pollution degree (board) according to [EN 50124-1] 7HA 10057 AA** PD1 7HA 10058 AA** (with coating) PD2 Shock and Vibration [EN 50125-3], outside track bed, distance to track 1 to 3 m and more; [EN 61373], category 1B; AREMA Part 11.5.1 class I and J EMC [EN 50121-4] 1) [EN 50121-3-2] 2) Altitude 3000 m 3) (no pressure sensitive parts on board) 1) Except pulsed magnetic field in the 3 m range of the track (reference configuration is not defined for use in the 3-m-range of the track). 2) For rolling stock apparatus only one test case (Immunity, radio-frequency electromagnetic field, enclosure) has been carried out. Other tests for rolling stock apparatus are not covered because qualification tests have to be executed on a complete system which is currently not defined for rolling stock apparatus. 3) The thermal limits specified above shall be guaranteed by application. Wind, airflow, precipitation and solar radiation are not applicable due to the above defined field of application. The environmental conditions shall be kept by the application. The qualification has been carried out with reference set-ups. Therefore not all interfaces could be tested, as they are not used by the software of these reference set-ups. A new qualification statement is necessary, if the application-specific configuration deviates from the reference set-ups. For this statement additional qualification tests can be necessary. Dependent of the intended field of application also the not carried out tests have to be considered (see footer 1) and 2)). Regarding condensation, ESD/EMC and pollution, constructive measures on equipment level (sub-rack, rack, etc.) have to be taken by the application to keep the required conditions. If the conditions under which the Core System SCC is used are aggravated (e.g. due to dust, smoke, frost, etc.), the user shall specify such conditions and prove that the system is compliant with these conditions. |  |
| Status: active Architecture: red_and_non-red | Comment: Note that this SAC originally was CORE-2.x-HW:SAC:045. Number changed to CORE-2.x-HW:SAC:048 in 3CR 00055 5070 DUAFQ / 16 RL for TAS PLF 2.1.3a according to CQD 6260. Class: SCC |
| Implementation in ELEKTRA: | ELEKTRA does not use SCC. EEC uses SCC. The environmental conditions shall be defined as part of the development of the EEC HW. |

| [CORE-2.x-HW:SAC:0024] | Safety qualification tests |
| --- | --- |
| SAC-Text: Each application shall plan and perform safety qualification tests with the used TAS PLF 2.x software and hardware configuration. |  |
| Status: active Architecture: red_and_non-red | Comment: The use of a railway application is necessary for safety qualification tests. No safety qualification tests were performed since TAS PLF Core is a generic product and not containing a railway application. Class: CIQ3 CIQ6 CU3 D2 D6 D10 EPCM MCC SC33 SCC |
| Implementation in ELEKTRA: | ELEKTRA and EEC will do qualification test for the cabinets. |

| [CORE-2.x-HW:SAC:0001] | Reliability and availability calculations of the applications |
| --- | --- |
| SAC-Text: The MTTR values for the least repairable units of the TAS PLF Core HW assumed in the related RAM Analyses are: Core System D2 (2oo3, 2oo2): - CPU board D2, Quad Ethernet Controller, Compact Flash memory card, Transition adapter and Hex-switch, backplane, power supply, fan: 12 h Core System D6 (2oo3, 2oo2): - CPU board D6, Compact Flash memory card, Transition adapter and Hex-switch, backplane, power supply, fan: 12 h Core System EPCM 2oo3: - CPU board EPCM, Compact Flash memory card, power supply, (MVB board, Profibus board): 8 h - backplane 2.6 h Core System EPCM 2oo2: - CPU board EPCM, Compact Flash memory card, power supply, backplane: 3 h MCC 2oo3: - MCC board: MTTR: 3 h / MRT: 10 min (repair on system failure) SCC 2oo3: - CPU board, power supply: 8 h - sub-rack with backplane: 3 h SCC 2oo2: - CPU board, power supply, sub-rack with backplane: 3 h The availability calculations in the RAM Analyses for the Core Systems (D2, D6, EPCM, MCC, SCC , CIQ6, CIQ3, D10 and SC33) can be used as basis for further reliability and availability calculations of the applications only, if it is ensured that these time values can be realized by means of suitable diagnosis procedures, the construction of the application and respective logistics. The boot times of the software must be taken into account, too. The RAM Analyses are based on assumptions and partly on reference configurations. The assumptions for CIQ6 are e.g.: - mean annual temperature of 40°C - continuous operation (24 h / 7 days a week) - recovery functionality implemented - automated refresh or manual refresh during operational pauses of memory card - exchange of fan rack possible without shutting down the system - worst case calculation, e.g. with regard to power supplies Assumptions for other Core Systems are similar. The applications shall check, if these assumptions fit with the application conditions or if some assumptions have to be reconsidered. CIQ6: The results of the RAM Analysis are not the final application results. For the application the complete MRT values, which include application specific times, shall be considered. In addition beta factors are not considered in the analysis, as these factors are also application-specific. If, these factors are necessary, the pre-filled table for the beta factor determination shall be completed on application level. In case of using the Core System EPCM MCC or SCC, the application shall check whether the actual used power supplies, backplane and sub-rack are compliant to the reliability values assumed in the respective RAM Analysis. |  |
| Status: active Architecture: red_and_non-red | Comment: For EPCM, MCC, SCC, CU3, D10 and SC33 power supplies, backplane and sub-rack are not a part of the Core System. Refer to the RAM Analyses of the respective Core System: - D2: 3CR 30911 0017 VFAFC, - D6: 3JA 80001 AAAA VFAFA, - EPCM: 3JA 80300 AAAA VFZZA, - MCC: 3JA 81000 AAAA 909EN - SCC: 7HA 10056 AAAA 909EN - CIQ6: 7HA-10077-AAAA-909EN This SAC is a simple reminder that the results of the RAM analysis is only applicable if the preconditions as defined in the respective manual are fulfilled. According to the rules (SAC definition and Handling [SACDEF]), This does not qualify this topic for a SAC. The handbooks have to be observed anyway. Therefore, this SAC does not mention the CIQ3, CU3, D10 and SC33 explicitly, see the RAM analysis in 7HA 10143 AAAA 909EN for CIQ3 and 3BU 80812 4846 QZAPA for CU3, 3BY 09015 0008 DUAPA for D10, 3CU 17035 0036 DUZZA for the SC33. Class: CIQ6 D2 D6 EPCM MCC SCC |
| Implementation in ELEKTRA: | ELEKTRA will get its specific RAM analysis. |

| [CORE-2.x-HW:SAC:0170] | Refresh of 128 MB CF memory cards |
| --- | --- |
| SAC-Text: In case applications use a 128 MB compact flash card, a loss of data on the CF card is possible o after about 1 million read accesses without data write access o after a data retention time of 10 years without data write access either in operation or storage Due to the introduction of MD5 for the start-up tests, this is no hazard, but it may lead to a loss of availability (2oo2) or to a constrained maintenance action. Therefore, a refresh utility remains provided with Core 2.x. Applications, while deciding to use the refresh utility with Core 2.x, shall take into account: o If the boards are submitted to a loss of power while writing the flash, the flash might be destroyed or at least the board(s) might not reboot. o If applications do not read from flash cards after start-up, the safety reaction to failed disks in an already booted system can be delayed to next boot or maintenance action (see Core-2.x-SW:SAC:100). o the period should be set as to at most 10 years either in operation or storage. If an application does not use the refresh utility, but requires an operative time in the order of magnitude of 10 years, it shall provide its own concept to refresh the 128 MB CF cards. |  |
| Status: active Architecture: red_and_non-red | Comment: The data retention time of the 128MB CF card is 10 years. 32MB CF cards do not need a refresh. Class: D2 D6 EPCM |
| Implementation in ELEKTRA: | The refresh utility will be used by ELEKTRA and ECC for MEN-D2 and MEN-D6 (see also [CORE-2.x-HW:SAC:0173] for MEN-D10). Details are available in chapter 5.19.1 "Use TAS Platform including its hardware" in the paragraph for this SRAC. |

| [CORE-2.x-HW:SAC:0040] | Prevention of electrostatic discharge |
| --- | --- |
| SAC-Text: Electrostatic discharge shall be prevented during handling with the TAS PLF Core HW. Therefore appropriate ESD protective measures shall be taken and thereby the grounding concept of the system shall be considered. |  |
| Status: active Architecture: red_and_non-red | Comment: According to the Rules (SAC definition and Handling [SACDEF]), a reference to a standard need not be an explicit SAC. Therefore this SAC is not applicable to the Core System Hardware after CIQ6 although the relevant standard has to be applied also to them. Class: CIQ6 D2 D6 EPCM MCC SCC |
| Implementation in ELEKTRA: | The grounding concept will be part of the design of the cabinets. The correct handling of the HW will be described in maintenance manuals. |

| [CORE-2.x-HW:SAC:0173] | Refresh of programmed memory cards |
| --- | --- |
| SAC-Text: Programmed memory cards, shall be refreshed in regular intervals, because: - it is recommended not to exceed 100,000 read cycles without a refresh to avoid the NAND read disturb phenomenon. - only a data retention for 5 years is guaranteed. The user shall ensure, that none of these limits is exceeded during operation and storage. The application shall ensure that it shall either not write more than 100.000 times to the card or shall use an intelligent algorithm to minimize the erase cycles similar to the TAS Platform refresh algorithm to extend the number of possible write actions. |  |
| Status: active Architecture: red_and_non-red | Comment: The refresh of a memory card can be achieved in two different ways: - Using an automated refresh mechanism embedded in e.g. the operating system. - Refreshing the memory card manually during a maintenance activity. For detailed information regarding write cycles see also 7HA 10080 AAAA 980EN and 001SCM00347970. Class: CIQ6 CU3 D10 |
| Implementation in ELEKTRA: | The refresh utility will be used by ELEKTRA and ECC for MEN-D10 (see also [CORE-2.x-HW:SAC:0170] for MEN-D2 and MEN-D6). Details are available in chapter 5.19.1 "Use TAS Platform including its hardware" in the paragraph for this SRAC. |

| [CORE-2.x-HW:SAC:0019] | Visual inspection of the fans (D2 and D6 only) |
| --- | --- |
| SAC-Text: In case of using the Core System D2 or D6, the fans of the fan rack shall be checked every 12 months by a visual inspection for their operability to guarantee a reliable operation of the system. |  |
| Status: active Architecture: red_and_non-red | Comment: As with all hardware parts, the regulations for usage issued by the vendor apply for fans used by all other TAS Platform Core System Hardware. Class: D2 D6 |
| Implementation in ELEKTRA: | A periodical check of the fans is not necessary. ELEKTRA is able to handle the failure of one fan and will give an alarm to exchange the fan rack. (see also [CORE-2.x-HW:SAC:0002]). |

| [CORE-2.x-HW:SAC:0002] | Exchange of the fan rack every 5 years (D2 and D6 only) |
| --- | --- |
| SAC-Text: In case of using the Core System D2 or D6 the fan rack shall be exchanged every 5 years. |  |
| Status: active Architecture: red_and_non-red | Comment: The fan rack has a limited life time (wear out parts). As with all hardware parts, the regulations for usage issued by the vendor apply for fans used by all other TAS Platform Core System Hardware. Class: D2 D6 |
| Implementation in ELEKTRA: | A periodical exchange of the fans is not necessary. ELEKTRA is able to handle the failure of one fan and will give an alarm to exchange the fan rack. (see also [CORE-2.x-HW:SAC:0019]). |

#### Template for DOORS export SRACs TAS-PLF CORE-HW

| <<SAC PUID including Bookmark>> | <<SAC Synopsis including Bookmark>> |
| --- | --- |
| SAC-Text: <<SAC Description>> |  |
| Status: <<SAC Status>> Architecture: <<System Architecture>> | Comment: <<SAC Comments>> Class: <<class>> |
| Implementation in ELEKTRA: | <<Object Text including DocRef>> |

## SRACs OCS

### SRACs OCS System 2.x

The SRACs of OCS System are described in the following documents:

- Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform OCS - 2.x - System & OCS-RED [TP_OCS_Sys]

The following sub-chapters are based on an extract of the referenced document (including the head lines) as no SRAC database is available in HITACHI Austria. The referenced document is valid for OCS 2.1.9 (OCS - 2.x - System & OCS-RED).

#### Integration

##### Processes

ID: OCS:SAC:009

Synopsis: Application hazard analysis and OCS integration.

Description: In the scope of its hazard analysis, the application shall consider following two aspects:

System level focus: integration of OCS in the application safety concept

Detailed focus: potential conflicts of OCS configuration parameters with the application’s safety requirements - the configuration of OCS is made by means of the templates described in [UG OCS API 2.x].

Comment: The safety management performs the identification of potentially hazardous API and configuration constellations. Unavoidable constellations are pointed out as SAC - for instance the configuration of the authentication, see OCS:SAC:017.

Avoidable constellations are corrected - see for instance GNATS 2447. This analysis can only be based on TAS Platform’s estimation of the application expectations/uses of OCS - a limited view. Therefore, a complementary step appears necessary before use.

See also [UG OCS API 2.x].

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | For ELEKTRA the reference document [UG OCS API 2.x] has to be replaced by [UG_OCSNR_API_2.x]. OCS-EL: Appendix A of “Sicherheitsnachweis, Interface IXL-RBC über ERS”, 3BU 15000 3019 DUAPC OCS-SB: “Hazard Analysis, Interface IXL-AzLM über OCS-SB, Elektra 2.x”, ÖBB, 3BU 15000 2242 DUAPC “Gefaehrdungsanalyse Interface IXL-AzLM ueber OCS-SB”, 3BU 15000 3256 DUAPC |
| --- | --- |

##### Underlying Layers

ID: OCS:SAC:008

Synopsis: Complementarity of Safety Handbook Core to Safety Handbook OCS

Description: Fulfilling the SAC in current document does not replace fulfilling those of [SAC CORE 2.x].

Projects using OCS shall show evidence of fulfillment of the safety handbook associated to the TAS Platform Core version they run OCS on.

Comment: None.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | The SRACs of TAS Platform are considered in chapter 6.1. |
| --- | --- |

ID: OCS:SAC:046

Synopsis: Limitations in regard to OCS variants

System architecture: redundant

##### Parallel Use of Protocols

ID: OCS:SAC:077

Synopsis: SBS for not safety related communication only.

System architecture: redundant

ID: OCS:SAC:084

Synopsis: Assurance of functional independence and prevention of masquerade due to corruption

System architecture: redundant

ID: OCS:SAC:078

Synopsis: Impact of SAHARA protocol on communication of EKIR components in a network.

System architecture: redundant

#### Using the framework

##### Instantiation

ID: OCS:SAC:015

Synopsis: Framework: Multiple OCS instantiation forbidden.

Description: Running more than one OCS on the same CN is not allowed.

Comment: See [SC OCS 2.x] for the embedding of this SAC in the OCS safety concept.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | Only one OCS instantiation shall be started on an ELEKTRA computer. This restriction applies to the computers used for RCU and SCU for connection to HITACHI RBC - see “ADD Interface IXL - RBC over OCS” [EL_RBC_OCS]. This restriction applies to computers used for EC for connection to AzLM – see “ADD IF Elektra-AzLM over OCS-SB” [EL_AZLM_OCSSB]. Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. |
| --- | --- |

##### OCS User Guides

ID: OCS:SAC:054

Synopsis: Compliance to OCS API User Guide and Configuration Reference Manual

System architecture: redundant

##### Configuration

ID: OCS:SAC:048

Synopsis: Use of the official configuration template mandatory

Description: Applications shall run OCS versions only with the configuration template delivered with this version, ocs.tmpl. In case the application alters this template, it shall provide evidence that the safety of the system is not affected.

Comment: Applications shall set OCS only with the help of their configuration file (and that only).

System architecture: redundant

| Implementation in ELEKTRA: | This SRAC is also relevant for ELEKTRA in a modified way. ELEKTRA shall use the configuration template ocs_nonred.tmpl. The template shall be part of the configuration files that are written to the flashcard and used by the application. |
| --- | --- |

ID: OCS:SAC:079

Synopsis: Setting MD5 checks in OCS configuration files

Description: OCS is subject to CORE-2.x-SW:SAC:177. To fulfill it, the cdb MD5 check shall be enabled on its configuration files for safety relevant data. This succeeds with the help of the cdbmd5 tool as specified by [GDE REF CORE]. The MD5 check requires that each safety relevant configuration file is instrumented with an offline generated MD5 signature as specified by [GDE REF CORE].

Comment: If this is not performed, applications will not succeed in demonstrating their compliance to CORE-2.x-SW:SAC:177. CORE-2.x-SW:SAC:177 addressed systematic faults in the Linux file system - see [SAC CORE 2.x] for details.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | All configuration files used for CS-remote, EP, XEP and OCS shall be instrumented with the above mentioned MD5 signature – see [CORE-2.x-SW:SAC:0177] and chapter 5.19.2. Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. |
| --- | --- |

##### Message Queues

ID: OCS:SAC:012

Synopsis: Configuration of OCS internal message queues

System architecture: redundant

##### Safety Reaction

ID: OCS:SAC:033

Synopsis: Particular relevance of SAC CORE-2.x-SW:SAC:019 for OCS

System architecture: redundant

ID: OCS:SAC:072

Synopsis: Loss of communication not necessarily notified in the specified time.

Description: Requirement OCS-2.x:SRS:0099 of the OCS requirement specification tells:

“The TAS PLF OCS shall deliver a notification to the API within t<=500 msec. after an error occurred in an OCS communication link.”.

Meeting this requirement is conditioned by

- Process priorities and machine load,

- the chosen protocol and its settings about the life-cycle monitoring (the dedicated parameter for the different protocols are described in the SAC mentioned below).

The calculation of the failure detection times for the different protocols, which are guaranteed by OCS, are documented in OCS:SAC:028 and OCS:SAC:071.

The application shall define OCS priority and life-cycle monitoring settings according to its requirements. It shall test that its requirements are met, especially under high load.

As high CPU load conditions have the potential to extend the failure detection time, as preventive measure the TAS Platform Core's real-time monitoring service (Timeliness Supervision) shall be enabled if necessary.

Compared to single-core CPUs, such CPU load conditions come along with additional specific scenarios on multi-core CPUs which have been evaluated in the study [TIMANA]. Applications on multi-core CPUs shall consider these specific scenarios and take appropriate measures, if necessary.

Comment: In [SRS OCS 2.1], OCS-2.x:SRS:0099 replaces former OCS:FUR:181 in [SRS OCS 1.10].

In [SSRS OCS GEN 2.x] genOCS-2.x:SSRS:OCS-gen:0091 replaces former gen-OCS:FUR:48.

OCS:SAC:037, originally referred to along with  OCS:SAC:028 and OCS:SAC:071 in 1.x  is no longer referred to as ANSALDO is no more on the OCS road map.

OCS:SAC:063, originally referred to along with OCS:SAC:028 and OCS:SAC:071 in 1.x is not referred to either, as UNISIG is not supported in OCS 2.1. Previous formulation referred also to (deleted) requirement genOCS-2.x:SSRS:OCS-gen:0091.

For the TAS Platform Core's real-time monitoring service on dual-/multi-core CPUs (provided since Core 2.3.0) please refer to the study [TIMANA].

System architecture: redundant

| Implementation in ELEKTRA: | This SRAC is also relevant for ELEKTRA although non redundant ELEKTRA and non redundant SB is missing in the System architecture (see comment of OCS:SAC:076). OCS non-red will inform ELEKTRA about the status of a connection (OCS_UP or OCS_DOWN). Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. |
| --- | --- |

ID: OCS:SAC:080

Synopsis: Platform Core real-time supervision necessary for OCS

Description: OCS uses intensively timers to monitor the health of communication channels. These timers may not expire on time if the computer is overloaded. To avoid these conditions, OCS requires TAS Platform Core's real-time monitoring service to be enabled. To activate this feature, the TIMELINESS configuration item has to be set.

Comment: In Core 2.x, this service replaces former Leo/Add-on 2 for 1.x.

The TAS Platform Core's real-time monitoring service, also referred to as Timeliness Supervision, safeguards the reactiveness of the system at runtime, so that OCS is able to increment its timers and to supervise their values. Concerning the reactiveness of the system the Timeliness Supervision is as reliable on dual-/multi-core CPUs as on single-core CPUs. This has been evaluated in the study [TIMANA].

Multi-core CPUs have been introduced with Core 2.3.0.

System architecture: redundant

| Implementation in ELEKTRA: | This SRAC is also relevant for ELEKTRA although non redundant ELEKTRA and non redundant SB is missing in the System architecture (see comment of OCS:SAC:076). ELEKTRA will supervise its realtime behaviour with the following mechanisms: Reactivity Supervision (see chapter 5.18 “Supervision of reactivity”) Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. |
| --- | --- |

##### Safety relevant Monitoring

ID: OCS:SAC:016

Synopsis: Handling OCS diagnosis messages

System architecture: redundant

#### Common SAC for all Protocols

ID: OCS:SAC:081

Synopsis: Limitations toward security

Description: OCS offers no provisions against unauthorized access.

Applications shall show that OCS is deployed in networks where this probability of the event is negligible, as defined by cat. 1 and 2 [EN 50159:2010].

For end-of-life ALCATEL protocol, the conformity to EN 50159 has been shown only for closed network, thus applications using the ALCATEL protocol shall prove that it is deployed in closed networks only.

Comment: None.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | ELEKTRA shall use only networks of category 1 or category 2 (see PSAH:SAC:014). ELEKTRA shall use SAHARA protocol only (see [OCS-2x-nonred:SAC:0004]). Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. ALCATEL protocol is not supported by OCS 2.x non-red (see [OCS-2x-nonred:SAC:0004]). |
| --- | --- |

##### Safety Code Configuration

ID: OCS:SAC:025

Synopsis: Safety code configuration.

Description: The field ‘.safety’ permits to tailor the safety code of a given protocol with all safety codes provided by OCS.

This field shall be configured in compliance with the protocol definition of the used protocol:

- ALCATEL:  'MD4'

- SAHARA: 'MD4S'

- SAHARA: 'MD4' (only from OCS 2.1.4 onwards)

Comment: The protocols are analyzed toward [EN 50159:2001-1] (ALCATEL) or [EN 50159:2010], Cat 1&2, (SAHARA) only with these safety codes. In case another safety code is used, these analyses are no longer valid.

For protocol SAHARA, the field '.safety' shall be configured with particular care with respect to the SAHARA variant. The MD4 safety code is configurable in OCS 2.1.4 only (formerly known as OCS 1.10.9-ADDON-4)  and shall be used only in context with .dilo = 'dilo_md4U'. See [UG OCS CONF], Chapter 4.5.8.

System architecture: redundant

| Implementation in ELEKTRA: | This SRAC is relevant for ELEKTRA in a modified way. The document relevant for ELEKTRA is [UG_OCSNR_CONF_2.x]. ELEKTRA shall use the following configuration: SAHARA: 'MD4'. Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. OCS 2.x non-red supports only SAHARA protocol (see [OCS-2x-nonred:SAC:0004]). |
| --- | --- |

##### Guarantee of Message Delivery

OCS provides a one-channel safe layer for [EN 50159:2010], Cat 1&2. As specified by the norm its protocols provide in general following features:

Detection of loss of communication

Authentication (of both sender and receiver)

Detection of sequence failure

SAHARA is the only protocol implemented by OCS and providing timeliness, implemented in form of a round-trip monitoring.

ID: OCS:SAC:076

Synopsis: Guarantee of delivery not given

Description: OCS protocols do not provide on sender side a monitoring of the delivery of application messages (timely or not).  For instance, even with SAHARA, OCS does not provide diagnosis about the last acknowledged telegram before the closure of the connection.

Therefore, applications shall not base their safety case on the assumption that OCS guarantees the delivery of messages. Instead that safety case shall be based on the acknowledgement of execution of requests/commands provided at application level.i.e. on messages received.

Comment: Delivery of messages is not monitored in case of  closure of channel. For message delivery while a connection is open, please pay attention to OCS:SAC:072 and OCS:SAC:080.

This SAC replaces OCS:SAC:042  which evolved during OCS 1.x. In the end, it contained a proposal for solution that was not in line with the introduction and with a safe praxis.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | The application protocols ELEKTRA - AzLM and ELEKTRA - RBC do not depend on the safe delivery of a message before closing the connection. Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. OCS 2.x non-red supports only SAHARA protocol (see [OCS-2x-nonred:SAC:0004]). |
| --- | --- |

##### Common Settings /  Gemeinsame Voreinstellungen

Parameter .wait_for_dl_ack of template OCS_CHAN is an important timeout for the safe interaction between PL and DL. The correct PL behavior is guaranteed only with a value of this parameter bigger than a given minimum.

ID: OCS:SAC:041

Synopsis: Minimum value for .wait_for_dl_ack

System architecture: redundant

#### SAHARA Protocol

Current chapter groups SAC relevant for the use of the SAHARA protocol only.

##### Evidence of Compliance to [EN 50159:2010]

ID: PSAH:SAC:014

Synopsis: Deployment restricted to transmission systems according to categories 1 and 2

Description: Applications using the SAHARA protocol, as defined by [PS SAHARA 2.2], shall ensure that networks in which the protocol is deployed satisfy the requirements set by the definition of categories 1 and 2, as defined by [EN 50159:2010].

Comment: This SAC replaces PSAH:SAC:001, which was restricted to closed networks due to the limited scope of [EC SAHARA 50159-1].

[PS SAHARA 2.2] offers no defences against unauthorized access.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | ELEKTRA shall use only networks of category 1 or category 2 (see OCS:SAC:081). Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. |
| --- | --- |

ID: PSAH:SAC:013

Synopsis: Lower half of MD4 safety code reserved for safety relevant communication

Description: In networks where SAHARA is deployed for safety critical applications, the lower half of the MD4 safety code shall be used only for safety critical purposes.

In particular it excludes the use of this safety code for following purpose:

in transmission systems category 1 in the (defined) layers under the transport adaptation layer of SAHARA.

Communication between application that are not safety critical using the same non-safety relevant transmission system.

Communication between safety critical applications and not safety critical applications using the same non-safety relevant transmission system.

Comment: For category 2 transmission systems the underlying layers are possibly not defined. Note that the MD4 safety code in general is not used by commercial transmission systems.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | ELEKTRA shall use the MD4 safety code as mentioned above. Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. Within OCS 2.x non-red the lower half of MD4 is not used according to the design. |
| --- | --- |

ID: OCS:SAC:085

Synopsis: Lower half of MD4 safety code reserved for safety relevant communication

System architecture: redundant

##### Limited Scope

ID: OCS:SAC:044

Synopsis: SAHARA with UDP transport layer only

Description: Applications shall configure only the UDP transport layer for SAHARA virtual channels.

Comment: TCP is not validated. For TCP, a change request shall be submitted to the CCB.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | ELEKTRA shall configure only the UDP transport layer for SAHARA virtual channels. Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. |
| --- | --- |

##### Loss of communication

ID: OCS:SAC:028

Synopsis: Failure detection time for SAHARA - receiving

Description: Application using the SAHARA protocol shall determine how long their failure detection time shall be. Then, they shall set OCS parameters in order to meet their requirements.

The failure detection time of OCS, considered as receiver, is with SAHARA 2.2 protocol independent on retransmission feature activation:

Tworstcase = .max_msg_rtd.

For details about the configuration of .max_msg_rtd the application shall refer to [UG OCS CONF 2.x], chapter 4.7.2.

Comment: None.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | The document relevant for ELEKTRA is [UG_OCSNR_CONF_2.x]. SAHARA for the AzLM connections .max_msg_rtd calculation is documented in 3BU 15000 5138 DSAPC chapter 7.2; the value is defined as max_msg_rtd = 1200ms. SAHARA for RBC interface: the value is defined as max_msg_rtd = 1800ms. Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. |
| --- | --- |

##### Authentication

ID: OCS:SAC:075

Synopsis: Configuration of authentication for SAHARA

Description: In case of using the SAHARA protocol, the configuration of fields ‘local_auth’ and ‘remote_auth’ shall ensure that the combination of ‘local_auth’ and ‘remote_auth’ is unique for each communication between computing nodes (CNs) - i.e. for each virtual channel using SAHARA within one network.

Comment: In case of using the SAHARA protocol each transferred message contains the sender identification (field ‘local_auth’) and the ‘receiver identification’ (field ‘remote_auth’). Thus a safe authentication is ensured in case that the combination of sender identification and receiver identification is unique within one network.

As specified by [PS SAHARA 2.2] ‘.remote_auth’ shall be bigger than ‘.local_auth’ on master side.

See  e.g. OCS:SAC:017.

System architecture: redundant

non redundant ELEKTRA

non redundant SB

| Implementation in ELEKTRA: | SAHARA for the AzLM connections: the authentication numbers are defined in documented 3BU 15000 5138 DSAPC chapter 7.3.2.1. SAHARA for the RBC connection: .remote_auth = 100, .local_auth = 200. Compliance statement in [TP_OCS_EoC_NR]: The application has to be aware of this SAC according to [OCS-2x-nonred:SAC:0001] in [TP_OCS_EL] and [TP_OCS_SB]. |
| --- | --- |

##### Field Monitoring

The field observation remains necessary in the scope of [EN 50128]. How it is performed is, in this context, general responsibility of the application and therefore not worth a SAC as OCS Level.

#### SAHARA Protocol variant 'OCS Addon-4'

This protocol variant will not be used by ELEKTRA. Therefore this chapter does not contain further details.

#### ALCATEL Protocol

Current chapter groups SAC relevant for the use of the ALCATEL protocol only.

This protocol variant will not be used by ELEKTRA. Therefore this chapter does not contain further details.

### SRACs OCS-EL / OCS-SB

The SRACs of OCS-EL and OCS-SB (Version OCS 2.1.9 nonred) are described in the following documents:

- Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform OCS - 2.x - OCS EL [TP_OCS_EL]

- Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform OCS - 2.x - OCS SB [TP_OCS_SB]

The following part of this chapter is based on a DOORS export of the according SRAC database.

#### Identification of the DOORS databases

| DOORS/RMF Version: | Client 9.7.0.0 – Server 9.7.0.0 / RMF 6.1.0.9 |
| --- | --- |
| DOORS/RMF Module Name: | ELEKTRA PV GEN_OCSnonred_EoC |
| DOORS/RMF Module Version (Baseline): | 1.2 |
| DOORS/RMF Module Path: | /SAC-DB AT/Generic Components/Thales-AT VIE/ |
| DOORS/RMF Export View: | SAE WEXP Export View Extended |
| DOORS/RMF Current Filter: | ((WEXP Bookmark is not empty) OR (((NOT (Column 'SAC Status' contains 'deleted' (case sensitive))) AND (NOT (Column 'SAC Status' contains 'obsolete' (case sensitive)))) AND ((Column 'SAC Variant' contains 'OCS-EL' (case sensitive)) OR (Column 'SAC Variant' contains 'OCS-SB' (case sensitive))))) |

Table 25: DOORS EoC module identification of OCS-EL / OCS-SB

| DOORS/RMF Version: | Client 9.7.0.0 – Server 9.7.0.0 / RMF 6.1.0.9 |
| --- | --- |
| DOORS/RMF Module Name: | OCS 2.x non-red SACs |
| DOORS/RMF Module Version (Baseline): | 5.2 |
| DOORS/RMF Module Path: | /SAC-DB AT/Generic Components/Thales-AT VIE/ |
| DOORS/RMF Export View: | SAC WEXP Export View |
| DOORS/RMF Current Filter: | (((TA SAC Variant includes OCS-SB) OR (TA SAC Variant includes OCS-EL)) AND (TA SAC Status != obsolete)) AND (TA SAC Status != deleted) |

Table 26: DOORS SRAC module identification of OCS-EL / OCS-SB

#### Implementation of the conditions

| [OCS-2x-nonred:SAC:0001] | Compliance with OCS system safety handbook |
| --- | --- |
| SAC-Text: Applications using OCS 2.x non-red shall prove the compliance to the set of SACs in this document and additionally the set of SACs from the safety handbooks from subordinate systems: - Safety Application Conditions, TAS Platform OCS - 2.x, System and Red (3CR 00055 5329 DUAFQ) - Safety Application Conditions, TAS Platform Core - 2.x (3CR 00055 5070 DUAFQ). In Evidence of compliance - TAS Platform OCS 2.x non-red versus SAC (3CR 00055 5772 DUAPA) the SACs valid for an application using OCS 2.x non-red are identified. A checklist is an accepted method to document the evidence. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: |
| Implementation in ELEKTRA: | This is done in chapter 6 "Taking account of SRACs and SecRACs of underlying systems in the design of ELEKTRA" of this document. |

| [OCS-2x-nonred:SAC:0002] | Voting of any message incoming from OCS 2.x non-red |
| --- | --- |
| SAC-Text: The application shall vote any message coming from both OCS 2.x non-red software channels. In case of a mismatch the Application shall perform a Safety reaction as defined in [OCS-2x-nonred:SAC:0011]. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: |
| Implementation in ELEKTRA: | ELEKTRA will use OCS in the following way: • OCS-EL shall be used by ELEKTRA Switzerland for the connection to HITACHI RBC. OCS-EL shall run on on the computers of RCU and SCU. Therefore RCU and SCU shall run on different computers. Voting of messages coming from RBC to ELEKTRA shall be done by the RCU CHILL processes rca_rbc_controller and rcb_rbc_controller. The safety reaction in case of an error will be either a disconnect of the communication to RBC or a reboot of the computer. Details of the design will be available in document "ADD Interface IXL - RBC over OCS" [EL_RBC_OCS]. • OCS-SB shall be used by all ELEKTRAs for the connection to AzLM. OCS-SB shall run on the computers of ECs. Voting of messages coming from AzLM to ELEKTRA shall be done by the EC CHILL processes gpa_ocs_con and gpb_ocs_con. The safety reaction in case of an error will be either a disconnect of the communication to AzLM or a reboot of the computer. Details of the design will be in document "ADD IF Elektra-AzLM over OCS-SB" [EL_AZLM_OCSSB]. See also [OCS-2x-nonred:SAC:0006]. |

| [OCS-2x-nonred:SAC:0003] | Life cycle mechanism between Application and OCS-PL (and vice versa) over the same message path (message queue) as application data messages |
| --- | --- |
| SAC-Text: The Application shall send after OCS_SYNC_REPLY periodical Life Cycle messages to OCS 2.x non-red and check with a Timeout incoming Life Cycle messages from OCS 2.x non-red. In case of Timeout expiration the Safety Reaction as defined in [OCS-2x-nonred:SAC:0011] shall be performed. The communication between the Application and OCS 2.x non-red in both directions shall ensure that neither path is congested. This is provided by using the same message path for the Life Cycle messages and the User Data messages. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: |
| Implementation in ELEKTRA: | For connection to AzLm: This will be ensured by the configuration of the communication within ELEKTRA. For connection to RBC: This will not be fulfilled by the configuration. Rationale for the deviation: Based on the behaviour and based on the realisation no solution to fulfill the SAC is possible. In case a life cycle message is missing, OCS initiates a reboot. If application data messages and Llife cycle messages would use the identical path, the life cycle message to the passive redundancy would also get lost and the passive redundancy would also reboot. Therefore the life cycle messages are individually sent by each redundancy, while the application messages are only sent by the active redundancy to both redundancies using route copy. |

| [OCS-2x-nonred:SAC:0004] | Protocol support restricted to SAHARA |
| --- | --- |
| SAC-Text: The application shall use only the SAHARA protocol. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: Note that according to [EC SAHARA 59 Cat 1&2] the SAHARA protocol is restricted to category 1 and 2 networks, as defined in [EN 50159] . |
| Implementation in ELEKTRA: | This shall be ensured by the configuration provided by ELEKTRA (see also OCS:SAC:081). |

| [OCS-2x-nonred:SAC:0005] | Voting mechanism in OCS 2.x non-red |
| --- | --- |
| SAC-Text: OCS 2.x non-red votes all application messages - Channel A vs. B. Only on successful vote, the message is sent. The Application shall respect that on failed vote (data, header discrepancy) or on expired voter time-out (message from one channel excessively delayed or missing), the out-going message is not sent. In that case, OCS 2.x non-red performs its safety reaction. The application needs to keep safe as defined in [OCS-2x-nonred:SAC:0006]. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: |
| Implementation in ELEKTRA: | ELEKTRA shall behave as required by [OCS-2x-nonred:SAC:0006]. • OCS-EL: The design for the RBC interface shall take into consideration that OCS uses AND voting for all messages from ELEKTRA to RBC. In case the voting fails (messages are not equal or voting timeout), the messages will not be sent by OCS. • OCS-SB: OCS uses AND voting for all messages from ELEKTRA to RBC. The design for the AzLM shall include the following consideration: • Command axle counter reset (“Achszählergrunstellung”): If the command is not executed, ELEKTRA remains in a safe state. • Messages for general inquiry (“Generalabfrage”), version check, diagnosis request and status request (for 2v3 AzA’s): the operational state will not change if one of these messages is missing. For information about the safety reaction see [OCS-2x-nonred:SAC:0011]. |

| [OCS-2x-nonred:SAC:0006] | No safety impact to Application from OCS availability |
| --- | --- |
| SAC-Text: The Application shall guarantee, with the help of the monitoring of OCS-Application messages from both OCS 2.x non-red software channels, that the Application remains safe by occurrence of a safety reaction in OCS 2.x non-red. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: If an application channel is hosted on the same HW board as the OCS 2.x non-red software channel and OCS 2.x non-red performs a safety reaction, this will lead to a reboot of the HW board according to [OCS-2x-nonred:SAC:0010]. |
| Implementation in ELEKTRA: | ELEKTRA will either use timeout supervision for messages expected from OCS (e.g. life cycle messages) or use a voter for messages (e.g. state messages). For each of this mechanisms an appropriate safety reaction will be defined. • OCS-EL: Details of the design will be available in document "ADD Interface IXL - RBC over OCS" [EL_RBC_OCS]. • OCS-SB: Details of the design will be in document "ADD IF Elektra-AzLM over OCS-SB" [EL_AZLM_OCSSB]. See also [OCS-2x-nonred:SAC:0002]. If the OCS specific startup scripts of ELEKTRA detect that an OCS process is missing it shall initiate the killing of ELEKTRA application. |

| [OCS-2x-nonred:SAC:0009] | Check for OCS interface version |
| --- | --- |
| SAC-Text: The application shall check for the correct OCS interface version. If the major version is not identical with the parameter defined at the SW-Build of the Application, the OCS Link shall not be established. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: OCS 2.x non-red sends its interface version to the application at startup. A list of compatible versions is defined in the validation report resp. the release notes. |
| Implementation in ELEKTRA: | ELEKTRA shall check the OCS interface version at the beginning of the communication. |

| [OCS-2x-nonred:SAC:0010] | Configuration of "appl_init" |
| --- | --- |
| SAC-Text: The OCS 2.x non-red safety reaction results in an "exit" of the corresponding process. The necessary safety reaction is a HW-Reset. Therefore the configuration of the "appl_init" within the Application shall take care of the necssary hardware reboot. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: OCS 2.x non-red does not require the usage of "appl_init". Any other procedure with equivalent functionality may be used instead, too. |
| Implementation in ELEKTRA: | ELEKTRA will use “appl_init” to start the initial startup script of ELEKTRA. This initial startup script will call other ELEKTRA scripts. All these ELEKTRA scripts ensure the following actions: • Start all necessary parts of ELEKTRA. • Supervise the correct start of some parts • Wait for the stop of some parts to initiate an appropriate reaction: either kill all remaining processes and restart the application completely or initiate a reboot of the computer. |

| [OCS-2x-nonred:SAC:0011] | Safety Reaction in the Application |
| --- | --- |
| SAC-Text: If the timer for the life cycle mechanism between the application and OCS 2.x non-red (please refer to [OCS-2x-nonred:SAC:0003]) expires, or in case the voting of any message coming from the two OCS 2.x non-red software channels fails (please refer to [OCS-2x-nonred:SAC:0002]), the application shall execute a safety reaction (e.g. reboot). |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: |
| Implementation in ELEKTRA: | ELEKTRA shall define appropriate safety reactions in the following documents: • OCS-EL: Details of the design will be available in document "ADD Interface IXL - RBC over OCS" [EL_RBC_OCS]. • OCS-SB: Details of the design will be in document "ADD IF Elektra-AzLM over OCS-SB" [EL_AZLM_OCSSB]. See also [OCS-2x-nonred:SAC:0002] and [OCS-2x-nonred:SAC:0003]. |

| [OCS-2x-nonred:SAC:0012] | Configuration of ELEKTRA Personality (EP) |
| --- | --- |
| SAC-Text: The configuration of the "ELEKTRA Personality" (EP/XEP) shall provide the "route-copy" mechanism according to API User Guide, TAS Platform OCS 2.x non-red (3CR 00055 5786 PCAPA), to allow the correct voting as well in OCS 2.x non-red as in the application. "OCS_CLOSE" shall be copied to all OCS 2.x non-red software channels by means of a specific EP-path (different one as for all other messages). |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: The mechanism “route-copy” is used within the ELEKTRA application and is mentioned as example. The principle and for OCS 2.x non-red necessary configuration of communication paths is mentioned in API User Guide, TAS Platform OCS 2.x non-red (3CR 00055 5786 PCAPA). |
| Implementation in ELEKTRA: | This shall be ensured by the configuration of the communication within ELEKTRA. |

| [OCS-2x-nonred:SAC:0013] | Environment Variable not set to Test mode |
| --- | --- |
| SAC-Text: The Environment Variable SYS_FLAGS=end_to_end_checks in the application must not be set to “0” in a field version using OCS 2.x non-red. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: See API User Guide, TAS Platform OCS 2.x non-red (3CR 00055 5786 PCAPA) for details. |
| Implementation in ELEKTRA: | This shall be ensured by the ELEKTRA start-up scripts. |

| [OCS-2x-nonred:SAC:0014] | OCS 2.x non-red timeout values set to meet application requirements |
| --- | --- |
| SAC-Text: OCS 2.x non-red provides the following configurable timeouts: - OVO / DLC Time-Out: (Default = 8 sec.) - Application Life-Cycle Timeout: (Default = 20 sec.) These Timeouts have to be set by the Application in order to meet its system safety reaction time. The values have to be defined by the application and verified in the System integration test. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: See API User Guide, TAS Platform OCS 2.x non-red (3CR 00055 5786 PCAPA) for details. The SAHARA specification expects that the time stamp of a message is set when it is generated. In case of OCS 2.x non-red and TAS PLF non-red the time stamp is generated within OCS 2.x non-red (and not related to the generation of the message), which needs to be considered for the timeout considerations. |
| Implementation in ELEKTRA: | The timeouts shall be defined by the design of the different functions: • OCS-EL: The design for the RBC interface shall define the timeout values in document "ADD Interface IXL - RBC over OCS" [EL_RBC_OCS]. • OCS-SB: The design for the AzLM interface shall define the timeout values in document "ADD IF Elektra-AzLM over OCS-SB" [EL_AZLM_OCSSB]. |

| [OCS-2x-nonred:SAC:0016] | Only validated OCS communication partner |
| --- | --- |
| SAC-Text: A communication between OCS partners is only allowed, if this combination has been successfully validated. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: The validation can be done either by the Application project or is already done by TAS PLF OCS non-red Validation. |
| Implementation in ELEKTRA: | ELEKTRA shall use only validated combinations of OCS versions for communication. In case of using a not validated combination of OCS versions ELEKTRA shall do the validation as part of the baseline tests. The necessary tests shall be defined in the test plan of the ELEKTRA baseline. |

| [OCS-2x-nonred:SAC:0020] | Hardware support restricted to validated HW/SW combinations |
| --- | --- |
| SAC-Text: OCS 2.x non-red may only be used with validated HW/SW combinations. Other options or combination of options not validated (e.g. protocols) shall not be used. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: The validated HW/SW combinations are defined in the validation report. Only HW targets - D2, D6 and D10 for OCS-EL variant - D6, and D10 for OCS-SB variant are allowed. Before using OCS SB with any other hardware the suitability of the processor regarding the diversification approach (i.e. single processor with superscalar architecture) has to be investigated. |
| Implementation in ELEKTRA: | ELEKTRA will use only D6 and D10. D2 cannot be used as it does not provide the additional Ethernet interfaces needed for the OCS connections. |

| [OCS-2x-nonred:SAC:0021] | External clock supervision |
| --- | --- |
| SAC-Text: The application has to provide a supervision of the local clock based on a check with an external, physical independent clock, if the provisions of TAS Platform internal check with RT-Clock and the timing supervision of the remote partner are not sufficient for hazard control in the application context. |  |
| Status: active Variant: OCS-SB | Comment: The timing checks within OCS SB (especially SAHARA communication paths) and the possible hazards from delayed messages are the reason for this SAC. Within a consideration on application level regarding these hazards the external clock supervision may be found as not necessary. |
| Implementation in ELEKTRA: | ELEKTRA will supervise its realtime behaviour with the following mechanisms: • Reactivity Supervision (see chapter 5.18 “Supervision of reactivity”) Additionally the value for round trip time of OCS-SB used for the AzLM interface shall be defined in document "ADD IF Elektra-AzLM over OCS-SB" [EL_AZLM_OCSSB]. |

| [OCS-2x-nonred:SAC:0022] | OLT progress supervision |
| --- | --- |
| SAC-Text: The application has to provide a supervision of the successful progress and execution of the OLT in two independent ways (two software channels). |  |
| Status: active Variant: OCS-SB | Comment: This SAC is included already in [OCS-2x-nonred:SAC:0001] based on the evaluation of SACs of TAS Platform. Due to its importance for OCS SB it is mentioned here explicitely again. |
| Implementation in ELEKTRA: | ELEKTRA supervises the OLT progress in two different ways – see [CORE-2.x-SW:SAC:0097] and [CORE-2.x-SW:SAC:0098]. |

| [OCS-2x-nonred:SAC:0023] | Physical independence of subsequent system |
| --- | --- |
| SAC-Text: The subsequent system shall be physically independent from the OCS SB application (the application as well as the OCS/SAHARA communication partner). It is allowed that the physical independence is achieved by a series of applications, but in this case the direct message flow has to be shown explicitly (including voting). Between OCS SB and its subsequent application an input voting from the subsequent application is necessary in any case (see [OCS-2x-nonred:SAC:0002]). |  |
| Status: active Variant: OCS-SB | Comment: |
| Implementation in ELEKTRA: | Communication direction from ELEKTRA to AzLM: The subsequent system of ELEKTRA’s OCS-SB is AzLM. AzLM will be physically independent of an ELEKTRA computer and therefore physically independent of ELEKTRA’s OCS-SB. Communication direction from AzLM to ELEKTRA: The subsequent system of ELEKTRA’s OCS-SB is the EC application. EC will never be physically independant of ELEKTRA’s OCS-SB. The subsequent system of the application will be the application CC. CC will be physically independant of EC. For details see "ADD IF Elektra-AzLM over OCS-SB" [EL_AZLM_OCSSB]. |

| [OCS-2x-nonred:SAC:0025] | OCS 2.x non-red configuration |
| --- | --- |
| SAC-Text: The configuration of OCS 2.x non-red shall be set according to the rules given in Configuration Reference Manual, TAS Platform OCS 2.x non-red (3CR 00055 5787 PCAPA) and API User Guide, TAS Platform OCS 2.x non-red (3CR 00055 5786 PCAPA). |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: The threshold for safety code failures shall be 1, because the OCS SB communication partner has to ensure integrity of OCS SB in its function as subsequent system. Any other failure scenarios have to be considered with regard to the round-trip delay time from the application's point of view. |
| Implementation in ELEKTRA: | This shall be ensured by the configuration provided by ELEKTRA. |

| [OCS-2x-nonred:SAC:0026] | Safety case for overall system |
| --- | --- |
| SAC-Text: A safety case for the overall system, which includes OCS 2.x non-red, has to be provided. Herein it has to be shown that in the hazard analysis the integration of OCS 2.x non-red has been considered and that there exist no conflicts of OCS configuration parameters with the application's safety requirements (see also [OCS-2x-nonred:SAC:0010], [OCS-2x-nonred:SAC:0012], [OCS-2x-nonred:SAC:0014], [OCS-2x-nonred:SAC:0025]). For OCS-SB additionally the functional independence between the OCS SB application and other applications within the same hardware board needs to be considered. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: [OCS-2x-nonred:SAC:0002] supports the functional independence by input voting of the application (which is not existing for OCS 2.x non-red itself). |
| Implementation in ELEKTRA: | This shall be ensured by each country specific ELEKTRA that uses OCS 2.x non-red. |

| [OCS-2x-nonred:SAC:0027] | Supervision of master-master situation |
| --- | --- |
| SAC-Text: For all redundant configurations of OCS SB the application shall provide a supervision of the master-master connection. In case of a master-master situation the application shall shut down at least one of the redundant OCS applications (depending on available countermeasuers to control this situation the appliation may shut down both redundant OCS applications). |  |
| Status: active Variant: OCS-SB | Comment: There exist different ways to cover this topic: The hazard log PLF0004600 includes some example calculations for different time spans and their effect in terms of hazard rates, in case that no measures are implemented in the application (only supervision of connection status by field monitoring). Since OCS 2.1.1 the redundancy layer of SAHARA is able to detect different messages with the same sequence number and reacts appropriatly. It shall be used for argumentation within the application context, because the malfunction due to a master-master situation will be detected by the communication partner. With OCS 2.x non-red the application receives channel status from OCS SB to detect master-master situations, see API User Guide, TAS Platform OCS 2.x non-red (3CR 00055 5786 PCAPA), that can be used for active supervision by the application. |
| Implementation in ELEKTRA: | This shall be ensured by ELEKTRA – a master-master situation is detected by active CC. For details see "ADD IF Elektra-AzLM over OCS-SB" [EL_AZLM_OCSSB]. |

| [OCS-2x-nonred:SAC:0028] | Validation of incompatibility of application protocol |
| --- | --- |
| SAC-Text: The format of the application protocol (structure of data telegrams) when using OCS SB shall be different to the OCS protocols. |  |
| Status: active Variant: OCS-SB | Comment: Within ASD the data telegrams from application as well as incoming messages from the network are processed and sent to PL. In case of a fault confusion is handled by different structure of telegrams, in order to handle a mismatch between incoming and outgoing messages.. |
| Implementation in ELEKTRA: | This shall be shown in the design of the function of ELEKTRA. For details see "ADD IF Elektra-AzLM over OCS-SB" [EL_AZLM_OCSSB]. |

| [OCS-2x-nonred:SAC:0029] | Performance test in application context |
| --- | --- |
| SAC-Text: An application using OCS 2.x non-red, which is communicating with another application using a different OCS version has to perform a system integration test. This specifically applies to scenarios with high communication load. |  |
| Status: active Variant: OCS-EL, OCS-SB | Comment: |
| Implementation in ELEKTRA: | The necessary tests shall be defined in the test plan of the country specific ELEKTRA baseline that uses OCS 2.x non-red. |

#### Template for DOORS export SRACs OCS-EL/OCS-SB

| <<SAC PUID including Bookmark>> | <<SAC Synopsis including Bookmark>> |
| --- | --- |
| SAC-Text: <<SAC Description>> |  |
| Status: <<SAC Status>> Variant: <<SAC Variant>> | Comment: <<SAC Comments>> |
| Implementation in ELEKTRA: | <<Object Text including DocRef>> |

## Conditions CAR

CAR does not define SRACs – see chapter 6.5 of “Funktionsnachweis, ELEKTRA 2.x, Evidence of Functionality, CAR/EP” [CA_EOF_CAR]. CAR defines conditions instead in chapter 3 of [CA_EOF_CAR]. The version of CAR considered in this chapter is 5.1.2.

The following part of this chapter is based on a DOORS export of the according SRAC database.

### Identification of the DOORS databases

| DOORS/RMF Version: | Client 9.7.0.0 – Server 9.7.0.0 / RMF 6.1.0.9 |
| --- | --- |
| DOORS/RMF Module Name: | ELEKTRA PV GEN_CAR_EoC |
| DOORS/RMF Module Version (Baseline): | 1.2 |
| DOORS/RMF Module Path: | /SAC-DB AT/Generic Components/Thales-AT VIE/ |
| DOORS/RMF Export View: | SAE WEXP Export View Extended |
| DOORS/RMF Current Filter: | ((WEXP Bookmark is not empty) OR (((NOT (Column 'SAC Status' contains 'draft' (case sensitive))) AND (NOT (Column 'SAC Status' contains 'obsolete' (case sensitive)))) AND (NOT (Column 'SAC Status' contains 'deleted' (case sensitive))))) |

Table 27: DOORS EoC module identification of CAR_EP

| DOORS/RMF Version: | Client 9.7.0.0 – Server 9.7.0.0 / RMF 6.1.0.8 |
| --- | --- |
| DOORS/RMF Module Name: | CAR_EP |
| DOORS/RMF Module Version (Baseline): | 1.8 (this version is consistent to the document) |
| DOORS/RMF Module Path: | /SAC-DB AT/Generic Components/Thales-AT VIE/ |
| DOORS/RMF Export View: | SAC WEXP Export View |
| DOORS/RMF Current Filter: | ((TA SAC Status != draft) AND (TA SAC Status != obsolete)) AND (TA SAC Status != deleted) |

Table 28: DOORS SRAC module identification of CAR_EP

### Implementation of the conditions

| [CAR:GEN_COND:01] | Integration tests |
| --- | --- |
| Cond-Text: If CAR is used in a new system CAR/EP has to be tested within an integration test with the new system. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | This condition shall be part of the test design specification of ELEKTRA. |

| [CAR:GEN_COND:02] | Environment |
| --- | --- |
| Cond-Text: CAR/EP has to be re-integrated into an overall system on changes in the environment CAR/EP. I.e. new HW board is used, as CAR is released for a platform, but not for a specific HW board only. |  |
| Status: active Variant: CAR, EP, XEP | Comment: See point “x86 compatible machines” in Table 11 in chapter 2.9.2) in Evidence of Functionality, CAR/EP; 3BU 15000 5191 DUAPA, Ed 05. |
| Implementation in ELEKTRA: | The relevant boards of table 11 are MEN-D2, MEN-D6 and MEN-D10. ELEKTRA will only use these boards (see chapter 5.19.1 Table 20: HW boards of TAS-PLF to be used by ELEKTRA applications). |

| [CAR:GEN_COND:03] | Impact of Changes |
| --- | --- |
| Cond-Text: If the CAR/EP changes the impact of the changes has to be analysed. The analysis has to clarify, if new tests are necessary. Note: A new update of this document will be issued for the new CAR/EP SW version (based on the decision of the CCB). |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | This condition shall be part of the test design of the ELEKTRA baseline that uses changed CAR/EP version. |

| [CAR:APPL_COND:01] | No Safety Functions |
| --- | --- |
| Cond-Text: The application does not require any safety functions from CAR/EP. The application uses CAR/EP as test tool as described in chapter 2.8 and as communication and schedule medium on top of the operating system. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | The needs of ELEKTRA 2.x for CAR are described in chapter 5.1.3 "CAR on TAS Platform". |

| [CAR:USE_COND:01] | CAR/EP Version |
| --- | --- |
| Cond-Text: CAR 5.1.2, EP of CAR 5.1.2, XEP 5.1.2 shall only be applied when identified with identifier strings as given in Version Delivery Description Release Notes CAR 5.1.2 3BU 15000 5288 FMAPA, ED 02. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | The use of the correct CAR version will be ensured by configuration management of ELEKTRA. |

| [CAR:USE_COND:02] | CAR/EP Environment |
| --- | --- |
| Cond-Text: CAR/EP have been tested with platform 2.4.2. If used with a different version the application shall provide a reasoning for the difference |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | If CAR/EP is used with another TAS-PLF version than the one mentioned above this must be mentioned in the scope of change in test plan of ELEKTRA. |

| [CAR:USE_COND:03] | Routing Table |
| --- | --- |
| Cond-Text: The correctness of the Routing-Table must be shown. |  |
| Status: active Variant: EP, XEP | Comment: |
| Implementation in ELEKTRA: | The correctness of the configuration files for CS and EP will be shown by the tests of ELEKTRA. |

| [CAR:USE_COND:04] | TAS Platform Configuration for CAR/(X)EP |
| --- | --- |
| Cond-Text: CAR and (X)EPs configuration shall follow the rules for a TAS Platform non-redundant application (“Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform Core - 2.x” 3CR 00055 5070 DUAFQ, ED 25). |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | The intended fulfilment of SACs of TAS-PLF is provided in chapter 6.1 "SRACs TAS-PLF". |

| [CAR:USE_COND:05] | Data Base Protection |
| --- | --- |
| Cond-Text: In case the applications use the DBS service of CAR to store safety relevant data on non-volatile memory the application has to show fulfilment of SAC CORE-2.x-SW:SAC:092 from Safety Application Conditions/Sicherheitsbezogene Anwendungs Vorschrift - TAS Platform Core - 2.x 3CR 00055 5070 DUAFQ, ED 25 for that purpose. |  |
| Status: active Variant: CAR | Comment: |
| Implementation in ELEKTRA: | See [CORE-2.x-SW:SAC:0092]. |

| [CAR:USE_COND:06] | Test Tools TFU and CARCONS |
| --- | --- |
| Cond-Text: Test tools TFU5 and carcons of CAR 5.1.2 must not be applied on an application in the field. |  |
| Status: active Variant: CAR | Comment: |
| Implementation in ELEKTRA: | ELEKTRA will handle the test tools in the following way: • Tools of TFU5: These test tools shall not be contained on the flashcard. • Tool carcons: This tool shall be contained on the flashcard. It is not necessary for normal operation of ELEKTRA, but it shall be available in case debugging in the customer installation is necessary. |

| [CAR:USE_COND:07] | Environment Variables |
| --- | --- |
| Cond-Text: CAR must not be called in the field having environment variable ANNOTATE_SIGNAL set. |  |
| Status: active Variant: CAR | Comment: |
| Implementation in ELEKTRA: | The environment variable ANNOTATE_SIGNAL shall not be set in any of the startup scripts of ELEKTRA contained on the flashcard. |

| [CAR:USE_COND:09] | Includes |
| --- | --- |
| Cond-Text: An application shall use the include files of CAR 5.x provided in /elektra/486t/sybl-3os/car/releases/5.1.2/include or provide proof that its data structures defined correspond to those of the include files. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | ELEKTRA shall ensure the consistency to the include files in the following ways: • Use the appropriate include file in case the declarations of the file have to be used. • In case ELEKTRA uses its own declaration file, a consistency check shall be done at compile time to ensure the equality of the ELEKTRA declaration to the declaration of the include file. |

| [CAR:USE_COND:10] | Diagnosis Message Length |
| --- | --- |
| Cond-Text: Applications shall be aware that diagnosis messages of CAR are limited to a length of 70 bytes; the part of the diagnosis message exceeding 70 bytes, is truncated. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | This limitation will be automatically fulfilled by the correct definition of the diagnosis signal. |

| [CAR:USE_COND:11] | Hostname Length |
| --- | --- |
| Cond-Text: Hostnames shall be configured with a size of at most 20 bytes. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | The mode declaration used for a hostname shall use the limit of 20. |

| [CAR:USE_COND:12] | CHILL Compiler |
| --- | --- |
| Cond-Text: CHILL Applications executing on CAR 5.1.2 resp. EP of CAR 5.1.2 shall be compiled using GNUCHILL 2.3.5 or if not shall show that a different CHILL compiler does not matter. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | ELEKTRA will use the GNUCHILL compiler version mentioned above. In case a compiler error is detected, ELEKTRA will change to the new compiler containing the error correction. In this case an evaluation is necessary, if CAR must also be compiled with the new compiler or if it can remain unchanged. |

| [CAR:FMEA:RULE:01] | CHILL Exceptions |
| --- | --- |
| Cond-Text: The application must catch the chill exception and ensure a safe handling (e.g. calling exit itself, forwarding another exception). |  |
| Status: active Variant: CAR | Comment: As a last resort CAR would handle uncaught exceptions with the handler for unhandled exceptions. |
| Implementation in ELEKTRA: | See chapter 5.1.3 "CAR on TAS Platform". |

| [CAR:FMEA:RULE:02] | Source and Destination Check |
| --- | --- |
| Cond-Text: The application must check the message source and safety code for all safety critical messages. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | See chapter 5.14 "End-to-End Checks". |

| [CAR:FMEA:RULE:03] | Diverse Messages |
| --- | --- |
| Cond-Text: The application must use two messages for all messages that lead to a less safe state. There is no need that the messages themselves are different, but 2 messages are needed logically. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | See chapter 5.14 "End-to-End Checks". |

| [CAR:FMEA:RULE:04] | Diverse Timeouts |
| --- | --- |
| Cond-Text: The application must use two different timeouts for safety critical function. |  |
| Status: active Variant: CAR | Comment: A systematic error of the clock (e.g. no clock progress) is assumed to be detected by missing life cycles to the subsequent system. So the timeouts can be checked by two SW channels on the same HW as well as this rule aims rather at a wrong timeout function within CAR and not an error on the clock chip. |
| Implementation in ELEKTRA: | Each application channel of ELEKTRA shall use its own timeout. The length of the timeout shall fulfill the needs required by the synchronisation of CCA and CCB (see "Synchronisierung CC-Ebene, Funktionelle Spezifikation" [EL_SYNC]). |

| [CAR:FMEA:RULE:05] | Time Handling |
| --- | --- |
| Cond-Text: The application must handle the cases of persistent time over power off and the possibility to set the clock outside CAR/EP. The application must avoid setting of the real-time clock on Linux targets when using XCEL (device queues). CAR performs no time synchronisation. |  |
| Status: active Variant: CAR, EP, XEP | Comment: Call of the function ABSTIME will use the real-time clock as specified, so no rules need to be given, But for XCEL the EP generate timers sensitive to changes of the real-time clock. This tracked with CELCAR-2443 to be changed in a future release. |
| Implementation in ELEKTRA: | ELEKTRA shall provide the possibility to set the real time. The following options are possible: • The MMI of ELEKTRA provides a command for the operator to set the real time. • The time is provided by a communication partner of ELEKTRA (e.g. X25 protocol defined by customer ÖBB). • A radio controlled clock is connected to ELEKTRA to provide the actual time. |

| [CAR:FMEA:RULE:06] | Configuration of Process ID |
| --- | --- |
| Cond-Text: The route-, queue- and CAR-process configuration used by the application has to be free from errors (e.g. by reviews or tests). The checking of the configuration and the assignment of Process IDs are no safety function of CAR. In case free allocated IDs are used it is the responsibility of the application to ensure that it works correctly with these IDs. |  |
| Status: active Variant: CAR | Comment: |
| Implementation in ELEKTRA: | ELEKTRA shall not use free allocated IDs for processes and signals. For tests of the configuration files for CS and EP see [CAR:USE_COND:03]. |

| [CAR:FMEA:RULE:07] | Termination of CAR |
| --- | --- |
| Cond-Text: The application must react safe in case of the termination of CAR.CAR calls the defined exit_routine of the application: This alone must be safe even if the application is not stopped (e.g. life cycles stopped by exit-routine call). In case the application has not defined the exit_routine for CAR, CAR executes its default exit_routine. |  |
| Status: active Variant: CAR | Comment: |
| Implementation in ELEKTRA: | The termination of CAR will be detected by the startup script of ELEKTRA that has started CAR. If this script detects the termination of CAR it shall unconditionally kill the processes of EP, LWATCH and CS. |

| [CAR:FMEA:RULE:08] | Start-up Scripts |
| --- | --- |
| Cond-Text: The application is responsible for the start-up script handling and configuration of CAR. The maximum independence is provided if two applications are stared with two separate CAR commands (separate EP, separate RMT, but still common CAR library). |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | CAR provides support for one application only. If more than one ELEKTRA application (resp. channel) needs to be started on one computer, each application shall get its own CAR. One ELEKTRA application may consist of one or more executables. The startup scripts shall ensure that the configuration of each CAR meets the needs of an application and its related executables. |

| [CAR:FMEA:RULE:09] | Handle Deletion |
| --- | --- |
| Cond-Text: The application must detect the loss of all safety critical messages (e.g. feedback message handshake, sequence numbers,..). CAR cannot ensure that a message is always delivered. |  |
| Status: active Variant: CAR, EP, XEP | Comment: |
| Implementation in ELEKTRA: | ELEKTRA shall use timeout supervision and voting to detect the loss of messages. |

| [CAR:FMEA:RULE:10] | DBS Integrity |
| --- | --- |
| Cond-Text: The application must not rely only on CAR/EP (mainly DBS) for the integrity of safety relevant data themselves. |  |
| Status: active Variant: CAR | Comment: DBS secures the data via CRC, if configured, but cannot ensure the integrity of the write operations. One possibility is the storage of diverse data and voting of safety critical data at start-up. Another possibility is the check by the subsequent application which would notice a deviation between the SW channels based on the corrupted data of one channel. |
| Implementation in ELEKTRA: | See chapter 5.1.3 "CAR on TAS Platform". |

| [CAR:FMEA:RULE:11] | Error Buffer |
| --- | --- |
| Cond-Text: The application must not rely on the error buffer for safety critical reactions. |  |
| Status: active Variant: CAR | Comment: |
| Implementation in ELEKTRA: | See chapter 5.1.3 "CAR on TAS Platform". |

| [CAR:FMEA:RULE:12] | LWATCH |
| --- | --- |
| Cond-Text: The supervision of connections by EP (LWATCH) must not be used for safety critical function. It is intended for diagnose only. |  |
| Status: active Variant: EP, XEP | Comment: |
| Implementation in ELEKTRA: | See chapter 5.1.6 "LWATCH: Get diagnosis from CS about connection status". |

| [CAR:FMEA:RULE:13] | Delay or Deletion |
| --- | --- |
| Cond-Text: The application must ensure that a single delayed or lost message does not delay critical commands (e.g. by sending 2chan_com via 2 logical communication channels). |  |
| Status: active Variant: EP, XEP | Comment: |
| Implementation in ELEKTRA: | The definition of the voting rules shall ensure that a delayed or lost message does not delay the sending of a critical command. |

### Template for DOORS export Conditions CAR

| [<<SAC Legacy ID>>]$$BMK_<<SAC Legacy ID>>$$ | <<SAC Synopsis>>$$BMK_OST_<<SAC Legacy ID>>$$ |
| --- | --- |
| Cond-Text: <<SAC Description>> |  |
| Status: <<SAC Status>> Variant: <<SAC Variant>> | Comment: <<SAC Comments>> |
| Implementation in ELEKTRA: | <<Object Text including DocRef>> |

## SecRACs TAS Platform

Security-Related Application Conditions forwarded from TAS Platform are listed and discussed in the Evidence of Compliance IEC 62443-4-2 [SecEoC].

# Index of SRACs and Requirements

## Index of SRACs

The following table gives an overview of the SRACs and conditions relevant for ELEKTRA and the documents that contain considerations for the SRACs.

The keywords used for the consideration are:

- Completely: The SRAC is considered completely in the mentioned document or chapter.

- Partly: The SRAC is considered only partly in the mentioned document or chapter.

- No: The SRAC is considered not at all in a design document.

The document reference “THIS” means the actual document, otherwise it is a link to a reference document.

### Index of SRACs TAS-PLF CORE-SW

| ID | Headline | Considered in document |
| --- | --- | --- |
| [CORE-2.x-SW:SAC:0008] | Choice of a safe programming model. | Completely: THIS ch. 5.19.1 |
| [CORE-2.x-SW:SAC:0018] | Check of TaskSets. | Completely: THIS ch. 5.2.1 “General functionality” [EL_SYS_ADM] ch. 3.4.5 “Periodical Supervision of Connections” |
| [CORE-2.x-SW:SAC:0026] | Limitation of TAS Platform with regard to security | Completely for ELEKTRA internal communication: THIS [CORE-2.x-SW:SAC:0026] “Limitation of TAS Platform with regard to security” No for communication to external partners: Must be defined in a document of the interface. |
| [CORE-2.x-SW:SAC:0073] | Periodic check of Safety relevant data | Completely: THIS only in SRAC |
| [CORE-2.x-SW:SAC:0079] | Compiler restrictions | Completely: This in SRAC and [EL_VP_SSCA] |
| [CORE-2.x-SW:SAC:0085] | Safety concept for the non-redundant TAS Platform configuration | Completely: [EL_SC] |
| [CORE-2.x-SW:SAC:0092] | Limitations while using permanent storage. | Completely: THIS ch. 5.19.2 |
| [CORE-2.x-SW:SAC:0096] | Absence of tolerance to human errors | No: Has to be described in maintenance documents |
| [CORE-2.x-SW:SAC:0097] | Neighbourhood check for startup-test | Completely: THIS ch. 5.19.2 and ch. 5.19.3 |
| [CORE-2.x-SW:SAC:0098] | Safety reaction associated to delayed success of on-line tests | Completely: THIS ch. 5.19.2 and ch. 5.19.3 |
| [CORE-2.x-SW:SAC:0109] | Restrictions about the single user mode | No: Has to be described in maintenance documents |
| [CORE-2.x-SW:SAC:0112] | boot configuration | Completely: THIS ch. 5.19.1 |
| [CORE-2.x-SW:SAC:0114] | Neighbourhood and version checks | No (for TAS-PLF and CAR): TAS-PLF and CAR is ensured by configuration management of ELEKTRA. Completely (for ELEKTRA SW and station data)): [EL_SYS_ADM] ch. 3.4.6 |
| [CORE-2.x-SW:SAC:0115] | Only relative timers are allowed for use in safety relevant applications | Completely: THIS only in SRAC |
| [CORE-2.x-SW:SAC:0123] | Tracing tools allowed for lab usage only | Completely: THIS only in SRAC |
| [CORE-2.x-SW:SAC:0151] | Compliance to restrictions of Programmers Guide | Completely: [EL_VP_SSCA] |
| [CORE-2.x-SW:SAC:0157] | Avoiding erroneous combinations of TAS Platform Core versions and configuration data | No: ensured by configuration management of ELEKTRA |
| [CORE-2.x-SW:SAC:0158] | Handling components with different Software Integrity Levels. | Completely: THIS only in statement of SRAC. |
| [CORE-2.x-SW:SAC:0172] | QoS Priority usage | Completely: THIS ch. 5.2.1, ch. 5.15, ch. 5.18 [EL_SYS_ADM] ch. 3.4.5, ch. 3.4.13, ch. 3.4.12, ch. 3.4.17, ch. 3.4.18 |
| [CORE-2.x-SW:SAC:0173] | MD5 Files for protected partitions or image files to be produced off-line. | Completely: THIS ch. 5.19.2 |
| [CORE-2.x-SW:SAC:0177] | Check of safety relevant data during start-up | Completely: THIS ch. 5.19.2 |
| [CORE-2.x-SW:SAC:0180] | Adherence to realtime constraints | Completely: THIS ch. 5.18 and ch. 5.15 |
| [CORE-2.x-SW:SAC:0182] | Non safe diagnosis data | Completely: THIS ch. 5.19.1 |
| [CORE-2.x-SW:SAC:0183] | No impact of diagnosis on real time properties | Completely: THIS ch. 5.19.1 |
| [CORE-2.x-SW:SAC:0184] | No impact of diagnosis on safety critical operation | Completely: THIS ch. 5.19.1 |
| [CORE-2.x-SW:SAC:0188] | Priority of application threads | Completely: THIS ch. 5.19.4 |
| [CORE-2.x-SW:SAC:0191] | Data download via network | Completely: [SE_ADD_SW_GEN] for EEC. No download for other components |
| [CORE-2.x-SW:SAC:0192] | Non-“elf” text segments shall be tested by the application for data integrity | Completely: THIS ch. 5.19.2 |
| [CORE-2.x-SW:SAC:0194] | Restricted settings in field mode | Completely: THIS ch. 5.19.2 |
| [CORE-2.x-SW:SAC:0196] | Check for validated hardware | Partly: THIS ch. 5.19.1 defines the supported HW boards. Restrictions concerning the allowed mix of the HW boards are not part of a design document. |
| [CORE-2.x-SW:SAC:0197] | Safe digital I/O | Completely: THIS ch. 5.19.1 |

### Index of SRACs TAS-PLF CORE-HW

| ID | Headline | Considered in document |
| --- | --- | --- |
| [CORE-2.x-HW:SAC:0001] | Reliability and availability calculations of the applications | No: ELEKTRA will get its specific RAM analysis. |
| [CORE-2.x-HW:SAC:0002] | Exchange of the fan rack every 5 years (D2 and D6 only) | No: Necessary exchanges of the fans will be defined in maintenance manuals. |
| [CORE-2.x-HW:SAC:0008] | Components without fault detection measures | Completely: THIS only in statement of SRAC |
| [CORE-2.x-HW:SAC:0009] | Additional interfaces without fault detection measures | Completely: THIS only in statement of SRAC |
| [CORE-2.x-HW:SAC:0010] | Additional functions without fault detection measures | Completely: THIS only in statement of SRAC |
| [CORE-2.x-HW:SAC:0011] | Hardware installation (D2, D6 and CIQ6 only) | No: The cabinet design and its tests have to observe this SRAC. |
| [CORE-2.x-HW:SAC:0012] | Environmental conditions (Core System D2) | No: The environmental conditions shall be defined by product ELEKTRA. |
| [CORE-2.x-HW:SAC:0015] | Protection against unauthorized access | No: The protection against unauthorized access shall be ensured by the customer. |
| [CORE-2.x-HW:SAC:0017] | The BIOS / U-BOOT software shall remain unchanged | Completely: THIS only in statement of SRAC. |
| [CORE-2.x-HW:SAC:0018] | No replacement under voltage | No: HW maintenance will be described in maintenance manuals. |
| [CORE-2.x-HW:SAC:0019] | Visual inspection of the fans (D2 and D6 only) | No: Necessary checks of the fans will be defined in maintenance manuals. |
| [CORE-2.x-HW:SAC:0020] | Supported hardware configurations | Completely: THIS ch. 5.19.1 |
| [CORE-2.x-HW:SAC:0021] | 230 V (AC) power supply 3CR 30604 ADAA (D6 only) | No: The cabinet design and its tests have to observe this SRAC. |
| [CORE-2.x-HW:SAC:0022] | Personal safety | No: The necessary measures will be defined on product level. |
| [CORE-2.x-HW:SAC:0023] | Repair or firmware update of components only at manufacturer | No: This restriction will be part of the repair process. |
| [CORE-2.x-HW:SAC:0024] | Safety qualification tests | No: ELEKTRA and EEC will do qualification test for the cabinets. |
| [CORE-2.x-HW:SAC:0025] | Suitably trained personnel | No: Training is not part of ELEKTRA development. HW maintenance will be described in maintenance manuals. |
| [CORE-2.x-HW:SAC:0028] | Insulation coordination (D2 and D6 only) | No: The cabinet design and its tests have to observe this SRAC. |
| [CORE-2.x-HW:SAC:0029] | Extension of the power supply | No: The cabinet design and its tests have to observe this SRAC. |
| [CORE-2.x-HW:SAC:0030] | Common mode influence of the power supplies | No: The cabinet design and its tests have to observe this SRAC. |
| [CORE-2.x-HW:SAC:0033] | Environmental conditions (Core System D6) | No: The environmental conditions shall be defined by product ELEKTRA. |
| [CORE-2.x-HW:SAC:0037] | Usage of the diagnosis interface on front for application specific purposes | Completely: THIS only in statement of SRAC. |
| [CORE-2.x-HW:SAC:0039] | Environmental conditions (Core System EPCM) | No: The environmental conditions shall be defined as part of the development of the EEC HW. |
| [CORE-2.x-HW:SAC:0040] | Prevention of electrostatic discharge | No: Grounding concept will be part of the design of the cabinets. HW handling will be described in maintenance manuals. |
| [CORE-2.x-HW:SAC:0042] | Non safe diagnosis | Completely: THIS only in statement of SRAC. |
| [CORE-2.x-HW:SAC:0048] | Environmental conditions (SCC) | No: The environmental conditions shall be defined as part of the development of the EEC HW. |
| [CORE-2.x-HW:SAC:0170] | Refresh of 128 MB CF memory cards | Completely: THIS only in statement of SRAC. |
| [CORE-2.x-HW:SAC:0172] | Download to (creation of) loading medium | Completely: THIS only in statement of SRAC for ELEKTRA. [SE_ADD_SW_GEN] for EEC. |
| [CORE-2.x-HW:SAC:0173] | Refresh of programmed memory cards | Completely: THIS only in statement of SRAC. |
| [CORE-2.x-HW:SAC:0174] | Power supply concept | No: The cabinet design and its tests have to observe this SRAC. |
| [CORE-2.x-HW:SAC:0198] | No virtualization for safety relevant operation | Completely: THIS ch. 5.19.1 |
| [CORE-2.x-HW:SAC:0199] | Additional interfaces without fault detection measures (D10, SC33) | Completely: THIS only in statement of SRAC |
| [CORE-2.x-HW:SAC:0200] | Additional functions without fault detection measures (D10, SC33) | Completely: THIS only in statement of SRAC |
| [CORE-2.x-HW:SAC:0201] | Environmental conditions | No: The environmental conditions shall be defined by product ELEKTRA. |

### Index of SRACs OCS System 2.x

| ID | Headline | Considered in document |
| --- | --- | --- |
| OCS:SAC:008 | Complementarity of Safety Handbook Core to Safety Handbook OCS | Completely: THIS ch. 6.1. |
| OCS:SAC:009 | Application hazard analysis and OCS integration. | No: Hazard analysis documents are available |
| OCS:SAC:015 | Framework: Multiple OCS instantiation forbidden. | No: OCS-EL: [EL_RBC_OCS] OCS-SB: [EL_AZLM_OCSSB] |
| OCS:SAC:025 | Safety code configuration. | Completely: THIS |
| OCS:SAC:028 | Failure detection time for SAHARA - receiving | Completely: THIS |
| OCS:SAC:044 | SAHARA with UDP transport layer only | Completely: THIS in SAC |
| OCS:SAC:048 | Use of the official configuration template mandatory | Completely: THIS in SRAC |
| OCS:SAC:072 | Loss of communication not necessarily notified in the specified time. | Completely: THIS in SRAC |
| OCS:SAC:075 | Configuration of authentication for SAHARA | Completely: THIS |
| OCS:SAC:076 | Guarantee of delivery not given | Completely: THIS in SRAC |
| OCS:SAC:079 | Setting MD5 checks in OCS configuration files | Completely: THIS ch. 5.19.2. |
| OCS:SAC:080 | Platform Core real-time supervision necessary for OCS | Completely: THIS in SRAC |
| OCS:SAC:081 | Limitations toward security | Completely: THIS |
| PSAH:SAC:013 | Lower half of MD4 safety code reserved for safety relevant communication | Completely: THIS |
| PSAH:SAC:014 | Deployment restricted to transmission systems according to categories 1 and 2 | Completely: THIS |

### Index of SRACs OCS-EL / OCS-SB

| ID | Headline | Considered in document |
| --- | --- | --- |
| [OCS-2x-nonred:SAC:0001] | Compliance with OCS system safety handbook | Completely: THIS ch. 6 |
| [OCS-2x-nonred:SAC:0002] | Voting of any message incoming from OCS 2.x non-red | Completely: OCS-EL: [EL_RBC_OCS] OCS-SB: [EL_AZLM_OCSSB] |
| [OCS-2x-nonred:SAC:0003] | Life cycle mechanism between Application and OCS-PL (and vice versa) over the same message path (message queue) as application data messages | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0004] | Protocol support restricted to SAHARA | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0005] | Voting mechanism in OCS 2.x non-red | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0006] | No safety impact to Application from OCS availability | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0009] | Check for OCS interface version | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0010] | Configuration of "appl_init" | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0011] | Safety Reaction in the Application | Completely: OCS-EL: [EL_RBC_OCS] OCS-SB: [EL_AZLM_OCSSB] |
| [OCS-2x-nonred:SAC:0012] | Configuration of ELEKTRA Personality (EP) | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0013] | Environment Variable not set to Test mode | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0014] | OCS 2.x non-red timeout values set to meet application requirements | Completely: OCS-EL: [EL_RBC_OCS] OCS-SB: [EL_AZLM_OCSSB] |
| [OCS-2x-nonred:SAC:0016] | Only validated OCS communication partner | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0020] | Hardware support restricted to validated HW/SW combinations | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0021] | External clock supervision | Completely: THIS ch. 5.18 OCS-EL: [EL_RBC_OCS] OCS-SB: [EL_AZLM_OCSSB] |
| [OCS-2x-nonred:SAC:0022] | OLT progress supervision | Completely: THIS ch. 6.1.1 |
| [OCS-2x-nonred:SAC:0023] | Physical independence of subsequent system | Completely: THIS in SRAC OCS-SB: [EL_AZLM_OCSSB] |
| [OCS-2x-nonred:SAC:0025] | OCS 2.x non-red configuration | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0026] | Safety case for overall system | Completely: THIS in SRAC |
| [OCS-2x-nonred:SAC:0027] | Supervision of master-master situation | Completely: THIS in SRAC OCS-SB: [EL_AZLM_OCSSB] |
| [OCS-2x-nonred:SAC:0028] | Validation of incompatibility of application protocol | No: OCS-SB: [EL_AZLM_OCSSB] |
| [OCS-2x-nonred:SAC:0029] | Performance test in application context | Completely: THIS in SRAC |

### Index of Conditions CAR

| ID | Headline | Considered in document |
| --- | --- | --- |
| [CAR:APPL_COND:01] | No Safety Functions | Completely: THIS ch. 5.1.3. |
| [CAR:FMEA:RULE:01] | CHILL Exceptions | Completely: THIS ch. 5.1.3. |
| [CAR:FMEA:RULE:02] | Source and Destination Check | Completely: THIS ch. 5.14. |
| [CAR:FMEA:RULE:03] | Diverse Messages | Completely: THIS ch. 5.14. |
| [CAR:FMEA:RULE:04] | Diverse Timeouts | Completely: THIS in condition. [EL_SYNC]. |
| [CAR:FMEA:RULE:05] | Time Handling | Completely: THIS in condition. |
| [CAR:FMEA:RULE:06] | Configuration of Process ID | Completely: THIS in condition. |
| [CAR:FMEA:RULE:07] | Termination of CAR | Completely: THIS in condition. |
| [CAR:FMEA:RULE:08] | Start-up Scripts | Completely: THIS in condition. |
| [CAR:FMEA:RULE:09] | Handle Deletion | Completely: THIS in condition. |
| [CAR:FMEA:RULE:10] | DBS Integrity | Completely: THIS ch. 5.1.3. |
| [CAR:FMEA:RULE:11] | Error Buffer | Completely: THIS ch. 5.1.3. |
| [CAR:FMEA:RULE:12] | LWATCH | Completely: THIS ch. 5.1.6. |
| [CAR:FMEA:RULE:13] | Delay or Deletion | Completely: THIS in condition. |
| [CAR:GEN_COND:01] | Integration tests | No: See test design specification. |
| [CAR:GEN_COND:02] | Environment | Completely: THIS ch. 5.19.1 |
| [CAR:GEN_COND:03] | Impact of Changes | No: See test design specification. |
| [CAR:USE_COND:01] | CAR/EP Version | Completely: THIS in condition. |
| [CAR:USE_COND:02] | CAR/EP Environment | Completely: THIS in condition. |
| [CAR:USE_COND:03] | Routing Table | Completely: THIS in condition. |
| [CAR:USE_COND:04] | TAS Platform Configuration for CAR/(X)EP | Completely: THIS ch. 6.1. |
| [CAR:USE_COND:05] | Data Base Protection | Completely: THIS in condition. |
| [CAR:USE_COND:06] | Test Tools TFU and CARCONS | Completely: THIS in condition. |
| [CAR:USE_COND:07] | Environment Variables | Completely: THIS in condition. |
| [CAR:USE_COND:09] | Includes | Completely: THIS in condition. |
| [CAR:USE_COND:10] | Diagnosis Message Length | Completely: THIS in condition. |
| [CAR:USE_COND:11] | Hostname Length | Completely: THIS in condition. |
| [CAR:USE_COND:12] | CHILL Compiler | Completely: THIS in condition. |

### Macro to extract bookmarks defined in the document

The following VB macro generates a table containing references to all bookmarks defined in this document. The generated table is stored in a document where the filename starts with “Bookmarks in”. This generated table is provided to easily check the completeness of tables in the previous chapters.

Macro “ExtractBookmarksInADoc” as file:

Macro “ExtractBookmarksInADoc” as source code:

Attribute VB_Name = "Module1"

Sub ExtractBookmarksInADoc()

Dim xRow As Long

Dim xTable As Table

Dim xDoc As Document

Dim xBookMark As Bookmark

Dim xBookMarkDoc As Document

Dim xParagraph As Paragraph

Set xDoc = ActiveDocument

If xDoc.Bookmarks.Count = 0 Then

MsgBox "There is no bookmark in this document", vbInformation, "KuTools for Word"

Exit Sub

End If

Set xBookMarkDoc = Documents.Add

xRow = 1

Selection.TypeText "BookMarks in " & "'" & xDoc.Name & "'"

Set xTable = Selection.Tables.Add(Selection.Range, 1, 3)

xTable.Borders.Enable = True

With xTable

.Cell(xRow, 1).Range.Text = "Name"

.Cell(xRow, 2).Range.Text = "Texts"

.Cell(xRow, 3).Range.Text = "Page Number"

For Each xBookMark In xDoc.Bookmarks

xTable.Rows.Add

xRow = xRow + 1

.Cell(xRow, 1).Range.Text = xBookMark.Name

.Cell(xRow, 2).Range.Text = xBookMark.Range.Text

.Cell(xRow, 3).Range.Text = xBookMark.Range.Information(wdActiveEndAdjustedPageNumber)

xDoc.Hyperlinks.Add Anchor:=.Cell(xRow, 3).Range, Address:=xDoc.Name, _

SubAddress:=xBookMark.Name, TextToDisplay:=.Cell(xRow, 3).Range.Text

Next

End With

xBookMarkDoc.SaveAs xDoc.Path & "\" & "Bookmarks in " & xDoc.Name

End Sub

## Index of Requirements

DPR:0201 (TRT285)	54

DPR:0202 (TRT286)	54

DPR:0203 (TRT287)	54

DPR:0204 (TRT288)	55

DPR:0205 (TRT289)	55

DPR:0206 (TRT290)	55

DPR:0207 (TRT291)	55

DPR:0209 (TRT293)	55

DPR:0212 (TRT296)	55

DPR:0216 (TRT304)	54

DPR:0222 (TRT313)	50

DPR:0223 (TRT315)	49

DPR:0224 (TRT316)	27, 67

DPR:0228 (TRT321)	54

DPR:0229 (TRT322)	54

DPR:0314 (TRT5)	48

DPR:0316 (TRT6)	28, 49

DPR:0401 (TRT218)	55

DPR:0810 (TRT8)	49

DPR:1001 (TRT229)	55

ELSEC:SRS:REQ:0003	30

ELSEC:SRS:REQ:0004	30

ELSEC:SRS:REQ:0008	30

ELSEC:SRS:REQ:0009	30

ELSEC:SRS:REQ:0012	30, 37

ELSEC:SRS:REQ:0014	30, 35, 37

ELSEC:SRS:REQ:0015	30

ELSEC:SRS:REQ:0016	30

ELSEC:SRS:REQ:0017	30

ELSEC:SRS:REQ:0018	35, 79

ELSEC:SRS:REQ:0023	30

ELSEC:SRS:REQ:0024	79

FUR:0117 (TRT18)	43

FUR:0118 (TRT19)	43

FUR:0119 (TRT20)	76

FUR:0120 (TRT21)	40

FUR:0123 (TRT22)	76

FUR:0124 (TRT23)	76

FUR:0125 (TRT24)	76

FUR:0126 (TRT25)	76

FUR:0127 (TRT26)	51

FUR:0128 (TRT27)	51

FUR:0210 (TRT294)	55

FUR:0211 (TRT295)	55

FUR:0306 (TRT44)	35, 39

FUR:0307 (TRT45)	35, 39

FUR:0309 (TRT46)	35, 39

FUR:0330 (TRT47)	41, 50

FUR:0331 (TRT48)	41, 50

FUR:0332 (TRT49)	39

FUR:0333 (TRT50)	51, 62

FUR:0334 (TRT51)	51, 62

FUR:0337 (TRT52)	34, 62

FUR:0338 (TRT53)	34, 41

FUR:0339 (TRT54)	34, 40

FUR:0341 (TRT55)	35, 62

FUR:0343 (TRT56)	62

FUR:0344 (TRT57)	62

FUR:0345 (TRT58)	39, 62

FUR:0346 (TRT59)	50

FUR:0350 (TRT60)	50

FUR:0354 (TRT61)	39

FUR:0355 (TRT62)	39

FUR:0357 (TRT63)	39, 41, 43

FUR:0362 (TRT65)	63

FUR:0363 (TRT66)	27, 45, 63

FUR:0364 (TRT67)	53

FUR:0367 (TRT69)	52, 63

FUR:0368 (TRT70)	52

FUR:0369 (TRT71)	53

FUR:0370 (TRT72)	52

FUR:0372 (TRT74)	53

FUR:0803 (TRT86)	25

FUR:0804 (TRT279)	26

FUR:0814 (TRT89)	41, 50

FUR:0821 (TRT90)	63

FUR:0825 (TRT92)	63

FUR:0829 (TRT299)	63

FUR:0830 (TRT302)	63

FUR:0843 (TRT446)	62

FUR:0846 (TRT667)	43

FUR:0847 (TRT668)	43

FUR:0848 (TRT669)	43

FUR:1103 (TRT235)	53

FUR:1104 (TRT236)	53

FUR:1105 (TRT284)	52

FUR:1106 (TRT506)	26, 28, 57

FUR:1107 (TRT507)	26, 27, 29, 55, 57

FUR:1303 (TRT105)	53

FUR:1304 (TRT106)	52

FUR:1574 (TRT219)	63

FUR:1575 (TRT220)	40

FUR:1576 (TRT221)	38

FUR:1577 (TRT317)	63

FUR:1578 (TRT1433)	78

FUR:1701 (TRT222)	63

FUR:1703 (TRT224)	49

FUR:1929 (TRT662)	59

FUR:3001 (TRT484)	57

FUR:3004 (TRT487)	57

FUR:3005 (TRT488)	57

FUR:3006 (TRT489)	57

FUR:3026 (LRT297)	60

FUR:3801 (TRT517)	28, 54

FUR:3802 (TRT518)	28, 54

FUR:3803 (TRT519)	28, 54

FUR:3804 (TRT520)	29, 57

FUR:3805 (TRT521)	29, 57

FUR:3806 (TRT522)	29, 57

FUR:4101 (TRT683)	32, 60

FUR:4107 (TRT1100)	58, 59

FUR:4200 (TRT1330)	63

FUR:4400 (TRT721)	61

FUR:4401 (TRT722)	61

FUR:4403 (TRT964)	61

FUR:4404 (TRT971)	61

FUR:4405 (TRT970)	61

FUR:4406 (TRT969)	61

FUR:4420 (TRT1106)	61

FUR:4421 (TRT1107)	61

FUR:4422 (TRT1110)	61

FUR:4423 (TRT1109)	62

FUR:4430 (TRT1118)	62

FUR:4431 (TRT1119)	62

FUR:4436 (TRT1127)	62

FUR:6205 (TRT1088)	55

GCR:0162 (TRT107)	76

GCR:0301 (TRT110)	50

GCR:0303 (TRT111)	41, 50

GCR:0304 (TRT112)	27, 50

GCR:0305 (TRT113)	27, 45

GCR:0347 (TRT115)	39, 40, 41, 43, 50

GCR:0348 (TRT116)	39

GCR:0349 (TRT117)	50

GCR:0352 (TRT118)	45

GCR:0359 (TRT119)	45

GCR:0361 (TRT120)	63

GCR:0812 (TRT124)	34

GCR:0823 (TRT1090)	63

MDR:0213 (TRT297)	45

MDR:0311 (TRT128)	39, 45

MDR:1002 (TRT230)	45

MDR:1003 (TRT231)	54

MDR:1007 (TRT245)	45

MDR:1009 (TRT247)	54

MDR:1010 (TRT248)	54

MDR:1011 (TRT249)	54

MDR:1012 (TRT250)	54

MDR:1019 (TRT283)	54

MIR:0136 (TRT145)	53

MIR:0317 (TRT152)	47

MIR:0318 (TRT153)	63

OAR:1004 (TRT232)	26

OAR:1005 (TRT243)	26

OAR:1008 (TRT246)	45

OAR:1013 (TRT251)	54

OAR:1016 (TRT280)	26

OAR:1017 (TRT281)	26

OAR:1018 (TRT282)	26

PRR:0319 (TRT162)	69

PRR:0320 (TRT163)	48

PRR:0321 (TRT164)	49

PRR:0322 (TRT165)	39, 40, 41, 43

PRR:0323 (TRT166)	54

PRR:0324 (TRT167)	54

PRR:0602 (TRT228)	54

PRR:4906 (TRT1436)	78

SAR:0214 (TRT298)	55

SAR:0315 (TRT175)	48

SAR:0335 (TRT176)	54

SAR:0353 (TRT178)	45, 48

SAR:0402 (TRT238)	45

SAR:0403 (TRT241)	45

SAR:0404 (TRT242)	67

SAR:0405 (TRT269)	67

SAR:0406 (TRT311)	59

SAR:0807 (TRT184)	29

SAR:1101 (TRT505)	28, 30, 49

SAR:1102 (TRT512)	49

SAR:1201 (TRT189)	28, 30, 48, 49

SAR:1202 (TRT190)	53

SAR:1204 (TRT192)	30, 50

SAR:1205 (TRT193)	30, 50

SAR:1207 (TRT195)	53

SAR:1208 (TRT196)	53

SAR:1209 (TRT197)	53

SAR:1211 (TRT276)	45

SAR:1212 (TRT277)	45

SAR:1213 (TRT278)	67

SAR:1214 (TRT314)	49

SAR:1901 (TRT663)	59

SAR:1902 (TRT664)	59

SAR:2501 (TRT659)	58

SAR:2502 (TRT660)	58

SAR:2504 (TRT1071)	58

SAR:2505 (TRT1070)	58

SAR:2506 (TRT1068)	58

SAR:3001 (TRT490)	57

SAR:3025 (LRT296)	60

SAR:3027 (LRT298)	60

SAR:3028 (LRT299)	60

SAR:3031 (LRT302)	61

SAR:3032 (LRT303)	61

SAR:4411 (TRT963)	61

TIR:0308 (TRT209)	40

TIR:6700 (TRT1435)	78

TVR:1102 (TRT234)	53

END OF DOCUMENT