System Requirements Specification           ELEKTRA 2.x

ELEKTRA Generic

| APPROVAL |  |  |
| --- | --- | --- |
|  | Name and Role/Job Title | Esignature |
| Written by | M. Schneider / RM |  |
| Verified by | C. Janneschitz / QAM |  |
| Approved by | A. Paul / PDA |  |

| LOG OF CHANGES |  |  |  |
| --- | --- | --- | --- |
| Revision | Date | Author | Change Description |
| 01 | 1998-10-28 | AUTVA/RTSS PAM (DM) D. Kral | First edition |
| 02 | 1999-03-19 | AUTVA/RTSS PAM (DM) D. Kral | Additional Requirements: |
| 03 | 2000-09-12 | AUTVA/RTSS PAM (DM) D. Kral | See Report 000904_CCB_rr.doc |
| 04 | 2000-10-17 | AUTVA/RTSS PAM (DM) D. Kral | New requirements: |
| 05 | 2001-01-31 | AUTVA/RTSS PAM (DM) D. Kral | Adjustment of requirements status to database. |
| 06 | 2003-08-08 | AUTVA/RTIC RM J. Plank | Includes the generic requirements |
| 07 | 2003-09-01 | AUTVA/RTIC RM J. Plank | Includes the IMO requirements and FUR:°0831, FUR:°0843 |
| 08 | 2003-12-10 | AUTVA/RTIE RM J. Plank | Includes the requirements of OMO and redundant MMI, see chapter 16.3. |
| 09 | 2004-08-18 | AUTVA/RTIE RM J. Plank | New requirements see chapter 16.4 |
| 10 | 2004-11-24 | AUTVA/RTIE RM J. Plank | New requirements see chapter 16.5 |
| 11 | 2005-06-10 | AUTVA/RTIE RM J. Plank | New requirements see chapter 16.6 |
| 12 | 2005-10-24 | AUTVA/RTIE RM J. Plank | New/changed requirements see chapter 16.7. |
| 13 | 2006-01-13 | AUTVA/RTIE RM J. Plank | New/changed requirements see chapter 16.8. |
| 14 | 2006-03-06 | AUTVA/RBII RM G. Eber | New/changed requirements see chapter 16.9. |
| 15 | 2006-09-06 | AUTVA/RBII RM G. Eber | Slight changes due to replacement of HUB by switch see chapter 16.10 |
| 16 | 2006-11-09 | ATS-AT/RBII RM G. Eber | New safety requirements concerning FBI handler safety concept |
| 17 | 2007-11-22 | TRSS-AT/RBII RM G. Eber | Update due to consideration of D6. See chapter 16.11 |
| 18 | 2008-03-14 | TRSS-AT/RBII RM G. Eber | New requirements concerning ehternet interface DGP – Webride New requirements concerning monitoring of reactivity of ELEKTRA system components. Further details see chapter 16.12. |
| 19 | 2008-10-15 | TRSS-AT/RBII RM Ch. Cerncic | New requirements concerning ELEKTRA-ELEKTRA Communication via Ethernet LAN (required for Bulgaria). New requirements concerning ELEKTRA Schweiz. Further details see chapter 16.14. |
| 20 | 2009-05-07 | TRSS-AT/RBII RM Ch. Cerncic | New requirements concerning ELEKTRA Schweiz (EC - TTP Bus Interface acceleration, serial connection ELEKTRA – AzLM). Further details see chapter 16.15. |
| 21 | 2011-06-17 | Thales Austria/TDE RM R. Samsegger | Changed requirements due to ELEKTRA ÖBB BL27.10. For further details refer to chapter 16.16. |
| 22 | 2012-01-12 | Thales Austria/TDE RM R. Samsegger | Changed requirements due to ELEKTRA ÖBB BL28.00 and ELEKTRA SBB BL19.20. For further details refer to chapter 16.17 |
| 23 | 2012-06-15 | Thales Austria/TIE RM M. Gollinger / G. Eber | Changed requirements due to adaptations in chapter “Serial Connection ELEKTRA – AzLM (52)” and splitting into 0300 and 0309. Within scope of CH19.20. Modifications within scope INT01.00 regarding splitting into 0300 and 4107. For further details see chapter 16.18. |
| 24 | 201210-12 | Thales Austria/TIE RM G. Eber | Adaptations for SBB CH19.20 in chapter “Monitoring of reactivity of ELEKTRA system components (44)” For further details see chapter 16.19. |
| 25 | 2013-01-24 | Thales Austria/TIE RM M. Gollinger | Minor modification in chapter “RCU/SCU Communication between two ELEKTRA interlockings via Ethernet LAN (49)” and “Serial Connection ELEKTRA - AzLM (52)”. For further details see chapter 16.20. |
| 26 | 2013-05-15 | Thales Austria/TIE RM M. Gollinger | Adaptions for INT01.20 in chapter “Serial Connection ELEKTRA - AzLM (52)” and new requirement in chapter “Man-Machine Interface”. For further details see chapter 16.21 |
| 27 | 2014-01-15 | Thales Austria/TAI RM M. Gollinger | Adaptions in context of INT01.30 in chapter “Monitoring of reactivity of ELEKTRA system components (44)” and minor correction for INT01.20 “Man-Machine Interface”, minor correction in “Serial Connection ELEKTRA – AzLM (52)”. For further details see chapter 16.22 |
| 28 | 2015-03-16 | Thales Austria/TAS RM R. Samsegger | Updated requirements due to ELEKTRA ÖBB BL28.10 and INT 02.00 For further details refer to chapter 16.23 |
| 29 | 2015-05-20 | Thales Austria/TAS RM I. Miler | Changed requirements due to EL-INT 02.00 For further details refer to chapter 16.24 |
| 30 | 2015-09-03 | Thales Austria/TAS RM I. Miler | Adaptions in context of EL-INT 02.00 in chapter “Monitoring of reactivity of ELEKTRA system components (44)” and update of requirements regarding to CAR. For further details refer to chapter 16.25 |
| 31 | 2015-12-04 | Thales Austria/TAS RM I. Miler | Corrections in context in chapter “Monitoring of reactivity of ELEKTRA system components (44)” and minor single corrections. For further details refer to chapter 16.26 |
| 32 | 2016-01-20 | Thales Austria/TAS RM I. Miler | Corrections in context in chapter “Monitoring of reactivity of ELEKTRA system components (44)”. For further details refer to chapter 16.27 |
| 33 | 2016-02-29 | Thales Austria/TAS RM I. Miler | Released per signature. Adaptions in context in chapter “Monitoring of reactivity of ELEKTRA system components (44)”. For further details refer to chapter 16.28 |
| 34 PD 01 | 2018-01-25 | Thales Austria/TEJS RM I. Miler | Proposal version with adaptions in context of ELOBB BL29.01, ELINT RC18.04.00 and ELSBB BL21.30. For further details refer to chapter 16.29 |
| 34 | 2018-02-02 | Thales Austria/TEJS RM I. Miler | Released version with adaptions in context of ELOBB BL29.01, ELINT RC18.04.00 and ELSBB BL21.30. For further details refer to chapter 16.29 |
| 35 | 2019-03-15 | Thales Austria/TEJS RM G. Eber | Released edition for PV GEN 2.19.0. For further details refer to chapter 16.30. |
| 36 | 2019-11-06 | Thales Austria/TEJS RM G. Eber | Released edition in context of CH21.40. For further details refer to chapter 16.31. |
| 37 | 2020-06-29 | Thales Austria/TEJS RM G. Eber | Released edition in context of GenSys 2.19.0. For further details refer to chapter 16.32. RELEASE PER SIGNATURE. |
| 38 | 2021-04-21 | Thales Austria/TEJS RM R. Samsegger | Released edition in context of GenSys 2.19.0 and ELOBB BL28.30 For further details refer to chapter 16.33 |
| 39 | 2022-09-09 | Thales Austria/TEJS RM G. Eber | Released edition in context of GenSys 2.20.0. For further details refer to chapter 16.34. |
| 40 | 2026-07-15 | Hitachi Austria/OEDCR RM M. Schneider | Released edition in context of GenSys 2.20.10. For further details refer to chapter 16.35. |

Contents

1	PREFACE	12

1.1	Purpose	12

1.2	Intended Audience	12

1.3	Information for Document Generation	12

1.4	Referenced Documents	13

1.5	Abbreviations used by Requirement report	14

1.6	Abbreviations	14

1.7	Definitions	14

2	OVERVIEW	16

2.1	Evolution of the System Requirements Specification	16

2.2	Structure of the Requirements	16

2.3	Abbreviations of Categories of Requirements (= Types of Requirements)	19

2.4	Abbreviations of Customers	22

3	GENERAL DESCRIPTION	23

4	Technical Requirements	24

4.1	Functional Requirements	24

4.1.1	General Requirements	24

4.1.2	Journaling (43)	63

4.1.3	Monitoring of Reactivity of ELEKTRA system components (44)	63

4.1.4	Enhancement Interface DGP-Webride (MDS) (45)	71

4.1.5	RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN (49)	73

4.1.6	Serial Connection ELEKTRA - AzLM (52)	75

4.1.7	DGP Enhancements (53)	79

4.1.8	Remote Control: decentralized architecture (62)	79

4.1.9	ELEKTRA 2 components on ELEKTRA 1 (67.1)	80

4.2	Operational Requirements	80

4.2.1	RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN  (49)	84

4.3	Constraints	85

5	Interfaces	92

5.1	Technical Interface	92

5.1.1	Enhancement Interface DGP-Webride (45)	92

5.1.2	RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN  (49)	93

5.1.3	EC - TTP Bus Interface (51)	93

5.1.4	Serial Connection ELEKTRA - AzLM (52)	95

5.2	User Interface / Man-Machine Interface	99

6	Requirements to the Customer	107

6.1	Serial Connection ELEKTRA - AzLM (52)	107

7	Safety	108

7.1	Monitoring of reactivity of ELEKTRA system components (44)	125

7.2	Enhancement Interface DGP-Webride (45)	126

7.3	Serial Connection ELEKTRA - AzLM (52)	126

7.4	CHILL Instruction Test & Supervision of CHILL Instruction Test (68)	129

7.4.1	CHILL Instruction Test	129

7.4.2	Supervision of CHILL Instruction Test	131

8	Security	134

9	Reliability	135

9.1	Serial Connection ELEKTRA - AzLM (52)	144

10	System Integration and Acceptance	145

11	Putting into Operation	146

12	Diagnosis and Maintenance	149

13	Documentation	154

14	Tool Chain	155

15	OPEN ISSUES	157

16	HISTORY	158

16.1	Edition 06	158

16.2	Edition 07	158

16.3	Edition 08	158

16.4	Edition 09	158

16.5	Edition 10	158

16.6	Edition 11	158

16.7	Edition 12	158

16.8	Edition 13	159

16.9	Edition 14	159

16.10	Edition 15	159

16.11	Edition 16	159

16.12	Edition 17	160

16.13	Edition 18	160

16.14	Edition 19	160

16.15	Edition 20	160

16.16	Edition 21	161

16.16.1	Miscellaneous	161

16.16.2	New Requirements	161

16.16.3	Modified Requirements	162

16.17	Edition 22	164

16.17.1	Miscellaneous	164

16.17.2	New Requirements	164

16.17.3	Modified Requirements	165

16.17.4	Deleted Requirements	167

16.18	Edition 23	167

16.19	Edition 24	168

16.20	Edition 25	169

16.21	Edition 26	169

16.22	Edition 27	169

16.22.1	Edition 27 Proposal 01	169

16.22.2	Edition 27 Proposal 02	169

16.22.3	Edition 27 Proposal 03	170

16.22.4	Edition 27 Released	170

16.23	Edition 28	170

16.23.1	Miscellaneous	170

16.23.2	New Requirements	170

16.23.3	Modified Requirements	170

16.24	Edition 29	172

16.24.1	New Requirements	172

16.24.2	Modified Requirements	172

16.25	Edition 30	174

16.25.1	Modified Requirements	174

16.25.2	New Requirements	175

16.26	Edition 31	175

16.26.1	Edition 31 Proposal 01	175

16.26.2	Edition 31 Released	176

16.27	Edition 32	176

16.27.1	Edition 32 Proposal 01	176

16.27.2	Edition 32 Released	176

16.28	Edition 33	176

16.29	Edition 34	178

16.29.1	Edition 34 Proposal 01	178

16.29.2	Edition 34 Released	179

16.30	Edition 35	179

16.31	Edition 36	180

16.32	Edition 37	180

16.33	Edition 38	180

16.34	Edition 39	181

16.34.1	Proposal 01	181

16.34.2	Proposal 02	182

16.34.3	Proposal 03	184

16.34.4	Released Version	186

16.35	Edition 40	187

16.35.1	Proposal 01	187

16.35.2	Released Version	189

APPENDIX A	INDEX OF REQUIREMENTS	191

List of Tables

No table of figures entries found.

List of Figures

No table of figures entries found.

# PREFACE

## Purpose

The present document contains the system/generic requirements needed for the project ELEKTRA.

The primary purpose of the document is the description of system/generic requirements for the system architecture variant ELEKTRA 2.x. If a requirement is also valid for the variant ELEKTRA 1.x this is marked in the requirement (attribute “Architecture). However the document does not make any claim to completeness for a description of system/generic requirements of the variant ELEKTRA 1.x.

## Intended Audience

- Product Design Authority

- Solution Engineering Manager

- Technical Project Manager Elektra System / Solution Engineering Manager ELEKTRA System

- Product Line Manager

- Test Manager

- Validator

- Verifier

- Safety Assurance Manager

- Quality Assurance Manager

- Project Security Manager

- RAM Manager

- System Architect

- Work Package Manager

## Information for Document Generation

The present document was generated by using IBM™ Rational DOORS.

Settings applied:

| DOORS/RMF Version: | DOORS 9.7.2.5/RMF 6.1.0.9 |
| --- | --- |
| DOORS/RMF Module Name: | ELEKTRA SA - SRS |
| DOORS/RMF Module Version (Baseline): | 49.0 (PV GEN 2.20.10) |
| DOORS/RMF Module Path: | /ELEKTRA Projects/ELEKTRA Generisch Länderübergreifend/ELEKTRA System |
| DOORS/RMF Export View: | Exp.V. SRS 0300 (COM) |
| DOORS/RMF Current Filter: | (A_Status != deleted) AND (A_SRS includes 0300 (COM)) |
| DOORS/RMF Export Options 1: | 2,false,0,0,false,false,0,true,false, |
| DOORS/RMF Export Options 2: | true,false,true,1,false,false,true,PostprocessAllStyles,\\Thales-AT-Data\THALES-AT\PROJECTS\ReqMng\DOORS\Word Export\DOORS-EXPORT-TEMPLATES\Generic\SRS\0300\exports\0300.log, |
| DOORS/RMF Export Options 3: | \\Thales-AT-Data\THALES-AT\PROJECTS\ReqMng\DOORS\Word Export\DOORS-EXPORT-TEMPLATES\Generic\SRS\0300\exports\0300.log,false,true,false,false,false,true,100,true,4, |
| DOORS/RMF Export Options 4: | 3,###,===,true,false,false,false,false, |
| Documentation-Vob Template Name: | 3BU_15000_0300_DTAPA-Template.docx |

## Referenced Documents

| [ADD Elektra Evolution] | Architectural Design Document, Elektra Evolution, System Architecture, 3BU 15000 0400 PEAPA Ed. 26 |
| --- | --- |
| [DDD Reaktionsverzoegerungen] | Detailed Design Document, ELEKTRA 2.x, Mechanismus zur Beherrschung von Reaktionsverzögerungen, 3BU 15000 4970 PEAPC Ed. 06 |
| [Defs ELEv] | Definitions, Acronyms and Abbreviations, Technical Project ELEKTRA EVOLUTION, 3BU 15000 0081 DCAPA Ed. 06 |
| [EN 50129] | EN 50129:2018 + AC:2019 – Railway applications – Communication, signalling and processing systems - Safety related electronic systems for signalling |
| [EN 50159] | EN 50159:2010 + A1:2020 – Railway applications – Communication, signalling and processing systems – Safety-related communication in transmission systems |
| [EN 50716] | EN 50716:2023 (supersedes EN 50128) Railway Applications - Requirements for software development |
| [FL INT] | Feature List ELEKTRA International, 3BU 15000 9000 FLAPQ Ed. 21 |
| [FL ÖBB] | Feature List ELEKTRA ÖBB, 3BU 15000 4301 FLAPC Ed. 43 |
| [FMEA] | FMEA Table, CAR/EP, 5.x 3BU 15000 5190 DUAPA Ed. 03 |
| [IF-Spec CC/DGP] | Interface Specification CC/DGP, 3BU 61400 0080 PBAPA Ed. 06 |
| [IF-Spec X25] | Spezifikation X25 – Schnittstelle / Specification X25 Interface, 3BU 15000 4167 DTAPQ Ed. 60 |
| [MM Reactivity] | EL-SYS-16-042_MM_Reaktivitaetsueberwachung-Elektra-Plf2x-SACs-HAZLG98.doc Ed. 03 |
| [SRS BF 5] | System Requirements Specification, ELEKTRA Schweiz, Betriebliche Funktionen Teil 5, 3BU 15000 3401 DTAPC Ed. 24 |
| [SRS INT SA] | System Requirements Specification, Elektra Evolution, INT, 3BU 15000 4107 DTAPA Ed. 32 |
| [SRS ÖBB SA] | System Requirements Specification, Elektra Evolution, ÖBB, 3BU 15000 0315 DTAPA Ed. 27 |
| [SRS SBB SA] | System Requirements Specification, Elektra Evolution, SBB, 3BU 15000 0309 DTAPA Ed. 42 |
| [SRS Security] | System Requirements Specification, Security Elektra, 3BU 15000 5310 DTAPA Ed. 05 |
| [SysDesc-RC-ILS-CTC] | System Description Remote Control - Interlocking System CTC, 3BU 15000 7009 EDAPA Ed. 03 |
| [TAS Plf Safety Handbook] | Safety Handbook, TAS Platform Core 2.x, non-redundant, 3BY 87000 0837 DUAPA Ed. 01 |
| [TSR EL 2SW] | Verfahrenssicherheitsnachweis – 2 SW-Kanäle auf einer Hardware, ELEKTRA, 3BU 15000 0664 DUAPC Ed. 01 |

## Abbreviations used by Requirement report

See Definitions, Acronyms and Abbreviations [Defs ELEv].

## Abbreviations

| AFE | Anlagenfehler | installation error |
| --- | --- | --- |
| CRA | n/a | Cyber Resilience Act (EU regulation) |
| CRC | Zyklische Redundanzprüfung | Cyclic Redundancy Check |
| CS | Kommunikationssystem (Teil der TAS Platform | Communication System (part of TAS platform) |
| DBS | Datenbankserver | Data Base Server |
| EBS | Fehlerpufferserver | Error Buffer Server |
| FMEA | Fehlermöglichkeits- und Einflussanalyse | Failure Mode and Effects Analysis |
| GA | Geschwindigkeitsanzeiger | speed indicator |
| GVA | Geschwindigkeitsvorananzeiger | advanced speed indicator |
| IAU | n/a | Interlocking Adaptation Unit |
| JPS | n/a | Juridical Protocol Service |
| LSA | Lichtsignal Abfahrt | light signal departure |
| MDS | n/a | Monitoring and Datacollection System |
| OCS-SBJ | n/a | One Channel Safe - Single Board Java |
| OLT | Online Test der TAS Platform | Online Test of TAS Platform |
| OS | Betriebssystem | Operating System |
| RaSTA | n/a | Rail Safe Transport Application |
| SNA | Signalnachahmer | signal repeater |
| SRAC | Sicherheitsbezogene Anwendungs-bedingung | Safety Related Application Condition |
| TCD | n/a | Transmission Controller Double Channel |
| TFFR | Tolerierbare funktionale (unsichere) Fehlerrate | Tolerable Functional (unsafe) Failure Rate |
| VM | Virtuelle Maschine | Virtual Machine |
| ZAML | Zugführerabfertigungsmeldelampe | dispatch lamp for train guard |
| ZUML | Zustimmungsmeldelampe | departure indication lamp for train driver |

## Definitions

| LAN/Ethernet | In the sense of ELEKTRA items “LAN” and “Ethernet” are used interchangeably |
| --- | --- |
| Supervised Area | Defined line consisting of several stations for which the concept of remote control is applied. |
| projectable/preparable | Entries of the data preparation are set by the field project planning engineer by means of a data preparation tool. |
| configurable | Entries of the data preparation are not set by the field project planning engineer but are derived e.g. from other parameters or from a patch-file. |

# OVERVIEW

## Evolution of the System Requirements Specification

This document will be continuously updated.

## Structure of the Requirements

| Identifier: | Unique identifier of the requirement preceded by category |
| --- | --- |
|  | Category (Type) of Requirement → see Ch. 2.3 |
| Synopsis: | Unique brief description of the requirement |
| Description: | Detailed description of the requirement |
| Status: | Current status of the requirement. Possible values: draft requirement is collected and written down, not yet approved and reviewed active requirement is valid, approval and review performed to be descoped requirement is planned to be postponed (no system baseline former product version generic stated), not yet approved and reviewed descoped requirement is postponed (no system baseline former product version generic stated), approval and review performed to be deleted requirement is planned to be deleted, not yet approved and reviewed deleted requirement is deleted (no longer valid), approval and review performed. A ‘deletion reason’ in the ‘Comments’ field |
| Stability: | Expected stability of the requirement over the whole life cycle of the system. Possible values: stable will probably not change during the life time of the product volatile subject to change instable subject to change frequently |
| Customer: | Customer the requirement applies for and is released for. Possible values: OBB Austria SBB Switzerland BLS Switzerland ATG Switzerland MAV Hungary BKV Hungary GySEV Hungary NRIC Bulgaria |
| Priority: | Relative importance of implementing the requirement. Possible values mandatory product will not be accepted unless the requirement is provided optional enhancement of the product may be implemented in this or a later release absence doesn´t make the product unacceptable |
| Architecture: | The system architecture (ELEKTRA 1 and/or ELEKTRA 2) to which the requirement applies. |
| See also: | References to other requirements. |
| Allocation: | Functional, architectural component or other aspect to which the requirement is allocated. Possible values: CAR CCX-SW Configuration CS Data preparation Data prep. guideline DGP DGP-SW ECX-SW ELEKTRA ELEKTRA application ELEPRO EP EQP ESM Ext. network FE High-load IMO International JPS MI MI-2 MI-3 MI-IBO MI-HW MI-red Linux OMO OREST PMO PS SA SRS SSU-SW TAS platform Tooling VCX-SW Webride |
| Originator: | Source/backward reference of the requirement. Possible values: CENELEC customer customer (OBB) customer (SBB) customer (SBB, MAV) Development Manual HW development maintenance migration strategy product management product strategy safety management system technical management technical strategy |
| Checking: | Verification method of the requirement. Possible values: acceptance test EQP analysis BusExe test CAR test code reading configuration checking design review EC SUBIT EC SUBIT mit Zeitmessung EP test FBI test FE integration test HUB/Switch test HW integration test HW module test IC SUBIT IMO SUBIT integration test MI integration test MMI SUBIT module test OMO SUBIT on-site test corresponds to the German Anlagenprüfung performance test qualification test RAM analysis RCU SUBIT review review data sheet review HW review TOOL safety analysis system administration SUBIT system design system test TAS Platform test TOOLS test XGIOS integration test XGIOS module test XGIOS performance test |
| System Baseline: | The first value with the lowest number indicates the version (generic system baseline) in which the requirement was released for the first time. Further values with higher numbers indicate versions by which the requirement was substantially modified. Some possible values are given below: PV_GEN_2.x.x All “old” requirements before system baseline PV2.15.3 PV_GEN_2.15.3 Requirements concerning “Integration SEA” (baseline OBB27.20) PV_GEN_2.15.4 Requirements concerning “Monitoring of reactivity of ELEKTRA system components“ (baseline OBB28.00) PV_GEN_2.15.10 Requirements in context “International” (baseline INT01.00) PV_GEN_2.16.0 Requirements concerning “Serial connection ELEKTRA - AzLM” (baseline CH19.20, CH20.10) PV_GEN_2.16.2 Requirements in context “EBO2 Resolution International” PV_GEN_2.16.7 Requirements in context “Monitoring of reactivity of ELEKTRA system components” PV_GEN_2.17.0 Requirements concerning migration to the TAS Platform 2.x |
| Comments: | Any additional information; e.g. reason for deleting a requirement |

## Abbreviations of Categories of Requirements (= Types of Requirements)

| CSR | Customer Specifics Requirement - Additional requirements from the customer: Customer creates site specific data Customer requires specific tools for site specific data creation (the customer intends to purchase the tools) Customer requirements for diagnosis Training equipment |
| --- | --- |
| DAT | Data Requirement - These describe specific system behaviour regarding data, examples could be: Data management (e.g. management of user accounts) Data types Data structure Views |
| DOR | Documentation Requirement - Which documentation does the customer receive? Planning handbook Installations and putting into operation handbook Maintenance handbook Training material User manual(s) |
| DPR | Dependability Requirement - This includes: Mean time between failures (MTBF) (with how to calculate) Availability (with how to calculate) Lifetime Customer defined or expected |
| ECR | Environmental Conservation Requirement - This includes: Materials you have to use (e.g., non-polluting materials) Waste management |
| FUR | Functional Requirement - These describe what the system will do (i.e, the behavior of the system). Which functions does the system/product fulfill? Examples of the functional requirements include: Functions, entry criteria Results, rules Description of reactions to unexpected inputs (error behavior, reaction) |
| GCR | General Constraints Requirement - This includes all necessary systems and prerequisites for the system/product to work correctly. Some examples are given below: Dynamic load balancing relating to connection setup from rolling stock has to be provided by the GSM-R network i.e. mobile switching center (MSC). Power for the system must be provided by a uninterruptible power supply (UPS) 3x230/400V 50Hz. |
| MCR | Management and Commercial Requirement - This includes: Special requirements for management Cost estimation Schedule |
| MDR | Maintenance and Diagnosis Requirement - This includes: Required equipment for diagnosis Mean time to repair (MTTR) Mean time between maintenance activities Spare parts |
| MIR | Man-Machine Interface Requirement - This includes: Inputs Messages Commands Output media Way to present Way to operate Screen layouts Keyboards and switches Help |
| MPR | Manufacturing and Production Requirement - This includes: Specifically required technology solutions (e.g., ASICs) Manufacturing procedures Testability and test concept Requirements for tools to create site specific data |
| OAR | Operational Aliveness Requirement - This includes: Mode of operations Operational states Conditions System parameters Timing, performance |
| PER | Physical/Environmental Requirement - This includes: Weight, size Temperature, humidity, equipment temperature, dust Vibration, shock Power supply Grounding Labels Storage and transportation conditions |
| PHR | Prevention of Hazards Requirement - This includes: Warning labels Labels (e.g., CE-label) Overload protection, isolation Requirements related to electrical shock, fire, radiation Electro magnetic interference Special handling instructions |
| PRR | Performance and Real-Time Requirement - This includes: Mode of operations Operational states Conditions System parameters Timing, performance |
| PSR | Product Startup Requirement - This includes: Status of product when you deliver Required documentation Procedure to put product into operation Additional tests, test tools How to replace the old product Installation and assembly How to replace the old product Installation and assembly |
| RSR | Resource Support and Consumption Requirement - This includes: Mode of operations Operational states Conditions System parameters Timing, performance |
| SAR | Safety Requirement - This includes: Safety integrity level Safety concept Safety standards which are applicable Customer defined safety requirements |
| SCR | Supplier to Customer Requirement - This includes: Location and/or facilities Training required to use the product |
| SER | Security Requirement - Protection from malicious or inadvertent interference with operations. Security of networks Security of data transmissions Physical security |
| TCR | Tool Chain Requirement - This includes all necessary tools for the development and testing of the product. |
| TIR | Technical Interface Requirement - What interfaces does the product have? Electrical Mechanical Logical - both internal and external Which are the requirements for those interfaces? Format, protocol Range Timing conditions Frequency, sequences Procedures |
| TVR | Testing and Validation Requirement - This includes: Test specifications System test by simulation Entry criteria Exit criteria - Functional tests - Verification of the documentation Acceptance criteria - Test of availability - Field trial Requirements for test tools |

## Abbreviations of Customers

| MAV | Ungarische Staatsbahnen (Magyar Államvasutak) |
| --- | --- |
| BKV | Nahverkehrsgesellschaft der ungarischen Hauptstadt Budapest (Budapesti Közlekedési Vállalat) |
| GySEV | Raaberbahn (Győr-Sopron-Ebenfurti Vasút) |
| NRIC | Bulgarische Staatsbahnen (National Railway Infrastructure Company) |
| ÖBB | Österreichische Bundesbahnen |
| SBB | Schweizerische Bundesbahnen |
| BLS | Lötschbergbahn (Bern–Lötschberg–Simplon) |
| ATG | Tochtergesellschaft der Schweizerischen Bundesbahnen (AlpTransit Gotthard AG) |

# GENERAL DESCRIPTION

Unless stated otherwise this SRS applies to ELEKTRA 2 only.

# Technical Requirements

## Functional Requirements

### General Requirements

| FUR:0112 (TRT14) |  | VC compatible handling of input devices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The handling (initialization, setting, etc.) of mouse and keyboard must be like at VC of ELEKTRA 1. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: | GCR:°0163 (TRT108) |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | XGIOS integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For details about the handling of mouse and keyboard refer to today's SGIOS documentation. |  |  |  |  |

| FUR:0113 (TRT15) |  | SGIOS compatible call interface |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The call interface of XGIOS (XGIOS library) must be compatible to SGIOS callgate interface of ELEKTRA 1. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0163 (TRT108) |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For details about SGIOS callgates refer to today's SGIOS documentation. |  |  |  |  |

| FUR:0114 (TRT16) |  | SGIOS compatible signal interface |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The signal interface of XGIOS must be compatible to SGIOS signal interface of ELEKTRA 1. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0163 (TRT108) |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | XGIOS integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For details about SGIOS (CHILL-)signals refer to SGIOS documentation of ELEKTRA 1. |  |  |  |  |

| FUR:0116 (TRT17) |  | Multi-screen support (for multi-headed systems) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The X Windows Server must support multi-screens. The X Windows server must support up to 6 screens. In case of EBO 1.5 moving a window between these screens shall not be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-HW |  |  | Originator: | system |
| Checking: | XGIOS integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For example, the 'Metro-X Server' (version 4.3) supports up to 4 'Matrox Millenium II' graphics cards within one Linux-PC (i.e. exactly on the Intel-based 'Red Hat Linux 5.x'). Notice: There are graphic cards available which support up to 6 screens. |  |  |  |  |

| FUR:0117 (TRT18) |  | Environment (pthreads, timeouts, shared-memory, etc.) for CAR-handler on 'Linux' |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A development- and runtime-environment to implement and to run CAR-handlers (i.e. all necessary software components like pthread-library, timeout-library, shared-memory, etc.) must be available for the operating system 'Linux'. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0162 (TRT107) |
| Allocation: | CAR |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For details about CAR-handler refer to CAR documentation. |  |  |  |  |

| FUR:0118 (TRT19) |  | CAR for testing on operating system Linux |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For testing purposes only, CAR on Linux (as far as allowed by Linux) shall provide the same functionality as CAR on TAS platform (with components RMT, EBS, DBS and all libraries belonging to them). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: | For details about CAR refer to CAR documentation. |  |  |  |  |

| FUR:0166 (TRT1619) |  | DBS: Checksum handling |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The CAR component DBS shall support checksum handling. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | CAR |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0119 (TRT20) |  | CS (Communication System) on operating system 'Linux' |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CS (with all functions like within TAS platform) shall be able to run inside a TAS platform runtime container on 'Linux'. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0162 (TRT107) |
| Allocation: | ELEKTRA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: | For details about CS refer to TAS platform documentation. |  |  |  |  |

| FUR:0123 (TRT22) |  | Multi-threading support |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The operating system 'Linux' chosen for EBO1,5 and EBO2 must support multi-threading. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0162 (TRT107) |
| Allocation: | Linux |  |  | Originator: | system |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0124 (TRT23) |  | Shared-memory access |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The chosen operating system 'Linux' must support shared-memory access. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0162 (TRT107) |
| Allocation: | Linux |  |  | Originator: | system |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0125 (TRT24) |  | Support of timer interrupts |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The chosen operating system 'Linux' must support (system-)timer interrupts. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0162 (TRT107) |
| Allocation: | Linux |  |  | Originator: | system |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0126 (TRT25) |  | TCP/IP and Ethernet support |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The chosen operating system 'Linux' must support TCP/IP over Ethernet. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0162 (TRT107) |
| Allocation: | MI-HW |  |  | Originator: | system |
| Checking: | MI integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0127 (TRT26) |  | VCB removal |  |  |  |
| --- | --- | --- | --- | --- | --- |
| All passages of code within VCA, CCA, CCB, RCU and DGP must be changed dealing with the conception of two channels of VC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | product strategy |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0128 (TRT27) |  | Cancel lifecycle telegrams between VCA and VCB |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Exchange of lifecycle telegrams between VCA and VCB has to be canceled. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: | FUR:°0127 (TRT26) |
| Allocation: | ELEKTRA application |  |  | Originator: | system |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0131 (TRT30) |  | Cancel synchronisation between VCA and VCB |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Today synchronisation of VCA and VCB is made in the case of displaying system troubles and the quitting of them. Within the new MMI conception the synchronisation between VCA and VCB must be omitted. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: | FUR:°0127 (TRT26) |
| Allocation: | ELEKTRA application |  |  | Originator: | system |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0132 (TRT31) |  | Cancel signal exchange between VCA and VCB |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Today there are passages of code, which deal with signal exchange between VCA and VCB in the case of requesting a shut-down, receiving PIOS informations and occurrence of device error signals from SGIOS. These signal exchanges must be canceled. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: | FUR:°0127 (TRT26) |
| Allocation: | ELEKTRA application |  |  | Originator: | system |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0210 (TRT294) |  | LAN distances > 100m |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Increasing the distance between a hub/switch and an attached device from 100 metres up to 2 kilometres requires the use of a 100Base-FX bridge module (additional module). The optical fiber cable length shall be limited to 412 m if operated in half duplex mode and shall be limited to 2000 m if operated in full duplex mode. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0211 (TRT295) |  | Hub distance extender module, switch buffering, modem buffering |  |  |  |
| --- | --- | --- | --- | --- | --- |
| When using a distance extender module (DEM)/switch/modem interface we are faced with common switching problems. The DEM/switch/modem acts as a switch interface including a port buffer. If a DEM/switch/modem receives a sustained high volume of traffic the buffer can become full. So the buffers of the DEM/switch/modem must be adequately dimensioned to avoid retransmission activities or even reliability problems. For error-proneness reasons the DEM/switch/modem shall not support additional mechanisms such as back-pressure flow control, address filtering and the like. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°0210 (TRT294) |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0306 (TRT44) |  | Signal sequence |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The sequence of signals with the same priority, which are sent from the same application to the same applications, has to be preserved. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | CAR EP TAS platform |  |  | Originator: | system |
| Checking: | CAR test EP test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Derived from VOTRICS requirements. |  |  |  |  |

| FUR:0307 (TRT45) |  | Number of signal priorities |  |  |  |
| --- | --- | --- | --- | --- | --- |
| At least two signal priorities have to be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | CAR EP |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | This requirement deals with signal priorities between applications and not within a taskset. eg: In ELEKTRA SBB currently two signal priorities are used: Signals received from ILTIS are less prior than ELEKTRA internal signals. |  |  |  |  |

| FUR:0309 (TRT46) |  | Routing mechanism of ELEKTRA Classic |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Routing mechanism of ELEKTRA Classic has to be supported due to interoperability reasons. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EP |  |  | Originator: | system |
| Checking: | EP test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Requirement for communication with ELEKTRA Classic components. Routing mechanism in ELEKTRA Classic is based on VOTRICS Network Indices (VNI). |  |  |  |  |

| FUR:0330 (TRT47) |  | Scheduling strategy for ELEKTRA applications |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Run-to-completion scheduling strategy is used for each ELEKTRA application independent of the system configuration and application location. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | E.g. in case CCA and CCB are non redundant and executed on the same computer, run-to-completion scheduling strategy is used for CCA and for CCB as well. |  |  |  |  |

| FUR:0331 (TRT48) |  | Number of ELEKTRA applications served by CAR |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR has to serve at least 8 ELEKTRA applications executing on one computer. Target platform is TAS Platform (AEOS 4.x, AEOS 5.x) and Linux. This implies also multiple database support. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0301 (TRT110) GCR:°0823 (TRT1090) |
| Allocation: | CAR |  |  | Originator: | product strategy |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 |
| Comments: | - RCA, RCB and SCU are a typical example for a configuration. - SCWS (AWS) currently requires 8 ELEKTRA applications executing on one board: CCA/B, RCA/B, ECA/B, DGP, SCU. - AEOS 4.x: one CAR serves all applications using separated data structures. - AEOS 5.x, Linux: each application has its own CAR. |  |  |  |  |

| FUR:4103 (TRT1096) |  | CAR - Shutdown |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR has to provide two shutdown functions – enter_halt and car_exit – that stop all running applications controlled by this CAR. The two shutdown functions must provide different exit codes for the chill image. Autoreset is configured: - Function enter_halt provides exit code 100. - Function car_exit provides exit code 2. Autoreset is not configured: - Both functions have to provide the exit code 1. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR EP |  |  | Originator: | safety management |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.17.0 |
| Comments: | The behavior of these functions shall be identical for all operating systems supported by CAR, as long as the different operating systems do not inhibit the equality. |  |  |  |  |

| FUR:4106 (TRT1099) |  | CAR - Protection from outside applications |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR must start the CHILL applications as separate processes to separate them from processes running outside the CAR environment. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR |  |  | Originator: | safety management |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.17.0 |
| Comments: | For more information see [TSR EL 2SW]. (Voraussetzung 5.2) |  |  |  |  |

| FUR:4107 (TRT1100) |  | CAR - Callbacks at sending/receiving of message |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR must call the configured callback functions of the application when sending or receiving a message. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°2501 (TRT659) SAR:°2502 (TRT660) |
| Allocation: | CAR |  |  | Originator: | safety management |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.17.0 |
| Comments: | comment: This is the CAR part of SAR:2501 and SAR:2502 |  |  |  |  |

| FUR:4108 (TRT1101) |  | CAR - Signal dispatching, Signal Priority 0 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Two parts of CAR are involved in signal handling – RMTS and EP. RMTS must dispatch received signals in the following order: - Signals of the internal message queue - Signals of the external message queue RMTS must handle signals sent by the own application in the following way: - Signals sent to an external instance must be forwarded to EP. - Signals sent to an internal instance must be put into the internal message queue. - Special case: if votrics flag is set and signal priority is 0, the signal must be forwarded to EP even if the receiver is an internal instance. This special treatment is only done by the functions send_signal and __send_signal. EP must put each signal that has to be delivered to own application into the external message queue. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°0307 (TRT45) |
| Allocation: | CAR |  |  | Originator: | safety management |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.17.0 |
| Comments: | This behaviour is partly described in FUR:0307 but it does not cover special case: related to VOTRICS_FLAG to cause special handling for CAR (send to EP instead of local). |  |  |  |  |

| FUR:4109 (TRT1102) |  | CAR - Stacksize check |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR has to check the instances stack usage against a configured maximum size. If the configured size is exceeded the application has to be terminated and a runtime exception has to be raised (STACKOVERFLOW). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR |  |  | Originator: | safety management |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.17.0 |
| Comments: | This partly interacts with the used CHILL compiler (depending on its version). The diagnosis generated by CAR in case of stackoverflow shall be identical to the implementation of CAR4 running on TAS-PLF 1.x as long as there is no technical limitation that inhibits it. |  |  |  |  |

| FUR:4110 (TRT1108) |  | CAR 5 – Functionality provided for ELEKTRA |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR 5 running on TAS-PLF 2.x (AEOS5) shall provide the same functionality as CAR 4 running on TAS-PLF 1.x (AEOS4). The functionality of CAR 5 shall be identical for all supported operating systems as long as the different OS do not force a different behavior. The functionality of CAR 4 provided for ELEKTRA is defined by the source code of CAR 4.2.11 compiled for operating system TAS-PLF 1.11. The following deviations are allowed: - TAS-PLF 2.x does not forbid to run CAR more than once. Therefore CAR 5 does not need to support more than one application any longer. Each application must start its own CAR. - Requirements demand a different behavior of CAR 5. - Differences between TAS-PLF 2.x and TAS-PLF 1.x force a change. - Differences between RHE5 and RHE3 force a change. - CQDs open in CAR 4 have been solved in CAR 5. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR EP |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.17.0 |
| Comments: | Explanation for “source code of CAR 4.2.11 compiled for operating system TAS-PLF 1.11”: If source code is compiled conditionally the following examples show, which part of the code is relevant for ELEKTRA (especially for CCA, CCB, ECA, ECB, RCA, RCB and SCU). #ifdef __TAGOPERT__ <statements relevant for ELEKTRA> #else <statements NOT relevant for ELEKTRA> #endif #if defined(LINUX) <statements NOT relevant for ELEKTRA> #else <statements relevant for ELEKTRA> #endif |  |  |  |  |

| FUR:4111 (TRT1288) |  | CAR - Memory initialization |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR shall initialize all memory allocated by the application through CAR to zero. Note: If the memory initialisation value is made configurable, the default shall be zero. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR |  |  | Originator: | safety management |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.17.5 |
| Comments: | "All memory" means both global memory allocated at startup as well as dynamically allocated memory blocks. |  |  |  |  |

| FUR:0332 (TRT49) |  | Periodical supervision of the connections between computers |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Connections between computers have to be periodically supervised and diagnosed. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: | MDR:°0311 (TRT128) |
| Allocation: | CS EP |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Open/close has to be done periodically or on demand. The current status of connections has to be transferred to DGP. |  |  |  |  |

| FUR:0333 (TRT50) |  | Memory to be supported |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TAS Platform has to support at least 1 GB RAM and at least 2 GB ROM (Flashcard). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: | D2 computer board is no longer supported starting with PV_GEN_2.20.10. |  |  |  |  |

| FUR:0334 (TRT51) |  | Non volatile RAM |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each HW Platform used for an ELEKTRA application shall provide at least 20MB non-volatile memory. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Non volatile RAM has to be used for CMOS data and diagnosis data. Non volatile RAM can be realised with reset stable memory or RAM disk. Reset stable memory might not be provided by off-the-shelf HW. RAM disk might not be able to be used for reset stable memory, because of performance reasons and because currently it has to be allocated from reset stable RAM. In ELEKTRA Classic: DGP(OBB) requests currently 7MB (used for diagnosis data), the other applications request not more than 560KB. |  |  |  |  |

| FUR:0338 (TRT53) |  | CAR on top of TAS Platform |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR on top of TAS Platform (AEOS 4.x, AEOS 5.x) has to be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0337 (TRT52) GCR:°0823 (TRT1090) |
| Allocation: | CAR |  |  | Originator: | product strategy |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 |
| Comments: | Used for CC, EC, DGP, RCU, SCU, TCD and IAU applications. For AEOS 4.x the CAR 4.x version is used. Respectively for AEOS 5.x the CAR 5.x version is used. |  |  |  |  |

| FUR:0339 (TRT54) |  | XCEL on top of TAS Platform |  |  |  |
| --- | --- | --- | --- | --- | --- |
| XCEL on top of TAS Platform (AEOS 5.x) has to be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: | GCR:°0337 (TRT52) GCR:°0823 (TRT1090) |
| Allocation: | EP |  |  | Originator: | system |
| Checking: | EP test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.19.0 |
| Comments: | Used for DGP and RCU applications. XCEL is an additional function of EP, which allows communication via remote message queues and in addition over serial devices. XCEL is used to connect ELEKTRA 1 with ELEKTRA 2 components. |  |  |  |  |

| FUR:0341 (TRT55) |  | Services of TAS Platform |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TAS Platform (AEOS 4.x, AEOS 5.x) shall provide a redundant communication service. Application recovery services are not needed. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0337 (TRT52) GCR:°0823 (TRT1090) |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 |
| Comments: | Used for CC, EC, DGP, RCU, SCU, TCD and IAU applications. The CAR/EP uses CS communication. Therefore these functions are tested in every single application tests implicitly as well in CAR. |  |  |  |  |

| FUR:0343 (TRT56) |  | Location of application code and system code |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Application code and system code located on the same or on two different flashcards have to be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Only one flashcard is usually provided by off-the-shelf HW. |  |  |  |  |

| FUR:0344 (TRT57) |  | Configurable serial interface |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A configurable serial interface has to be provided by TAS Platform (used e.g. for DCF77 or GPS receiver) |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Derived from "TAS Platform, System Requirements Specification",FUR-61 and generalized. HW has to support a serial interface. |  |  |  |  |

| FUR:0345 (TRT58) |  | Configuration of routing |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The routing of logical communication channels must be static off-line configurable with files. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EP TAS platform |  |  | Originator: | system |
| Checking: | EP test TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0346 (TRT59) |  | Exit and restart of applications on one computer |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Currently an exit (called by DISASTER function) is not handled by CAR. It just leads to a reset of the whole computer. A restart of an application is only possible if the whole computer is reset as well. In case of more than one application on one computer an exit of one application shall depending on the existing problem lead to a reset of the whole computer or restart of the applications. CAR/EP should provide the necessary functionality by the exit code. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°4103 (TRT1096) |
| Allocation: | TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.5 |
| Comments: | The DISASTER function of ELEKTRA calls an exit function provided by CAR. This exit function terminates all application images and all CAR functions. At the end it provides an exit code to the script that started CAR. This exit code has to be handled by the script. Every exit code not equal 2 leads to a reboot of the computer. Exit code 2 only leads to a restart of the application without executing a reboot. |  |  |  |  |

| FUR:0350 (TRT60) |  | Execution time supervision |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The execution time of each application located on the same computer has to be supervised in order to prevent infinite loops. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: | GCR:°0301 (TRT110) |
| Allocation: | CAR |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0354 (TRT61) |  | Routecopy mechanism |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Routecopy mechanism as implemented in VOTRICS has to be provided. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | EP |  |  | Originator: | system |
| Checking: | EP test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The multicast mechanism of TAS Platform might be used. Routecopy mechanism must not change signal sequence. |  |  |  |  |

| FUR:0355 (TRT62) |  | Signal arrival sequence |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Certain signal arrival sequence is required by the synchronisation algorithm between CCA and CCB: In CCB the signal of PCA has to be received before the corresponding SYNC signal sent from CCA. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: | GCR:°0301 (TRT110) |
| Allocation: | EP |  |  | Originator: | system |
| Checking: | EP test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | If CCA and CCB are executed on one computer, the synchronisation between CCA and CCB can be performed implicitly by the communication system and not explicitly by the application-based algorithm. This would increase the system performance. If CCA and CCB are executed on different computers, it has to be checked if the synchronisation can be performed by the communication system as well. |  |  |  |  |

| FUR:0357 (TRT63) |  | Signal length |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The maximum signal length has to be configured. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Due to performance reasons the signal length is restricted to 150 bytes in ELEKTRA Classic. |  |  |  |  |

| FUR:0362 (TRT65) |  | Scalability of communication media |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Communication media must be able to be used in redundant and non redundant configuration. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0361 (TRT120) |
| Allocation: | TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0363 (TRT66) |  | Identification concept for spare hardware computers |  |  |  |
| --- | --- | --- | --- | --- | --- |
| An identification concept for spare hardware computers has to be provided in order to fulfill the following requirements: easy exchange by customer, independent of the application running on the hardware component to be replaced. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | EPROM on backplane might be the most suitable solution. Setup for each board via frontend (LINUX PC) might be not easy enough for the customer. |  |  |  |  |

| FUR:0367 (TRT69) |  | Number of external communication lines to be supported |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LDC component has to support external communication lines. The number of used lines must be configurable between 1 and 20. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0368 (TRT70) |  | Integration of RCU and SCU |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Integration/separation of RCU and SCU on the same/different computer has to be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | RCU SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0370 (TRT72) |  | Communication interface between ELEKTRA systems |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The communication interface between ELEKTRA systems must not consist of proprietary data (RMT Header, VNI Index, Layer 4). This is necessary for transferring data between ELEKTRA Evolution systems and ELEKTRA Classic systems. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Remote control (ELEKTRA OeBB, ELEKTRA MAV) are applications to be considered. Communication interface between ELEKTRA components is not part of this requirement. |  |  |  |  |

| FUR:0372 (TRT74) |  | Printer replacement by electronic storage |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to attach an electronic storage device instead of a printer. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0508 (TRT79) |  | Backpanel divisor |  |  |  |
| --- | --- | --- | --- | --- | --- |
| for optimizing the space in a subrack only length with 167M, 83M, 55M or 41M shall be used |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EQP FE |  |  | Originator: | product strategy |
| Checking: | acceptance test EQP |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | when using this divisors for the backpanel length, then 1,2,3,4,BP/subrack can be mounted |  |  |  |  |

| FUR:0509 (TRT80) |  | Backpanel thickness 3,2mm +/- 0,3mm |  |  |  |
| --- | --- | --- | --- | --- | --- |
| to avoid additional stiffeners on the backpanel (against bending when mounting the boards) the required thickness is necessary |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EQP FE |  |  | Originator: | HW development |
| Checking: | qualification test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0510 (TRT81) |  | Backpanel mounting conductive/isolated |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Both possibilities shall be foreseen, mounting the backplane isolated or conductive to the subrack |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EQP FE |  |  | Originator: | HW development |
| Checking: | qualification test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0601 (TRT303) |  | Power supply D6/D10 backplane |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The power supply of the D6/D10 backplanes needs to have all screw connections pre-mounted. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | acceptance test EQP |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.19.0 PV_GEN_2.20.10 |
| Comments: | D2 computer board is no longer supported starting with PV_GEN_2.20.10. |  |  |  |  |

| FUR:0802 (TRT85) |  | Communication load for the fieldbus system. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The communication system (fieldbus system) within the FE subsystem must handle at least the communication load of up to 21 ICs. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0803 (TRT86) |  | EC must handle up to 4 fieldbus subsystems |  |  |  |
| --- | --- | --- | --- | --- | --- |
| An Element Controller must handle up to four fieldbus subsystems. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0804 (TRT279) |  | Handling of Element Controllers |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to handle up to eight Element Controllers (ECs). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Information: Checking to be applied: integration test -> customer OBB system test -> all other customers |  |  |  |  |

| FUR:0809 (TRT87) |  | 70 digital I/O lines for each ELEKTRA channel per IC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| An IO-Controller has to handle all digital I/O lines of a backpanel. In the current configuration for each channel A and B 20 output lines and 50 input lines are required. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | EC SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0813 (TRT88) |  | Intelligent IO-controller must take over some interface board's functionality |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Basic IO-functions like scanning of an Input-Line and periodical output (blinker functionality) control must be implemented at the IO-Controllers. This is required to realize a type reduction and cost reduction. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | IC SUBIT |  |  | System Baseline: |  |
| Comments: |  |  |  |  |  |

| FUR:0814 (TRT89) |  | Execution of A/B-Software on the same computer |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The PC software for channel A and channel B of the current ELEKTRA system shall be executed on the same computer. This concept should help to reduce the number of required computer replicas. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | FE integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0821 (TRT90) |  | TAS Platform must provide appropriate fieldbus interface |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The EC will be a TAS Platform computer. For that reason the TAS Platform has to provide the appropriate fieldbus interface (fieldbus driver), e.g., TTP or CAN. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The preferred candidate for the fieldbus is TTP. |  |  |  |  |

| FUR:0825 (TRT92) |  | Replacement for the plant ID plug |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to identify the railway station and to provide this information to other subsystems (e.g. CC). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EQP SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0826 (TRT93) |  | Hot pull-out/plug-in of FE fieldbus components |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to switch off a component of the FE internal fieldbus subsystem, to replace this component, to switch on and integrate this component. These activities must be possible under "hot" operation of the fieldbus system. The components are the computer boards which are connected on the FE fieldbus. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Result of a meeting with product strategy on 8.3.1999. |  |  |  |  |

| FUR:0827 (TRT94) |  | Realisation of the "Betriebsersatzstecker" in hardware |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The simulation of the outdoor equipment should be realised in hardware with the "Betriebsersatzstecker" concept from the ELEKTRA Classic. The following improvement of the ELEKTRA Classic concept should be considered: Not all HW interfaces should be designed to hold a "Betriebsersatzstecker". Only special HW interface (Pruefleiterplatten) should be used for system integration. This HW interface will be replaced before setting the system in operation. With such an procedure the HW interfaces with the "Betriebsersatzstecker" equipment are reuseable. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | system test |  |  | System Baseline: |  |
| Comments: | A replacement of the hardware parts "Betriebsersatzstecker" by a simulation software which is executed at the ICs was discussed with the product strategy (8.3.1999). A SW solution was rejected because of following reasons: high effort for perfoming the safety case, costs for the development of the simulation SW, and reduced quality of the simulation (HW interface boards are not included). |  |  |  |  |

| FUR:0828 (TRT95) |  | Reduction of cabling costs between interlocking system and field elements |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The number of parallel cables from the interlocking system to the field elements at the track site should be reduced. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | analysis |  |  | System Baseline: |  |
| Comments: |  |  |  |  |  |

| FUR:0829 (TRT299) |  | TAS Platform must provide an access to a PC-MIP module for parallel I/O |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA Plant-ID setting is realised with a rear adapter board for the MEN D6/D10 computer board. This setting is realised with 12 BCD switches. In order to read these switches the PC-MIP modul '15P013' from MEN will be used on the MEN D6/D10 computer board. The handling of this PC-MIP module must be provided by the TAS Platform. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: | D2 computer board is no longer supported starting with PV_GEN_2.20.10. |  |  |  |  |

| FUR:0830 (TRT302) |  | Fixed numbering and allocation scheme for communication links |  |  |  |
| --- | --- | --- | --- | --- | --- |
| There must be a fixed numbering and allocation scheme for the communication links which are provided by the TAS Platform. The relevant links are all external links which are realised as PCI-devices, e.g., PCI-MIP-Modules (Ethernet, parallel I/O) and M-Modules (TTP, and HDLC). Also the Ethernet-Links on the LINUX-PC for the realisation of the redundant LAN (redundant CS) must be considered. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°1211 (TRT276) |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0831 (TRT445) |  | CAR has to support up to 32 applications |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAR has to serve up to 32 ELEKTRA applications executing on one computer. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR |  |  | Originator: | product strategy |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:0832 (TRT434) |  | Using of a COTS-microcontroller module on IMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A COTS-microcontroller module shall be used instead of realising the microcontroller functionality directly on the IMO "motherboard". |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | system |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0833 (TRT435) |  | At least 50 inputs on each IMO channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
| An IMO board shall handle at least 50 inputs per channel. Each of them is insulated against ground and frame. All inputs are referenced to the same grounds. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | product strategy |
| Checking: | IMO SUBIT qualification test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0834 (TRT436) |  | Same TTP-bus configuration for EC - IC communication and EC - IMO communication |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No changes in the TTP-bus configuration must be required to realise the communication between EC and IC via the TTP-bus. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | product strategy |
| Checking: | IMO SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0835 (TRT437) |  | For the communication via the TTP-bus the IMO-SW supports 50 inputs for each channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
| With respect to the used TTP-bus configuration the IMO-SW supports 50 inputs for each channel |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | product strategy |
| Checking: | IMO SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0837 (TRT439) |  | Power supply on IMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A single 60VDC power supply must be realised on the IMO (e.g. according to realisation on the IC). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | HW development |
| Checking: | HW module test review HW |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0838 (TRT440) |  | Input voltage on IMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each insulated input on the peripheral side of the IMO must be able to handle 10V - 72V. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | HW development |
| Checking: | HW module test review HW |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0839 (TRT441) |  | Serial console on IMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The IMO-board has to provide a serial RS232 compatible device for programming the onboard flash and misc. output during operation |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | HW development |
| Checking: | review HW |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0841 (TRT443) |  | Identification pins on IMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 12 identification pins readable from the backplane and insulated between both channels on one IMO-board are required |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | HW development |
| Checking: | review HW |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0842 (TRT444) |  | Printed board size for IMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The IMO-board has to correspond to the extended double European standard size. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | HW development |
| Checking: | HW module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0844 (TRT510) |  | IMO - Safe detection of missing reference flash frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
| IMO must provide a safe detection of missing reference flash frequency. All input ports that are configured as flash inputs are provided with information if flash, permanent voltage or no voltage is provided. In case of detection of missing reference flash frequency all input ports that are configured as flash inputs must be reported to the EC as faulty (by reporting flashing and permanent voltage at the same time). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | system |
| Checking: | IMO SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:0846 (TRT667) |  | XEP (EP running without CAR system) required for LINUX |  |  |  |
| --- | --- | --- | --- | --- | --- |
| XEP and lwatch are required to support applications without running a CAR system providing the EP functionality and shall be able to run inside a TAS platform runtime container on Linux. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0162 (TRT107) |
| Allocation: | ELEKTRA |  |  | Originator: | product strategy |
| Checking: | integration test module test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

| FUR:0847 (TRT668) |  | XEP (EP running without CAR system) required |  |  |  |
| --- | --- | --- | --- | --- | --- |
| XEP and lwatch are required to support applications without running a CAR system providing the EP functionality for TAS Platform (AEOS 5.x). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: | GCR:°0823 (TRT1090) |
| Allocation: | EP |  |  | Originator: | product strategy |
| Checking: | EP test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

| FUR:0848 (TRT669) |  | XEP has to provide TCP/IP connections |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Additional to twofold redundant LAN the XEP shall provide TCP/IP connections in parallel. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EP |  |  | Originator: | product strategy |
| Checking: | EP test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The TCP/IP connection connects XEP and the application. |  |  |  |  |

| FUR:1103 (TRT235) |  | DGP running on Linux PC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DGP has to run on Linux PC too. The PC-monitor can be used as DGP-terminal. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | system test |  |  | System Baseline: |  |
| Comments: | DESCOPE REASON: Pending further notice there is no need for this requirement. |  |  |  |  |

| FUR:1105 (TRT284) |  | More than one LDC computer |  |  |  |
| --- | --- | --- | --- | --- | --- |
| To handle up to 20 external communication lines it must be possible to use more than one LDC computer. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°0367 (TRT69) |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:1106 (TRT506) |  | VC for temporary use |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to mark a VC to be used only temporarily. If this VC is missing, no error has to be reported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:1107 (TRT507) |  | Non-redundant components |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to use CC, VC, EC, RCU, SCU, TCD, IAU and Ethernet connection with only one redundancy. The second redundancy is not realized and therefore, must not lead to an error message. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | MAV |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: TTP-bus is always redundant. |  |  |  |  |

| FUR:1108 (TRT671) |  | WebRide is used to retrieve the diagnosis data from DGP |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WebRide shall be used to retrieve the diagnosis data from DGP. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: |  |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:1109 (TRT672) |  | Transfer of diagnosis data from WebRide to Vienna. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| There shall be a communication link to transfer the diagnosis data from WebRide to HITACHI Rail Austria / Vienna. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: |  |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:1110 (TRT676) |  | CMOS-copy ready for sending/receiving data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The process CMOS-copy shall be in the state "ready for sending data" or "ready for receiving data" within a defined time after its start. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The "defined time" will be set to 20min. Info for implementation: Process gcx_cmos_standby must leave INIT state within 20 minutes after process start. Therfore gcx_first_appl has to send a signal to gcx_cmos_standby to change state to "receive data" or to "active". |  |  |  |  |

| FUR:1302 (TRT104) |  | Maximum Detection time of HW subsystem failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The time between the complete failure of a HW subsystem (e.g. a computer, an Interface controller) and its detection (and appropriate reaction to it) must not exceed 1 sec. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE TAS platform |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:1303 (TRT105) |  | Maximum Detection time of SW subsystem failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The time between the complete failure of a SW subsystem (e.g. an application, the communication system etc.) and its detection (and appropriate reaction to it) must not exceed 1 sec. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:1304 (TRT106) |  | Integration of LDC functions on EC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to integrate Long Distance Communication Functions (such as TCD, SSU) into one board with the EC function. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EP |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: |  |
| Comments: |  |  |  |  |  |

| FUR:1574 (TRT219) |  | Twofold redundant LAN connection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A 2 fold redundant LAN is necessary to connect the ELEKTRA boards to increase the availbility to the comunication. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Timeout supervision of the LAN connection must be configurable and less than one second. It must be possible to run the LAN connections for boards one ETHERNET connector only (e.g. ACP). 2-fold redundant and single LAN connections must be operational in parallel. |  |  |  |  |

| FUR:1577 (TRT317) |  | OCS protocol |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Communication between ELEKTRA and another product which is based on TAS Platform (e.g. AzLM) by the OCS protocol |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | system |
| Checking: | system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Requirements to TAS Platform for use of OCS in ELEKTRA2 still have to be defined. |  |  |  |  |

| FUR:1578 (TRT1433) |  | TAS-PLF OCS-SBJ communication framework |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Generic System has to provide the TAS-PLF OCS-SBJ communication framework as generic component for the applications. OCS-SBJ must support the HW plattforms D6 and D10. OCS-SBJ has to support the communication protocols RaSTA and SAHARA in the latest versions. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.19.0 |
| Comments: | With PV_GEN_2.19.x only the RaSTA protocol is supported. Starting with PV_GEN_2.20.10 also the SAHARA protocol 2.2 is supported. Notice: OCS-SBJ is the replacement of the OCS-EL/OCS-SB framework. With Plf 2.5.x OCS-EL/OCS-SB is no longer supported. |  |  |  |  |

| FUR:1701 (TRT222) |  | Identification of redundancies |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MEN hardware must include a mechanism (supported by TAS platform) to identify redundancies by assigning a small integer number starting with 1 to each redundant component. Assignment must be unique for each subsystem. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°0363 (TRT66) |
| Allocation: | TAS platform |  |  | Originator: | technical strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Current proposal: 8-bit dip switch for component identification. From this, the redundancy numbers for each application executing on this component, as well as the IP address of the component are determined. |  |  |  |  |

| FUR:1702 (TRT223) |  | New version of reset function |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The RMT library has to provide a new version of the reset function with the semantic that the application and RMT are stopped and restarted without booting the hardware. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR |  |  | Originator: | technical strategy |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Possible syntax: enter_halt (e_rmt_only). This type of reset may be used if the reset is due to reasons that are external to the resetting component (e.g. reset request from CC, loss of CC life cycle). |  |  |  |  |

| FUR:1929 (TRT662) |  | Detection of high system load - measurement of time drift |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The time drift between time of application and real time must be measured at CC level (CCA and CCB). The time drift must be measured separately in each channel. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°1902 (TRT664) |
| Allocation: | ELEKTRA application High-load |  |  | Originator: | system |
| Checking: | performance test system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:1928 (TRT1673) |  | Detection of high system load - configurable data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The time drift threshold value shall be provided in the DB. Default DB values of the time drift threshold for CCA and CCB shall be defined as follows: - CCA: 4s (triggered on t >= 5s) - CCB: 9s (triggered on t >= 10s) |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application High-load SCWS |  |  | Originator: | system |
| Checking: | code reading system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | "Configurable" means that the values are represented as database items but shall not be editable through the data preparation tool and shall only be modifiable via database patch files. Information for "Checking": The request of configurable attributes will be proven by means of "code reading". |  |  |  |  |

| FUR:2801 (TRT467) |  | Using of a microcontroller module on OMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A microcontroller module shall be used instead of realising the microcontroller functionality directly on the OMO "motherboard". |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | product strategy |
| Checking: | review |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2802 (TRT468) |  | At least 16 outputs on each OMO channel in failsafe mode and 32 outputs in not-failsafe mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| An OMO board shall handle at least 16 outputs per channel in failsafe mode and 32 outputs with both channels together in not-failsafe mode. Each of them is insulated against ground and frame. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | product strategy |
| Checking: | OMO SUBIT review |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2803 (TRT469) |  | Variable assemblies of components for each mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The actual mode must be defined by configuration of resistors. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | product strategy |
| Checking: | OMO SUBIT |  |  | System Baseline: |  |
| Comments: | This has to be defined before assembling of the component. Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2804 (TRT470) |  | Same TTP-bus configuration for EC - IC communication and EC - OMO communication |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No changes in the TTP-bus configuration must be required to realise the communication between EC and IC via the TTP-bus. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | product strategy |
| Checking: | OMO SUBIT |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2806 (TRT472) |  | Single 60VDC power supply on OMO backpanel |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A single 60VDC ±10% power supply is realised on the OMO backpanel. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | HW development |
| Checking: | HW module test review HW |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2807 (TRT473) |  | 100VDC output voltage on OMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each insulated output on the peripheral side of the OMO must be able to handle 100VDC, 1A/50W. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | HW development |
| Checking: | HW module test review HW |  |  | System Baseline: |  |
| Comments: | assigned to Relais 65. Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2808 (TRT474) |  | Serial interface on OMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The OMO-board has to provide a serial RS232 compatible device for programming the onboard flash and misc. output during operation. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | HW development |
| Checking: | HW module test review HW |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2809 (TRT476) |  | Identification pins on OMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 12 identification pins readable from the backpanel and insulated between both channels on one OMO-board are required. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | HW development |
| Checking: | OMO SUBIT review HW |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2810 (TRT477) |  | Printed board size for OMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The OMO-board corresponds to the extended double European standard size. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | HW development |
| Checking: | review HW |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2811 (TRT478) |  | Duplicated Outputs |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Every Output of the OMO-board has to provide 2 independent contacts. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | HW development |
| Checking: | HW module test review HW |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:2812 (TRT479) |  | Switch between both outputs of one channel |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In failsafe mode, one output must be realised according to 891 65211 EDAPC (3BU 10701 AAAA EDAPC). |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | customer (OBB) |
| Checking: | HW module test review HW |  |  | System Baseline: |  |
| Comments: | Safety Case: 891 65211 DUAPC (3BU 10701 AAAA DUAPC). Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| FUR:3001 (TRT484) |  | Fahrdienstleiter-Arbeitsplatz mit Redundanz |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ist in einem Bahnhof nur 1 Fahrdienstleiter vorgesehen, muss der Fahrdienstleiter-Arbeitsplatz mit Redundanz äquivalent zum Arbeitsplatz der ELEKTRA 1.x ausgeführt werden können. Es sind Projektierungsvarianten vorzusehen, durch welche alle Arbeitsplätze einer Anlage redundant oder alle Arbeitsplätze einer Anlage nichtredundant ausgeführt werden können. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-red |  |  | Originator: | customer (OBB) |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:3004 (TRT487) |  | Umschalten bei Ausfall eines PC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Im Falle eines Ausfalls des aktiven MMI-PC muß der Fahrdienstleiter die Möglichkeit haben, mit dem redundanten PC weiterzuarbeiten, wobei der Komfort äquivalent zur ELEKTRA 1.x sein muss (z.B. Umsteckvorrichtung) |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-red |  |  | Originator: | customer (OBB) |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Sind die PCs nur dem Instandhaltungspersonal zugänglich, so ist ein Tausch nur möglich, wenn alle Anschlüsse der Endgeräte steckbar ausgeführt sind. |  |  |  |  |

| FUR:3005 (TRT488) |  | Endgeräte |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Die Endgeräte - Monitor(e), Maus, Tastatur, Lautsprecher - dürfen auf dem Fahrdienstleiter-Tisch nur einmal vorhanden sein. (Das Reservematerial dazu ist in einem separaten Schrank im Fahrdienstleiter-Raum untergebracht). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-red |  |  | Originator: | customer (OBB) |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:3006 (TRT489) |  | Tausch eines defekten Endgerätes |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Der Fahrdienstleiter muss die Möglichkeit haben, selbst einen defekten Monitor, Maus, Tastatur oder Lautsprecher zu tauschen. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-red |  |  | Originator: | customer (OBB) |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Sind die PCs nur dem Instandhaltungspersonal zugänglich, so ist ein Tausch nur möglich, wenn alle Anschlüsse der Endgeräte steckbar ausgeführt sind. |  |  |  |  |

| FUR:3801 (TRT517) |  | LAN failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The failure of a LAN connection (ethernet connection) must lead to the error "Anlagenfehler"; a corresponding error message has to be printed on the error printer ("Störungsdrucker"). Additional LAN-errors are neither indicated to the operator nor printed. The error "Anlagenfehler" is canceled only if no LAN-error is pending. The failure of a LAN connection (LAN error) is defined as the interruption of the connection between two computers (even if the redundant connection is still living). There is no differentiation which network component causes the interruption. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: "Anlagenfehler" is "AFE" in ELEKTRA OBB and INT and "ANL" in ELEKTRA Schweiz. |  |  |  |  |

| FUR:3802 (TRT518) |  | LAN diagnosis - WebRide |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to enquire the state of every single LAN connection ("OK"/"not OK") on the diagnosis system (WebRide). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:3803 (TRT519) |  | LAN diagnosis |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The diagnosis of LAN connections must include only projected computers and currently active MMI-PCs. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:3804 (TRT520) |  | LAN diagnosis - switched-off redundant MMI-PC (EBO1,5) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The failure of a LAN connection to a redundant MMI-PC which is switched off must not lead to AFE/ANL. This is valid for EBO1,5. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°4101 (TRT683) |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: Not applicable for NRIC and INT. |  |  |  |  |

| FUR:3805 (TRT521) |  | LAN diagnosis - display for switched-off redundant MMI-PC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A redundant MMI-PC which is switched off has to be shown on the diagnosis system and the corresponding LAN connections have to be indicated as "not OK". |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:3806 (TRT522) |  | LAN diagnosis - VC for temporary use |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A VC for temporary use is always ignored in the LAN diagnosis system. LAN errors will therefore not lead to the error "Anlagenfehler" and no error message is printed on the error printer ("Störungsdrucker"). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:4101 (TRT683) |  | LAN diagnosis - switched-off redundant MMI-PC (EBO2) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The failure of a LAN connection to a redundant MMI-PC which is switched off must lead to AFE/ANL. This is valid for EBO2. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°3804 (TRT520) |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| FUR:4200 (TRT1330) |  | Board Serial Number D6/D10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each individual computer board of the types D6 or D10 shall be identified by a serial number. It must be possible by analyzing the DGP data to assign the individual board serial number to each used board in the system. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | technical strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.19.0 |
| Comments: | It is not requested to see the serial numbers on the SMT. The board serial number shall be sent at reboot of the board and logged periodically (24h) in the rdu diagnosis log files. (reference: ELSYS-667) |  |  |  |  |

| FUR:4201 (TRT1331) |  | EC and EC-MUX on one computer board D10 – Basic Feature |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The system has to support the execution of EC and EC-MUX on one computer board D10. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°0346 (TRT59) |
| Allocation: | TAS platform |  |  | Originator: | technical strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.19.0 |
| Comments: | Because there are existing different reboot concepts for EC (Chill application) and EC-MUX (Java application) the requirement FUR:0346 must be considered specially. The execution of the applications EC and EC-MUX on one board is restriced to the computer board D10 because of performance reasons. Furthermore it is allowed to define additional restrictions to maximum number of elements, IC segments, number of AzLM connections etc. if required. The limits must be documented in the [ADD Elektra Evolution]. |  |  |  |  |

| FUR:4202 (TRT1332) |  | EC and EC-MUX on one computer board D10 – AzLM interface |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EC and EC-MUX on one computer board have to support variants with and without AzLM interface. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | technical strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.19.0 |
| Comments: | Notice: The AzLM interface is part of EC application system. The base AzLM interface allows 20 AzA connections per EC. Because of performance restrictions it is allowed to define a lower limit of connections, if required. The limits must be documented in the [ADD Elektra Evolution]. |  |  |  |  |

### Journaling (43)

The product JPS (function name "ESM 2") meets the European CRA and is the successor of the ESM.
Requirements for the JPS itself are not handled within the generic system (see SSS JPS, 3BU 51019 AAAA DTAPA).

| FUR:4301 (TRT685) |  | Journaling - General |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The "journaling" (protocolling of operational and technical events) shall be realized by means of a highly-reliable electronic storage device. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°0372 (TRT74) |
| Allocation: | JPS SA |  |  | Originator: | product management |
| Checking: | design review |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

| FUR:4302 (TRT686) |  | Journaling - ELEKTRA technology |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The function "journaling" shall be available for ELEKTRA 2. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°0372 (TRT74) |
| Allocation: | JPS SA |  |  | Originator: | product management |
| Checking: | design review |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

| FUR:4305 (TRT689) |  | Journaling - JPS - Indication of faults |  |  |  |
| --- | --- | --- | --- | --- | --- |
| JPS faults shall be indicated on the man machine interface of the interlocking. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°0372 (TRT74) |
| Allocation: | JPS SA |  |  | Originator: | technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: | Info: A registration fault / registration error has to be indicated. |  |  |  |  |

### Monitoring of Reactivity of ELEKTRA system components (44)

Each ELEKTRA system component must be able to monitor the age of data received by any other ELEKTRA system component. At the moment the ELEKTRA system component CC monitors the ELEKTRA system components EC, RCU and TCD and the CC is monitored by the EC, RCU and TCD system components (for more details see requirements below,  and [ADD Elektra Evolution] and [DDD Reaktionsverzoegerungen] ).

In general, the function "Monitoring of Reactivity" can be implemented for any other ELEKTRA system component in future.

In this chapter the term "System Component" is used for explanation of the functionality "Monitoring of Reactivity". In this context the system component has the meaning of component of the ELEKTRA system (such as CC, EC, RCU and TCD).

The requirements of the function "Monitoring of Reactivity" fulfill the SRAC [Core-2.x:SAC:0232] of the [TAS Plf Safety Handbook].

#### General Monitoring of Reactivity

| FUR:4400 (TRT721) |  | Monitoring of reactivity of ELEKTRA system components - detection of max. age of data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each ELEKTRA system component must be able to monitor the age of data received by any other ELEKTRA system component, to detect the exceeding of an upper time limit of the age of data and to perform a safety reaction depending on the specific application of the monitoring component. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | system administration SUBIT |  |  | System Baseline: | PV_GEN_2.15.4 |
| Comments: |  |  |  |  |  |

| FUR:4401 (TRT722) |  | Monitoring of reactivity of ELEKTRA system components - configurable max. age of data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The upper limit of the age of data must be configurable in the monitoring component for each component that has to be monitored. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | code reading system administration SUBIT |  |  | System Baseline: | PV_GEN_2.15.4 |
| Comments: | Info: Configurable means that the upper age limit is a database item but cannot be changed via user interface of the data preparation tool. |  |  |  |  |

#### Monitoring of reactivity of system components by the CC and vice versa

Note:
In the following context the term “EC“ refers to EC and also to EC-MUX.

The mechanism described by the requirements below provides supervision of the communication between system components by monitoring the reactivity of system components EC, RCU or TCD by the CC, or vice versa, by monitoring the reactivity of CC by one of the system components (EC, RCU or TCD). The monitoring is based on the supervision of response time according to FUR:°4400 (TRT721).

Constraint:
The function has to be implemented for ELEKTRA 2 only.

| FUR:4403 (TRT964) |  | Monitoring of reactivity of the system components - determination of response time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The monitoring of reactivity functionality has to provide a SW component in both system components which communication is monitored. This SW component measures the response time on the supervision side by sending messages periodically (each second) to the active component which is monitored. The monitored side must echo each message back to the supervisory side. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | module test |  |  | System Baseline: | PV_GEN_2.15.4 PV_GEN_2.16.7 PV_GEN_2.17.0 |
| Comments: | Example: If the CC monitors reactivity of the EC, in both system components SW component has to be provided. This SW component measures the response time on the supervision side (CC) by sending messages each second to the active component which is monitored (EC). The EC system component has to echo each message back to the supervisory side - CC. This mechanism is in general same for any variant of monitoring of the reactivity between system components. |  |  |  |  |

| FUR:4404 (TRT971) |  | Monitoring of reactivity of the system components - congestion reaction |  |  |  |
| --- | --- | --- | --- | --- | --- |
| When the response time, determined by the monitoring mechanism, exceeds the threshold value as described in FUR:°4406 (TRT969), a congestion reaction has to be initiated. The SW that performs this reaction is provided in country specific code. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°4406 (TRT969) FUR:°4405 (TRT970) |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | module test system administration SUBIT |  |  | System Baseline: | PV_GEN_2.15.4 PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | - The threshold valuse and congestion reactions for customer ÖBB is defined in FUR:°4407 (TRT968) and FUR:°4408 (TRT967), see [SRS ÖBB SA]. - The threshold values and congestion reactions for customer SBB is defined in FUR:°4412 (TRT999), FUR:°4413 (TRT1000), FUR:°4424 (TRT1112), FUR:°4425 (TRT1113), FUR:°4426 (TRT1114), FUR:°4427 (TRT1115), FUR:°4428 (TRT1116) and FUR:°4429 (TRT1117), see [SRS SBB SA]. The threshold values and congestion reactions for customer INT is defined in FUR:°4414 (TRT1064), FUR:°4415 (TRT1065), FUR:°4417 (TRT1093), FUR:°4418 (TRT1094) and FUR:°4419 (TRT1095), see [SRS INT SA]. In case of monitoring of reactivity of the EC or RCU by CC, delay or loss of individual reactivity messages from secondary EC/RCU is monitored but it shall not result in a congestion reaction. This is also valid in the case if the redundancy switch-over occured and the primary EC/RCU turns to secondary. - In case of monitoring of reactivity of the TCD by CC, delay or loss of individual reactivity messages from only one TCD redundancy is monitored but it shall not result in a congestion reaction in the CC (notice: CC-LDI(TCD) is a HOT/HOT connection). |  |  |  |  |

| FUR:4405 (TRT970) |  | Monitoring of reactivity of the system components - return to normal operation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| When the response time is within the threshold value for n (reset counter value) consecutive times, the congestion reaction has to be reverted to establish normal operation. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | module test system administration SUBIT |  |  | System Baseline: | PV_GEN_2.15.4 PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | The reset counter value between system components EC-CC (EC monitors CC) and RCU-CC (RCU monitors CC) is not configurable in the DB. This value is hard-coded defined and it has the value 3. Reset counter value for the monitoring of reactivity between system components CC-EC, CC-RCU, CC-TCD and TCD-CC has hard-coded value of 5 but the value can be overruled if other value is defined in the DB (configurable in the DB). |  |  |  |  |

| FUR:4406 (TRT969) |  | Monitoring of reactivity of the system components - threshold values |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The threshold value of the response time is provided by a procedure interface to country specific code. If no values are provided by this interface, default values have to be assigned via constants. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | module test |  |  | System Baseline: | PV_GEN_2.15.4 PV_GEN_2.16.7 PV_GEN_2.17.0 |
| Comments: |  |  |  |  |  |

| FUR:4422 (TRT1110) |  | Monitoring of reactivity of the EC by the CC - usage of EC-FlowControl |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In the case of monitoring of reactivity "CC monitors EC", the echoed response message, sent by EC, shall be sent via the EC-FlowControl. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | code reading module test |  |  | System Baseline: | PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | In this way the message-jam in the EC will be detected and the EC-FlowControl will be supervised. Info Checking: "Module test" means module test of generic functions. |  |  |  |  |

| FUR:4423 (TRT1109) |  | Monitoring of reactivity of the CC by the RCU - usage of RCU-FlowControl |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In the case of monitoring of reactivity "RCU monitors CC", the supervisory signal, sent by RCU, shall be sent via the RCU-FlowControl. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | code reading module test |  |  | System Baseline: | PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | If this mechanism is activated all 2-channel X25 connections will be disconnected (as described in FUR:4419 (TRT1095)) and the connection to the substations will remain unchanged. This not applies to customer SBB because in this case the supervisory signal, sent by RCU, is not sent via the RCU-FlowControl. Info Checking: "Module test" means module test of generic functions. |  |  |  |  |

| FUR:4436 (TRT1127) |  | Monitoring of reactivity of the RCU by the CC - usage of RCU-FlowControl |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In the case of monitoring of reactivity "CC monitors RCU", the echoed response message, sent by RCU, shall be sent via the RCU-FlowControl. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | code reading module test |  |  | System Baseline: | PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | This not applies to customer SBB, as the RCU-FlowControl of RCU SBB does not control the message flow from RCU to CC. Origin see also: ELSYS00000501 Info Checking: "Module test" means module test of generic functions. |  |  |  |  |

| FUR:4430 (TRT1118) |  | Monitoring of reactivity of the CC by the RCU - the CC sends to RCU the longest response time of the monitored ECs |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In the case of monitoring of reactivity "RCU monitors CC", the CC shall additionally send the information to RCU about the longest value of response time of the ECs monitored by CC, measured at the moment of response to RCU component. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | code reading module test |  |  | System Baseline: | PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | The value of the response time of the ECs monitored by CC, sent by CC to RCU, is the longest value measured from monitored ECs (EC1, EC2 ...ECn). Info Checking: "Module test" means module test of generic functions. |  |  |  |  |

| FUR:4431 (TRT1119) |  | Monitoring of reactivity of the CC by the RCU - supervision of 2 time limits |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In the case of monitoring of reactivity "RCU monitors CC", the RCU shall monitor two separate limits. - the value of the response time from CC - the value which is calculated as the value transferred by CC, about the longest measured response time of the monitored ECs, summed to the value of the response time of the CC which is measured by RCU. These two values for the threshold limit have to be independent of each other. As soon as one of the time limits, defined above, exceeds the allowed threshold value, the RCU shall initiate congestion reaction. The congestion reaction is the same for both cases and can only be canceled if none of the time limits is exceeded any more. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | code reading module test |  |  | System Baseline: | PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | Origin see also: ELSYS00000497 Info Checking: "Module test" means module test of generic functions. |  |  |  |  |

| FUR:4420 (TRT1106) |  | Monitoring of reactivity of the system components - congestion reaction and diagnosis |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For being able to identify explicitly the change from normal operation to congestion state, a diagnosis message has to be sent to DGP containing the identification of the supervised component. This message causes a spontaneous output by DGP on data logger. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | module test system administration SUBIT |  |  | System Baseline: | PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | Info Checking: "Module test" means module test of generic functions. |  |  |  |  |

| FUR:4421 (TRT1107) |  | Monitoring of reactivity of the system components - return to normal operation and diagnosis |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For being able to identify explicitly the change from congestion state to normal operation, a diagnosis message has to be sent to DGP containing the identification of the supervised component. This message causes a spontaneous output by DGP on data logger. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management safety management |
| Checking: | module test system administration SUBIT |  |  | System Baseline: | PV_GEN_2.16.7 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | Info Checking: "Module test" means module test of generic functions. |  |  |  |  |

### Enhancement Interface DGP-Webride (MDS) (45)

In a first step (baseline CH18.30) the enhancement relates to the interface between DGP data logger and Webride only.

Representative test cases for station Wanzwil (Switzerland) will show if an acceleration by means of an ethernet interface between DGP data logger and Webride is sufficient or if further steps have to be taken.

In a second step (baseline CH21.40) the enhancement is extended also to

- the interface between DGP-SMT (Signalmeister-Terminal) and Webride

- the interfaces between DGP fault printer and protocol printer and Webride

One reason for the enhancement is the increase of the performance of the interface between DGP data logger and Webride due to

- the necessity to prevent loss of data, especially in high-load situations (disturbance, shutdown or reintegration of system components)

- the necessity to reduce the delay time between the occurrence of a disturbance and its indication on the Webride user interface (improvement of the actuality of diagnosis data)

A further reason for the enhancement is the simplification of the interface by replacing the 4 serial interfaces by one ethernet interface.

Note: unless otherwise stated the following requirements apply to all interfaces (data logger, SMT, fault printer, protocol printer) between DGP and Webride.

See also [SRS SBB SA] (SBB) and [SRS ÖBB SA] (ÖBB)  for country specific requirements.

Note that MDS is used for ELEKTRA-ÖBB and ELEKTRA-International as successor of the Webride.

| FUR:4505 (TRT737) |  | Enhancement Interface DGP-Webride - Detection and indication of faulty transmission or loss of data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Both DGP and Webride have to detect a faulty transmission or loss of data via interface DGP - Webride and have to generate diagnostic information. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | DGP-SW Webride |  |  | Originator: | technical management |
| Checking: | code reading integration test |  |  | System Baseline: | PV_GEN_2.17.8 PV_CH_2.13.0 PV_CH_2.30.0 |
| Comments: | Info Checking: The part "faulty transmission" has to be verified by code reading. |  |  |  |  |

| FUR:4506 (TRT738) |  | Enhancement Interface DGP-Webride - Re-transmission of faulty or lost data by DGP |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In case of a faulty transmission or loss of data via interface DGP - Webride the DGP has to re-transmit the corresponding data when the transmissiom media is available again until the data transmission is successful resp. as long as the diagnostic data are available. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | DGP-SW |  |  | Originator: | technical management |
| Checking: | code reading integration test |  |  | System Baseline: | PV_GEN_2.17.8 PV_CH_2.13.0 PV_CH_2.30.0 |
| Comments: | Info: The amount of diagnostic data that can be stored is limited by the DGP buffer size. In case of a long-term interface failure this limit may be exceeded. The loss of data has to be detected and indicated according to FUR:4505. Info Checking: The part "faulty transmission" has to be verified by code reading. |  |  |  |  |

| FUR:4509 (TRT748) |  | Enhancement Interface DGP-Webride - Time stamp for outgoing DGP diagnostic data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| All diagnostic data sent to Webride have to be marked by the DGP with an additional time stamp added just before sending the data. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | DGP-SW |  |  | Originator: | technical management |
| Checking: | code reading integration test |  |  | System Baseline: | PV_GEN_2.17.8 PV_CH_2.13.0 PV_CH_2.30.0 |
| Comments: | Info: This time stamp is added to the existing time stamp assigned when diagnostic data enter DGP. Info: With these two time stamps it is possible to check the delay for the transmission of diagnostic data to Webride (see [SRS SBB SA], PRR:4508). Info Checking: The time stamps may be verified during integration test by checking the Webride log files. |  |  |  |  |

| FUR:4507 (TRT739) |  | Enhancement Interface DGP-Webride - Max. detection and reconnection time of interface failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DGP has to detect a failure of the interface DGP - Webride within 30 seconds. If the physical connection to the Webride is available again, the DGP has to establish the connection within 30 seconds |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | DGP-SW Webride |  |  | Originator: | technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.17.8 PV_CH_2.13.0 PV_CH_2.30.0 |
| Comments: | Info Checking: The time values may be verified during integration test by checking the DGP log files. |  |  |  |  |

### RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN (49)

RCU/SCU communication between ELEKTRA systems was originally handled via the low-level X.25 protocol (an ITU-T standard) over serial connections. This communication system now can be replaced by TCP/IP over Ethernet communication (individually selectable per line by database entry). This applies also to the RCU/SCU communication between ELEKTRA IXL and other external components via Ethernet LAN.
The application protocol (e.g. X25 an application protocol defined by the Austrian national railways), which is on top of the transport layer, remains unchanged.
In general, the RCU/SCU communication between ELEKTRA IXL and other external components via Ethernet LAN can be implemented as one-channel or two-channel communication.

Furthermore in the sense of this feature and requirements, respectively the terms “LAN” and “Ethernet” are used interchangeably.

| FUR:4901 (TRT791) |  | RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN - Type of Interface TCP/IP-based |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to communicate according to [IF-Spec X25] “Spezifikation X25 - Schnittstelle (3BU 15000 4167 DTAPC)" via a TCP/IP-based type of interface (RCU/SCU). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SSU-SW |  |  | Originator: | technical management |
| Checking: | integration test system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The main reason for this requirement is to reduce the number of SCUs, if many ELEKTRA interlocking systems have to be connected. Additionally, this requirement is to ease testing and demonstration. Information: Checking to be applied: integration test -> customer OBB system test -> all other customers |  |  |  |  |

| FUR:4902 (TRT792) |  | RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN - Configuration of type of Interface |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The type of the interface for communication according to [IF-Spec X25] “Spezifikation X25 - Schnittstelle (3BU 15000 4167 DTAPC)" must be configurable. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SSU-SW |  |  | Originator: | technical management |
| Checking: | code reading integration test system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: Configurable means that the interface type (X25/ethernet) is a database item (per line) which can be changed via user interface of the data preparation tool or is derived from a configuration file (depending on the particular customer). Info Checking: By means of "integration test" the impact of different configurations is tested, by means of "code reading" the configurability itself is checked. Information: Checking to be applied (integration test/system test): integration test -> customer OBB system test -> all other customers |  |  |  |  |

| FUR:4903 (TRT793) |  | RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN - Maximum detection time of interface failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Both communication sides (ELEKTRA -ELEKTRA or ELEKTRA - other external component), connected via Ethernet LAN, have to detect a failure of the interface within 6 seconds. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SSU-SW |  |  | Originator: | technical management |
| Checking: | integration test system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info for checking: Loss of connection must lead to a reaction within 6 seconds. Information: Checking to be applied: integration test -> customer OBB system test -> all other customers |  |  |  |  |

| FUR:4904 (TRT794) |  | RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN - Indication of interface failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A failure of the interface in the ELEKTRA - ELEKTRA or ELEKTRA - other component communication via Ethernet LAN, has to be indicated on the ELEKTRA local VDU (and on the remote VDU in the case of ELEKTRA - ELEKTRA connection). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SSU-SW VCX-SW |  |  | Originator: | technical management |
| Checking: | integration test system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: A failure of the ELEKTRA - ELEKTRA or ELEKTRA - other components interface is recorded by the DGP. Info: VDU is an abbreviation for "Vital Display Unit" and is the display unit of the MMI subsystem. See also document [SysDesc-RC-ILS-CTC] (System Description Remote Control - Interlocking System CTC). Information: Checking to be applied: integration test -> customer OBB system test -> all other customers |  |  |  |  |

### Serial Connection ELEKTRA - AzLM (52)

For general notes on the functionality refer to the subsection having the same name in the "Technical Interface" section.

| FUR:5204 (TRT846) |  | Serial connection ELEKTRA <-> AzLM - projectable AzLM |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA system must support a projectable axle counter dedicated as OCS serial connected AzLM. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW OREST |  |  | Originator: | product management |
| Checking: | code reading integration test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: Checking: The request of projectable attributes will be proven by means of "code reading". |  |  |  |  |

| FUR:5206 (TRT848) |  | Serial connection ELEKTRA <-> AzLM - AZGR |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The serial connection ELEKTRA <-> AzLM must support the following three projectable variants of the "reset" action (dt. Achszähler-Grundstellung) per AzLM section: - unconditional reset, - conditional reset, - preparatory reset without acknowledgment. The reset variant is selected by data preparation. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°5218 (TRT953) |
| Allocation: | ECX-SW OREST |  |  | Originator: | product management |
| Checking: | code reading integration test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: Reference document: "Data Interface Specification Az LM - ESTW" (3CR 01800 ALAA PBAFA). Info: Checking: The request of projectable attributes will be proven by means of "code reading". |  |  |  |  |

| FUR:5214 (TRT948) |  | Serial connection ELEKTRA <-> AzLM - Indication: Loss of Outgoing Telegram |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For successive messages to the OCS-System the ELEKTRA has to provide a sequence number which is strictly monotonic increasing. With this sequence number the OCS-System is able to detect a loss of telegrams. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | module test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The sequence number is not synchronous between software channel A and B. Info Checking: "Module test" is the only possibility of verification. Info: This requirement is needed for OCS-SB/OCS-SBJ. |  |  |  |  |

| FUR:5215 (TRT949) |  | Serial connection ELEKTRA <-> AzLM - Indication: Wrong Outgoing Message Routing |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If the ELEKTRA sends data to the OCS-System the ELEKTRA has to provide its own application instance. This allows the OCS to detect a wrong correlation between virtual channel and sending application instance (wrong message routing). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | module test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The OCS destination instance is the same for all virtual channels and therefore useless for verification. Info Checking: "Module test" is the only possibility of verification. Info: This requirement is needed for OCS-SB/OCS-SBJ. |  |  |  |  |

| FUR:5216 (TRT950) |  | Serial connection ELEKTRA <-> AzLM - Data Preparation of AzA-Limit |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA system must handle up to 40 AzAs. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°5217 (TRT951) |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The old AzLM system supports a total number of 20 AzAs. All AzAs are connected to one logical EC. In the new system a distributed AzA-configuration is possible using each logical EC. In this configuration a minor load factor for one AzA is expected (less sections configured for one AzA) compared to the old system. For an assumed average load factor of 50% (results in 16 sections per AzA) the number of AzAs has to be duplicated to provide the same amount of sections than before. The limit of 20 AzAs for one EC is still valid (handled via ELEKTRA system limits). |  |  |  |  |

| FUR:5218 (TRT953) |  | Serial connection ELEKTRA <-> AzLM - Treadle |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA system must support the projectable reset command (dt. Achszähler-Grundstellung) for the treadle (dt. Radsensor). |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°5206 (TRT848) |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | system test |  |  | System Baseline: |  |
| Comments: | Info: Reset command for a treadle is also named "activate treadle". |  |  |  |  |

| FUR:5219 (TRT954) |  | Serial connection ELEKTRA <-> AzLM - Restricted Treadle Support |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Restricted treadle support has to be integrated into ELEKTRA by treating treadles as axle counters and not as rail detectors (from MMI and CC point of view). |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | system test |  |  | System Baseline: |  |
| Comments: | Info: To accomplish this the ELEKTRA element controller (EC) has to map the reset commands and state infos. Info: Direction specific information of the treadle is not evaluated at the moment. |  |  |  |  |

| FUR:5227 (TRT975) |  | Serial connection ELEKTRA <-> AzLM - Force Detection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA system must support a projectable force detection by serial connected AzLM. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW OREST |  |  | Originator: | product management |
| Checking: | code reading integration test system test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The serial force detection is used for high speed switches. Info: AzLM force detection (parallel) already exists since BL18.00 (see FUR:°1819 (TRT394) SRS SFW, 3BU_15000_0310_DTAPA). Info: Checking: The request of projectable attributes will be proven by means of "code reading". |  |  |  |  |

| FUR:5228 (TRT976) |  | Serial connection ELEKTRA <-> AzLM - AZGR, Force Detection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For the serial force detection in the ELEKTRA system only reset type "unconditional reset" is allowed. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°5206 (TRT848) |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test module test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The serial force detection is used for high speed switches. Info: For other variants see: FUR:°5206 (TRT848). |  |  |  |  |

| FUR:5229 (TRT977) |  | Serial connection ELEKTRA <-> AzLM - State Mapping, Force Detection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For the serial force detection the ELEKTRA system has to treat the state info "disturbed" as "occupied". |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: Other states are treated unchanged. Info: In the AzLM-System a track section providing the force detection is projected as dead-end track (ch. Stumpengleis, dt. Stumpfgleis). |  |  |  |  |

### DGP Enhancements (53)

#### Time signal via NTP (53.2)

| FUR:5306 (TRT1373) |  | DGP extensions - Time signal via NTP - Time signal |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The DGP shall be able to receive periodically the time signal via ethernet interface from the NTP server of Webride and shall distribute the signal in the system. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | DGP-SW |  |  | Originator: | product management technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.17.8 PV_CH_2.30.0 |
| Comments: | - The radio clock used to date must not be connected in parallel any more. - The use of the time signal from the NTP server for diagnostic purposes is uncritical. - At the moment the period for reception of the time signal is 8s. - This DGP extension must not be enabled if the NTP-synchronization according to requirement ELSEC:SRS:REQ:0023 of [SRS Security] is activated. |  |  |  |  |

### Remote Control: decentralized architecture (62)

The following requirements are referring to feature INT:FL:FEA:0237 (Multiple EBO2-CC connections) of  Feature List International [FL INT].

The functions defined in this feature are requested for ELEKTRA 2. The cause of the introduction of these functions is to enable the new concept for remote control of stations according to “Duty Book Remote Control” of MÁV.

| FUR:6205 (TRT1088) |  | Remote Control: decentralized architecture - maximal number of subnet |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The maximum number of possible subnets within ELEKTRA LAN in the supervised area shall be 40. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management technical management |
| Checking: | code reading integration test |  |  | System Baseline: | PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: |  |  |  |  |  |

### ELEKTRA 2 components on ELEKTRA 1 (67.1)

| TIR:6700 (TRT1435) |  | Hybrid System - ELEKTRA 2 components on ELEKTRA 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Generic System ELEKTRA 2 SW/HW components have to support the interconnection to ELEKTRA 1 system (hybrid version) (e.g. cabeling for interconnection, EndToEnd checks, clock synchronisation, reactivity supervision, route configuration, …) |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL1 Hybrid |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.19.0 |
| Comments: | Currently the following realizations are required. Usage in the field: RCU/SCU/DGP ELEKTRA 2 must communicate with CC/PC ELEKTRA 1. Usage for testing: DGP/CC-Simulation ELEKTRA 2 must communicate with PC ELEKTRA 1. TCC (DGP with ELEKTRA simulation) ELEKTRA 2 must communicate with CC ELEKTRA 1. |  |  |  |  |

## Operational Requirements

| PRR:0115 (TRT161) |  | 100 changes of visualization points per second |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The throughput rate between the XGIOS call entry and the change of the visualization point on the screen must be at least 100 changes of visualization points per second. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | XGIOS performance test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Refer to document "ELEKTRA, ESTW, Pflichtenheft eines Prozess-Videosystem fuer den Einsatz in ELEKTRA, 1988-06-10". Because of the new MMI conception (e.g. with one VC channel only) this requirement must be revised, i.e. we have to look at the performance of the entire MMI! Possibly the "update" conception (usage of SGIOS update-callgates) of today's VC application must be revised. |  |  |  |  |

| PRR:0319 (TRT162) |  | Boot time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Boot of the system has to be performed within 180 seconds. Boot time is the time interval between system reset and operation (AFG, WIN) |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | optional |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Also a dependability requirement. The size of the station has to be defined. Boot time depends on the system configuration since image load time increases if more applications are executed on the same computer. |  |  |  |  |

| PRR:0320 (TRT163) |  | Standby switch-over time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The switch from the active component to the standby component has to be performed within 3 seconds. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Also a dependability requirement. Interruption of regular operation is defined by the standby switch over time and the time for GA. |  |  |  |  |

| PRR:0321 (TRT164) |  | EC update time (Generalabfrage) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The update of an EC (i.e. Generalabfrage) has to be done within 30 seconds. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Also a dependability requirement. Time interval depends on amount of elements serviced by the EC. Currently the update of a PC has an impact for the operator only during boot time and during normal operation, since this is performed in background. Currently the update of a PC is around 40 sec (maximum). |  |  |  |  |

| PRR:0322 (TRT165) |  | Signal throughput of the communication system |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The signal throughput of the communication system has to be at least 300 CHILL signals of 150 bytes per second. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | performance test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Derived from the fact that 70 peripheral event changes must be processed within 1 second. Valid for the case that CCA and CCB are executed on two different computers. If CCA and CCB are executed on the same computer the signal throughput of the communication system must be higher in order to handle the required number of event changes. |  |  |  |  |

| PRR:0323 (TRT166) |  | Command reaction time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The command reaction time has to be less than 1 second. The command reaction time is the time interval between a command and the corresponding response for the user. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| PRR:0324 (TRT167) |  | High load situations |  |  |  |
| --- | --- | --- | --- | --- | --- |
| High load situations must not lead to component brake-down. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| PRR:0601 (TRT168) |  | Function for setting the priority of messages |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to give certain messages a higher priority than other messages (from the HW-interfaces) |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | FE integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| PRR:0602 (TRT228) |  | Definition of configuration and scaling |  |  |  |
| --- | --- | --- | --- | --- | --- |
| There has to be a concept how to configure and scale the system in dependency of availability and performance |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | analysis |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | There have to be rules how to project the number of computers depending on the number of elements (points, signals, ...). |  |  |  |  |

| PRR:0805 (TRT169) |  | Period of input data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The input data (scanner values) must be available at the Element Controller within a period of 100ms. This means also that only one state change within 100ms must be detected. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| PRR:0806 (TRT170) |  | Period of output data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The output setting of an element aspect must not exceed one second. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | EC SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | An appropriate timing requirement for the FE subsystem must be derived! (L-Signal) |  |  |  |  |

### RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN  (49)

For general notes on the functionality refer to chapter "Functional Requirements", subchapter "RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN (49)".

| PRR:4905 (TRT795) |  | RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN - Sufficient performance of ethernet interface |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be guaranteed by the system that the use of an ethernet interface between two ELEKTRA IXLs, or between ELEKTRA IXL and other external components, is of sufficient performance to prevent loss of data. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | technical management |
| Checking: | integration test system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: Requirement FUR:4505 states, that a diagnostic information shall be generated. Info: The performance can be verified by some test cases showing normal operation and one test case for high load. Info: For ÖBB, the limits (number of stations, points, …) remain unchanged, i.e. the limits for communication via Ehternet LAN are the same as for communication via X.25. Information: Checking to be applied: integration test -> customer OBB system test -> all other customers |  |  |  |  |

| PRR:4906 (TRT1436) |  | External RCU communication by using TAS-PLF OCS-SBJ communication framework |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Generic System shall support the startup and stop of OCS-SBJ on the application RCU. OCS-SBJ shall be supported on the HW platforms MEN-D6 and MEN-D10. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | system |
| Checking: | integration test system test |  |  | System Baseline: | PV_GEN_2.19.0 PV_GEN_2.20.10 |
| Comments: | Notice: In the first step OCS-SBJ will be used for OBB "X25 over UDP/IP with RaSTA" connections. Starting with PV_GEN_2.20.10 this requirement applies also to customer SBB. |  |  |  |  |

## Constraints

| GCR:0162 (TRT107) |  | Use Intel-based Linux-PC for ELEKTRA MMI |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ELEKTRA MMI must be an Intel-based personal computer with the operating system 'Linux'. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-HW |  |  | Originator: | technical strategy |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0163 (TRT108) |  | Based on today's ELEKTRA-VC application |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ELEKTRA MMI (EBO1.5) must be based on today's ELEKTRA-VC application. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI |  |  | Originator: | system |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: Not applicable for NRIC and INT. |  |  |  |  |

| GCR:0164 (TRT109) |  | LCD screens |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ELEKTRA MMI must support LCD screens. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-HW |  |  | Originator: | product strategy |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0301 (TRT110) |  | Scalability of CC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The execution of CCA and CCB on the same computers and on two different computers as well has to be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The system configuration (i.e. integration/separation of CC) has to be a configuration issue and therefore application-transparent. |  |  |  |  |

| GCR:0303 (TRT111) |  | System configuration transparency concerning database access |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The system configuration (i.e. location of applications on computers) has to be transparent concerning database access for all ELEKTRA applications. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0301 (TRT110) |
| Allocation: | CAR |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0304 (TRT112) |  | System configuration transparency for operational functionality |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System configuration (i.e. redundancy degree of applications and location of applications) has to be transparent for operational functionality. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0301 (TRT110) |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0305 (TRT113) |  | System configuration handling of technical functionality |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Technical functionality must handle different redundancy degrees of applications. Also the location of the applications has to be transparent for technical functionality. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0301 (TRT110) |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0347 (TRT115) |  | System configuration transparency for run-time behaviour of applications |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System configuration (i.e. location of applications) has to be transparent for the run-to-completion semantic of the applications. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0301 (TRT110) |
| Allocation: | CAR EP |  |  | Originator: | system |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The signal priorities have to be preserved. I.e. internal signals (e.g. signals within CCA) derived from an external signal have to be processed before another external signal. Run-to-completion behaviour is not necessarily needed for EC, but due to simplicity reasons the same run-to-completion behaviour as for CC will be used. |  |  |  |  |

| GCR:0348 (TRT116) |  | Time increments |  |  |  |
| --- | --- | --- | --- | --- | --- |
| As in ELEKTRA Classic time increments have to be based on signals. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0301 (TRT110) |
| Allocation: | EP |  |  | Originator: | system |
| Checking: | EP test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | In ELEKTRA Classic time increments are provided via update_time signals generated by VOTRICS. It is recommended to use the same method as in ELEKTRA Classic in order to maintain the current application run-time behaviour. |  |  |  |  |

| GCR:0349 (TRT117) |  | System configuration transparent for time increments. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Time increments for CCA and CCB located on the same computer have to be independent. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0301 (TRT110) |
| Allocation: | CAR EP |  |  | Originator: | system |
| Checking: | module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The run-time behaviour is not clear in case CCA and CCB located on the same computer are provided with the same time increments. In ELEKTRA Classic SBB the clock is explicitly synchronised between CCA and CCB for certain functionality. For ECA and ECB independent clocks are not required. |  |  |  |  |

| GCR:0352 (TRT118) |  | Scalability of application resources |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Application resources (memory, stacks, ...) have to be scalable. Otherwise computers with the maximum required memory size are needed - especially for small stations where CCA and CCB are executed on one computer. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0359 (TRT119) |  | Source code basis for technical functionality |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The same source code basis of technical functionality has to be used for the ELEKTRA product for all customers in all countries. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | optional |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0361 (TRT120) |  | Highly available communication media |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A highly available communication media has to be supported for ELEKTRA Evolution system architecture. This is achieved with the use of redundant communication paths. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0601 (TRT121) |  | No support for usage of old PC and new EC in one plant |  |  |  |
| --- | --- | --- | --- | --- | --- |
| no mixture of the PC ELEKTRA Classic and the EC ELEKTRA Evolution in one interlocking is possible |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | design review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0602 (TRT122) |  | Replacement of the relay 65E |  |  |  |
| --- | --- | --- | --- | --- | --- |
| encouragement of industrial relays respectively electronic components |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | system design |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | valid only for variant FE-3b, date of runout not fixed |  |  |  |  |

| GCR:0808 (TRT123) |  | Scaleable FE architecture |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The FE system architecture must be scaleable for small, medium and large interlocking systems. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA |  |  | Originator: | product strategy |
| Checking: | design review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0812 (TRT124) |  | Using TAS Platform computers for EC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The element controller (EC) must be a TAS Platform Computer. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0337 (TRT52) |
| Allocation: | SA |  |  | Originator: | technical strategy |
| Checking: | design review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | In this context a TAS Platform computer is a computer board where the TAS Platform system software is executed. Sub-Requirement of GCR:°0337 (TRT52). |  |  |  |  |

| GCR:0818 (TRT125) |  | Preservation of the current ELEKTRA's field element controlling software |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The field element controlling software of the ELEKTRA Classic has to be used. I.e. the current PC software has to be used (ported). Additionally the master-slave activation of the interface boards has to be used. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| GCR:0823 (TRT1090) |  | ELEKTRA IXL using TAS Platform 2.x on MEN-D10 and MEN-D6 computers |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA interlocking system, running on the TAS Platform 2.x shall be executable on MEN-D10 and MEN-D6 computer boards. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: |  |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.17.0 PV_GEN_2.17.5 PV_GEN_2.19.0 PV_GEN_2.20.10 |
| Comments: | D2 computer board is no longer supported starting with PV_GEN_2.20.10. |  |  |  |  |

| GCR:0337 (TRT52) |  | Use of TAS Platform |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA components shall use TAS Platform Core (AEOS 5.x) "non-redundant" as their software platform. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EP |  |  | Originator: | system |
| Checking: | design review |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 PV_GEN_2.20.10 |
| Comments: | Used for CC, EC, DGP, RCU, SCU, TCD and IAU applications. See [TAS Plf Safety Handbook]. |  |  |  |  |

# Interfaces

## Technical Interface

| TIR:0308 (TRT209) |  | Protocols to supported for communication with ELEKTRA Classic applications |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Following protocols have to be supported in order to communicate with ELEKTRA Classic applications: VNODE protocol for CC, VC, DGP, RCU / DTSE protocol for PC, TCD. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EP |  |  | Originator: | system |
| Checking: | EP test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Currently the following realizations are required. Usage in the field: RCU/DGP ELEKTRA 2 must communicate with CC/PC and SCU ELEKTRA 1. Usage for testing: DGP/CC-Simulation ELEKTRA 2 must communicate with PC ELEKTRA 1. Usage for Testing: DGP/ELEKTRA Simulation ELEKTRA 2 must communicate with CC ELEKTRA 1. |  |  |  |  |

| TIR:0601 (TRT210) |  | master-slave-procedure, correct order of outputs |  |  |  |
| --- | --- | --- | --- | --- | --- |
| the hw-interfaces must maintain this principle |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | FE integration test |  |  | System Baseline: |  |
| Comments: | master-slave procedure and order of outputs needed to supervise the outdoor equipment |  |  |  |  |

### Enhancement Interface DGP-Webride (45)

For general notes on the functionality refer to chapter 4.1.4.

| TIR:4504 (TRT743) |  | Enhancement Interface DGP-Webride - Transmission bit rate |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The transmission bit rate of data via ethernet interface DGP - Webride must be 100Mbit/s full-duplex. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | DGP-SW Webride |  |  | Originator: | technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.17.8 PV_CH_2.13.0 PV_CH_2.30.0 |
| Comments: |  |  |  |  |  |

### RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN  (49)

Note: The following requirements apply for the functionality "Communication between ELEKTRA and ILTIS via TCP/IP (54)", too.

For general notes on the functionality refer to chapter "Functional Requirements", subchapters "RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN (49)" resp. "Communication between ELEKTRA and ILTIS via TCP/IP (54)".

| TIR:4907 (TRT796) |  | RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN - Transmission bit rate |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The transmission bit rate of data via interface ELEKTRA - ELEKTRA or ELEKTRA - other component, must be configurable. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: Intended bit rates for the two available types of hardware platforms: D6/D10: Auto-negotiation is possible; the transmission bit rate depends on the network, but can also be configured. Possible values are: 10 Mbit/s half-duplex or full-duplex 100 Mbit/s half-duplex or full-duplex 1 Gbit/s half-duplex or full-duplex |  |  |  |  |

### EC - TTP Bus Interface (51)

Between two SW components inside the EC, called FBI (Field Bus Interface) and BusExe, data transfer is done via a shared memory area. The access to this data area happens in a non-synchronized manner. In order to accelerate the data transfer the following requirements demand a synchronization between FBI and BusExe.

The following figure, which is taken out of [4], gives a rough overview of the situation:

| TIR:5101 (TRT828) |  | EC - TTP Bus Interface - Optimized Synchronized Data Transfer IC -> EC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| TTP-bus, BusExe and FBI must be synchronized by the following mechanism: A read access of the FBI process to the shared memory (SHM) shall be triggered by a write access of the BusExe (synchronous access). Also the TTP-read access of the BusExe must be triggered by the TTP-bus. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | BusExe test EC SUBIT mit Zeitmessung FBI test module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: This mechanism covers the IC -> EC communication direction. |  |  |  |  |

| TIR:5102 (TRT838) |  | EC - TTP Bus Interface - Optimized Synchronized Data Transfer EC -> IC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| To optimize the data transfer from EC to IC the BusExe must fetch the data stored in the shared memory shortly before the next TTP-bus cycle. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | BusExe test EC SUBIT mit Zeitmessung module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Info: This mechanism covers the EC -> IC communication direction. |  |  |  |  |

### Serial Connection ELEKTRA - AzLM (52)

In the Swiss ELEKTRA application software baseline BL19.00 a new interface to the AzLM system of HITACHI Rail Austria is realized via OCS.

See also [SRS SBB SA] for further requirements concerning Switzerland.

In the OEBB ELEKTRA application software baseline BL28.10 a new interface to the AzLM system of HITACHI Rail Austria is realized via OCS.
This has to be implemented for ESTW (EL2) only.

See also [SRS ÖBB SA] for further requirements concerning ELEKTRA ÖBB.

| TIR:5201 (TRT830) |  | Serial connection ELEKTRA <-> AzLM - OCS Connection with SAHARA Protocol |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A serial connection between an axle counter system of type AzLM and the ELEKTRA must be realized by an OCS connection. For the OCS connection the SAHARA protocol must be selected. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | code reading configuration checking integration test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: This requirement implies using the existing ELEKTRA OCS binding. |  |  |  |  |

| TIR:5205 (TRT857) |  | Serial connection ELEKTRA <-> AzLM - interrupt-free switch-over |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ELEKTRA must guarantee an interrupt-free switch-over of an EC redundancy without disturbance of axle-counter-sections. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: |  |  |  |  |  |

| TIR:5206 (TRT858) |  | Serial connection ELEKTRA <-> AzLM - AzLM protocol version |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ELEKTRA must receive and check a version number indicating the used AzLM protocol from AzLM. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | code reading integration test module test |  |  | System Baseline: |  |
| Comments: | Info: At the moment no AzLM protocol version (number) is available. Info: In case of a version mismatch the connection to the AzLM must not be established. |  |  |  |  |

| TIR:5209 (TRT865) |  | Serial connection ELEKTRA <-> AzLM - CAED version |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ELEKTRA must receive and check the customer application engineering data (CAED) version number from AzLM. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | code reading integration test module test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The CAED version number must be consistent with the corresponding ELEKTRA database entry. Info: The CAED is specified in the document "Data Interface Az LM - ESTW 3CR_01800_ALAA_PBAFA". Info: In case of a version mismatch the connection to the AzLM must not be established. |  |  |  |  |

| TIR:5202 (TRT837) |  | Serial connection ELEKTRA <-> AzLM - Hardware Base Platform |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The serial connection ELEKTRA <-> AzLM must be realized for the MEN D10 and MEN D6 boards. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The OCS-SB/OCS-SBJ may only be used with validated HW. |  |  |  |  |

| TIR:5207 (TRT859) |  | Serial connection ELEKTRA <-> AzLM - diagnosis |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Diagnosis information must be available for: - the operator, - support personnel, - internal on-site-diagnosis. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test system test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: Operator includes MMI, ILTIS (for Switzerland) and "Störungs-/Protokolldrucker". |  |  |  |  |

| TIR:5208 (TRT860) |  | Serial connection ELEKTRA <-> AzLM - loss of connection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In case of a loss of connection the EC must set the relevant elements to state "undefined" (KAZ). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test module test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: On the MMI the affected track sections are displayed red and the "Störlistenfenster GFM" shows "KAZ" for each of these track sections. |  |  |  |  |

| TIR:5210 (TRT861) |  | Serial connection ELEKTRA <-> AzLM - re-establishment of connection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In case of a re-establishment of a connection the current AzLM state must be updated. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test module test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: |  |  |  |  |  |

| FUR:5211 (TRT876) |  | Serial connection ELEKTRA <-> AzLM - state messages of AzLM |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The EC must convert the state messages of AzLM to elem_state messages as follows: - AC section free -> e_tr_free_es - AC section occupied -> e_tr_occupied_es - AC section disturbed -> e_tr_not regular_es |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test module test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: On the MMI a disturbed track section is displayed red and the "Störlistenfenster GFM" shows "TECHNISCHE STOERUNG" for this track section. Info: Reference document: "Data Interface Specification Az LM - ESTW" (3CR 01800 ALAA PBAFA). |  |  |  |  |

| TIR:5221 (TRT956) |  | Serial connection ELEKTRA <-> AzLM - AzLM-Support for Logical ECs |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA connection to AzLM must not be restricted to dedicated logical ECs. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: AzLM track sections are no longer limited to EC-1. |  |  |  |  |

| TIR:5222 (TRT957) |  | Serial connection ELEKTRA <-> AzLM - Support of MEN-D6 Ethernet Ports |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA must support unused ethernet ports on a MEN-D6 for connections to the AzLM-System under the following restrictions: - in case of using more than one Ethernet-Port only ACEs are allowed to be connected to a single EC. - the usage of switch is not allowed. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: |  |
| Comments: | ##DESCOPE REASON: For BL19.20 it is only needed to connect AzLM and MEN-D6 via switch, see TIR:5224 ## |  |  |  |  |

| TIR:5224 (TRT972) |  | Serial connection ELEKTRA <-> AzLM - Support of MEN-D6/D10 over Switch |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA must support the connection of AzLM to MEN-D6/D10 via switch. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | TIR:°5222 (TRT957) |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.16.0 PV_GEN_2.19.0 |
| Comments: | Info: A combination of using ethernet ports and switch is not allowed. |  |  |  |  |

## User Interface / Man-Machine Interface

| MIR:0103 (TRT133) |  | Graphics display |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The MMI must have a graphics display. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-HW |  |  | Originator: | customer |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Most of the customers (e.g. OBB, SBB) ask for an ergonomic graphics display. Few customers (e.g. "Wiener U-Bahn") demand a graphics display, which has not to be ergonomic in all parts. |  |  |  |  |

| MIR:0104 (TRT134) |  | Character oriented screen layout |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Because of compatibility to ELEKTRA 1-VC (incl. ELEPRO and database) the screen layout must be character (symbol) oriented, i.e. screen and visualization parameters (like size and position) are defined (and handled) with 'character columns by character lines'. If a window system is used all windows are text windows (using customized character sets). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | GCR:°0163 (TRT108) |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | MI integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MIR:0105 (TRT135) |  | Amount of displayed information per workplace like today |  |  |  |
| --- | --- | --- | --- | --- | --- |
| With the ergonomic graphics displays it must be possible to display as much information per workplace as in ELEKTRA 1, i.e. the character size and the screen resolution must be defined so that the MMI is able to display the whole information of 6 ELEKTRA 1-VC-screens, i.e. the MMI must be able to display 18240 characters (from a set of 512 different symbols) simultaneously on one workplace. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | In ELEKTRA 1, displayed information per workplace consists of max. 18240 characters (= 6 screens with 80 columns of characters by 38 lines of characters per screen. If we change the screen resolution from 80x38 characters to, for example, 113x57 characters per screen then we have to change ELEKTRA 1 screen layouts too. |  |  |  |  |

| MIR:0106 (TRT136) |  | On one monitor twice as much displayed information as in ELEKTRA 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| With the graphics displays it must be possible to display at least twice as much information per monitor as in ELEKTRA 1, i.e. the character size and the screen resolution must be defined so that the MMI is able to display twice as much information per screen as on one of ELEKTRA 1-VC-screens, i.e. the MMI must be able to display 6080 characters (from a set of 512 different symbols) simultaneously on one screen. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | customer |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | In ELEKTRA 1 the displayed information per monitor consists of 3040 characters (= 80 columns of characters by 38 lines of characters). Note that it may not be possible to display symbols in the same obsolete size as on the BARCO monitor. |  |  |  |  |

| MIR:0107 (TRT137) |  | Amount of displayed information per workplace like 7 monitors of ELEKTRA 1-VC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| With the graphics displays it must be possible to display as much information per workplace as on 7 monitors of ELEKTRA 1-VC, i.e. the character size and the screen resolution must be defined so that the MMI is able to display the whole information of 7 ELEKTRA 1-VC-screens, i.e. the MMI must be able to display 21280 characters (from a set of 512 different symbols) simultaneously on one workplace. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | customer (OBB) |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | In ELEKTRA 1 the displayed information per monitor consists of 3040 characters (= 80 columns of characters by 38 lines of characters). The displayed information on 7 monitors would consist of 21280 (= 3040 x 7) characters. Note that ELEKTRA 1-VC supports the maximum of 6 screens. Customer "BOS system for OBB-Wels"; reported by G. Staffel |  |  |  |  |

| MIR:0108 (TRT138) |  | 512 symbols |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 512 symbols are required. So it must be possible to display 512 symbols simultaneously on one screen. Given that text window is used, two fonts must be useable (with 256 characters per font). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | XGIOS integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Today a symbol table consists of 256 characters. If more than 512 symbols are required then more than two fonts must be useable. |  |  |  |  |

| MIR:0109 (TRT139) |  | Supported Character Resolutions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The system must support the following character resolutions: 8 columns pixels x 9 lines pixels 10 columns pixels x 19 lines pixels 11 columns pixels x 20 lines pixels 16 columns pixels x 26 lines pixels |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | XGIOS module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MIR:0110 (TRT140) |  | EBO compatible symbols |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The characters of the character oriented graphics display must be EBO compatible, i.e. the character resolution must be defined so that it's possible to display characters like today's EBO symbols. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | customer (OBB) |
| Checking: | XGIOS integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MIR:0111 (TRT141) |  | EBO compatible color attributes |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The color setting of the graphics display must be EBO compatible, i.e. the colors must be defined so that it's possible to display 16 different static foreground colors, 8 static background colors, the blinking of the 16 foreground colors and all combinations of them like at today's EBO. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | customer (OBB) |
| Checking: | XGIOS integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For details refer to today's SGIOS documentation. |  |  |  |  |

| MIR:0134 (TRT143) |  | Common sound card as signal horn |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Today's VC controls the signal horn of the control desk via PARIOH and PIOS. Within the new MMI we have to use a common sound card for Linux-PCs as replacement of the signal horn (e.g. sound cards for industrial or for office PCs (COTS sound cards)). It must be possible to connect an amplifier and loudspeakers to the sound card. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-HW |  |  | Originator: | system |
| Checking: | MI integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For details about PARIOH refer to today's PARIOH documentation. |  |  |  |  |

| MIR:0137 (TRT146) |  | PIOS replacement |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The new MMI conception no longer supports the PIOS, therefore static information ("Testmode is on or off", "Workplace number of VC") must be obtained by using additional hardware (Hardlock) to store these information data. Redundancy information ("VC is the primary VC", "VC is the default primary VC") are obsolete. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-3 |  |  | Originator: | system |
| Checking: | MI integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MIR:0164 (TRT226) |  | MMI identification via Hardlock |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The static information "Workplace number of VC" (= MMI identification number) must be obtained by using a Hardlock (Dongle) which has to be plugged on a hardware interface port of the Linux-based MMI-PC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | MIR:°0137 (TRT146) |
| Allocation: | MI-HW |  |  | Originator: | product strategy |
| Checking: | MI integration test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.15.5 |
| Comments: | Depending on the MMI-PC (EBO 1.5 or EBO 2) and the PC-HW Architecture, a USB port (Universal Serial Bus) or standard parallel port (LPT) can be used. |  |  |  |  |

| MIR:0165 (TRT227) |  | MMI test mode definition via Hardlock |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The static information "VC test mode is ON" must be obtained by using an additional (second) Hardlock which has to be plugged on the first Hardlock which includes the MMI identification number ("Workplace number"). Without this second Hardlock the VC test mode is off. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | MIR:°0137 (TRT146) |
| Allocation: | MI-HW |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MIR:0317 (TRT152) |  | Hex-display information |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Substitution for hex-display information has to be provided for maintenance and diagnosis reasons. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | customer |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Using MEN-D6/D10 only a subset of the current Hex-display is possible (2 LEDs instead of a Hex display). When changing to a new hardware this function has to be changed. |  |  |  |  |

| MIR:0318 (TRT153) |  | System reset |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manual reset and self-reset of the whole system resp. for each computer has to be provided. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application TAS platform |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MIR:0336 (TRT154) |  | Boot mode functionality |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Boot mode functionality has to be supported. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | system |
| Checking: | system test |  |  | System Baseline: |  |
| Comments: |  |  |  |  |  |

| MIR:1402 (TRT271) |  | COTS MMI Speakers |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Speakers shall be standard COTS products. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-HW |  |  | Originator: | product strategy |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MIR:0170 (TRT1063) |  | EBO2 MMI - Monitor Ratios/Resolutions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The MMI (EBO2) must support the following monitor ratios/resolutions: - 5:4 / 1280 x 1024 - 16:9 / 1920 x 1080 |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-HW |  |  | Originator: | product strategy |
| Checking: | MI integration test |  |  | System Baseline: | PV_GEN_2.16.2 |
| Comments: |  |  |  |  |  |

# Requirements to the Customer

## Serial Connection ELEKTRA - AzLM (52)

Note: With TAS platform 2.5.x "OCS-SB" is no longer supported and replaced by "OCS-SBJ", initially applied in PV_GEN_2.20.10 (using TAS platform 2.6.x).

| SCR:5226 (TRT974) |  | Serial connection ELEKTRA <-> AzLM - Configuration Instruction using OCS-SBJ |  |  |  |
| --- | --- | --- | --- | --- | --- |
| An AzLM must be connected to ELEKTRA by OCS-SBJ. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product strategy |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.16.0 PV_GEN_2.20.10 |
| Comments: | Info: Checking: The request of configurable attributes will be proven by means of "code reading". |  |  |  |  |

| SCR:5225 (TRT973) |  | Serial connection ELEKTRA <-> AzLM - Data Preparation Instruction |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In case of a serial connection AzLM <-> ELEKTRA, in each concerned EC at least one topological element must be connected via conventional HW interfaces. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Data prep. guideline |  |  | Originator: | technical management |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: Topological situations without elements connected via HW interfaces are not expected (at least a power supply element is connected to the ELEKTRA via conventional HW interface). Hence a corresponding ELEKTRA configuration is not tested. |  |  |  |  |

# Safety

| SAR:0144 (TRT172) |  | Critical command on element via CC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| To overrule the electronic interlocking in non-regular situations there shall be a method that a cirtical command from MMI is sent to CC. The command has to be generated in diverse ways and has to use diverse format in regard to regular commands. On MMI there has to be an information about the command in CC. This information has to be committed, if the command has been correctly delivered to CC (in data and time). There shall be measures that the commitment cannot be produced falsely by the MMI itself. The command has to be sent also to a printer or another device for permanent storage. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | By mouse click on a critical command in menu a signal is sent to CC with this command. CCA and CCB create signals with date, time, element ID and command ID in textual form. The text is kept in memory. The signals from CCA and CCB are exchanged and voted in the display comparator. SW channel, which is not sending states for regular display, sends signal to VC. Same text is sent to printer or harddisk for later control. VC displays text in a status line. If the command is correct the station master has to repeat text via keyboard in an answer line. Answer text is send in a signal to CC. CCA and CCB compare answer text with original text they have kept in memory. If the text is equal and the printer has sent a "quit" on the text the command is executed. |  |  |  |  |

| SAR:0214 (TRT298) |  | Galvanic separation of Ethernet-ports on TAS-Platform computer |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each Ethernet-port on TAS-Platform computer must provide galvanic separation (IEEE 802.3). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EQP SA |  |  | Originator: | product strategy |
| Checking: | qualification test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0221 (TRT310) |  | Galvanic separation of Ethernet-ports on HUB or switch |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each Ethernet-port on HUB or switch must provide galvanic separation (IEEE 802.3: 1500V RMS). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EQP SA |  |  | Originator: | product strategy |
| Checking: | review data sheet |  |  | System Baseline: | PV_GEN_2.1x.x |
| Comments: |  |  |  |  |  |

| SAR:0315 (TRT175) |  | Availability of CMOS data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CMOS data has to be available in the standby component when switching from the active to the standby component. The synchronisation of CMOS data has to be performed immediately (and not periodically) when CMOS data is changed. This ensures that all CMOS data is preserved when switching to the standby component. Still, CMOS data can be lost due to checksum error. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | DPR:°0314 (TRT5) |
| Allocation: | EP |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0335 (TRT176) |  | Error reaction time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Error reaction time must not exceed 1 second. The error reaction time is the time between detection of an error and the corresponding (safety) reaction (e.g. system halt, Nothalt). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Also a dependability requirement. Reboot time to the system consists of error reaction time and system boot time. Error detection time is not considered, since this time interval is application specific. |  |  |  |  |

| SAR:0353 (TRT178) |  | Availability of printer-buffer data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Printer-buffer data has to be available in the standby component when switching from the active to the standby component. The synchronisation of the printer-buffer data has to be done according to that of CMOS data. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | DPR:°0314 (TRT5) |
| Allocation: | ELEKTRA application |  |  | Originator: | customer |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0401 (TRT237) |  | MMI-Dongle |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In the dongle must be an identifier which describes the number of the MMI (readable for the MMI). Furthermore in the dongle must be a coded information (which is only useable by the CC) which also includes the number of the MMI. The MMI can not use these data but it must be able to send it in a transparent way. The CC must check that the number sent by the MMI correspondes to the coded number. This must be done before every safety critical command. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-HW |  |  | Originator: | safety management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0402 (TRT238) |  | Same Dongle-identifier |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If two reactions with the same Dongle-identifier will be sent as a reaction of a request of the CC, the CC must ignore all commands from MMIs with this Dongle-identifier. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | safety management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0403 (TRT241) |  | CHILL-Signals |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The chill signals from one channel must be inverted. Afterwards the signals (the inverted and the not inverted) must be protected with a CRC-Code on application level. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°2501 (TRT659) SAR:°2502 (TRT660) |
| Allocation: | ELEKTRA application |  |  | Originator: | safety management |
| Checking: | system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0405 (TRT269) |  | Clock-Test |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A Clock-Test must be implemented which proofs that the system clock (and the oscillator) runs correctly (in a specificied variation interval). The error detection time must not exceed 24 hours.. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | safety management |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Fur further details see TAS platform on-line tests. |  |  |  |  |

| SAR:0406 (TRT311) |  | Safe communication over Ethernet |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The internal ELEKTRA IXL Ethernet communication shall be a safe communication according to CENELEC [EN50159] category 1 (closed network between safe components). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | analysis |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.5 |
| Comments: | For the possibility to use an open network according to CENELEC [EN50159] category 2 for the communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN, see country specific requirements SAR:4908 (TRT797) in [SRS SBB SA] and SAR:4908 (TRT797) in [SRS INT SA]. |  |  |  |  |

| SAR:0407 (TRT312) |  | Safe communication over Fieldbus |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The communication over the fieldbus must be a safe communication according to [EN50159] category 1 (between safe components). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA |  |  | Originator: | safety management |
| Checking: | analysis |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0408 (TRT323) |  | Reduction of communication between CC and nodes with wrong SW-ID |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CC compares the SW-ID during the startup. CCA has to send a shutdown message to nodes with wrong IDs. it also has to reduce the communication to this nodes onto a safety level. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | integration test module test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Example for solution: EC: mark logical node as defect; don't send commands. MMI: don't accept commands. RCU (incl. el. block): don't send commands; don't accept commands. TCD: is transparent. |  |  |  |  |

| SAR:0603 (TRT180) |  | Control the setting of two outputs in A- and B-channel, which belong together |  |  |  |
| --- | --- | --- | --- | --- | --- |
| prevent consequences in case of unintended setting by special design of HW-Interfaces |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | customer (SBB) |
| Checking: | FE integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | shifting of microcontroller outputs and inputs, usage of more than 2 relays for each output, no influence on microcontroller SW |  |  |  |  |

| SAR:0604 (TRT181) |  | short-circuit between two cable cores outside the signal box room |  |  |  |
| --- | --- | --- | --- | --- | --- |
| detection of this error, maintain the safety in case of this error |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | customer (SBB, MAV) |
| Checking: | HW integration test |  |  | System Baseline: |  |
| Comments: | new form of lamp-current supervision, realization through microcontroller SW |  |  |  |  |

| SAR:0605 (TRT182) |  | supervision of outdoor circuits |  |  |  |
| --- | --- | --- | --- | --- | --- |
| supervise circuits which leave the signal box room, where this is postulated |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | customer (SBB, MAV) |
| Checking: | HW integration test |  |  | System Baseline: |  |
| Comments: | master-slave principle necessary, similar to M-relay in N-Signal- and L-Signal-SW |  |  |  |  |

| SAR:0606 (TRT183) |  | Software life-cycle mechanism |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Shutoff of the HW-interfaces outputs in case of missing life-cycle signal from EC / EC-MUX |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°1210 (TRT198) |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | FE integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0807 (TRT184) |  | Preservation of ELEKTRA's two diverse channel approach |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The ELEKTRA's two diverse channel approach must be preserved within the FE system architecture. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | CENELEC |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0817 (TRT185) |  | Safe end-to-end fieldbus communication |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The fieldbus protocol has to support the implementation of a safe end-to-end communication. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | CENELEC |
| Checking: | IC SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0820 (TRT186) |  | Support for fail-safe switch |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The FE subsystem has to maintain the functionality to switch the track side elements into a fail-safe state. In the current ELEKTRA the 60V power supply (at the POBE level) is switched off. (Since in an FE subsystem the POBE functionality will be provided by the IO-controllers such a fail-safe switch should be realized at this level.) |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°1206 (TRT194) SAR:°1210 (TRT198) |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | FE integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:0831 (TRT433) |  | Galvanic separation of IMO inputs between channel A and channel B |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A galvanic separation of 2000 V AC between the inputs of channel A and channel B must be realized |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | CENELEC |
| Checking: | qualification test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| SAR:0836 (TRT438) |  | Different assignment of a particular pin on A and B channel of IMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The same external peripheral input is coded different for the two IMO channels A and B. This mechanism voids common mode errors. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | safety management |
| Checking: | review HW |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| SAR:0840 (TRT442) |  | Watchdog on IMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The IMO-board has to provide a HW watchdog to reset the board when it expires for safety reactions. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | IMO |  |  | Originator: | safety management |
| Checking: | HW module test review HW |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| SAR:1101 (TRT505) |  | Delete CMOS data - double redundant |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If CC is configured as redundant, CMOS data have to be deleted after booting. This has to be done to prevent ELEKTRA to boot with too old data. Upon entering warm stand-by mode each component has to copy CMOS data from the active component. A stand-by CCA copies from the active CCA, a stand-by CCB copies from the active CCB. In addition, CMOS are deleted, if they are found to be corrupt. CMOS data of active components are deleted if there is an inconsistency between the active CCA and the active CCB. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1102 (TRT512) |  | Delete CMOS data - single redundant |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If CC is configured as single redundant, CMOS data have to be deleted, if they are found to be corrupt (test by each channel independent - eg checksum error) or if there is an inconsistency between CCA and CCB during booting. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1111 (TRT677) |  | State in CC must not change from active to standby |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If the state of one CC channel is active and it receives state standby from the other channel of the same redundancy, a safety reaction has to be done. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | As CCB needs a certain time to change from standby to active, CCA has to tolerate the inconsistency 2 times. |  |  |  |  |

| SAR:1112 (TRT678) |  | CC must not accept life cycle from other redundancy, if it is not configured |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If a redundant CC is not configured (mode SA-1), CC shall do a safety reaction when receiving a life cycle from the other redundancy. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | system administration SUBIT |  |  | System Baseline: |  |
| Comments: | Configuration currently not used. |  |  |  |  |

| SAR:1201 (TRT189) |  | Safe switch-over |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For a safe switch-over to a stand-by component there have to be measures that information is not lost in both channels. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | CENELEC |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Best way would be that no information is lost. |  |  |  |  |

| SAR:1202 (TRT190) |  | Loss of information |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If information is lost in both channels, a safety-reaction has to be performed. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | CENELEC |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1203 (TRT191) |  | Independent output and input |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Ports for input and output to the hardware-interfaces have to be independent items in the two channels. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | CENELEC |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1204 (TRT192) |  | Two channels on one hardware |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If the two diverse software-channels perform safety-related function on one item, they have to be independent to avoid common-mode faults. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | CENELEC |
| Checking: | safety analysis |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: | It has to be demonstrated, that no single random hardware failure is hazardous. |  |  |  |  |

| SAR:1205 (TRT193) |  | Memory-space |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The memory-space (code, data, stack, ...) for two software-channels on one hardware has to be disjoint except for shared objects (e.g. shared libraries, shared executables). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CAR TAS platform |  |  | Originator: | CENELEC |
| Checking: | CAR test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1206 (TRT194) |  | Safety-reaction of interfaces |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A safety-reaction of the system has to include a switching of all output-ports so that a powerless fail-safe state (Grundzustand) on all interfaces is reached and retended until full service of the system is reached. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | CENELEC |
| Checking: | FE integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1207 (TRT195) |  | Disclosure time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The system has to be designed in a way, that a disclosure time of random faults of 48 hours is sufficient. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | customer |
| Checking: | analysis |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Disclosure time inludes time for detection and repair. Concept of TAS Platform regarding disclosure time for FFR calculation is the basis. |  |  |  |  |

| SAR:1208 (TRT196) |  | Non restrictive actions during safety-reaction |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In case of a safety-reaction all non restrictive actions (ausführungssichere Kommandos) to reach the safe ground-state have to be performed within 1 sec. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1209 (TRT197) |  | Restrictive actions during safety-reaction |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In case of a safety-reaction a restrictive action (hemmungssichere Kommandos) must not be performed and this state has to be retented until full service is reached. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1210 (TRT198) |  | Partial shut down of a multiple component |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If there are many components and one component is shut down, there have to be measures that in the direction to field-elements a powerless fail-safe state (Grundzustand) is reached and in direction to central controller a "safe restrictive state" is reached for all elements concerned with this component. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For example one IC is shut down. Then the output-ports of all IF controlled by this IC have to be switched that a "powerless ground state" is reached. To all element-processes of EC controlling this IF's messages have to be sent so that they reach a "save restrictive state" (in most cases element is undefined). For example one TTP-line is shut down. Then all IC's on this line have to be shut down. To all element-processes of EC getting information from this IC's messages have to be sent so that they reach a "save restrictive state" (in most cases element is undefined). For example one EC is shut down. Then all IC's controlled by this EC have to be shut down. To all element-processes of CC controlling the elements of this EC messages have to be sent so that they reach a "save restrictive state" (in most cases element is KAZ). |  |  |  |  |

| SAR:1211 (TRT276) |  | Identification of links |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Every link in the system must have a static identification, which has to be checked end-to-end before use. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | system |
| Checking: | TAS Platform test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1212 (TRT277) |  | Identification of installation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For a specific installation of the system an unambigous identification has to be provided for control with SW identification in CCA and CCB. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1213 (TRT278) |  | Control of selfchecks |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Every application has to control if the selfchecks are performed in time. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA |  |  | Originator: | system |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | This requirement holds not only for CCA and CCB but also for ECA, ECB, RCA, RCB, SCU and LDA, LDB. |  |  |  |  |

| SAR:1901 (TRT663) |  | Detection of high system load - safety mechanism |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The time drift between time of application and real time must be checked separately in each channel.If the time drift exceeds a defined level a shut down must be performed.This level must be defined for each channel separately. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°1902 (TRT664) |
| Allocation: | ELEKTRA application High-load |  |  | Originator: | safety management system |
| Checking: | safety analysis system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:1902 (TRT664) |  | Detection of high system load has to be done in a safe way |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The calculation and the checking for the detection of high system load must be done in a safe way according to CENELEC SIL 4.The levels for the checking must be protected against change (at storage media and during runtime). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application High-load |  |  | Originator: | safety management |
| Checking: | safety analysis |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2022 (TRT694) |  | Mutual scan cycle supervision BusExe <-> FBI |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Per channel and TTP segment a shared memory field for BusExe <–> FBI scan cycle supervision is necessary. The supervision must be performed in the following manner: (1a) Channel A - BusExe behavior: BusExe checks after every scan cycle if the field is zero. After the check the BusExe must increase the scan cycle flag. If the field isn’t zero and the scan cycle flag value is equal or greater 6 (300ms) a safety reaction must be performed by the BusExe. (1b) Channel A - FBI behavior: FBI checks after every scan cycle if the field is greater than zero. After the check the FBI must set the scan cycle flag to zero. If the field isn’t greater than zero after 6 scan cycles (300ms) a safety reaction must be performed. If the counter value is greater than 6 a safety reaction must be performed (missing safety reaction by BusExe). (2a) Channel B - BusExe behavior: BusExe checks after every scan cycle if the field is greater than zero. After the check the BusExe must set the scan cycle flag to zero. If the field isn’t greater than zero after 6 scan cycles (300ms) a safety reaction must be performed. If the counter value is greater than 6 a safety reation must be performed (missing safety reaction by FBI). (2b) Channel B - FBI behavior: FBI checks after every scan cycle if the field is zero. After the check the FBI must increase the scan cycle flag. If the field isn’t zero and the scan cycle flag value is equal or greater 6 (300ms) a safety reaction must be performed by the FBI. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | safety management technical management |
| Checking: | BusExe test FBI test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2023 (TRT695) |  | Mutual life cycle supervision FBI <-> EC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| There must be a channel independent life cycle (LC) supervision between FBI handler and EC application. FBI and EC must send periodically (every second) a LC signal. After a LC timeout of 3 seconds FBI handler or EC application must perform a safety reaction. The LC signals must have the lowest (default) priority. The lowest priority is necessary to detect delayed signals between FBI and EC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | safety management technical management |
| Checking: | FBI test system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2024 (TRT696) |  | Busexe - IC sequence number check |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BusExe must check the IC sequence number. In one of the following cases BusExe must reset an IC and set the appropriate IC shared memory field to the default state: (1) BusExe is missing more than one sequence number by one IC. (2) BusExe is receiving duplicated sequence number by one IC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | safety management technical management |
| Checking: | BusExe test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2102 (TRT697) |  | Mutual life cycle supervision Busexe <-> EC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| There must be a channel independent life cycle (LC) supervision between Busexe and EC application. After a LC timeout of 3 seconds Busexe or EC application must perform a safety reaction. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | safety management technical management |
| Checking: | BusExe test system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2103 (TRT698) |  | FBI IC sequence number check |  |  |  |
| --- | --- | --- | --- | --- | --- |
| There must be a mechanism in FBI handler to check that: - in real mode, IC sequence numbers are changing. If a sequence number does not change within 3 scan cycles (150ms) the corresponding IC has to be marked as unavailable - in simulation mode, IC sequence numbers are always zero. If a sequence number does not remain zero a safety reaction has to be performed. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | safety management technical management |
| Checking: | FBI test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2311 (TRT673) |  | CC redundancy - failure of one redundancy |  |  |  |
| --- | --- | --- | --- | --- | --- |
| There shall be no inconsistency of CMOS-data between active CC and standby CC, if the failure of the standby CC is only supposed by the active CC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | safety analysis system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2312 (TRT674) |  | CC redundancy - supervision of CMOS-copy-function |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The CMOS-copy function shall be performed in the following way (safety view): - The CMOS-copy function shall be supervised in the standby CC, in order to assure that CMOS-data in the standby CC are always consistent with CMOS-data of the active CC. - The supervision cycle must not be longer than 6s due to quick revealing of inconsistencies. - In case of detection of an inconsistency, if the standby CC becomes later active, a message "DATENVERLUST" has to be indicated on the MMI by the active CC after switch-over. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | safety analysis system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | This function shall use the same mechanism as the copy function (including the put_record hook). |  |  |  |  |

| SAR:2313 (TRT675) |  | CC redundancy - inconsistent CMOS-data indicated to operator |  |  |  |
| --- | --- | --- | --- | --- | --- |
| After a switch-over to the standby CC the new active CC shall indicate a CMOS-data loss to the operator in a safe way in the following cases: - The comparison of CMOS data between CCA and CCB identifies differences. - CMOS data consist of default values. - Copying of CMOS-data after reboot is not completed. - A data loss in the former active CC has not been indicated (i.e. AFG (Schweiz)/WIN (ÖBB) was not performed from any user interface). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | safety management |
| Checking: | safety analysis system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2501 (TRT659) |  | Safety mechanism for sending CHILL-Signals based on ELEKTRA LAN communication between different applications |  |  |  |
| --- | --- | --- | --- | --- | --- |
| All sent signals (the inverted and the not inverted) must be protected with a modified CRC-Code on application level. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | safety management |
| Checking: | system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| SAR:2502 (TRT660) |  | Safety mechanism for sending CHILL-Signals - Station-ID |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The sender of chill signals shall add a Station-ID to RMT data packet. The receiver of chill signals shall check the correctness of the received Station-ID. Exception: Timeout and shutdown signals. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | safety management |
| Checking: | system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 |
| Comments: | Including of Station-ID in messages enables the receivers of the messages to verify that messages are intended for them (if this information is not present in the content of the message data). This fulfills the requirement of the uniqueness of the identifiers for entities in the entire transmission system (according to [EN50159]). Timeout signals may be exempt from this mechanism because they do not leave the application and therefore are not found on the transmission system. Shutdown signals are also exempt in order to not hinder their delivery by a potentially failing check of the Station-ID. Definition: Station-ID: Bitstring provided in file station_check.inc located in the station directory of the file system and is used by end-to-end check. |  |  |  |  |

| SAR:2504 (TRT1071) |  | CHILL – Signals – Station-ID, safety reaction |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CHILL signals received from different "processor" board containing a wrong Station-ID shall result in a safety reaction i.e. shutdown/reboot. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | safety management |
| Checking: | system administration SUBIT |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 |
| Comments: |  |  |  |  |  |

| SAR:2505 (TRT1070) |  | CHILL – Signals – Station-ID unique |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Station-ID(s) used by ELEKTRA LAN(s) shall be unique in the entire network. Adding the Station-ID to the RMT data packet and checking of it shall be implemented by a diverse mechanism. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | safety management |
| Checking: | safety analysis TOOLS test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 |
| Comments: |  |  |  |  |  |

| SAR:2506 (TRT1068) |  | CHILL – Signals – Station-ID activation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Processing of the Station-ID is activated, by providing the dedicated environment variable "STATION_CHECK". |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application |  |  | Originator: | safety management |
| Checking: | review TOOL |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.17.0 |
| Comments: |  |  |  |  |  |

| SAR:2801 (TRT466) |  | Galvanic separation of OMO outputs against all other components |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A galvanic separation of 2000 V AC against all other components must be realized (at least 2,7mm). |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | product strategy |
| Checking: | qualification test |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| SAR:2802 (TRT475) |  | Watchdog on OMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The OMO-board has to provide a HW watchdog to reset the board for safety reactions when it expires. The reaction time is fixed at a value between 1 and 2 seconds. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | safety management |
| Checking: | HW module test review HW |  |  | System Baseline: |  |
| Comments: | Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| SAR:2807 (TRT491) |  | Separation of channel A and channel B in OMO |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The minimum distance between channel A and channel B has to be 0.5mm. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | OMO |  |  | Originator: | safety management |
| Checking: | review HW |  |  | System Baseline: |  |
| Comments: | according to CENELEC 15124.1 - see 3BU_15000 5030 PCAPC; Checking by visual inspection of layout. Note: IMO/OMO is an item for customer ÖBB only (until further notice). |  |  |  |  |

| SAR:3001 (TRT490) |  | Nur der aktiv verwendete MMI-PC (EBO1.5) darf laufen |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Es ist nicht zulässig, dass ein nicht verwendeter MMI-PC in "hot-stand-by" ohne angeschaltete Endgeräte mitläuft, da ein Einzelfehler sonst nicht offenbart wird und ein zweiter Fehler angenommen werden muss. Der redundante PC darf erst vor der notwendigen aktiven Verwendung eingeschaltet werden. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB MAV |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | MI-red |  |  | Originator: | safety management |
| Checking: | system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

## Monitoring of reactivity of ELEKTRA system components (44)

The term "System Component" is used in this chapter for explanation of the functionality "Monitoring of Reactivity". In this context the system component has the meaning of component of the ELEKTRA system (such as CC, EC, RCU and TCD).

| SAR:4411 (TRT963) |  | Monitoring of reactivity of ELEKTRA system components - SIL-4 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The functionality "Monitoring of the reactivity of ELEKTRA system components" has to be implemented as a SIL-4 function (according to CENELEC [EN 50129]). This especially includes a diverse implementation of this functionality. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA |  |  | Originator: | safety management |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.15.4 PV_GEN_2.17.0 PV_GEN_2.17.5 |
| Comments: | The reason for diverse implementation is, that the monitoring functionality is directly safety relevant in case of long execution time within ELEKTRA. |  |  |  |  |

## Enhancement Interface DGP-Webride (45)

For general notes on the functionality refer to chapter 4.1.4.

| SAR:4501 (TRT741) |  | Enhancement Interface DGP-Webride - Network type |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be guaranteed by the site/installation project that the transmission system between DGP and Webride and the external network for the remote access to Webride fulfills the requirements for a category 1 network according to CENELEC [EN50159]. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | SBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | DGP Ext. network Webride |  |  | Originator: | product management safety management technical management |
| Checking: | safety analysis |  |  | System Baseline: | PV_GEN_2.17.8 PV_CH_2.13.0 PV_CH_2.30.0 |
| Comments: | - In the currrent architecture the transmission system between DGP and Webride is only a class 1 network. - The fulfillment can be achieved by additional technical measures (e.g. firewalls). |  |  |  |  |

## Serial Connection ELEKTRA - AzLM (52)

For general notes on the functionality refer to the subsection having the same name in the "Technical Interface" section.

Further requirements regarding OCS-SBJ can be found in "Safety Application Conditions/SAV - TAS Platform OCS - SBJ - 1.x" (3CR 00055 7751 DUAPQ).

| SAR:5208 (TRT835) |  | Serial connection ELEKTRA <-> AzLM - SIL-4 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The serial connection ELEKTRA - AzLM must fulfill the safety integrity level SIL-4 according to the CENELEC [EN 50129] standard. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | safety management |
| Checking: | safety analysis |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: |  |  |  |  |  |

| SAR:5209 (TRT862) |  | Serial connection ELEKTRA <-> AzLM - network |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Communication between ELEKTRA and AzLM must take place in a category 1 or 2 network according to the CENELEC [EN 50159]. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | safety management |
| Checking: | safety analysis |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The IP addresses of AzLMs are configurable. |  |  |  |  |

| SAR:5210 (TRT863) |  | Serial connection ELEKTRA <-> AzLM - fault detection time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Fault detection time (until the relevant elements are set to "disturbed" or "undefined" by the EC-process) in the subsystem comprising the serial connection ELEKTRA <-> AzLM and AzLM must not exceed 1s. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | safety management |
| Checking: | integration test safety analysis |  |  | System Baseline: | PV_GEN_2.16.0 PV_GEN_2.17.5 |
| Comments: |  |  |  |  |  |

| SAR:5211 (TRT864) |  | Serial connection ELEKTRA <-> AzLM - SAC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Developing the serial connection ELEKTRA <-> AzLM the safety application conditions (SAC) of the following products must be observed: - TAS platform core - AzLM - OCS-SBJ |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | safety management |
| Checking: | safety analysis |  |  | System Baseline: | PV_GEN_2.16.0 PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

| SAR:5212 (TRT946) |  | Serial connection ELEKTRA <-> AzLM - Detection: Loss of Incoming Telegram |  |  |  |
| --- | --- | --- | --- | --- | --- |
| While receiving data from the OCS-System the ELEKTRA has to detect a loss of telegrams by checking the received OCS sequence number. In case of a wrong sequence number the ELEKTRA has to perform a SitAb for the application on the affected EC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°5214 (TRT948) |
| Allocation: | ECX-SW |  |  | Originator: | safety management |
| Checking: | module test safety analysis |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The provided sequence number increases strictly monotonic for successive messages over the local interface (not synchronous between software channel A and B). See also FUR:°5214 (TRT948)). Info: This requirement is needed for OCS-SB/OCS-SBJ. |  |  |  |  |

| SAR:5213 (TRT947) |  | Serial connection ELEKTRA <-> AzLM - Detection: Wrong Incoming Message Routing |  |  |  |
| --- | --- | --- | --- | --- | --- |
| While receiving data from the OCS-System the ELEKTRA has to detect wrong message routing by checking the expected destination instance provided by OCS. The expected instance has to be equal to the own instance. In case of deviation the ELEKTRA has to perform a SitAb for the application on the affected EC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°5215 (TRT949) |
| Allocation: | ECX-SW |  |  | Originator: | safety management |
| Checking: | module test safety analysis |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: This requirement is needed for OCS-SB/OCS-SBJ. |  |  |  |  |

| SAR:5217 (TRT951) |  | Serial connection ELEKTRA <-> AzLM - Limitation of AzAs |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The total number of AzAs must be limited to 40 within the ELEKTRA system. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | FUR:°5216 (TRT950) |
| Allocation: | Data prep. guideline |  |  | Originator: | product management |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Assumption: The limit must not be higher than 40 to prevent from negative performance influence. Info: This requirement must be added to the data preparation guidelines (dt. Projektierungsrichtlinien). |  |  |  |  |

| SAR:5220 (TRT955) |  | Serial connection ELEKTRA <-> AzLM - Manage OCS-SBJ Master-/Master-Situation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The application must be able to manage master-/master-situations indicated by information received from OCS-SBJ. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | CCX-SW ECX-SW |  |  | Originator: | product management |
| Checking: | integration test module test safety analysis |  |  | System Baseline: | PV_GEN_2.16.0 PV_GEN_2.20.10 |
| Comments: | Info: It is not allowed that two masters are active at the same time within the system. |  |  |  |  |

## CHILL Instruction Test & Supervision of CHILL Instruction Test (68)

The following requirements are derived from the [TAS Plf Safety Handbook].

| DAT:6853 (TRT1631) |  | CHILL Instruction Test & Supervision - Failure Disclosure Time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The failure disclosure time for the CHILL Instruction Test and the Supervision of the CHILL Instruction Test shall be configurable. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Generic System |  |  | Originator: | technical management |
| Checking: | system test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

### CHILL Instruction Test

The CHILL Instruction Test is an application specific CPU test. It is based on the test suite from the validation of the CHILL compiler and serves as substitute for the missing CPU test of the TAS Platform.

| SAR:6802 (TRT1489) |  | CHILL Instruction Test - Execution on each CPU Core |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The execution of the CHILL Instruction Test shall be realized according to the rules of a SIL-4 function according to CENELEC [EN 50716]. The execution of the CHILL Instruction Test shall be performed on each CPU core of the applied HW platform (MEN computer board). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Generic System |  |  | Originator: | safety management |
| Checking: | safety analysis |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: | The CHILL Instruction Test has no allocated SIL by itself. Analogous to the former OLT a single implementation is sufficient. See SRAC [Core-2.x:SAC:0228] of the [TAS Plf Safety Handbook]. |  |  |  |  |

| SAR:6800 (TRT1488) |  | CHILL Instruction Test - Periodical Execution |  |  |  |
| --- | --- | --- | --- | --- | --- |
| After booting, the system shall perform the CHILL Instruction Test periodically. The time between the test runs shall be configurable. The default value shall be 12 hours. The range shall be between 0 hour and 24 hours. The time value shall be specified as integral number in hours. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Generic System |  |  | Originator: | technical management |
| Checking: | code reading integration test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: | 0 hours means continuous test runs. Information for "Checking": The request of configurable time values will be proven by means of "code reading". |  |  |  |  |

| SAR:6849 (TRT1618) |  | CHILL Instruction Test - Delay of Application Start |  |  |  |
| --- | --- | --- | --- | --- | --- |
| After system reboot the system shall perform a CHILL Instruction Test. The system shall delay the application start until the test is completed in the following cases: - if the previous test was not successful, or - if no previous positive test result is available or the previous test was not performed within the configurable failure disclosure time (see DAT:6853). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Generic System |  |  | Originator: | technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: | See SRAC [Core-2.x:SAC:0233] of the [TAS Plf Safety Handbook]. |  |  |  |  |

| SAR:6851 (TRT1621) |  | CHILL Instruction Test - System Reboot |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The system shall reboot if the CHILL Instruction Test fails. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Generic System |  |  | Originator: | technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

| SAR:6852 (TRT1622) |  | CHILL Instruction Test - Blocking of Application Start |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If the CHILL Instruction Test fails 2 times in a row on any CPU core the system shall block the application start permanently (exception see SAR:6801). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°6801 (TRT1490) |
| Allocation: | Generic System |  |  | Originator: | technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: | See SRAC [Core-2.x:SAC:0233] of the [TAS Plf Safety Handbook]. |  |  |  |  |

| SAR:6801 (TRT1490) |  | CHILL Instruction Test - System Reactions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The system shall allow the removal of the blocking of the application (see SAR:6852) only by manual interaction. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°6852 (TRT1622) |
| Allocation: | Generic System |  |  | Originator: | technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

| SAR:0404 (TRT242) |  | Inversion-function of CPU |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The inversion function(s) of the CPU must be checked by a hardware test program periodically. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Generic System |  |  | Originator: | safety management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: | Either the inversion function of the CPU must be checked, or A and B-channel have to be implemented diverse. Note: With PV_GEN_2.20.10 the check of the inversion function(s) must be performed via the "CHILL Instruction Test". |  |  |  |  |

### Supervision of CHILL Instruction Test

| SAR:6805 (TRT1491) |  | Supervision CHILL Instruction Test - Safety Level |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The supervision of the execution of the CHILL Instruction Test shall be realized as a SIL-4 function according to CENELEC [EN 50716]. The supervision shall be realized by two implementations in a diverse way. Both implementations shall be started on the same applied HW platform (MEN computer board). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°6800 (TRT1488) |
| Allocation: | Generic System |  |  | Originator: | safety management technical management |
| Checking: | integration test safety analysis |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: | As parts of the supervision of the former OLT CPU test were realized in a diverse way, also now a diverse implementation is desired to ensure that a possible non-execution of the CHILL Instruction Test is safely revealed. |  |  |  |  |

| SAR:6803 (TRT1493) |  | Supervision CHILL Instruction Test - Successful Execution at Boot Time |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The safety related application shall supervise that the CHILL Instruction Test has reported a positive test result at boot time. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Generic System |  |  | Originator: | safety management technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: | See SRAC [Core-2.x:SAC:0227] of the [TAS Plf Safety Handbook]. |  |  |  |  |

| SAR:6817 (TRT1524) |  | Supervision CHILL Instruction Test - Timeliness |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The safety related application shall supervise the complete and periodical execution of all necessary checks of the CHILL Instruction Test within the configurable failure disclosure time (see DAT:6853). The default value shall be 24 hours. The maximum value shall be 24 hours. The time value shall be specified as integral number in hours. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Generic System |  |  | Originator: | safety management technical management |
| Checking: | code reading integration test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: | See SRAC [Core-2.x:SAC:0227] of the [TAS Plf Safety Handbook]. Information for "Checking": The request of configurable time values will be proven by means of "code reading". |  |  |  |  |

| SAR:6804 (TRT1630) |  | Supervision CHILL Instruction Test - Supervision Reaction in Case of Failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The safety related application shall reboot the system in case one of the checks according to SAR:6803 or SAR:6817 fails. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | SAR:°6803 (TRT1493) SAR:°6817 (TRT1524) |
| Allocation: | Generic System |  |  | Originator: | safety management technical management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: |  |  |  |  |  |

# Security

For ELEKTRA security requirements see [SRS Security].

# Reliability

| DPR:0201 (TRT285) |  | System reliability (System down time < 3 min) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System failures leading to a system down time < 3 minutes: MTBF of the entire system must exceed 10 years |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | analysis |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0202 (TRT286) |  | System reliability (System down time < 8 h) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System failures leading to a system down time > 3 min < 8 hours: MTBF of the entire system must exceed 2000 years |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | analysis |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0203 (TRT287) |  | System availability |  |  |  |
| --- | --- | --- | --- | --- | --- |
| System down time due to a failure of an active central controller must not exceed 3 minutes. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0204 (TRT288) |  | LAN jabber protection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The switch must incorporate jabber protection disabling a port that receives a frame that is longer than a maximum allowed length and re-enabling the port when the condition clears (IEEE 802.3). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | HUB/Switch test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0205 (TRT289) |  | LAN excessive collision protection |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The switch must incorporate excessive collision protection including auto-partitoning and reconnecting of ports whose attached devices create excessive collisions (IEEE 802.3). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | HUB/Switch test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0206 (TRT290) |  | LAN environment |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For reliability reasons the switch shall be a pure 100-Mbps switch (IEEE 802.3). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0207 (TRT291) |  | LAN replacing of devices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The switch must be hot-pluggable, so that nodes can be replaced individually without disrupting the other ones. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0209 (TRT293) |  | LAN default mode of devices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For reliability reasons the attached devices within a network domain must run in case of: - D6 HW (network components: only Switch) with auto-negotiation mode (limited to 100 Mbps) - D10 HW (network components: only Switch) with auto-negotiation mode (limited to 100 Mbps) |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x PV_GEN_2.20.10 |
| Comments: | D2 computer board is no longer supported starting with PV_GEN_2.20.10. |  |  |  |  |

| DPR:0212 (TRT296) |  | CC-Ethernet link failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The failure of a switch and any fault of an active CC which only causes one Ethernet link to fail, must not lead to an automatic switch-over to the standby CC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0215 (TRT301) |  | Error correction code |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The processor boards must incorporate at least SECDED (single error correction, double error detection) ECC memories. The corrected errors must be reported. |  |  |  |  |  |
| Status: | descoped |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: |  |
| Comments: |  |  |  |  |  |

| DPR:0216 (TRT304) |  | Redundant fan-wheels |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The failure of a single fan-wheel must not overstress any single component and must not influence operation. The fans must be monitored and the failure must be indicated to the operator. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EQP SA |  |  | Originator: | product strategy |
| Checking: | qualification test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0217 (TRT305) |  | Failure of a single IC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Any failure (permanent, transient and non fail silent) of a single IC must not result in an interruption of service via the remaining fault-free ICs. Additionally, switching over activities must not take place. The faulty IC must be indicated to the operator. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA |  |  | Originator: | product strategy |
| Checking: | FE integration test integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0218 (TRT306) |  | Failure of a single TTP-module on hot standby - EC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In case of loss of the connection of a physical EC to an entire segment, due to failure of the corresponding TTP-module, switching over to the redundant software channel must take place. If both of the physical redundancies do not see a specific segment, switching over activities must not take place. The faulty unit must be indicated to the operator. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0219 (TRT307) |  | Uninterruptible switch over of hot standby - EC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Switching over to any redundant software channel must not have any impact on operation. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0220 (TRT308) |  | Failure of a single TTP-module on non redundant - EC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The failure of a single segment caused by a fault on a single TTP-module must not affect operation via the remaining segments. The faulty unit must be indicated to the operator. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0222 (TRT313) |  | Only one CC-processing board per redundancy |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For reliability reasons only one processing board per redundancy in the CC-subsystem is allowed. This holds for all sizes of railway installations. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0223 (TRT315) |  | No automatic switch-over of EC / EC-MUX |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For reliability reasons no automatic switch-over at stated intervals to the redundant EC/EC-MUX (SW-channel) must take place. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | code reading |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For safety reasons on an Elektra 1 PC automatic switch over (every 48 hours) was initiated. Elektra 2 doesn't need such a mechanism. |  |  |  |  |

| DPR:0224 (TRT316) |  | Compact flash card failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For reliability reasons compact flash card failures must not lead to a shut down of a processor board. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | An error of the flashcard is detected during the periodic check of the results of the TAS-PLF online tests. One of the supervisions runs every 3 hours. This supervision sends the actual status of all online tests to DGP. If the result of the flashcard tests shows an invalid state (e.g. error), a warning message is sent to DGP. All the information sent to DGP will become part of the datalogger file. The datalogger file allows the company to see the flashcard error. |  |  |  |  |

| DPR:0225 (TRT318) |  | No wearing parts on PBAs and devices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For reliability reasons all components used on PBAs and devices must show a constant failure behaviour (i.e. constant failure rate) over a time period of 25 years. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EQP SA |  |  | Originator: | product strategy |
| Checking: | analysis |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | This is to ensure that no preventive maintenance is necessary over the planned operation time. |  |  |  |  |

| DPR:0228 (TRT321) |  | Independence of external systems in case of faults |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Invalid messages coming from external sources must not result in an interruption of service provided by ELEKTRA-units, provided that no safety reaction is required. The same holds for hardware failures of external systems. The fault must be reported and indicated to the operator. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | - Safety reaction should only concern the error source. - The concrete handling of faults is described in the country specific documentation. |  |  |  |  |

| DPR:0229 (TRT322) |  | Independence of ELEKTRA-units in case of faults |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Invalid messages coming from one specific unit (e.g. first work place, second EC) must not affect operation via the remaining system units.The same holds for hardware failures of all categories of a specific unit.The fault must be reported and indicated to the operator. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | - Safety reaction should only concern the error source. - The concrete handling of faults is described in the country specific documentation. |  |  |  |  |

| DPR:0314 (TRT5) |  | Standby concept for CC applications |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A standby concept for the CC applications has to be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EP |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0316 (TRT6) |  | Redundancy concept for FE subsystem |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The redundancy concept for FE subsystem has to be supported. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Due to availability reasons it is recommended that ECA and ECB are located on two different computers. |  |  |  |  |

| DPR:0401 (TRT218) |  | Redundant LAN |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A fault or error in a single part of subsystem must not interrupt both LANs (permanent). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | PRR:°0323 (TRT166) |
| Allocation: | TAS platform |  |  | Originator: | safety management |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0810 (TRT8) |  | Two-fold redundant EC / EC-MUX |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For availability reason the EC / EC-MUX must be realized as a two-fold redundant computer. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | DPR:°0201 (TRT285) |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:0811 (TRT9) |  | Fieldbus system between EC and IC must provide communication channel redundancy. |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For availability reasons the fieldbus system between the EC and the IC must provide replicated communication channels in order to tolerate the failure of a single channel (bus cable). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: | DPR:°0201 (TRT285) |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:1001 (TRT229) |  | Failure of a component connected to the bus |  |  |  |
| --- | --- | --- | --- | --- | --- |
| In case of a failure of a component connected to the bus, the bus itself must not turned off, but only the relevant component itself. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | FE integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| DPR:1020 (TRT1671) |  | Robustness to year 2038 problem |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The functions of the ELEKTRA system shall work correctly for any epoch up to 7 February 2106, 06:28:15 UTC (one second before overflow of UTC seconds, when represented by an unsigned 32-bit integer value). They shall not be affected by the so-called "Year 2038 Problem". |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA TAS platform |  |  | Originator: | system |
| Checking: | integration test review |  |  | System Baseline: | PV_GEN_2.20.10 |
| Comments: | At "7 February 2106, 06:28:16 UTC" the UNIX time counter (seconds) will be reset to value 0 ("Year 2106 Problem"). Note - for customers using the X25-interface: The specification of the X25 interface [IF-Spec X25] defines its own range for the "year" value (2 digits only) and can therefore further restrict the supported range of the epoch. |  |  |  |  |

## Serial Connection ELEKTRA - AzLM (52)

For general notes on the functionality refer to the subsection having the same name in the "Technical Interface" section.

| DPR:5207 (TRT833) |  | Serial connection ELEKTRA <-> AzLM - No Availability Restrictions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The serial connection ELEKTRA <-> AzLM must keep the availability of the eStW within the RAM requirements. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV GySEV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ECX-SW |  |  | Originator: | product management |
| Checking: | RAM analysis |  |  | System Baseline: | PV_GEN_2.16.0 |
| Comments: | Info: The usage of OCS-SB/OCS-SBJ has no impact on the availability of the ELEKTRA ECs. |  |  |  |  |

# System Integration and Acceptance

| TVR:0815 (TRT217) |  | Module test concept for the FE subsystem |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The FE subsystem architecture has to support a module test concept. The FE subsystem must be testable without field elements and without an interlocking control layer (i.e. the ELEKTRA's CC). Additionally the EC-Layer (Element Controller) and the IC-Layer must be testable without each other. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | system |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| TVR:1102 (TRT234) |  | ELEKTRA Laboratory PC |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A Linux PC is needed for laboratory tests to be able to test all applications except X25. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | system |
| Checking: | system test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Virtual realization recommended (VE2). |  |  |  |  |

# Putting into Operation

| OAR:1004 (TRT232) |  | Putting into operation with CC only |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Booting of a system must also be possible without MMI, EC and IC in order to achieve error alarm of this component. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| OAR:1005 (TRT243) |  | Putting into operation without redundancy |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Booting of a system must also be possible without a redundancy including the later integration of missing components. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| OAR:1008 (TRT246) |  | Use of computers with wrong SW |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The use of computers with wrong SW must not lead to a break-down of the whole system. Additionally, this must be recognized within the diagnosis. Such computers must not be integrated, but have to lead to a reboot. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| OAR:1014 (TRT252) |  | Missing HW |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Missing IF printed boards must not have an influence on the availability of the remaining system. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| OAR:1016 (TRT280) |  | Putting into operation without LAN-redundancy |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Booting of the system must be possible without LAN-redundancy including the later integration of missing components. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| OAR:1017 (TRT281) |  | Putting into operation without fieldbus-redundancy |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Booting of the system must be possible without fieldbus-redundancy including the later integration of missing components. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| OAR:1018 (TRT282) |  | Putting into operation without LDC-redundancy |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Booting of the system must be possible without LDC-redundancy including the later integration of missing components. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

# Diagnosis and Maintenance

| MDR:0213 (TRT297) |  | Ethernet link failure |  |  |  |
| --- | --- | --- | --- | --- | --- |
| When having an Ethernet link failure, disclosure of the link failure and localization of the faulty component must be guaranteed. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For example, any CC fault which only causes one Ethernet link to fail must not lead to an automatic switch-over to the standby CC. Anyway, the link failure must be detected and the maintenance staff must be supported to find out that the CC is the faulty component (not the hub). Then one can manually initiate a switching to the standby CC. |  |  |  |  |

| MDR:0311 (TRT128) |  | Level of diagnosis data |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Following items on system level have to be diagnosed: computers, ELEKTRA applications, communication links, components of TAS Platform |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | EP |  |  | Originator: | system |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | The level of diagnosis data has to be the same as for ELEKTRA Classic. |  |  |  |  |

| MDR:1002 (TRT230) |  | Detection of defective components |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A defective component connected to the bus has to be easily detected. (To be indicated on the component itself or DGP-message). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MDR:1003 (TRT231) |  | Tracking of I/O commands ("Signalwegverfolgung") |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tracking the I/O commands within the system must be possible. (In today`s PC POP and SCE I/Os can be checked as well as within IFs the voting relais by LED). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Bei alten Interfaces erfüllt, bei neuen Interfaces offen. |  |  |  |  |

| MDR:1006 (TRT244) |  | Failure manifestation on printed boards (LRU) layer |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A failure manifestation within the LRU layer is required. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MDR:1007 (TRT245) |  | Easy failure recognition |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Failure recognition within the components layer has to be possible without specific test tools. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | DGP is not a special test tool. |  |  |  |  |

| MDR:1009 (TRT247) |  | Data transmission on the lines |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Data transmission on several bus-systems and X - 25 connections has to be read-out clearly with the aid of specific test tools. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE SA TAS platform |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MDR:1010 (TRT248) |  | Manual switch between redundancies |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A possibility has to be provided to put every HW and SW channel in primary mode. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MDR:1011 (TRT249) |  | Change (replacement) of printed boards |  |  |  |
| --- | --- | --- | --- | --- | --- |
| An "open bus" must not have any effect on the overall system. (Change of printed boards). |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | SA |  |  | Originator: | product strategy |
| Checking: | FE integration test integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MDR:1012 (TRT250) |  | Read-out of statuses of the outdoor equipment |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The statuses of the outdoor equipment have to be read out within EC. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application SA |  |  | Originator: | product strategy |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MDR:1015 (TRT253) |  | Change (replacement) of IF printed boards |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The replacement of IF printed boards must be possible without disconnecting the distribution voltages. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | FE |  |  | Originator: | product strategy |
| Checking: | FE integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| MDR:1019 (TRT283) |  | Read-out of statuses of elements within the station topology |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The statuses of the elements within the station topology have to be read out within the CC layer. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA application SA |  |  | Originator: | product strategy |
| Checking: | integration test |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

# Documentation

| DOR:0924 (TRT3) |  | Documentation for the customer (Power Supply) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Documentation like circuit diagrams, functional description, maintenance handbook is required. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | ELEKTRA PS |  |  | Originator: | product strategy |
| Checking: | review |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

# Tool Chain

| TCR:0141 (TRT204) |  | PUP extension (= PUPnew) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PUP must be extended so that it's possible to handle all new screen data like bitmaps (fonts) and new structures of 'X-like' color tables. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Tooling |  |  | Originator: | system |
| Checking: | review TOOL |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | For details about PUP refer to SGIOS documentation. |  |  |  |  |

| TCR:0161 (TRT205) |  | VC database differences |  |  |  |
| --- | --- | --- | --- | --- | --- |
| It must be possible to show the differences between two VC database releases. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Tooling |  |  | Originator: | system |
| Checking: | review TOOL |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: | Differences of ELEKTRA 1-VCB database are showable by ELEPRO. To show differences of ELEKTRA 1-VCA database are not showable by ELEPRO because of disadvantageous structures of "menue data". To solve this problem the structure of "menue data" from VCB must be implemented in VCA too. Another possible solution is to implement an offline tool, which shows the differences between two VCx database releases. reported by S. Grasegger |  |  |  |  |

| TCR:0358 (TRT206) |  | Tooling and development environment |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The i486-ELEKTRA tooling and development environment has to be adapted in order to use TAS Platform and all other ELEKTRA Evolution specific system components. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | mandatory |
| Architecture: | EL2 |  |  | See also: |  |
| Allocation: | Tooling |  |  | Originator: | system |
| Checking: | review TOOL |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

| TCR:1618 (TRT275) |  | Database - Difference - Tool (CLASSIC - Evolution) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The Database - Difference - Tool must support the comparison of VCA-, CCA-, PCA-, ECA-, RCU- databasefiles and CCA- with CCB- databasefiles in a UNIX-ClearCase enviroment. |  |  |  |  |  |
| Status: | active |  |  | Stability: | stable |
| Customer: | OBB SBB BLS ATG MAV BKV NRIC |  |  | Priority: | optional |
| Architecture: | EL1 EL2 |  |  | See also: |  |
| Allocation: | Tooling |  |  | Originator: | product strategy |
| Checking: | review TOOL |  |  | System Baseline: | PV_GEN_2.x.x |
| Comments: |  |  |  |  |  |

# OPEN ISSUES

Identification of requirements, applicable for the SCWS

The requirements contained in this SRS have to be examined regarding validity for the SCWS.
Requirements - identified as also applicable for the SCWS - have to be tagged accordingly, e.g. by attribute "Allocation"; see the already present example/requirement "FUR:1928 (TRT1673)".

# HISTORY

## Edition 06

ÖBB und SBB specific requirements are not included in this ELEKTRA 2.x generic SRS.

## Edition 07

IMO requirements and FUR:°0831, FUR:°0843 were added.

## Edition 08

New system requirements are added concerning OMO:

FUR:°2801 (TRT467), FUR:°2802 (TRT468), FUR:°2803 (TRT469), FUR:°2804 (TRT470), FUR:°2806 (TRT472), FUR:°2807 (TRT473), FUR:°2808 (TRT474), FUR:°2809  (TRT476), FUR:°2810 (TRT477), FUR:°2811 (TRT478), FUR:°2812 (TRT479), SAR:°2801 (TRT466), SAR:°2802 (TRT475), SAR:°2807 (TRT491).

and workplace redundancy:

FUR:°3001 (TRT484), FUR:°3004 (TRT487), FUR:°3005 (TRT488), FUR:°3006 (TRT489), SAR:°3001 (TRT490)

## Edition 09

New requirements:

FUR:°1106, FUR:°1107 , SAR:°1101, SAR:°1102, FUR:°0844, FUR:°0845.

## Edition 10

Added requirements:

SAR:°0403, SAR:°2501, SAR:°2502, FUR:°3801 – FUR:°3806 (LAN Diagnose)

Changed requirement: FUR:°0844, SAR:°1101 und SAR:°1102.

## Edition 11

Added requirements:

Detection of high system load/regulation of application load: FUR:°1929, SAR:°1901, SAR:°1902.

XEP: FUR:°0846, FUR:°0847, FUR:°0848, FUR:°0849.

## Edition 12

General rework with respect to ortography.

Changed Requirements:

FUR:°0333	Memory to be supported: inserted “at least”

FUR:°1304	Integration of LDC functions on EC: changed to optional

SAR:°0221	Galvanic separation of Ethernet-ports on HUB or switch: inserted: “switch”

Added Requirements:

Concerning Webride:

FUR:°1108	Webride is used to get the diagnosis data from DGP

FUR:°1109	Transfer the diagnosis data from Webride to Vienna.

Concerning CC redundancy/CMOS Copy

FUR:°1110	CMOS-copy ready for sending/receiving data

SAR:°1111	State in CC must not change from active to standby

SAR:°1112	CC must not accept life cycle from other redundancy, if it is not configured

SAR:°2311	CC redundancy - failure of one redundancy

SAR:°2312	CC redundancy - supervision of CMOS-copy-function

SAR:°2313	CC redundancy - inconsistent CMOS-data indicated to operator

## Edition 13

DPR:°0222	priority set to "recommended" (was: mandatory). 
Deleted : “This holds for all sizes of railway installations.”; 
Checking set to “review”.Edition 09

## Edition 14

FUR:°3804	LAN diagnosis - switched-off redundant MMI-PC: 
-> Constraint: Requirement valid for method safe EBO (V2)
-> adapted synopsis and description

FUR:°4101	LAN diagnosis - switched-off redundant MMI-PC (EBO2):
-> new requirement for EBO2

## Edition 15

Chapter 1.4	Clarification of items “LAN” and “Etnernet”

FUR:°0210	Description:	replace „Hub“ by „Hub/Switch“.

FUR:°0211 	Synopsis:	„Hub distance extender module: add „switch buffering“ and 		„modem buffering“.

Description:	 replace „DEM“ by„DEM/Switch/Modem“.

SAR:°0221 	Checking:	replace by “review data sheet”.

DPR:°0204 	Description:	replace „Hub“ by „Hub/Switch“

Checking:	replace “HUB Test” By “HUB/Switch test”

DPR:°0205 	Description:	replace „Hub“ by „Hub/Switch“
Checking:	replace “HUB Test” By “HUB/Switch test”

DPR:°0206 	Description:	replace „Hub“ by „Hub/Switch“

DPR:°0207	Description:	replace „Hub“ by „Hub/Switch“

DPR:°0209	Description:	„collision“ replaced by „network“

DPR:°0212	Description:	replace „Hub“ by „Hub/Switch“

MDR:°0213	Checking:	replace  „HUB/Switch test“ by „integration test“

## Edition 16

New requirements concerning FBI handler safetyconcept:

SAR:°2022, SAR:°2023 , SAR:°2024, SAR:°2102, SAR:°2103

SAR:°0221: Add value for galvanic separation.

## Edition 17

Miscellaneous:

- Adaptation of document to THALES layout

- Replacement of of chapter 2.4 by Appendix A (Index of Requirements)

- Show also requirements with status “descoped”.

Modified requirements:

- FUR:°0601: Synopsis, Description: added D6

- FUR:°0829: Description: added D6

- MIR:°0317: Comments: added D6

- FUR:°1104: Status: changed to descoped, requirement never realized for D2 nor within scope of D6

- SAR:°1112: Status changed to descoped, as configuration currently not used.

- FUR:°0832: Checking: changed to review.

- MDR:°1012: Checking: changed to review.

## Edition 18

Miscellaneous:

- Show also “instable” and “volatile” requirements (adapted export filter)

- Show attribute “Project” (adapted export filter)

- Show also requirements concerning Journaling (43) (adapted export filter)

New requirements:

- New requirements FUR:°4400, FUR:°4401 and SAR:°4402 concerning monitoring of reactivity of ELEKTRA system components (44).

- New requirements PRR:°4500, SAR:°4501, TIR:°4502, FUR:°4503, TIR:°4504, FUR:°4505, FUR:°4506, FUR:°4507, PRR:°4508, FUR:°4509 and FUR:°4510 concerning ethernet interface between DGP and webride (45)

## Edition 19

New requirements – ELEKTRA Bulgaria:

- New sections “Communication between two ELEKTRA via Ethernet LAN (49)” including new requirements FUR:°4901 - FUR:°4904, PRR:°4905, TIR:°4901, TIR:°4907, SAR:°4908.

New requirements – ELEKTRA Schweiz:

- New section “Extensions of the interface to the external system "Leittechnik" (46)” including new requirements FUR:°4601, FUR:°4602.

- New section “Hot-standby LDI connection (47)” including new requirements FUR:°4701, FUR:°4702, FUR:°4703.

- New section “Suppression of certain disturbance messages (48)” including new requirements FUR:°4801 to FUR:°4805.

Modified requirements – ELEKTRA Bulgaria:

- CSR:°0923 valid also for ELEKTRA Bulgaria.

## Edition 20

Corresponding edition for ELEKTRA 2 BL19.00.

Miscellaneous:

- Cover Sheet: Class 1 entry field: “ELEKTRA 2.x” replaced by “ELEKTRA”, since the document contains also ELEKTRA 1 requirements.

- Chapter 3: comment on validity regarding ELEKTRA architecture.

- For all requirements the attribute “Urgency” was removed (not used any longer).

- Empty headings removed, hence this chapter is chapter 15 now.

New requirements:

- New sections “EC - TTP Bus Interface (51)” including new requirements TIR:°5101, TIR:°5102 (acceleration of data transfer by synchronization).

- New sections “Serial Connection ELEKTRA - AzLM (52)” including new requirements TIR:°5201, TIR:°5202, TIR:°5204 - TIR:°5210, FUR:°5204 - FUR:°5206, DPR:°5207, SAR:°5208 - SAR:°5211.

Modified requirements:

- FUR:°4802: Update ELEKTRA2.

## Edition 21

Edition corresponding to ELEKTRA 2 BL27.10, PV gen 2.15.2.

### Miscellaneous

Chapter 1.3, Information for Document Generation

Chapter 1.4, Referenced Documents

Chapter 2.2, Structure of the Requirements

Updated to new Structure of Requirements.

Attributes: Hierarchy, Urgency and Project are not used anymore

New Attribute Target Release, shows First Target Release and Changed Target Release

Section, Note to Target Relase

Chapter 4.1.7, Changed introduction:
from:
Within the ELEKTRA baseline, upon which the Bulgarian product version is based, RCU/SCU communication between ELEKTRA systems is handled via the low-level X.25 protocol (an ITU-T standard) over serial connections. This communication system will be replaced by TCP/IP over Ethernet communication.
to:
RCU/SCU communication between ELEKTRA systems is originally handled only via the low-level X.25 protocol (an ITU-T standard) over serial connections. This communication system now canbe replaced by TCP/IP over Ethernet communication (individually selectable per line by database entry).

### New Requirements

Integration SEA (55):

- FUR:5500 – Integration SEA – Configuration

- FUR:5501 – Integration SEA – Diagnostics

- FUR:5502 – Integration SEA – Extension of SEA Diagnostic Messages to DGP

- FUR:5503 – Integration SEA – EC-MUX Switch of Redundancy in DGP

- FUR:5504 – Integration SEA – Remote Reset of OEC in DGP

- FUR:5505 – Integration SEA – Query State EC-MUX in DGP

- FUR:5506 – Integration SEA – Query SEA Diagnostic Extensions in DGP

- FUR:5507 – Integration SEA – Query Configuration of SEA Elements in DGP

- FUR:5508 – Integration SEA – Query State of Elements in DGP

- FUR:5509 – Integration SEA – Day/Night-Switching

- FUR:5510 – Integration SEA – Status of Day/Night-Signalvoltage when using an EC-MUX

- SAR:5500 – Integration SEA – Safetylevel

### Modified Requirements

- FUR:1575 – Status: active  descoped

- FUR:1576 – Status: draft  descoped

- SAR:2312 – Status: draft  active
Description:
added: "

- In case of detection of an inconsistency, if the standby CC becomes later active, a message "DATENVERLUST" has to be indicated on the MMI.

- In case of detection of an inconsistency, if the standby CC becomes later active, a message "DATENVERLUST" has to be indicated on the MMI by the active CC after switch-over. "

Project:
added: ELEv-BG

- FUR:2801 – Status: active  descoped

- FUR:2802 – Status: active  descoped

- FUR:2803 – Status: active  descoped

- FUR:2804 – Status: active  descoped

- FUR:2806 – Status: active  descoped

- FUR:2807 – Status: active  descoped

- FUR:2808 – Status: active  descoped

- FUR:2809 – Status: active  descoped

- FUR:2810 – Status: active  descoped

- FUR:2811 – Status: active  descoped

- FUR:2812 – Status: active  descoped

- SAR:2801 – Status: active  descoped

- SAR:2802 – Status: active  descoped

- SAR:2807 – Status: active  descoped

- FUR:4901 – Description:
from:
“It must be possible to exchange data between ELEKTRA eStws via a TCP/IP-based type of interface (RCU/SCU).“
to:
“It must be possible to communicate according to “Spezifikation X25 – Schnittstelle (3BU 15000 4167 DTAPC) via a TCP/IP-based type of interface (RCU/SCU).“
Project:
added: ELEv-ÖBB
Checking:
added: integration test
Comments:
added:
Additionally, this requirement is to ease testing and demonstration.
Information: Checking to be applied
integration test -> ELEv-ÖBB
system test -> ELEv-BG, ELEv-SYSTEM

- FUR:4902 – Description:
from:
“The type of the interface between ELEKTRA and ELEKTRA must be configurable”
to:
“The type of the interface for communication according to “Spezifikation X25 – Schnittstelle (3BU 15000 4167 DTAPC) must be configurable.”
Comments:
from:
“Info: Configurable means that the interface type (X25/ethernet) is a database item which cannot be changed via user interface of the data preparation tool but is derived from a configuration file.
Info Checking: By means of "system test" the impact of different configurations is tested, by means of "code reading" the configurability itself is checked.”
to:
“Info: Configurable means that the interface type (X25/ethernet) is a database item (per line) which can be changed via user interface of the data preparation tool or is derived from a configuration file (depending on the particular customer).
Info Checking: By means of "integration test" the impact of different configurations is tested, by means of "code reading" the configurability itself is checked.
Information: Checking to be applied (integration test/system test)
integration test -> ELEv-ÖBB
system test -> ELEv-BG, ELEv-SYSTEM”
Project:
added: ELEv-ÖBB
Checking:
added: integration test

- FUR:4903 – Description:
from:
“…interface within 5 seconds”
to:
“…interface within 6 seconds”
Comments:
added:
“Info for checking: Loss of connection must lead to a reaction within 6 seconds.
Information: Checking to be applied
integration test -> ELEv-ÖBB, ELEv-BG
system test -> ELEv-SYSTEM"
Project:
added: ELEv-ÖBB
Checking:
added: integration test

- FUR:4904 – Comments:
added:
"Information: Checking to be applied
integration test -> ELEv-ÖBB, ELEv-BG
system test -> ELEv-SYSTEM"
Project:
added: ELEv-ÖBB
Checking:
added: integration test

- PRR:4905 – Comments:
added:
“Info: The performance can be verified by some test cases showing normal operation and one test case for high load.
Info: For ÖBB, the limits (number of stations, points, …) remain unchanged, i.e. the limits for communication via Ehternet LAN are the same as for communication via X.25.
Information: Checking to be applied
integration test -> ELEv-ÖBB, ELEv-BG
system test -> ELEv-SYSTEM”
Project:
added: ELEv-ÖBB
Checking:
added: integration test
	system test
removed: performance test
	system integration

- SAR:4908 – Project:
added: ELEv-ÖBB

- TIR:4901 – Description:
from:
“The SCU board (both D2 and D6) must be equipped with a dedicated ethernet interface including RJ45 connectors for the RCU/SCU communication to another ELEKTRA“
to:
“The SCU board (D2) must be enabled to be equipped with a dedicated ethernet interface including RJ45 connectors for the RCU/SCU communication.“
Project:
added: ELEv-ÖBB

- TIR:4907 – Project:
added: ELEv-ÖBB
Checking:
added: integration test
removed: system test

## Edition 22

Edition corresponding to:

ELEKTRA 2 (ÖBB) BL27.20, PV gen 2.15.3 as well as BL28.00, PV gen 2.15.4
ELEKTRA 2 (SBB) BL19.20, PV gen 2.15.5

### Miscellaneous

Chapter 1.3, Information for Document Generation

Chapter 1.4, Referenced Documents

Chapter 2.2, Structure of the Requirements

Updated to new Structure of Requirements

New Attribute Architecture, shows system architecture (ELELKTRA 1 ELEKTRA 2),	
currently maintained for ELEKTRA SBB only.

Section, Note to Architecture

Chapter 6.5, Changed introduction:
added:
"Further requirements regarding OCS-SB can be found in the OCS-SB Safety Handbook (3BY 02000 0040 DUAPQ)."

### New Requirements

Monitoring reactivity of the ECs by the CC:

- FUR:4403 – Monitoring reactivity of the ECs by the CC - determination of response time

- FUR:4404 – Monitoring reactivity of the ECs by the CC - congestion reaction

- FUR:4405 – Monitoring reactivity of the ECs by the CC - return to normal operation

- FUR:4406 – Monitoring reactivity of the ECs by the CC - threshold values

- FUR:4407 – Monitoring reactivity of the ECs by the CC - configurable data

- FUR:4408 – Monitoring reactivity of the ECs by the CC - congestion reaction for ÖBB

- FUR:4409 – Monitoring reactivity of the ECs by the CC - congestion reaction and diagnosis

- FUR:4410 – Monitoring reactivity of the ECs by the CC - congestion reaction line status

- SAR:4411 – Monitoring reactivity of the ECs by the CC - safety level

Serial Connection ELEKTRA – AzLM (52):

- FUR:5214 – Serial connection ELEKTRA <-> AzLM - Indication: Loss of Outgoing Telegram

- FUR:5215 – Serial connection ELEKTRA <-> AzLM - Indication: Wrong Outgoing Message Routing

- FUR:5218 – Serial connection ELEKTRA <-> AzLM - Treadle

- FUR:5219 – Serial connection ELEKTRA <-> AzLM - Restricted Treadle Support

- FUR:5227 – Serial connection ELEKTRA <-> AzLM - Force Detection Serial

- FUR:5228 – Serial connection ELEKTRA <-> AzLM - AZGR, Force Detection Serial

- FUR:5229 – Serial connection ELEKTRA <-> AzLM - State Mapping, Force Detection Serial

- TIR:5221 – Serial connection ELEKTRA <-> AzLM - AzLM-Support for all Logical ECs

- TIR:5222 – Serial connection ELEKTRA <-> AzLM - Support of MEN-D6 Ethernet Ports

- TIR:5224 – Serial connection ELEKTRA <-> AzLM - Support of MEN-D6 over Switch

- SCR:5225 – Serial connection ELEKTRA <-> AzLM - Data Preparation Instruction

- SCR:5226 – Serial connection ELEKTRA <-> AzLM - Configuration Instruction using OCS-SB

- SAR:5212 – Serial connection ELEKTRA <-> AzLM - Detection: Loss of Incoming Telegram

- SAR:5213 – Serial connection ELEKTRA <-> AzLM - Detection: Wrong Incoming Message Routing

- SAR:5220 – Serial connection ELEKTRA <-> AzLM - Manage OCS-SB Master-/Master-Situation

### Modified Requirements

- FUR:0804 – Synopsis:
from:
"Handling of four Element Controllers"
to:
"Handling of Element Controllers"
Description:
from:
"It must be possible to handle four Element Controllers (ECs)."
to:
"It must be possible to handle up to eight Element Controllers (ECs)."
Comments:
added:
"Information: Checking to be applied
integration test -> ELEv-ÖBB
system test -> ELEv-Phase2"
Checking:
added:
integration test

- FUR:5204 – Synopsis:
from:
"Serial connection ELEKTRA <-> AzLM - Data Preparation of AzLM"
to:
"Serial connection ELEKTRA <-> AzLM - projectable AzLM"
Description:
from:
"The data preparation tool must support an axle counter dedicated as OCS serial connected AzLM."
to:
"The ELEKTRA system must support a projectable axle counter dedicated as OCS serial connected AzLM."
Comments:
added:
"Info: Checking: The request of projectable attributes will be proven by means of "code reading"."
Checking:
added:
code reading
system test
removed: OREST test

- FUR:5206 – Description:
from:
"..three variants of the "reset" (Achszähler-Grundstellung) action per AzLM section:.."
to:
"..three projectable variants of the "reset" action (dt. Achszähler-Grundstellung) per AzLM section:.."
Comments:
from:
"Info: An AzLM provides four types of reset. As mentioned above future  ELEKTRA implementations will support three of them.
Info: In future implementations the ELEKTRA command "AZGR" will be mapped onto one of the reset variants (types) mentioned above. Until BL19.00 AZGR is mapped onto "unconditional reset".
Info: Reference document: "Data Interface Specification Az LM - ESTW" (3CR 01800 AJAA PBAFA)."
to:
"Info: Reference document: "Data Interface Specification Az LM - ESTW" (3CR 01800 ALAA PBAFA).
Info: Checking: The request of projectable attributes will be proven by means of "code reading"."
Checking:
added:
code reading
removed: module test

- TIR:5202 – Comments:
added:
"Info: For BL19.20 only the variant MEN D6 is required."

- TIR:5209 – Comments:
from:
".."Data Interface Az LM - ESTW 3CR_01800_AJAA_PBAFA"."
to:
".."Data Interface Az LM - ESTW 3CR_01800_ALAA_PBAFA"."

- TIR:5211 – Comments:
from:
".."Data Interface Specification Az LM – ESTW" (3CR_01800_AJAA_PBAFA)."
to:
".."Data Interface Specification Az LM – ESTW" (3CR_01800_ALAA_PBAFA)."

- SAR:5209 – Description:
from:
"according to the CENELEC EN 50159-1 standard."
to:
"according to the CENELEC EN 50159 category 1 standard."

### Deleted Requirements

- FUR:5205 – Serial connection ELEKTRA <-> AzLM - Data Preparation of two EC

- TIR:5204 – Serial connection ELEKTRA <-> AzLM - number of AzLM sections

## Edition 23

General modifications:

- Splitting of SRS 0300 and country specific SRS (0309, 4107).

- Generic Product Version added:

  - Chapter “Integration SEA”: requirements tagged with generic product version 2.15.3.

  - Chapter “Serial Connection ELEKTRA – AzLM”: requirements tagged with generic product version 2.16.0.

  - Chapter “Monitoring of reactivity”: requirements tagged with generic product version 2.15.4.

  - Remaining requirements tagged with generic product version 2.x.x.

- Architecture (ELEKTRA 1/2) filled in for all requirements (input from K. Padourek).

- Attribute “Project” replaced by the more relevant attribute “Customer”:	
ELEv-ÖBB -> OBB
ELEv-SBB -> SBB, BLS, ATG
ELEv-HU -> MAV
ELEv-HU-BKV -> BKV
ELEv-BG -> NRIC
ELEv-Phase1, ELEv-Phase2, ELEv-HW, ELEv-SYSTEM -> addresses all customers

- Value “EL-INT” in attribute “Project” replaced by “International” in attribute “Allocation”.

- Checking: German “Anlagenprüfung” replaced by English “on-site test”.

- Priority: Values “desired” and “recommended” replaced by “optional” (further distinction not useful).

Requirements moved to other SRS (0309, 4107, 0315):

- FUR:°0845: Swiss specific therefore in SRS 0309 only.

- FUR:°4407 - 4410: ÖBB specific therefore in SRS 0315 only.

- Chapter 45: Swiss specific therefore in SRS 0309 only.

- Chapter 46: Swiss specific therefore in SRS 0309 only.

- Chapter 47: Swiss specific therefore in SRS 0309 only.

- Chapter 48: Swiss specific therefore in SRS 0309 only.

Requirements added from other SRS (0309, 4107, 0315):

- FUR:°0127, 0128, 0131, 0132, 0344: Generic requirement therefore added in SRS 0300.

- FUR:°5216: Generic requirement therefore added in SRS 0300.

- PRR:°0322 - 0324: Generic requirement therefore added in SRS 0300.

- GCR:°0163: Generic requirement therefore added in SRS 0300, comment added, EBO1.5 (based on an Elektra-VC application) added.

- GCR:°0359: Generic requirement therefore added in SRS 0300, description slightly modified.

- SAR:°0144, 0401, 0408: Generic requirement therefore added in SRS 0300.

- SAR:°5217. Generic requirement therefore added in SRS 0300.

- DPR:°0223: Generic requirement therefore added in SRS 0300.

- DPR:°0228, 0229: Generic requirement therefore added in SRS 0300, comment added that concrete handling of faults is described in the country specific SRS.

Requirements modified:

- TIR:°0308: Checking “none” replaced by “EP test” (EP test performed)

- GCR:°0601: Checking “none” replaced by “design review” (design review performed)

- FUR:°0832 – 0835, 0837 – 0839, 0841 – 0842, 0844, 2801 – 2804, 2806-2812: Comment added (IMO/OMO requirements are identified as generic requirements although it is item for customer ÖBB only -> requirements remain in SRS 0300 exclusively).

- SAR:°0831, 0836, 0840, 2801, 2802, 2807: Comment added (IMO/OMO requirements are identified as generic requirements although it is item for customer ÖBB only -> requirements remain in SRS 0300 exclusively).

- FUR:°0804, FUR:°4901, FUR:°4902, FUR:°4903, FUR:°4904, PRR:°4905: Comment slightly modified: reference to project replaced by customer.

- FUR:°1103: “descoped” as there is no need for this requirement at the moment.

- FUR:°1929: Allocation “High-load” added.

- SAR:°1901, 1902: Allocation “High-load” added.

- FUR:°3801: Comment slightly modified (INT added).

- FUR:°3804: Comment added (not applicable for NRIC and INT).

- FUR:4301, 4302, 4304 – 4306, 4401: Customer “BKV” deleted.

- FUR:°4403, 4406: “Code reading” deleted, “module test” added.

- FUR:°4404, 4405: Comment added (reference to SRS 0315).

- TIR:°4907: Information regarding checking in comment deleted.

- SAR:°0405: Description and comment modified (input from K. Padourek).

- SAR:°3001: Synopsis slightly modified.

- TVR:°1102: Comment added.

- SCR:°5225: Updated comment concerning obligatory power supply element. Substituted checking “on-site test” by “review” + allocation “Data prep. guideline”.

- SAR:°5217: Comment slightly modified. Substituted checking “on-site test” by “review” + allocation “Data prep. guideline”.

Requirements deleted:

- FUR:°0365: requirement not useful apart from the stated safety and security problems (decision during online-meeting on 21.05.2012 and succeeding acknowledgement of W. Fuss).

AzLM (SBB19.20) modifications:

- FUR:°5204: “integration test” instead of “system test”.

- FUR:°5227: “integration test” added, description and synopsis slightly modified.

- FUR:°5228: “module test” instead of “system test”, synopsis slightly modified.

- FUR:°5229: “integration test” instead of “system test”, synopsis slightly modified.

- TIR:°5207: Comment added, “system test” added.

- TIR:°5221: Description modified, “integration test” instead of “system test”.

- TIR:°5222, 5224: “integration test” instead of “system test”.

## Edition 24

General: Deleted Product Version for requirements in state “descoped”.

Chapter “Monitoring of reactivity of ELEKTRA system components (44)”:

- Introduction: Revised. Deleted ÖBB specific parts.

- Changed requirements:

  - FUR:°4400, FUR:°4401, FUR:°4403, FUR:°4404,  FUR:°4405, FUR:°4406, SAR:°4411: Added Swiss, Hungarian and Bulgarian customers . Added allocation “International”.

  - FUR:°4403: Added value for periodical messages: each second.

  - FUR:°4404, FUR:°4405: : Added references to requirements concerning threshold values and/or congestion reaction.

## Edition 25

Chapter “Structure of Requirements”:

- Former “Product Version” renamed to “System Baseline” and for “PV2.16.0” target release CH20.10 added.

Chapter “RCU/SCU Communication between two ELEKTRA interlockings via Ethernet LAN (49)”:

- Customers: MAV, BKV and GySEV added (see CQD ELINT-2133).

Chapter “Serial Connection ELEKTRA - AzLM (52)”:

- DPR:°5207 (TRT833): Comment corrected regarding the availability of the ECs when using OCS-SB.

Chapter “Abbreviation of Customers of Requirements” added.

## Edition 26

Chapter “Serial Connection ELEKTRA - AzLM (52)”:

- Customers: MAV, GySEV, BKV and NRIC added (due to Feature INT:FL:FEA:0002 for INT01.20).

Changed Requirements:

- SAR:°5211 (TRT864): Corrected OCS-ELEKTRA-binding -> OCS-SB-ELEKTRA-binding.

- TIR:°5202 (TRT837): Additional information added in comment regarding SAC concerning MEN D2, priority lowered therefore.

Chapter “Man-Machine Interface”:

New Requirements:

- MIR:°0170 (TRT1063): EBO2 MMI - Monitor Ratios/Resolutions (Verification Step: MMI integration test).

## Edition 27

### Edition 27 Proposal 01

Chapter “Man-Machine Interface”:

Changed Requirements:

- MIR:°0170 (TRT1063): System Baseline corrected to “PV2.16.2” (see CQD ELINT-2272).

Chapter “Monitoring of reactivity of ELEKTRA system components (44)”:

Changed Requirements:

- FUR:°4400, FUR:°4403, FUR:°4404, FUR:°4405, FUR:°4406: Customer “GySEV” added (see CQD ELINT-2273).

- FUR:°4401: Customer “NRIC” added (see CQD ELINT-2273).

- SAR:°4402: Customers “NRIC, GySEV, BKV, BLS, ATG” added (see CQD ELINT-2273).

### Edition 27 Proposal 02

Reference “SRS INT SA” added, abbreviation “INT” added, intended audience updated.

Chapter “Monitoring of reactivity of ELEKTRA system components (44)”:

Introduction sentence added regarding “ELEKTRA system component”.

Changed Requirements:

- FUR:°4404 (TRT971): First comment extended “or total EC breakdown”, further comments added with references to INT requirements.

- FUR:°4405 (TRT970): Further comment added with reference to INT requirement.

### Edition 27 Proposal 03

Chapter “Monitoring of reactivity of ELEKTRA system components (44)”:

Introduction sentence added regarding “ELEKTRA system component” modified.

Changed Requirements:

- FUR:°4404 (TRT971): First comment corrected, “or total EC breakdown” deleted.

Chapter “Serial Connection ELEKTRA – AzLM (52)”:

Changed Requirements:

- SCR:°5225 (TRT973): Description corrected.

### Edition 27 Released

Release version for INT01.20 and INT01.30.

## Edition 28

Edition corresponding to ELEKTRA 2 (ÖBB) BL28.10, PV gen 2.16.2,

ELEKTRA INT02.00, PV gen 2.17.0

### Miscellaneous

Conversion to CHORUS Template

Requirements are splitted into:

3BU_15000_0315_DTAPA [SRS ÖBB SA] and

3BU_15000_0300_DTAPA

Chapter 1.4, Referenced Documents

### New Requirements

CHILL - Signals

- SAR:2504 (TRT1071) - CHILL – Signals – system-ID, wrong system-ID

- SAR:2505 (TRT1070) - CHILL – Signals – system-ID unique

- SAR:2506 (TRT1068) - CHILL – Signals – system-ID activation

New chapter “Remote Control: decentralized architecture (62)”:

New Requirement (due to INT:FL:FEA:0237 - Multiple EBO2-CC connections):

- FUR:6205 (TRT1088) - Remote Control: decentralized architecture - maximal number of subnets

### Modified Requirements

- SAR:2502 – Description:
from:
"The sender of chill signals adds a system-ID in the signal header, and the receiver of chill signals checks the correctness of the received system-ID."
to:
"The sender of chill signals shall add a Station-ID to RMT data packet. The receiver of chill signals shall check the correctness of the received Station-ID."
Comments:
added:
"Including of Station-ID in messages enables the receivers of the messages to verify that messages are intended for them (if this information is not present in the content of the message data). This fulfills the requirement of the uniqueness of the identifiers for entities in the entire transmission system (according to [EN50159]).
Definition:
Station-ID: Bitstring provided in file station_check.inc located in the station directory of the file system and is used by end-to-end check."

- DPR:5207 – Customer:
added: OBB

- FUR:5204 – Customer:
added: OBB

- FUR:5206 – Customer:
added: OBB

- FUR:5211 – Customer:
added: OBB

- FUR:5214 – Customer:
added: OBB

- FUR:5215 – Customer:
added: OBB

- FUR:5216 – Customer:
added: OBB

- FUR:5218 – Customer:
added: OBB

- FUR:5219 – Customer:
added: OBB

- FUR:5227 – Customer:
added: OBB

- FUR:5228 – Customer:
added: OBB

- FUR:5229 – Customer:
added: OBB

- SAR:5208 – Customer:
added: OBB

- SAR:5209 – Customer:
added: OBB

- SAR:5210 – Customer:
added: OBB

- SAR:5211 – Customer:
added: OBB

- SAR:5212 – Customer:
added: OBB

- SAR:5213 – Customer:
added: OBB

- SAR:5217 – Customer:
added: OBB

- SAR:5220 – Customer:
added: OBB

- SCR:5225 – Customer:
added: OBB

- SCR:5226 – Customer:
added: OBB

- TIR:5201 – Customer:
added: OBB

- TIR:5202 – Customer:
added: OBB

- TIR:5205 – Customer:
added: OBB

- TIR:5207 – Customer:
added: OBB

- TIR:5208 – Customer:
added: OBB

- TIR:5209 – Customer:
added: OBB

- TIR:5210 – Customer:
added: OBB

- TIR:5221 – Customer:
added: OBB

- TIR:5222 – Customer:
added: OBB

- TIR:5224 – Customer:
added: OBB

## Edition 29

Edition corresponding to ELEKTRA INT02.00, PV gen 2.17.0

### New Requirements

New Requirement (due to INT:FL:FEA:0234 - MEN D6, TAS PL 2.x):

- GCR:0823 (TRT1090) - TAS Platform 2.x on MEN-D6 and MEN-D2 computers

### Modified Requirements

- FUR:0331 – Description:
from:
"… is TAS Platform including AEOS 4.x (not Linux). …"
to:
"… is TAS Platform (AEOS 4.x, AEOS 5.x) and Linux. …"
Comments:
from: 
" CCA, CCB and a diagnosis process on one computer might be the maximum configuration. "

to:

"RCA, RCB and SCU are a typical example for a configuration. AEOS 4.x: one CAR serves all

applications using separated data structures. AEOS 5.x, Linux: each application has its own CAR."

System Baseline:

added: PV2.17.0

- FUR:0337 – Description:
from:
"TAS Platform is the software platform to be used for the …"
to:
"TAS Platform (AEOS 4.x, AEOS 5.x) is the software platform to be used for the … "
Comments:
from:
"Used for CC, EC, DGP applications. "

to:

"Used for CC, EC, DGP, RCU, SCU and TCD applications."

System Baseline:

added: PV2.17.0

- FUR:0338 – Synopsis:

from:
"CAR on top of TAS Platform (AEOS 4.x)"
to:
"CAR on top of TAS Platform"
Description:
from:
"CAR on top of TAS Platform including AEOS 4.x has to be supported."
to:
"CAR on top of TAS Platform (AEOS 4.x, AEOS 5.x) has to be supported."
Comments:
from: 
"Used for CC, EC, DGP applications."

to:

"Used for CC, EC, DGP, RCU, SCU and TCD applications.

For AEOS 4.x the CAR 4.x version is used. Respectively for AEOS 5.x the CAR 5.x version is used."

System Baseline:

added: PV2.17.0

- FUR:0339 – Synopsis:

from:
"XCEL on top of TAS Platform (AEOS 4.x) "
to:
"XCEL on top of TAS Platform "
Description:
from:
"XCEL on top of TAS Platform including AEOS 4.x has to be supported."
to:
"XCEL on top of TAS Platform (AEOS 4.x, AEOS 5.x) has to be supported."
Comments:
from:
" Used for CC, EC, DGP applications."

to:

“Used for CC, EC, DGP, RCU, SCU and TCD applications.

System Baseline:

added: PV2.17.0

- FUR:0341 – Synopsis:

from:
"Services of TAS Platform (AEOS 4.x)"
to:
"Services of TAS Platform"
Description:
from:
"TAS Platform including AEOS 4.x has to …"
to:
"TAS Platform (AEOS 4.x, AEOS 5.x) has to …"
Comments:
from:
"Used for CC, EC, DGP applications."

to:

"Used for CC, EC, DGP, RCU, SCU and TCD applications."

System Baseline:

added: PV2.17.0

- FUR:0847 – Synopsis:

from:
"XEP (EP running without CAR system) required for AEOS 4.x"
to:
"XEP (EP running without CAR system) required."

Description:
from:
"… providing the EP functionality."
to:
"… providing the EP functionality for TAS Platform (AEOS 4.x, AEOS 5.x) and Linux."
System Baseline:

added: PV2.17.0

Customer:

added: SBB

## Edition 30

Edition corresponding to ELEKTRA INT02.00, PV GEN 2.17.0

The set of general requirements at the beginning of the Chapter “4.1 Functional Requirements”, which were listed without any grouping, are grouped to Subchapter “4.1.1 General Requirements”

### Modified Requirements

Monitoring of Reactivity:

The functionality of the monitoring of reactivity (Chapter 4.1.2) was enhanced. The existing requirements were partly modified and new requirements are added. The requirements to this functionality, which are specific to the ELEKTRA International, are also modified and few are added. Those can be found in [SRS INT SA]. The changes were made according to [MM Reactivity].

Modified explanations and requirements in Chapter 4.1.2:

- Explanations and number of components to which the monitoring of reactivity will aplly, were changed and added to the general Chapter 4.1.2

- FUR:4403 (TRT964): Monitoring of reactivity of the system components - determination of response time.

- FUR:4404 (TRT971): Monitoring of reactivity of the system components - congestion reaction.

- FUR:4405 (TRT970): Monitoring of reactivity of the system components - return to normal operation.

- FUR:4406 (TRT969): Monitoring of reactivity of the system components - threshold values.

Modified requirements in Chapter 7.1:

- SAR:4402 (TRT723) - Monitoring of reactivity of ELEKTRA system components - safety level.

- SAR:4411 (TRT963) - Monitoring of reactivity of ELEKTRA system components - SIL-4.

### New Requirements

Monitoring of Reactivity:

New requirements to the enhancement of the monitoring of reactivity functionality:

- FUR:4420 (TRT1106): Monitoring of reactivity of the system components - congestion reaction and diagnosis.

- FUR:4421 (TRT1107): Monitoring of reactivity of the system components - return to normal operation and diagnosis.

- FUR:4422 (TRT1110): Monitoring of reactivity of the EC by the CC - usage of EC-FlowControl.

- FUR:4423 (TRT1109): Monitoring of reactivity of the CC by the RCU - usage of RCU-FlowControl.

CAR-requirements:

To cover already implemented behavior of CAR following new requirements are added:

- FUR:4103 (TRT1096): CAR - Shutdown.

- FUR:4106 (TRT1099): CAR - Protection from outside applications.

- FUR:4107 (TRT1100): CAR - Callbacks at sending/receiving of message.

- FUR:4108 (TRT1101): CAR - Signal dispatching, Signal Priority 0.

- FUR:4109 (TRT1102): CAR - Stacksize check.

- FUR:4110 (TRT1108). CAR 5 - Functionality provided for ELEKTRA

New Requirement (due to INT:FL:FEA:0234 - MEN D6, TAS PL 2.x):

- GCR:0823 (TRT1090) - TAS Platform 2.x on MEN-D6 and MEN-D2 computers

## Edition 31

### Edition 31 Proposal 01

Edition corresponding to ELEKTRA INT02.00, CH21.20, PV GEN 2.17.0

New Requirements:

New requirements to the enhancement of the monitoring of reactivity functionality:

- FUR:4430 (TRT1118): Monitoring of reactivity of the CC by the RCU - response of CC to RCU includes the longest time of response time of the ECs monitored by CC

- FUR:4431 (TRT1119): Monitoring of reactivity of the CC by the RCU -  summary threshold value of the response time

Deleted Requirements:

- SAR:4908 (TRT797) - The requirement was displaced without any changes from this document to country specific SRS documents [SRS INT SA], [SRS ÖBB SA] and [SRS SBB SA] (for more details see ELSYS00000403).

- SAR:4402 (TRT723): - In the modified description of the requirement SAR:4411 (TRT963), the software diversity and two independent channel implementation are covered due the SIL4 demand. Therefore the requirement SAR:4402 (TRT723) is not necessary anymore.

Modified Requirements:

- SAR:2502:
System Baseline: Label 2.17.0 added (see Chapter History, Edition 28)

- SAR:2504:
System Baseline: Label 2.17.0 added (see Chapter History, Edition 28)

- SAR:2505:
System Baseline: Label 2.17.0 added (see Chapter History, Edition 28)

- SAR:2506:
System Baseline: Label 2.17.0 added (see Chapter History, Edition 28)

### Edition 31 Released

Release edition corresponding to ELEKTRA INT02.00, CH21.20, PV GEN 2.17.0.

Modified Requirements:

- FUR:4420 (TRT1106) and FUR:4421 (TRT1107):

  - Checking modified: Code Reading deleted

## Edition 32

### Edition 32 Proposal 01

Edition corresponding to ELEKTRA INT02.00, CH21.20, OBB28.10, PV GEN 2.17.0

New Requirements:

New requirements to the enhancement of the monitoring of reactivity functionality:

- FUR:4435 (TRT1124): Monitoring of reactivity of the CC by the EC - usage of EC-FlowControl

Modified Requirements:

- FUR:4400, FUR:4401:
Allocation: International, SA -> ELEKTRA

- FUR:4403, FUR:4404, FUR:4405, FUR:4406:
Allocation: ELEKTRA, International -> ELEKTRA

### Edition 32 Released

Release edition corresponding to ELEKTRA INT02.00, CH21.20, PV GEN 2.17.0.

Modified Requirements:

- SAR:4411 (TRT963):

  - Following sentence added to the description of the requirement: "This especially includes a diverse implementation of this functionality".

  - Following sentence added to the comment of the requirement: "The reason for diverse implementation is, that the monitoring functionality is directly safety relevant in case of long execution time within ELEKTRA."

- FUR:4430 (TRT1118):

  - The text of the requirement changed from "the response signal" to "a signal".

- All requirements in the Chapter 4.1.3.2:

  - System Baseline: PV GEN 2.16.7 added.

## Edition 33

Release per signature: edition corresponding to ELEKTRA INT02.00, PV GEN 2.16.7 and PV GEN 2.17.0.

The table of Referenced Documents (Chapter 1.4) was updated. The reference [MM Reactivity] to the Meeting Minutes Report got new document name and edition (EL-SYS-16-042_MM_Reaktivitaetsueberwachung-Elektra-Plf2x-SACs-HAZLG98.doc Ed. 03).

This Report is the successor of the old report (EL-SYS-15-165_MM_Reaktivitaetsueberwachung-Elektra-Plf2x-SACs-ELMAV-1983-ELOBB-5069 Ed. 03) and it contains all relevant information about the reactivity functionality.

Monitoring of Reactivity:

The functionality of the monitoring of reactivity (Chapter 4.1.3) was changed. The changes were made acoording to ELSYS00000497 and ELSYS00000501 and described in [MM Reactivity].

Modified Requirements

- FUR:4430 (TRT1118):

Synopsis:

from:
"…- response of CC to RCU includes the longest time of response time of the ECs monitored by CC"
to:
"… - the CC sends to RCU the longest response time of the monitored ECs"

Description:
from:
"… the CC shall additionally send the information to RCU about the longest value..."
to:
"… a signal, sent by CC to RCU, shall include the information about the longest value..."

- FUR:4431 (TRT1119):

Synopsis:

from:
"…- summary threshold value of the response time"
to:
"… - supervision of 2 time limits"

Description:
from:
"In the calculation of the response time in the monitoring of the CC component, the RCU shall consider the response value, transmitted by CC, about the longest measured value of response time of the monitoring of the of the ECs. The RCu shall add the value transferred by CC to the value of response time of CC which is measured by RCU. If have the calculation of the response time, defined in this way, exceeds the allowed threshold value, the RCU shall initiate congestion reaction."
to:
"In the case of monitoring of reactivity "RCU monitors CC", the RCU shall monitor two separate limits.

- the value of the response time from CC

- the value which is calculated as the value transferred by CC, about the longest measured response time of the monitored ECs, summed to the value of the response time of the CC which is measured by RCU.

These two values for the threshold  limit have to be independent of each other.

As soon as one of the time limits, defined above, exceeds the allowed threshold value, the RCU shall initiate congestion reaction.

The congestion reaction is the same for both cases and can only be canceled if none of the time limits is exceeded any more."

Deleted Requirements

- FUR:4435 (TRT1124): Monitoring of reactivity of the CC by the EC - usage of EC-FlowControl.

New Requirements

- FUR:4436 (TRT1127): Monitoring of reactivity of the RCU by the CC - usage of RCU-FlowControl.

## Edition 34

### Edition 34 Proposal 01

Proposal version corresponding to  ELOBB BL29.01, ELINT RC18.04.00 and ELSBB BL21.30 (PV GEN 2.17.5).

New Requirement:

- FUR:4111 (TRT1288): CAR - Memory initialization

Modified Requirements:

Reason for the modification CQD ELSYS00000515:

- GCR:0823 (TRT1090): Text of the requirement modified according to the findings from the CQD, the baseline PV2.17.5 added to the attribute System Baseline.

- FUR:4420 (TRT1106), FUR:4421 (TRT1107), FUR:4422 (TRT1110), FUR:4423 (TRT1109), FUR:4430 (TRT1118), FUR:4431 (TRT1119) and FUR:4436 (TRT1127): “module test” added to the attribute  Checking , following information added to the comment of the requirement: “Info Checking: "Module test" means module test of generic functions”, the baseline PV2.17.5 added to the attribute System Baseline.

- SAR:4411 (TRT963): attribute Checking modified: “safety analysis” -> “code reading”, the baseline PV2.17.5 added to the attribute System Baseline.

- FUR:0341 (TRT55): following text added to the comment of the requirement: “The CAR/EP uses CS communication. Therefore these functions are tested in every single application tests implicitly as well in CAR”.

- History Chapter, Chapter 15.26.2: The note about modification of the requirement FUR:4417 deleted because this requirement is not part of this requirement specification.

Reason for the modification CQD ELSYS00000516:

- FUR:0346 (TRT59): Text of the requirement modified according to the findings from the CQD, baseline PV2.17.5 added to the attribute System Baseline.

Reason for the modification CQD ELSYS00000531:

- FUR:0339 (TRT54): The attribute Status changed "active" -> "descoped"

Reason for the modification CQD ELSYS00000590:

- SAR:0406 (TRT311): Text of the requirement modified according to the findings from the CQD, new note added to the attribute Comment of the requirement, the baseline PV2.17.5 added to the attribute System Baseline.

- Modification of the formulations in Chapter definition: “RCU/SCU Communication between two ELEKTRA interlockings via Ethernet LAN (49)” as well in the requirements FUR:4901 (TRT791), FUR:4902 (TRT792), FUR:4903 (TRT793), FUR:4904 (TRT794), PRR:4905 (TRT795), TIR:4901 (TRT790) and TIR:4907 (TRT796):

  - “between two ELEKTRA interlockings” -> “between two ELEKTRA IXLs or between ELEKTRA IXL and other external components”

Reason for the modification CQD ELSYS00000609:

- SAR:5210 (TRT863): “integration test” added to the attribute Checking.

- FUR:6205 (TRT1088): “integration test” added to the attribute Checking.

Reason for the modification CQD ELSYS00000610:

- FUR:4405 (TRT970): Text of the requirement modified from “When the response time goes below the threshold value…” to “When the response time is within the threshold value…”.

Other modifications:

- Formulation in the following requirements modified to enhance their validity also to the EC-MUX:

  - SAR:0606 (TRT183)

  - DPR:0223 (TRT315)

  - DPR:0810 (TRT8)

- Formulation in the following requirements modified for the clarity reasons:

  - MDR:1019 (TRT283)

  - SAR:1206 (TRT194)

  - SAR:1210 (TRT198)

### Edition 34 Released

Release version corresponding to  ELOBB BL29.01, ELINT RC18.04.00 and ELSBB BL21.30 (PV GEN 2.17.5).

Modifications according to the review comments:

- Typo and light errors

- Note about initial purpose for the generic ELEKTRA project in the Chapter 1.1 Purpose and the corresponding references [PMPlan ELEv-Ph1] and [PMPlan ELEv-Ph2] in the Chapter 1.4 Referenced Documents were deleted

- Entry 2.17.5 removed from the Baseline attribute of the requirement FUR:0339 (TRT54) which was descoped

- Reference to the requirement FUR:4103(TRT1096) added to the attribute See also of the requirement FUR:0346 (TRT59)

- Baseline PV2.17.5 added to the attribute System Baseline of the requirement FUR:4405 (TRT970)

- Requirements MDR:1019 (TRT283), SAR:1206 (TRT194), SAR:1210 (TRT198) added to the list of “other modifications”.

- Requirement FUR:6205 (TRT1088) added to the list of modifications according to CQD ELSYS00000609

- Requirements PRR:4905 (TRT795) , TIR:4901 (TRT790) and TIR:4907 (TRT796) added to the list of modifications according to CQD ELSYS00000590

- Text removed from the comment of the requirement SAR:0606 (TRT183) as obsolete

## Edition 35

Edition for PV GEN 2.19.0.

Updated chapter “Referenced Documents” and chapter “Abbreviations”.

New Requirements:

- FUR:4200: Board Serial Number D6/D10.

- FUR:4201: EC and EC-MUX on one computer board D10 – Basic Feature.

- FUR:4202: EC and EC-MUX on one computer board D10 – AzLM interface.

- FUR:4203: D2 computer cabinet with D10 communication computer boards.

Modified Requirements:

- FUR:0306: Allocation: Added CAR (CQ defect ELSYS-0633).

- FUR:0307: Allocation: Added EP (CQ defect ELSYS-0633).

- FUR:0333: Description: Increased memory values.

- FUR:0370: Comments: Removed electronic block.

- FUR:0601: Synopsis, Description: Added D10.

- FUR:0829: Description: Added D10.

- FUR:1107: Priority changed to optional.

- FUR:4103: Allocation: Added EP (CQ defect ELSYS-0633).

- FUR:4110: Allocation: Added EP (CQ defect ELSYS-0633).

- FUR:4200: Comments: Added note that data shall be sent at reboot of the board.

- GCR:0347: Allocation: Added EP (CQ defect ELSYS-0633).

- GCR:0823: Synopsis, Description: Added D10.

- TIR:4907: Comments: Updated configuration.

- TIR:5202: Description: Added D10. Comments: Deleted ad corrected notes concerning D2. Deleted notes concerning D6.

- TIR:5224: Synopsis, Description: Added D10.

- MIR:0317: Comments: Added D10. Added note that 2 LEDs are used instead of Hex display.

- SAR:1205: Allocation: Added CAR (CQ defect ELSYS-0633).

- SAR:5209: Corrected: Communication must take place in a category 1 or 2 network (CQ defect ELSYS-0634).

- DPR:0209: Description corrected for D2, D6 and D10 hardware (CQ defect ELSYS-0636).

- SAR:5209: Description corrected for category 1 and 2 networks (CQ defect ELSYS-0634).

## Edition 36

Edition for CH21.40.

Updated chapter “Referenced Documents”.

New Requirements (moved here from [SRS BF 5]):

- FUR:5306: DGP extensions - Time signal via NTP - Time signal (corresponds to former FUR:7602)

New Requirements (moved here from [SRS SBB SA]):

- FUR:4505: Enhancement Interface DGP-Webride - Detection and indication of faulty transmission or loss of data

- FUR:4506: Enhancement Interface DGP-Webride - Re-transmission of faulty or lost data by DGP

- FUR:4509: Enhancement Interface DGP-Webride - Time stamp for outgoing  DGP diagnostic data

- FUR:4507: Enhancement Interface DGP-Webride - Max. detection time of interface failure

- TIR:4502: Enhancement Interface DGP-Webride - Ethernet interface equipment

- TIR:4504: Enhancement Interface DGP-Webride - Transmission bit rate

- SAR:4501: Enhancement Interface DGP-Webride - Network type

Modified Requirements:

- SAR:2312: Description: Removed bullet 4 (is identical to bullet 3).

## Edition 37

Edition for GenSys 2.19.0.

The RELEASE of this edition is performed PER SIGNATURE in accordance with QAM and SEM.

Modified Requirements:

- FUR:4203: Status set to “descoped”.

## Edition 38

Edition for GenSys 2.19.0 for ÖBB 28.30.

Chapter 1.4, Referenced Documents

Chapter 2.2, Structure of the Requirements

General: In the whole document “CARIX” was replaced by “CAR”.

General: In the whole document “PV2.x.x” was replaced by “PV_GEN_2.x.x”.

New Requirements:

- FUR:1578: TAS-PLF OCS SBJ communication framework

- TIR:6700: Hybrid System - ELEKTRA 2 components on ELEKTRA 1.

- PRR:4906: External RCU communication by using TAS-PLF OCS SBJ communication framework

Modified Requirements:

- FUR:0118: Synopsis changed to “CAR on operating system ‘Linux’”

- FUR:0339: Status set to “active”
	Description removed “AEOS 4.x”
	System Baseline changed to “PV2.19.0”

- TIR:0308: Description removed “..during development phase..”
	Comments: “Currently the following realizations are required. Usage in the field: RCU/DGP ELEKTRA 2 must communicate with CC/PC and SCU ELEKTRA 1. Usage for testing: DGP/CC-Simulation ELEKTRA 2 must communicate with PC ELEKTRA 1. Usage for Testing: DGP/ELEKTRA Simulation ELEKTRA 2 must communicate with CC ELEKTRA 1.”

SAR:0404: 
	Comments: “..to be implemented divers“

## Edition 39

### Proposal 01

Edition for GenSys 2.20.0.

Update of document template.

Chapter 4.1.1 (Functional Requirements / General Requirements):

- FUR:0116: Description and Comments updated due to PCR CELSYS-752.

New chapter 4.1.10 (Functional Requirements / Two Diverse SW channels on the same HW Board)

- New requirement FUR:6800 (CHILL Instruction Test - Periodical Performance)

- New requirement FUR:6801 (CHILL Instruction Test - System Reactions)

- New requirement FUR:6806

- New requirement FUR:6807

- New requirement FUR:6808

- New requirement FUR:6810

- New requirement DAT:6811

- New requirement FUR:6813

- New requirement FUR:6814

New chapter 4.1.11 (Functional Requirements / Virtualization Server Operation and Maintenance)

New chapter 5.1.5 (Technical Interface / Virtualization Server Operation and Maintenance)

Chapter 5.2 (Man Machine Interface)

- Renamed to “User Interface / Man Machine Interface”

- MIR:0109: Description and Comments updated due to PCR CELSYS-752.

New chapter 5.2.1 (User Interface / Man Machine Interface / Virtualization Server Operation and Maintenance)

New chapter 6.2 (Requirements to the Customer / Virtualization Server Operation and Maintenance)

Chapter 7 (Safety):

- SAR:2501: Description updated due to PCRs CELSYS-756 and PELOBB-6021.

New chapter 7.4 (Safety / Two Diverse SW channels on the same HW Board):

- New requirement SAR:6802 (CHILL Instruction Test - Safety Level)

- New requirement SAR:6803 (Supervision CHILL Instruction Test - Performance at Boot Time)

- New requirement SAR:6804 (Supervision CHILL Instruction Test - Periodical Performance)

- New requirement SAR:6817 (Supervision CHILL Instruction Test – Timeliness)

- New requirement SAR:6805 (Supervision CHILL Instruction Test - Safety Level)

- New requirement SAR:6809 (Supervision TAS-PF Health Monitor (HMD) - SIL-4)

- New requirement SAR:6812 (Plant ID - SIL-4)

- New requirement SAR:6815 (RCU Output Voter - SIL-4)

- New requirement SAR:6841 (Diversity)

- New requirement SAR:6842 (OS process isolation)

- New requirement SAR:6843 (Safe Outputs)

- New requirement SAR:6844 (TAS Platform)

- New requirement SAR:6845 (Other applications)

- New requirement SAR:6846 (Communication between channels)

- New requirement SAR:6816 (Development standard)

- New requirement SAR:6818 (Hardware checks on application level – completion)

- New requirement SAR:6818 (Hardware checks on application level - development standard)

- New requirement SAR:6819 (Heartbeat messages - fully functional)

- New requirement SAR:6820 (Version check at start-up)

- New requirement SAR:6821 (Disclosure of CPU faults)

- New requirement SAR:6822 (Memory Integrity Checks)

- New requirement SAR:6823 (Diverse data structures)

- New requirement SAR:6824 (Diverse usage of underlying software)

- New requirement SAR:6825 (Communication with subsequent system - CENELEC EN50159)

- New requirement SAR:6827 (Communication with subsequent system - Heartbeat messages)

- New requirement SAR:6826 (Heartbeat messages per channel)

- New requirement SAR:6828 (Several processes in application channel)

- New requirement SAR:6829 (Common configuration data)

- New requirement SAR:6830 (Operative without subsequent system)

- New requirement SAR:6831 (Application specific identification with server setup)

- New requirement SAR:6832 (Network usage with server setup)

- New requirement SAR:6847 (Safe digital I/O)

- New requirement SAR:6833 (Application Runtime - development standard)

- New requirement SAR:6834 (Application Runtime - Shutdown procedure)

- New requirement SAR:6835 (Subsequent system - SIL for comparison)

- New requirement SAR:6835 (Subsequent system - reactivity supervision)

- New requirement SAR:6836 (Subsequent system - functionality for comparison)

- New requirement SAR:6837 (Subsequent system - kind of diversity)

- New requirement SAR:6838 (Subsequent system – independence)

- New requirement SAR:6839 (Retention of safe state)

- New requirement SAR:6840 (TFFR calculation)

New chapter 7.5 (Safety / Virtualization Server Operation and Maintenance)

New chapter 8.1 (Security / Virtualization Server Operation and Maintenance)

New chapter 9.2 (Reliability / Virtualization Server Operation and Maintenance)

New chapter 12.1 (Diagnosis and Maintenance / Virtualization Server Operation and Maintenance)

New chapter 13.1 (Documentation / Virtualization Server Operation and Maintenance)

### Proposal 02

Chapter 1.4 (Referenced Documents): Update.

Chapter 1.6 (Abbreviations): Update

Chapter 1.7 (Definitions): Added definitions for “projectable” and “configurable”.

Added Architecture EL4 to all requirements with System Baseline PV_GEN_2.20.0.

Chapter 3 (General Description): Added note concering ELEKTRA 4 requirements.”

Chapter 4.1.1 (Functional Requirements / General Requirements):

- FUR:0116: Updated description due to PCR CELSYS-752.

- FUR:0337: Type changed to GCR and moved to chapter 4.3 (Constraints).

- FUR:4108: Comments: Reference to presentation removed.

- New requirement FUR:0166 (DBS: CRC handling).

- New requirement FUR:4204 (Support of EL2 and sever hardware).

- FUR:0341: Upated description.

Chapter 4.1.3 (Functional Requirements / Monitoring of Reactivity of ELEKTRA system components):

- Added reference to SRAC [MS-2SW1HW:SAC:0028].

Chapter 4.1.10 (Functional Requirements / Two Diverse SW channels on the same HW Board)

- FUR:6810: Corrected synopsis and description. Checking changed to “system test”

- DAT:6811: Checking changed to “system test”. Removed comment.

- Subchapters “CHILL Instruction Test”, “Supervision of CHILL Instruction Test”, “Supervision of TAS Platform Health Monitor” and “RCU Output Voter” moved to chapter 7.4 (Safety / Two Diverse SW channels on the same HW Board). Reason: Requirements have safety impact. Requirement type changed to SAR.

Chapter 4.1.11 (Functional Requirements / Virtualization Server Operation and Maintenance):

- Refinement concerning migration of applications to server-of platform.

- Added missing figure.

Chapter 4.3 (Constraints):

- New requirement GCR:0850: ELEKTRA IXL using TAS Platform 2.x on Server HW, MEN-D10 and MEN-D6 computers (PV GEN 2.20.x and higher).

Chapter 5.2 (User Interface / Man-Machine Interface):

- MIR:0106: Comments: Note concerning U-Bahn removed.

Chapter 6.1 (Serial Connection ELEKTRA – AzLM):

- Added note that with PC GEN 2.20.x and higher the AzLM interface is no longer supported.

Chapter 7 (Safety):

- SAR:0144: Synopsis, Description, Comments: Consistent use of term “ciritical command”.

- SAR:0221: System Baseline restricted to PV GEN 2.1x.x.

- SAR:1205: Description: Added exception for shared objects.

- SAR:1212: Comments: Added reference to FUR:6810, FUR:6811 and SAR:6812..

- SAR:1213: Comments: RCA and RCB added.

- SAR:2313: Description: Removed Dublette.

- SAR:2502: Description: Added exception.

Chapter 7.4 (Safety / Two Diverse SW channels on the same HW Board):

- Added explanation for CHILL Instruction Test.

- Several requirements: Replaced “performance” by “execution”, “computer” by “application”. Consistent use of “shall” instead of “must.

- New requirement SAR:6849: CHILL Instruction Test - Execution at Boot Time

- SAR:6800: Description: Added configurability of time between test runs. Moved comments to explanation at beginning of chapter. Changed checking to “integration test”.

- New requirement SAR:6851: CHILL Instruction Test - System Reboot

- New requirement SAR:6852: CHILL Instruction Test - Blocking of Application Start

- SAR:6801: Description: Changed to removal of blocking. Changed checking to “integration test”.

- SAR:6802, SAR:6805, SAR:6809: Updated description and comments concerning two-channel realization.

- SAR:6803, SAR:6804: Changed checking to “integration test”.

- SAR:6817: Description: Added reference to [TAS Platform Safety Concept]. Changed checking to “integration test”.

- SAR:6803, SAR:6806, SAR:6819, SAR:6846, SAR:6825, SAR:6829: Corrected description.

- SAR:6806, SAR:6807, SAR:6808: Changed checking to “integration test”.

- SAR:6812: Corrected description. Added comment concerning query and reference to SAR:1212.

- SAR:6813, SAR:6814: Corrected description. Added checking.

- SAR:6818: Added checking “module test”.

- SAR:6818: Identifier changed to SAR:6850 (was used twice).

- SAR:6841, SAR:6842: Corrected description and checking.

- SAR:6819, SAR:6820, SAR:6822, SAR:6843, SAR:6844, SAR:6845, SAR:6846, SAR:6823 to SAR:6834, SAR:6847, SAR:6848, SAR:6836 to SAR:6839: Corrected checking.

- SAR:6843: Comments: Added note and reference concerning “subsequent systems”.

- SAR:6844, SAR:6845: Updated description and comments.

- SAR:6820: Added comment concerning configuration management.

- SAR:6821: Requirement deleted (covered by requirements for CHILL Instruction Test).

- SAR:6822: Corrected description and comments. Changed Priority to optional.

- SAR:6833, SAR:6834: Comments: Added note concerning checking.

- SAR:6835: Identifier changed to SAR:6848 (was used twice).

- SAR:6835: Requirement deleted (exists as detailed chapter “Monitoring of reactivity of ELEKTRA system components”)

- SAR:6837: Corrected description concerning kind of diversity.

- SAR:6838: Corrected description and comments.

- SAR:6839: Comments: Added reference to chapter “CHILL Instruction Test”.

### Proposal 03

Chapter 3 (General Description):

- Corrected sentence concerning ELEKTRA 4.0.

Chapter 4.1.1 (Functional Requirements / General Requirements):

- FUR:0116: Description refined.

- FUR:0118: Description re-formulated. Comments removed.

- FUR:0123: Description refined.

- FUR:0166: Synopsis and description updated. Added missing customers. Updated system baseline and architecture.

- FUR:0210: Description: Re-formulated 2nd sentence as requirement.

- FUR:0331: Description: Updated number of applications.

- FUR:0334: Description refined.

Chapter 4.1.2 (Functional Requirements / Journaling):

- FUR:4304: Description completed.

- FUR:4306: Completed description and comments.

Chapter 4.1.10.1 (Plant ID) moved to separate chapter 4.1.10.

Chapter 4.3 (Constraints):

- GCR:0850: Updated synopsis.

- GCR:0337: Re-formulated description.

Chapter 6.1 (Serial Connection ELEKTRA – AzLM):

- Corrected note that with PC GEN 2.20.0 the AzLM interface is not supported.

Chapter 7 (Safety):

- SAR:1213: Comments: SCU added.

- SAR:2312: Description: Removed dublette.

- SAR:2502: Comments: Added rationale concerning timeout signals and shutdown signals.

New chapter 7.4.1 (Safety / Two Diverse SW channels on the same HW Board / Common Requirements):

- New requirement DAT: 6853: Common Requirements - Failure Disclosure Time

Chapter 7.4.2 (Safety / Two Diverse SW channels on the same HW Board / CHILL Instruction Test):

- SAR:6800: Description: Refined. Added default value and range.

- SAR:6802: Description: Execution/implementation adapted analogous to SAR:6805. Added necessity to execute test on each CPU core. Comments: Removed sentence concerning 2 channels.

- SAR:6849: Synopsis updated. Description refined. Updated checking conditions. Added reference to DAT:6853.Comments: Added reference.

- SAR:6852: Description corrected. Comments: Added note that 2 failures on any CPU core result in a blocking. Added reference to SAR:6801.

Chapter 7.4.3 (Safety / Two Diverse SW channels on the same HW Board / Supervision of CHILL Instruction Test):

- SAR:6805: Description refined. Comments: Explanation restricted to diverse implementation. Added checking “integration test”.

- SAR:6804: Requirement deleted (content merged into SAR:6817).

- SAR:6817: Description: Added periodical execution (content from SAR:6804). Added reference to DAT:6853.

- New requirement SAR:6804: Supervision CHILL Instruction Test - Supervision Reaction in Case of Failure.

Chapter 7.4.4 (Safety / Two Diverse SW channels on the same HW Board / Supervision TAS Platform Health Monitor):

- SAR:6809: Description: Re-formulated and aligned with SAR:6805. Comments removed.

- SAR:6807: Description: Re-formulated and aligned with SAR:6803.

- SAR:6806: Description: Re-formulated and aligned with SAR:6817. Synopsis updated. Added reference to DAT:6853.

- SAR:6808: Description: Re-formulated and aligned with SAR:6804. Synopsis updated.

Chapter 7.4.6 (Safety / Two Diverse SW channels on the same HW Board / RCU Output Voter):

- Chapter renamed to “RCU Output Comparator”

- SAR:6815: Desription updated. Checking: Added “code reading”.

- SAR:6813: Description refined.

Chapter 7.4.7 (Safety / Two Diverse SW channels on the same HW Board / Further SRACs from [Safety Concept]):

- SAR:6843: Description updated.

- SAR:6844: Description: Reference moved to comments. Checking: Added code reading.

- SAR:6845: Comments: Sentence concerning virtualization removed.

- SAR:6816: Description: Added SIL-4.

- SAR:6818: Description re-formulated.

- SAR:6827: Checking: “Module test” replaced by “integration test”.

- SAR:6834: Added checking “module test”.

### Released Version

Chapter 4.1.1 (Functional Requirements./ General Requirements):

- FUR:0118: Description updated.

- FUR:0334: Description updated.

- FUR:0601: Synopsis, Description: D2 removed. System Baseline: Added PV GEN 2.20.0.

- FUR:0829: Description: D2 removed. System Baseline: Added PV GEN 2.20.0.

- FUR:1104: Requirement deleted (no D2 support for PV GEN 2.20.0 and higher).

- FUR:4203: Requirement deleted (no D2 support for PV GEN 2.20.0 and higher).

- MIR:0317: Comments: D2 removed. System Baseline: Added PV GEN 2.20.0.

Chapter 4.1.11 (Virtualization Server Operation and Maintenance):

- Figure 1: Caption updated

Chapter 4.3 (Constraints):

- GCR:0823: Synopis, Description: D2 removed, Server HW added. System Baseline: Added PV GEN 2.20.0.

- GCR:0850 deleted (content merged to requirement GCR:0823).

- GCR:0337:Description: AEOS 4.x removed. System Baseline: Added PV GEN 2.20.0.

- New requirement GCR:0851: Use of TAS Platform.

Chapter 5.1.1 (Enhancement Interface DGP-Webride):

- TIR:4502 deleted (no D2 support for PV GEN 2.20.0 and higher).

Chapter 5.1.2 (RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN):

- TIR:4901 deleted (no D2 support for PV GEN 2.20.0 and higher).

- TIR:4907: Comments: Removed notes concerning D2.

Chapter 6.1 (Requirements to the Customer / Serial Connection ELEKTRA - AzLM):

- Refined note that with PV GEN 2.20.1 and higher and OCS-SBJ the AzLM interface will be supported again..

Chapter 7.4.2 (Safety / Two Diverse SW channels on the same HW Board / CHILL Instruction Test):

- SAR:6802: Synopsis updated. Changed description and comments that similar to OLT a single implementation of the CIT is sufficient.

- SAR:6849: Description updated.

- SAR:6800: Description: Lower boundary set to 0 (continuous test runs). Comments: Added information concerning continuous test runs.

- New requirement SAR:6854: Shutdown procedure on application level (according to SAC:0034 of [Safety Concept]).

Chapter 7.4.7 (Safety / Two Diverse SW channels on the same HW Board / Further SRACs from [Safety Concept]):

- SAR:6825: System Baseline changed to PV GEN 2.20.1. Comments: Added note that in PV GEN 2.20.0 only category 1 communication is supported.

Chapter 9 (Reliability):

- DPR:0209: Description: D2 removed. System Baseline: Added PV GEN 2.20.0.

## Edition 40

### Proposal 01

Proposal for GenSys 2.20.10.

NOTE regarding ELEKTRA4 (architecture "EL4"):

Requirements for the ELEKTRA4 (digital interlocking system) had been initially incorporated in edition 39 of this document, originally planned for system baseline PV_GEN_2.20.0. All chapters, references and requirements, exclusively related to ELEKTRA4 (architecture "EL4") and PV_GEN_2.20.0, have been removed in this edition of the document, since EL4-related requirements will be contained in another/separate document (SSS Virtual Interlocking, 3BU 15000 6955 DTAPA).
Basically the requirements for "CHILL Instruction Test" and "Supervision of CHILL Instruction Test", initially introduced in edition 39, have been kept and taken over for ELEKTRA2 / PV_GEN_2.20.10 (requirement's attribute "Architecture" changed from EL4 to EL2).

General:

- Transition Thales->Hitachi:
update of document template; replacements of "Thales" by "Hitachi" in FUR:1109 and in introduction of chapter 5.1.4.

- Chapter 1.4: update of referenced documents:
References [Safety Concept] and [TAS Platform Safety Concept] deleted; safety concept incl. SRACs are covered in [TAS Plf Safety Handbook], which has been added as a new reference.
New reference [SRS Security] added.
References [SDD VRE] and [UG VRE] deleted, since chapter 4.1.11.1 of ed. 39 has also been deleted.

- Chapter 1.6: abbreviation "OCS-SBJ" added.

- Chapter 3: removed EL4-related description from chapter introduction.

MEN-D2 not supported anymore:
MEN-D2 computer boards are not supported anymore by TAS Platform 2.5.0 or higher, respectively from PV_GEN_2.20.x onwards. The following requirements had been already updated accordingly in ed. 39 ("MEN-D2" removed) and become effective now for system baseline PV_GEN_2.20.10. The system baseline PV_GEN_2.20.10 has been added to the following requirements:

- FUR:0601, FUR:0829, GCR:0823, DPR:0209

OCS-SB not supported anymore:
With TAS platform 2.5.x "OCS-SB" is no longer supported and replaced by "OCS-SBJ", initially applied in PV_GEN_2.20.10 (using TAS platform 2.6.x). "/OCS-SBJ" has been appended to "OCS-SB" in the comments of the following requirements:

- FUR:5214, FUR:5215, TIR:5202, SAR:5212, SAR:5213, DPR:5207

Removal of ELEKTRA4-related chapters/requirements/descriptions (initially added in ed. 39 of this SRS):

- Architecture "EL4" removed from the following requirements (architecture "EL2" kept):
FUR:0166, FUR:0601, FUR:0829, GCR:0823, GCR:0337, MIR:0317, DPR:0209

- Architecture "EL4" replaced by "EL2" (PV_GEN_2.20.10) for all requirements in
chapter 7.4 "CHILL Instruction Test & Supervision of CHILL Instruction Test"
chapter 7.4.1 "CHILL Instruction test"
chapter 7.4.2 "Supervision of CHILL Instruction Test"

- FUR:4204 deleted (pure "EL4" requirement).

- The following EL4-related chapters are not relevant for EL2 and have been deleted (see ed. 39 as reference), as discussed with PM and SAM:
chapters 4.1.10, 7.4.5  "Plant ID"
chapters 4.1.11, 5.1.5, 5.2.1, 6.2, 7.5, 8.1, 9.2, 12.1, 13.1  "Virtualization Server Operation and Maintenance"
chapter 7.4.4  "Supervision of TAS Platform Health Monitor (HMD)"
chapter 7.4.6  "RCU Output Comparator"
chapter 7.4.7  "Further SRACs from [Safety Concept]"

Chapter 4.1.1 "General Requirements":

- New FUR:1928 "Detection of high system load - configurable data" (see PCR PSCWS-3128).

Chapter 4.1.3 "Monitoring of Reactivity of ELEKTRA system components":

- Adaptation of chapter introduction: replaced SRAC [MS-2SW1HW:SAC:0028] of [Safety Concept] by SRAC [Core-2.x:SAC:0232] of the [TAS Plf Safety Handbook].

Chapter 4.3 "Constraints":

- GCR:0337: description and comment adapted for PV_GEN_2.20.10.

Chapter 6.1 "Serial Connection ELEKTRA - AzLM":

- Chapter introduction modified regarding OCS-SBJ in PV_GEN_2.20.10.

- SCR:5226: Synopsis and Description modified regarding OCS-SBJ in PV_GEN_2.20.10.

Chapter 7 "Safety":

- SAR:1212: references to deleted REQs FUR:6810, DAT:6811 (of deleted chapter 4.1.10) and SAR:6812 (of deleted chapter 7.4.5) removed.

Chapter 7.3 "Serial Connection ELEKTRA - AzLM":

- Chapter introduction modified regarding OCS-SBJ in PV_GEN_2.20.10: added reference "Safety Application Conditions/SAV - TAS Platform OCS - SBJ - 1.x" (3CR 00055 7751 DUAPQ).

- SAR:5211: description modified regarding OCS-SBJ in PV_GEN_2.20.10.

- SAR:5220: synopsis and description modified regarding OCS-SBJ in PV_GEN_2.20.10.

Chapter 7.4 "CHILL Instruction Test & Supervision of CHILL Instruction Test":

- Applies to this chapter including subchapters 7.4.1 "CHILL Instruction test" and 7.4.2 "Supervision of CHILL Instruction Test":
"Architecture" is changed from "EL4" to "EL2" for all requirements in these chapters:
Requirements are valid for EL2 / PV_GEN_2.20.10 (using TAS platform 2.6.0), because the "CPU test" part of the OLT is not supported anymore by the TAS platform (CHILL instruction test replaces the former CPU test).

- Chapter 7.4 replaces the former chapter/heading "Two Diverse SW channels on the same HW".

- Chapter introduction adapted for PV_GEN_2.20.10 (reference to [TAS Plf Safety Handbook] added).

- Subchapter 7.4.1 "Common Requirements" (see ed. 39) deleted / replaced by "CHILL Instruction Test".

- DAT:6853: synopsis and description adapted for EL2 / PV_GEN_2.20.10.

Chapter 7.4.1 "CHILL Instruction Test" (former chapter 7.4.2 of ed. 39) and
chapter 7.4.2 "Supervision of CHILL Instruction Test" (former chapter 7.4.3 of ed. 39):

- All requirements in these chapters have been adapted for EL2 / PV_GEN_2.10.10.

Chapter 8 "Security":

- Chapter introduction with reference to [SRS Security] added.

- SER:0327 deleted (was already descoped)

Chapter 9 "Reliability":

- New DPR:1020 "Robustness to year 2038 problem" (see also PCR CELSYS-487).

### Released Version

General:

- Chapter 1.2: added "Project Security Manager".

- Chapter 1.4: sorted the table alphabetically; "EN 50128" replaced by "EN 50716".

- Chapter 1.6: added abbreviations: CRA, JPS, MDS; correction of the English description for ZAML/ZUML.

- Chapter 15 "OPEN ISSUES" added/new (identification of requirements for SCWS to be done...):
NOTE: As an example the Allocation "SCWS" has been added to requirement FUR:1928 so far.

Modifications according to review of edition 40, proposal 01:

- Chapter 4.1.1 "General Requirements":
Modfied description:
 	FUR:0118, FUR:0119, FUR:0333, FUR:0846, FUR:0847, FUR:1107, FUR:1928
Modified comments:
 	FUR:0338, FUR:0339, FUR:0341, FUR:0601, FUR:0829, FUR:0848, FUR:1578,
 	FUR:1928
Removed comments:
 	FUR:0123
Re(moved) requirement:
 	FUR:0120 (decided to be moved to SBB-specific SRS 3BU 15000 0309 DTAPA).
Deleted requirement:
 	FUR:0843 (duplicate; already present/covered in FUR:0333)

- Chapter 4.1.2 "Journaling":
Chapter introduction added (subject: ESM is replaced by JPS).
Replaced the Allocation "ESM" by "JPS" for all requirements in this chapter.
Modified description (ESM -> JPS):
 	FUR:4305
Removed comments:
 	FUR:4301
Deleted requirements (requirements which concern the JPS functionality itself are not handled within the generic system and have been deleted):
 	FUR:4304, FUR:4306

- Chapter 4.1.3 "Monitoring of Reactivity of ELEKTRA system components":
Removed "IAU" from the chapter introduction (IAU has no reactivity supervision).

- Chapter 4.1.4 "Enhancement Interface DGP-Webride (MDS":
Added a note in the chapter introduction ("MDS is used for ELEKTRA-ÖBB and ELEKTRA-International as successor of the Webride").

- Chapter 4.1.7.1 "Time signal via NTP":
FUR:5306: Modified comments.

- Chapter 4.2.1 "RCU/SCU Communication between two ELEKTRA IXLs or between ELEKTRA IXL and other external components via Ethernet LAN":
PRR:4906: Customer "SBB" added (and additionally mentioned in "Comments").

- Chapter 4.3 "Constraints":
Modified comments:
 	GCR:0823, GCR:0337

- Chapter 5.1.4 "Serial Connection ELEKTRA - AzLM":
TIR:5202: Modified comments.

- Chapter 7 "Safety":
SAR:0404: Requirement has been moved to chapter 7.4.1 "CHILL Instruction Test", since with PV_GEN_2.20.10 the check of the inversion function(s) must be performed via the "CHILL Instruction Test". Requirement has been adapted accordingly (allocation "TAS platform" replaced by "Generic System"; note added under "Comments").
SAR:1204: Allocation "TAS platform" added (as since PLF 2.5.2 the safety concept for "2-SW on on HW" has been included to TAS Platform iself).
SAR:1207: Modified comments.

- Chapter 7.1 "Monitoring of reactivity of ELEKTRA system components":
Removed "IAU" from the chapter introduction (IAU has no reactivity supervision).

- Chapter 7.3 "Serial Connection ELEKTRA - AzLM":
SAR:5211: Modified description.

- Chapter 7.4.1 "CHILL Instruction Test":
SAR:6802: Modified description
SAR:6800: Modified description, added checking "code reading", added comment.

- Chapter 7.4.2 "Supervision of CHILL Instruction Test":
SAR:6805: Modified description and comment.
SAR:6803: Modified synopsis and description.
SAR:6817: Modified description, added checking "code reading", added comment.

- Chapter 9 "Reliability":
Word "hub" removed from the description of the following requirements (a hub was only used with D2 systems): DPR:0204, DPR:0205, DPR:0206, DPR:0207, DPR:0212.
DPR:0209: added comment.
DPR:0224: Modified description, added comment.
DPR:1020: added comment (note for customers using the X25-interface).

- Chapter 14 "Tool Chain":
Re(moved) requirement:
 	TCR:0121: decided to be moved to SBB-specific SRS 3BU 15000 0309 DTAPA;
 	Reason: TAS Plf. 2.6.0+ does not provide native Linux support anymore; a new concept
 	for EBO 1.5 based on containers has to be developed.
Deleted requirement:
 	TCR:0122: Requirement is obsolete / not applicable anymore (test environment had been
 	replaced by TFU).

INDEX OF REQUIREMENTS

DAT:6853 (TRT1631)	129

DOR:0924 (TRT3)	154

DPR:0201 (TRT285)	135

DPR:0202 (TRT286)	135

DPR:0203 (TRT287)	135

DPR:0204 (TRT288)	135

DPR:0205 (TRT289)	136

DPR:0206 (TRT290)	136

DPR:0207 (TRT291)	136

DPR:0209 (TRT293)	137

DPR:0212 (TRT296)	137

DPR:0215 (TRT301)	137

DPR:0216 (TRT304)	138

DPR:0217 (TRT305)	138

DPR:0218 (TRT306)	138

DPR:0219 (TRT307)	139

DPR:0220 (TRT308)	139

DPR:0222 (TRT313)	139

DPR:0223 (TRT315)	140

DPR:0224 (TRT316)	140

DPR:0225 (TRT318)	140

DPR:0228 (TRT321)	141

DPR:0229 (TRT322)	141

DPR:0314 (TRT5)	141

DPR:0316 (TRT6)	142

DPR:0401 (TRT218)	142

DPR:0810 (TRT8)	142

DPR:0811 (TRT9)	143

DPR:1001 (TRT229)	143

DPR:1020 (TRT1671)	143

DPR:5207 (TRT833)	144

FUR:0112 (TRT14)	24

FUR:0113 (TRT15)	24

FUR:0114 (TRT16)	24

FUR:0116 (TRT17)	25

FUR:0117 (TRT18)	25

FUR:0118 (TRT19)	25

FUR:0119 (TRT20)	26

FUR:0123 (TRT22)	26

FUR:0124 (TRT23)	27

FUR:0125 (TRT24)	27

FUR:0126 (TRT25)	27

FUR:0127 (TRT26)	28

FUR:0128 (TRT27)	28

FUR:0131 (TRT30)	28

FUR:0132 (TRT31)	28

FUR:0166 (TRT1619)	26

FUR:0210 (TRT294)	29

FUR:0211 (TRT295)	29

FUR:0306 (TRT44)	29

FUR:0307 (TRT45)	30

FUR:0309 (TRT46)	30

FUR:0330 (TRT47)	31

FUR:0331 (TRT48)	31

FUR:0332 (TRT49)	35

FUR:0333 (TRT50)	35

FUR:0334 (TRT51)	35

FUR:0338 (TRT53)	36

FUR:0339 (TRT54)	36

FUR:0341 (TRT55)	37

FUR:0343 (TRT56)	37

FUR:0344 (TRT57)	38

FUR:0345 (TRT58)	38

FUR:0346 (TRT59)	38

FUR:0350 (TRT60)	39

FUR:0354 (TRT61)	39

FUR:0355 (TRT62)	39

FUR:0357 (TRT63)	40

FUR:0362 (TRT65)	40

FUR:0363 (TRT66)	40

FUR:0367 (TRT69)	41

FUR:0368 (TRT70)	41

FUR:0370 (TRT72)	41

FUR:0372 (TRT74)	42

FUR:0508 (TRT79)	42

FUR:0509 (TRT80)	42

FUR:0510 (TRT81)	43

FUR:0601 (TRT303)	43

FUR:0802 (TRT85)	43

FUR:0803 (TRT86)	44

FUR:0804 (TRT279)	44

FUR:0809 (TRT87)	44

FUR:0813 (TRT88)	45

FUR:0814 (TRT89)	45

FUR:0821 (TRT90)	45

FUR:0825 (TRT92)	46

FUR:0826 (TRT93)	46

FUR:0827 (TRT94)	46

FUR:0828 (TRT95)	47

FUR:0829 (TRT299)	47

FUR:0830 (TRT302)	48

FUR:0831 (TRT445)	48

FUR:0832 (TRT434)	48

FUR:0833 (TRT435)	48

FUR:0834 (TRT436)	49

FUR:0835 (TRT437)	49

FUR:0837 (TRT439)	49

FUR:0838 (TRT440)	49

FUR:0839 (TRT441)	49

FUR:0841 (TRT443)	50

FUR:0842 (TRT444)	50

FUR:0844 (TRT510)	50

FUR:0846 (TRT667)	50

FUR:0847 (TRT668)	51

FUR:0848 (TRT669)	51

FUR:1103 (TRT235)	51

FUR:1105 (TRT284)	51

FUR:1106 (TRT506)	51

FUR:1107 (TRT507)	52

FUR:1108 (TRT671)	52

FUR:1109 (TRT672)	52

FUR:1110 (TRT676)	52

FUR:1302 (TRT104)	53

FUR:1303 (TRT105)	53

FUR:1304 (TRT106)	53

FUR:1574 (TRT219)	54

FUR:1577 (TRT317)	54

FUR:1578 (TRT1433)	54

FUR:1701 (TRT222)	55

FUR:1702 (TRT223)	55

FUR:1928 (TRT1673)	56

FUR:1929 (TRT662)	55

FUR:2801 (TRT467)	56

FUR:2802 (TRT468)	56

FUR:2803 (TRT469)	57

FUR:2804 (TRT470)	57

FUR:2806 (TRT472)	57

FUR:2807 (TRT473)	57

FUR:2808 (TRT474)	58

FUR:2809 (TRT476)	58

FUR:2810 (TRT477)	58

FUR:2811 (TRT478)	58

FUR:2812 (TRT479)	58

FUR:3001 (TRT484)	59

FUR:3004 (TRT487)	59

FUR:3005 (TRT488)	59

FUR:3006 (TRT489)	59

FUR:3801 (TRT517)	60

FUR:3802 (TRT518)	60

FUR:3803 (TRT519)	60

FUR:3804 (TRT520)	60

FUR:3805 (TRT521)	61

FUR:3806 (TRT522)	61

FUR:4101 (TRT683)	61

FUR:4103 (TRT1096)	31

FUR:4106 (TRT1099)	32

FUR:4107 (TRT1100)	32

FUR:4108 (TRT1101)	32

FUR:4109 (TRT1102)	33

FUR:4110 (TRT1108)	33

FUR:4111 (TRT1288)	34

FUR:4200 (TRT1330)	61

FUR:4201 (TRT1331)	62

FUR:4202 (TRT1332)	62

FUR:4301 (TRT685)	63

FUR:4302 (TRT686)	63

FUR:4305 (TRT689)	63

FUR:4400 (TRT721)	64

FUR:4401 (TRT722)	64

FUR:4403 (TRT964)	65

FUR:4404 (TRT971)	65

FUR:4405 (TRT970)	66

FUR:4406 (TRT969)	67

FUR:4420 (TRT1106)	70

FUR:4421 (TRT1107)	70

FUR:4422 (TRT1110)	67

FUR:4423 (TRT1109)	68

FUR:4430 (TRT1118)	69

FUR:4431 (TRT1119)	69

FUR:4436 (TRT1127)	68

FUR:4505 (TRT737)	71

FUR:4506 (TRT738)	72

FUR:4507 (TRT739)	72

FUR:4509 (TRT748)	72

FUR:4901 (TRT791)	73

FUR:4902 (TRT792)	73

FUR:4903 (TRT793)	74

FUR:4904 (TRT794)	74

FUR:5204 (TRT846)	75

FUR:5206 (TRT848)	75

FUR:5211 (TRT876)	98

FUR:5214 (TRT948)	76

FUR:5215 (TRT949)	76

FUR:5216 (TRT950)	77

FUR:5218 (TRT953)	77

FUR:5219 (TRT954)	77

FUR:5227 (TRT975)	78

FUR:5228 (TRT976)	78

FUR:5229 (TRT977)	79

FUR:5306 (TRT1373)	79

FUR:6205 (TRT1088)	79

GCR:0162 (TRT107)	85

GCR:0163 (TRT108)	85

GCR:0164 (TRT109)	85

GCR:0301 (TRT110)	86

GCR:0303 (TRT111)	86

GCR:0304 (TRT112)	86

GCR:0305 (TRT113)	87

GCR:0337 (TRT52)	91

GCR:0347 (TRT115)	87

GCR:0348 (TRT116)	87

GCR:0349 (TRT117)	88

GCR:0352 (TRT118)	88

GCR:0359 (TRT119)	88

GCR:0361 (TRT120)	89

GCR:0601 (TRT121)	89

GCR:0602 (TRT122)	89

GCR:0808 (TRT123)	90

GCR:0812 (TRT124)	90

GCR:0818 (TRT125)	90

GCR:0823 (TRT1090)	91

MDR:0213 (TRT297)	149

MDR:0311 (TRT128)	149

MDR:1002 (TRT230)	149

MDR:1003 (TRT231)	150

MDR:1006 (TRT244)	150

MDR:1007 (TRT245)	150

MDR:1009 (TRT247)	151

MDR:1010 (TRT248)	151

MDR:1011 (TRT249)	151

MDR:1012 (TRT250)	152

MDR:1015 (TRT253)	152

MDR:1019 (TRT283)	152

MIR:0103 (TRT133)	99

MIR:0104 (TRT134)	99

MIR:0105 (TRT135)	100

MIR:0106 (TRT136)	100

MIR:0107 (TRT137)	101

MIR:0108 (TRT138)	101

MIR:0109 (TRT139)	102

MIR:0110 (TRT140)	102

MIR:0111 (TRT141)	102

MIR:0134 (TRT143)	103

MIR:0137 (TRT146)	103

MIR:0164 (TRT226)	103

MIR:0165 (TRT227)	104

MIR:0170 (TRT1063)	105

MIR:0317 (TRT152)	104

MIR:0318 (TRT153)	104

MIR:0336 (TRT154)	105

MIR:1402 (TRT271)	105

OAR:1004 (TRT232)	146

OAR:1005 (TRT243)	146

OAR:1008 (TRT246)	146

OAR:1014 (TRT252)	147

OAR:1016 (TRT280)	147

OAR:1017 (TRT281)	147

OAR:1018 (TRT282)	147

PRR:0115 (TRT161)	80

PRR:0319 (TRT162)	81

PRR:0320 (TRT163)	81

PRR:0321 (TRT164)	81

PRR:0322 (TRT165)	81

PRR:0323 (TRT166)	82

PRR:0324 (TRT167)	82

PRR:0601 (TRT168)	82

PRR:0602 (TRT228)	83

PRR:0805 (TRT169)	83

PRR:0806 (TRT170)	83

PRR:4905 (TRT795)	84

PRR:4906 (TRT1436)	84

SAR:0144 (TRT172)	108

SAR:0214 (TRT298)	108

SAR:0221 (TRT310)	108

SAR:0315 (TRT175)	109

SAR:0335 (TRT176)	109

SAR:0353 (TRT178)	109

SAR:0401 (TRT237)	110

SAR:0402 (TRT238)	110

SAR:0403 (TRT241)	110

SAR:0404 (TRT242)	131

SAR:0405 (TRT269)	111

SAR:0406 (TRT311)	111

SAR:0407 (TRT312)	111

SAR:0408 (TRT323)	112

SAR:0603 (TRT180)	112

SAR:0604 (TRT181)	112

SAR:0605 (TRT182)	113

SAR:0606 (TRT183)	113

SAR:0807 (TRT184)	113

SAR:0817 (TRT185)	113

SAR:0820 (TRT186)	114

SAR:0831 (TRT433)	114

SAR:0836 (TRT438)	114

SAR:0840 (TRT442)	114

SAR:1101 (TRT505)	114

SAR:1102 (TRT512)	115

SAR:1111 (TRT677)	115

SAR:1112 (TRT678)	115

SAR:1201 (TRT189)	116

SAR:1202 (TRT190)	116

SAR:1203 (TRT191)	116

SAR:1204 (TRT192)	116

SAR:1205 (TRT193)	117

SAR:1206 (TRT194)	117

SAR:1207 (TRT195)	117

SAR:1208 (TRT196)	117

SAR:1209 (TRT197)	118

SAR:1210 (TRT198)	118

SAR:1211 (TRT276)	118

SAR:1212 (TRT277)	119

SAR:1213 (TRT278)	119

SAR:1901 (TRT663)	119

SAR:1902 (TRT664)	120

SAR:2022 (TRT694)	120

SAR:2023 (TRT695)	121

SAR:2024 (TRT696)	121

SAR:2102 (TRT697)	121

SAR:2103 (TRT698)	121

SAR:2311 (TRT673)	122

SAR:2312 (TRT674)	122

SAR:2313 (TRT675)	122

SAR:2501 (TRT659)	123

SAR:2502 (TRT660)	123

SAR:2504 (TRT1071)	123

SAR:2505 (TRT1070)	124

SAR:2506 (TRT1068)	124

SAR:2801 (TRT466)	124

SAR:2802 (TRT475)	124

SAR:2807 (TRT491)	125

SAR:3001 (TRT490)	125

SAR:4411 (TRT963)	125

SAR:4501 (TRT741)	126

SAR:5208 (TRT835)	126

SAR:5209 (TRT862)	127

SAR:5210 (TRT863)	127

SAR:5211 (TRT864)	127

SAR:5212 (TRT946)	128

SAR:5213 (TRT947)	128

SAR:5217 (TRT951)	128

SAR:5220 (TRT955)	129

SAR:6800 (TRT1488)	130

SAR:6801 (TRT1490)	131

SAR:6802 (TRT1489)	130

SAR:6803 (TRT1493)	132

SAR:6804 (TRT1630)	132

SAR:6805 (TRT1491)	131

SAR:6817 (TRT1524)	132

SAR:6849 (TRT1618)	130

SAR:6851 (TRT1621)	130

SAR:6852 (TRT1622)	131

SCR:5225 (TRT973)	107

SCR:5226 (TRT974)	107

TCR:0141 (TRT204)	155

TCR:0161 (TRT205)	155

TCR:0358 (TRT206)	155

TCR:1618 (TRT275)	156

TIR:0308 (TRT209)	92

TIR:0601 (TRT210)	92

TIR:4504 (TRT743)	92

TIR:4907 (TRT796)	93

TIR:5101 (TRT828)	94

TIR:5102 (TRT838)	95

TIR:5201 (TRT830)	95

TIR:5202 (TRT837)	96

TIR:5205 (TRT857)	95

TIR:5206 (TRT858)	96

TIR:5207 (TRT859)	97

TIR:5208 (TRT860)	97

TIR:5209 (TRT865)	96

TIR:5210 (TRT861)	97

TIR:5221 (TRT956)	98

TIR:5222 (TRT957)	98

TIR:5224 (TRT972)	99

TIR:6700 (TRT1435)	80

TVR:0815 (TRT217)	145

TVR:1102 (TRT234)	145

END OF DOCUMENT