Editions and Change History

| Edition | Date | Written by | Verified by | Approved by | Change Description |
| --- | --- | --- | --- | --- | --- |
| 01 RL | 2016-03-06 | Thales Austria / GRR RAM Manager Florian Moser | Thales Austria / GQA QAM Enrico Girod | Thales Austria / TIP PM Walter Bislavsky | First Edition |
| 02 RL | 2016-11-28 | Thales Austria / GRR RAM Manager Florian Moser, Markus Scheuer | Thales Austria / GQA QAM Enrico Girod | Thales Austria / TIP PM Walter Bislavsky | See 1.5.2 |

Table of Contents

1	SCOPE	4

1.1	Identification	4

1.2	Document Overview	4

1.3	Intended Audience	4

1.4	Evolution of the document	4

1.4.1	Distribution	4

1.4.2	Configuration Management	4

1.5	Change History	5

1.5.1	Edition 01	5

1.5.2	Edition 02	5

2	References and Terms	6

2.1	Referenced Documents	6

2.2	Standards and Procedures	6

2.3	Terms, Abbreviations and Acronyms	7

3	Installed Base Slovenia – Corridor D	8

3.1	LEU Variants Slovenia – Corridor D	8

3.1.1	LEU PV 3.5 Variants	8

3.1.2	LEU PV 3.5 components	8

3.2	Balises Slovenia – Corridor D	9

4	Assessment LEU repairs since deployment	10

5	Assessment LEU field diagnoses	11

6	Dependability Requirements	12

6.1	Fulfilment of Requirements	12

6.1.1	Fulfilment of MTBD Requirements	12

6.1.2	Fulfilment of MTBF Requirements	12

6.1.3	Fulfilment of balise Requirements	12

END OF DOCUMENT	12

Tables

Table 1 : Referenced Documents	6

Table 2: Standards and Procedures	6

Table 3 : Terms, Abbreviation and Acronyms	7

Table 4: Logical equations LEU variants PV 3.5	8

Table 5: Quantity of installed LEU variants in Corridor D	8

Table 6: Failure rates of LEU PV 3.5 components	9

Table 7: Quantity of components for LEU variants PV3.5 in Corridor D	9

Table 8: MTBF and availability of balises	9

Table 9: Quantity of installed balises in Corridor D	9

Table 10: Failures of components in 2015	10

Table 11: Comparison calculated – empirical MTBF	10

Table 12: LEU field diagnoses – incidents per section	11

Table 13: Corridor D - MTBD values	12

# SCOPE

## Identification

This document provides a repair report of the LEU variants PV 3.5 developed by THALES Austria for the ERTMS/ETCS equipment on Corridor D Infrastructure ETCS L1 System in the period of the year 2015. It covers a dependability requirement stated in Tender Documentation ETCS L1 Corridor D – Slovenia [Tender], which demands a RAM test to be conducted over a period of one year from the commissioning of equipment.

## Document Overview

Chapter 3 provides system relevant aspects together with references to the system documentation.

Chapter 4 presents an overview of the installed printed board assemblies and of the LEU variants PV 3.5 as well as of balises.

Chapter 5 presents an assessment of field diagnoses figures of LEU variants PV 3.5 as well as of balises.

Chapter 6 analyses the fulfilment of requirements specified in [Tender].

## Intended Audience

The intended audience of the repair report are the following functions:

- General Project Manager

- System Engineering Manager

- Project Managers

- RAMS Manager

- Validator

- Quality Assurance Manager

- Engineer

The contents of this repair report are to be treated confidentially.

## Evolution of the document

### Distribution

A new edition of this document has to be distributed to all persons, which are listed in chapter 1.3 Intended Audience.

### Configuration Management

This document will be archived in the THALES Austria document management system. It will be adjusted if significant changes to the valid version are recognized concerning content and extent of the described measures. A new version has to be approved by the Project Design Authority and will be available through the Thales network as described in 1.4.1 Distribution.

## Change History

### Edition 01

First edition

### Edition 02

Reference to tender document updated.

# References and Terms

## Referenced Documents

| Id | Reference |
| --- | --- |
| [RAM_ANA] | Reliability Analysis – ETCS L1 – Corridor D – Slovenia; 3BU 82800 5000 DUAPA, Ed.02 |
| [Tender] | Tender Documentation ETCS L1 Corridor D – Slovenia Design and Build of Level 1 ETCS System of Corridor D – Slovenia. Ed.01 |

Table 1 : Referenced Documents

## Standards and Procedures

| Id | Reference |
| --- | --- |
| [THALES_FAILRATE] | Reliability Standard Failure Rate Factor; ANV: 1AA 00210 0001 BCZZA, Ed. 04; (THALES Standard) |
| [EN_50126] | EN 50126-1:1999: Railway applications – The specification and demonstration of Reliability, Availability, Maintainability and Safety, 1999. |
| [IEC60050(191)] | IEC 60050-191 Ed. 1.0 b:1990: International Electrotechnical Vocabulary. Chapter 191: Dependability and quality of service |
| [Subset-026] | ERTMS/ETCS - Class 1 - System Requirement Specification, UNISIG 24.02.06, additional as "IN" classified CR of Subset-108 Version 1.2.0 (this is named Issue 2.3.0d, but never officially released), Subset-026, Issue 2.3.0 (24.02.2006), TSI - Decision 2012/88/EU |
| [MIL_HDBK217F] | US MIL-HDBK 217F, Reliability prediction for electronic equipment |
| [MIL-HDBK-338B] | US MIL-HDBK 338B, ELECTRONIC RELIABILITY DESIGN HANDBOOK |
| [Birolini] | Reliability Engineering. Theory and Practice; Alessandro Birolini; ISBN: 354040287X |

Table 2: Standards and Procedures

## Terms, Abbreviations and Acronyms

| Term | Meaning |
| --- | --- |
| BalF | Balise fixed |
| BalS | Balise switchable |
| BP | Backpanel |
| Failure rate | Limit for of the probability that an item will fail in the time interval , given that the item was new at and did not fail in the interval , divided by . [Birolini] |
| FiT | Failures in time (unit: ) |
| Ground Benign (GB) | Nonmobile, temperature and humidity controlled environments readily accessible to maintenance; includes laboratory instruments and test equipment, medical electronic equipment, business and scientific computer complexes, and missiles and support equipment in ground silos. [MIL_HDBK217F] |
| Ground Fixed (GF) | Moderately controlled environments such as installation in permanent racks with adequate cooling air and possible installation in unheated buildings; includes permanent installations of air traffic control radars and communication facilities [MIL_HDBK217F] |
| LEU | Lineside Electronic Unit |
| MTBF | The expectation of the operating time between failures. [IEC 60050(191)]. is usually reserved for items with constant failure rate . In this case, is the expected value (mean) of the exponentially distributed item’s failure-free operating time. In particular , where is the given, fixed cumulative operating time and the total number of failures (failed items) during . [Birolini] |
| MTBD | Mean Time Between Demand: A measure of system reliability related to demand for logistic support. The total number of system life units divided by the total number of system demands on the supply system during a stated period of time. [MIL-HDBK-338B] |
| MTTR | The expectation of the time to restoration. [IEC 60050(191)] is the expected value (mean) of an item’s repair time. It is obtained from the distribution function of the repair time as . For reliability or availability calculation a constant repair rate (i.e. exponentially distributed repair times with ) can often be used to get approximate results. [Birolini] |
| PBA | Printed Board Assembly |
| PDU | Power Device Unit |
| PICCG | Processor 2x Interface C with CAN expansion galvanically separated |
| RAB | Relay Adaptation Board |
| SAB | Signal Adaptation Board |

Table 3 : Terms, Abbreviation and Acronyms

# Installed Base Slovenia – Corridor D

This chapter gives a short overview of the LEU System and the components mountable in an LEU rack PV 3.5 and the quantities deployed in the sections of Slovenia – Corridor D.

A more detailed description of the components in use can be found in Reliability Analysis – ETCS L1 – Corridor D – Slovenia, [RAM_ANA].

The overall system structure is described in Tender Documentation ETCS L1 Corridor D – Slovenia, [Tender].

## LEU Variants Slovenia – Corridor D

This section subdivides into a listing of the LEU PV 3.5 variants deployed in Slovenia - Corridor D and into a listing of the components of LEU PV 3.5 variants.

### LEU PV 3.5 Variants

Table 4 below lists the different assemblies of all PV 3.5 LEU variants for the sections of Slovenia – Corridor D together with the corresponding MTBF values for ground fixed 40 ˚C outdoor.

| Product Version | Variant | Logical Equations | MTBF [y] |
| --- | --- | --- | --- |
|  |  |  | GF 40 |
| PV 3.5 | vP1R1 | BP, PICCG2, RAB2, PDU230V | 13,8 |
|  | vP2R1 | BP, 2 x PICCG2, RAB2, PDU230V | 9,5 |
|  | vP1S1 | BP, PICCG2, SAB, PDU230V | 16,9 |
|  | vP2S1 | BP, 2 x PICCG2, SAB, PDU230V | 10,8 |
|  | vP1 | BP, PICCG2, PDU230V | 20,8 |

Table 4: Logical equations LEU variants PV 3.5

Table 5 below lists the quantity of all installed PV 3.5 LEU variants in the sections of Slovenia – Corridor D.

| Product Version | Variant | P1 | P2 | S1 | S2 | S3 | S4 | S5 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PV3.5 | vP1S1 | 68 | 32 | 115 | 160 | 253 | 150 | 129 | 907 |
|  | vP2S1 | 12 | 7 | 32 | 60 | 61 | 19 | 10 | 201 |
|  | vP1R1 | 0 | 8 | 0 | 7 | 5 | 0 | 18 | 38 |
|  | vP2R1 | 0 | 0 | 11 | 9 | 2 | 0 | 4 | 26 |
|  | vP1 | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 2 |
|  |  | 80 | 47 | 160 | 236 | 321 | 169 | 161 | 1 174 |

Table 5: Quantity of installed LEU variants in Corridor D

### LEU PV 3.5 components

Table 6 below lists the MTBF values with the environmental conditions ground fixed 40 ˚C outdoor for the PV 3.5 LEU variants installed in the sections of Slovenia – Corridor D.

| Comp | Type | Identification | MTBF [y] |  |
| --- | --- | --- | --- | --- |
|  |  |  | GF 40 |
| BP | SAB/RAB | 3BU 80849 AxAA | 161,2 |
| PIC | PICCG2 | 3BU 80827 ABAA | 29,9 |
| SAB | SAB | 3BU 81503 AxAA | 88,8 |
| RAB | RAB1 | 3BU 81704 AxAA | 41,0 |
|  | RAB2 |  |  |
| PDU | PDU230V | 3BU 80821 AxAA | 119,4 |

Table 6: Failure rates of LEU PV 3.5 components

Table 7 below lists the quantity of all installed components for LEU variants PV 3.5 in the sections of Slovenia - Corridor D.

| Comp | P1 | P2 | S1 | S2 | S3 | S4 | S5 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PICCG2 | 92 | 54 | 203 | 305 | 384 | 188 | 175 | 1 401 |
| SAB | 80 | 39 | 147 | 220 | 314 | 169 | 139 | 1 108 |
| RAB | 0 | 8 | 11 | 16 | 7 | 0 | 22 | 64 |
| PDU | 80 | 47 | 160 | 236 | 321 | 169 | 161 | 1 174 |

Table 7: Quantity of components for LEU variants PV3.5 in Corridor D

## Balises Slovenia – Corridor D

For the sections of Corridor D highly available balises from ALSTOM are deployed. According to the data sheets, the MTBF values of fixed balises are stated with MTBF = 521 y and switchable balises with MTBF = 245 y.  Table 8 below lists the corresponding availability figures.

| Balise | Type | MTBF [y] | Availability [%] |
| --- | --- | --- | --- |
|  |  | GF 40 | GF 40 |
| ALSTOM | fixed | 521 | 99,999 9 |
|  | switchable | 245 | 99,999 8 |

Table 8: MTBF and availability of balises

Table 9 below lists the quantity of all installed balises in the sections of Slovenia – Corridor D.

| Type | P1 | P2 | S1 | S2 | S3 | S4 | S5 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| balise fixed | 93 | 71 | 184 | 293 | 431 | 230 | 169 | 1 471 |
| balise switch | 147 | 91 | 350 | 496 | 594 | 293 | 314 | 2 285 |

Table 9: Quantity of installed balises in Corridor D

# Assessment LEU repairs since deployment

This chapter gives an overview of the occurred repairs of the balises and the components mounted in an LEU rack PV 3.5 in the sections of Slovenia – Corridor D.

It contains a comparison of the calculated failure rates of each system/component with the empirical failure rates based on experience in the field in the year 2015.

In this timeframe a total of 36 repairs have been reported.

Table 10 gives an overview of the repairs of all affected components.

| MatNr | Comp | repairs |  |
| --- | --- | --- | --- |
| 3BU 80821 AAAA | PDU | 9 |  |
| 3BU 80827 ABAA | PICCG2 | 24 |  |
| 3BU 89005 0004 | balise fixed | 3 |  |

Table 10: Failures of components in 2015

Table 11 provides a comparison between the calculated MTBF values and the actual empirical MTBF values for ground fixed 40 ˚C outdoor. It can be observed, that the empirical MTBF values of printed board assemblies and power supplies are better than the calculated MTBF values. The high reliability figures of fixed balises indicate two or three repairs per year which shows that the number of repairs is in the expected range.

Please note that according to the customer requirements listed in chapter 6 the number of balise failures has to be less than 10, which is clearly fulfilled.

|  |  |  | calculated |  |  | empirical |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Comp | I.Base | Rep | MTBF [y] GF 40 | Availability [%] | Failure [%/y] | MTBF [y] GF 40 | Availability [%] | Failure [%/y] |
| PICCG2 | 1 401 | 24 | 29,9 | 99,9985 | 3,3 | 58,4 | 99,9992 | 1,7 |
| PDU | 1 174 | 9 | 119,4 | 99,9996 | 0,8 | 130,4 | 99,9996 | 0,8 |
| balise fixed | 1 471 | 3 | 521,3 | 99,9999 | 0,2 | 490,3 | 99,9999 | 0,2 |

Table 11: Comparison calculated – empirical MTBF

# Assessment LEU field diagnoses

LEU field diagnoses can be divided into following incidents:

- Error Shutdown (2)

Underlying is a HW failure where corrective maintenance is needed. This error has an operational consequence for the system.

  - Failure occurred (4)

This is a subdivision of “Error Shutdown”, whereby three or four “critical” diagnoses will lead to an “Error Shutdown”

- Balise disconnect (110)

The balise is not connected to the BD

- CAN bus failures (8)

Part of the HW failures of PICCG2 and taken into account in chapter 4

- Aspect 127 (1)

Aspect 127 indicates an invalid aspect.

- Sync time expiration (42)

Aspect change > 5 seconds

Table 12 gives an overview of the occurred incidents apportioned to all sections of Corridor D.

| Section | Aspect 127 | Balise disconnect | CAN bus | Failure occurred | Error Shut down | Sync time | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | 0 | 14 | 1 | 0 | 0 | 0 | 15 |
| P2 | 0 | 3 | 0 | 0 | 0 | 0 | 3 |
| S1 | 0 | 20 | 3 | 1 | 1 | 0 | 25 |
| S2 | 1 | 59 | 3 | 2 | 1 | 41 | 107 |
| S3 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| S4 | 0 | 41 | 1 | 1 | 0 | 1 | 7 |
| S5 | 0 | 9 | 0 | 0 | 0 | 0 | 9 |
|  | 1 | 110 | 8 | 4 | 2 | 42 | 167 |

Table 12: LEU field diagnoses – incidents per section

In total there are 167 incidents in Corridor D since deployment of the system.

Of these 167 incidents, 33 can be considered as permanent HW failures (see chapter 4, 24PICCG2 + 9PDU) and 134 (167-33=134) as transient failures that occur at irregular and unpredictable times.

Note that Table 12 shows a distinct maximum of balise disconnect incidents. It can be assumed that a reduction this type of incidents will improve the overall result significant.

# Dependability Requirements

The following dependability requirements are specified in Tender Documentation ETCS L1 Corridor D – Slovenia [Tender].

- MTBD for the main conventional lines shall be 16667 hours for train delays of up to 5 minutes.

- MTBF for the entire ETCS system in Corridor D in the RS shall be 100000 hours for total failure

- MTBF for the entire ETCS system in Corridor D in the RS shall be 15000 hours for major defects

- MDT shall be 4 hours

- MTTR shall be 4 hours

- Notwithstanding the specified RAM conditions the number of Eurobalise failures may not be more than 10 failures a year in the entire Corridor D in the RS.

## Fulfilment of Requirements

This section analyses the fulfilment of requirements specified in Tender Documentation ETCS L1 Corridor D – Slovenia [Tender].

### Fulfilment of MTBD Requirements

The MTBD value is a result of the ratio of operating hours with  downtime. As shown in Table 13, the total downtime of the system derives from 132 hours of downtime as a result of permanent HW failures and 11.2 hours as a result of transient failures (see chapter 5).

| Section | Number LEU | Operating hours total | Downtime total h | MTBD [h] | MTBD [h] factor5 |
| --- | --- | --- | --- | --- | --- |
| Corridor D | 1174 | 10 284 240 | 143,2 | 71 834 | 14 367 |

Table 13: MTBD values- Corridor D

With the assumption that any incident by transient failures causes a delay of 5 minutes and incidents of permanent failures cause a delay of MTTR=4h, the MTBD value gives 71 834 hours and the requirement is clearly fulfilled.

Taking also into consideration that there might be some additional delay due to follow-up train movements (indirect delay) each delay is weighted by a factor 5. This results in an MTBD value of 14 367 hours. Note, that even in this case the required MTBD value of 16 667h is nearly fulfilled.

As stated in chapter 5 it is planned to reduce the specific incident type balise disconnect which will further improve the MTBD value.

### Fulfilment of MTBF Requirements

The Reliability Analysis – ETCS L1 – Corridor D – Slovenia [RAM_ANA] provides a calculation model which indicates, that the MTBF requirements from the customer can be fulfilled.

Chapter 4 of this document presents the empirical MTBF calculation based upon LEU repairs since deployment of the system.  According to the figures in Table 11  the MTBF requirements can be considered as fulfilled.

### Fulfilment of balise Requirements

Table 10 shows, that with a total of three repairs of balises, the demanded requirement of no more than 10 failures a year is fulfilled.

# END OF DOCUMENT