Editions and Change History

| Edition | Date | Written by | Verified by | Approved by | Change Description |
| --- | --- | --- | --- | --- | --- |
| 01 RL | 2016-07-11 | Thales Austria / GRR RAM Manager Florian Moser | Thales Austria / GQA QMA Ivana Tolj | Thales Austria / TIH PDA Viktor Nemeth | See section 1.5.1 |
| 02 RL | 2016-08-18 | Thales Austria / GRR RAM Manager Florian Moser | Thales Austria / GQA QMA Ivana Tolj | Thales Austria / TIH PDA Viktor Nemeth | See section 1.5.2 |
| 03 RL | 2018-04-18 | Thales Austria / TRR RAM Manager Markus Scheuer | Thales Austria / TQA QMA Aleksander Balazic | Thales Austria / TEJR SEM Robert Strauss | See section 1.5.3 |
| 04 RL | 2022-11-25 | GTS Austria / TRR RAM-AM Florian Moser | GTS Austria / TQA QAM Denise Kovac | GTS Austria / TEJR PDA Attila Nemeth | See section 1.5.4 |
| 05 RL | 2023-07-13 | GTS Austria / TRR RAM-AM Florian Moser | GTS Austria / TQA QAM Christian Janneschitz | GTS Austria / TEJR PDA Attila Nemeth | See section 1.5.5 |

Table of Contents

1	SCOPE	5

1.1	Identification	5

1.2	Document Overview	5

1.3	Intended Audience	5

1.4	Distribution	5

1.5	Change History	6

1.5.1	Edition 01	6

1.5.2	Edition 02	6

1.5.3	Edition 03	6

1.5.4	Edition 04	6

1.5.5	Edition 05	6

2	References and Terms	7

2.1	Referenced Documents	7

2.2	Standards and Procedures	7

2.3	Terms, Abbreviations and Acronyms	7

3	System Overview	9

3.1	RaiLED	9

3.2	Hardware Interfaces	10

3.2.1	WEM-V2 Interfaces	10

3.2.2	Signal Interfaces	10

3.2.3	AzLM Interfaces	11

3.3	New concept of remote control	12

3.3.1	Redundancy characteristics of the new concept of remote control	13

3.3.2	New communication path 1	15

3.3.3	New communication path 2	15

3.3.4	Total availability of new concept of remote control	16

4	RAM Requirements	17

4.1	RaiLed Requirements	17

4.2	Hardware Interfaces Requirements	17

4.3	Remote Control Requirements	17

4.3.1	Fulfilment of RAM Requirements	18

Figures

Figure 1: Physical architecture of new communication EBO2 – ELEKTRA (category 1)	12

Figure 2: Physical architecture of new remote control concept related Communication EBO2 – ELEKTRA with Category 1 and Category 2 networks	13

Tables

Table 1 : Referenced Documents	7

Table 2 : Standards and Procedures	7

Table 3 : Terms, Abbreviations and Acronyms	8

Table 10: FiT and MTBF values of RaiLed 1.1.4	9

Table 5: Failure rates with WEM-V2 Interfaces	10

Table 6: Failure rates with Signal Interfaces	11

Table 7: Failure rates with AWM Interfaces	11

Table 4: MTBF values components of new communication	13

Table 5: New communication hardware path 1 (MEN-D6)	15

Table 6: New communication hardware path 1 (MEN-D10)	15

Table 7: New communication hardware path 2 (MEN-D6)	15

Table 8: New communication hardware path 2 (MEN-D10)	15

Table 9: Total availability new concept of remote control (MEN-D6)	16

Table 10: Total availability new concept of remote control (MEN-D10)	16

Table 11: MTBF requirements new concept of remote control	17

Table 12: Scheduled maintenance new concept of remote control	17

Table 13: MTBF values path 1 / path 2 (MEN-D6)	18

Table 14: MTBF values path 1 / path 2 (MEN-D10)	18

# SCOPE

## Identification

The following chapters describe ELEKTRA 2 product version International (short PV INT 2.10.0) which is fit for the intended purpose of the Hungarian and Bulgarian markets. See [PSEMP_ELINT]  and [SYS_Description] for details.

The contents of this RAM-analysis are to be treated confidentially.

## Document Overview

Chapter 3 provides system relevant aspects together with references to the system documentation.

Chapter 3.1 provides a description and references of RaiLed.

Chapter 3.2 provides a description and hardware calculations of ELEKTRA 2 interface boards.

Chapter 3.3 provides a description and hardware calculations of the new remote control.

Chapter 4 analyses the fulfilment of requirements specified in [Pflichtenheft_MAV].

## Intended Audience

The intended audience of this reliability analysis comprises the following roles:

- Project / Product Design Authority (PDA)

- Product Line Manager (PLM)

- Solution Engineering Manager (SEM)

- System Architect (ARC)

- Requirements Manager (RM)

- RAM Assurance Manager (RAM-AM)

- Safety Assurance Manager (SAM)

- Quality Assurance Manager (QAM)

- IVVQ Manager –Validator (IVVQ-VAL)

- IVVQ Manager – Verification (IVVQ-VER)

The content of this document is property of GTS Austria GmbH and has to be kept confidential within the organization. No part of this document may be reproduced in any form or by any means without the written permission of GTS Ground Transportation Systems Austria GmbH.

## Distribution

A new edition of this document has to be distributed to all persons, which are listed in section 1.3.

## Change History

### Edition 01

First edition

### Edition 02

- Redundancy concept descriptions added in section 3.3.1.

- Section 4.3.1 Fulfilment of RAM Requirements revised.

### Edition 03

- Refinement of different EBO2 characteristics in chapter 3.3.

- MTBF of switch-over unit (KVM-switch) added in Table 8.

- Figures in chapter 3.3 and chapter 4 updated according to the refinement of the different EBO2 characteristics.

- References updated.

### Edition 04

- References updated.

- New template.

- Figures in chapter 3.3 updated.

- Document/calculations updated with MEN-D10 board.

- Applicable for BL INT_23.02.

### Edition 05

- References updated.

- Added section 3.1, 3.2, 4.1 and 4.2

- Applicable for ELEKTRA 2 product version INT 2.10.0

# References and Terms

## Referenced Documents

| Id | Reference |
| --- | --- |
| [ARPR_AWM] | AlcaRel Reliability Prediction Report - 3BU95987AAAAHAAPC Ed.02 AWM; 3BU 95987 AAAA DUAPA, Ed.02 |
| [ARPR_WEM-V2] | AlcaRel Reliability Prediction Report - 3BU95971ABAA Ed.01 Weichenmodul - WEM-V2; 3BU 95971 ABAA DUAPA, Ed.01 |
| [EL2_SYS_ADD] | Architectural Design Document – System Architecture 3BU 15000 0400 PEAPA, Ed. 24 |
| [RAM_ANA_EL2] | Dependability Analysis - ELEKTRA 2.x 3BU 15000 4997 DUAPA, Ed.06 |
| [SSDD_Remote] | SSDD - Remote Control New 3BU 15000 9059 DSAPA, Ed.04 |
| [Pflichtenheft_MAV] | Pflichtenheft MAV – Gestaltung, Fernsteuerung und Fernbedienung; 3BU 15000 4848 DSAPC, Ed.01 |
| [SYS_Description] | System Description – Interlocking System; 3BU 15000 9010 EDAPA, Ed.11 |
| [EL_Maintenance] | Maintenance and Repair – ESTW – Generic ELEKTRA; 3BU 15000 7050 REAPA, Ed.11 |
| [ADD_EBO2] | SSDD EBO2 International; 3BU 15000 4481 DSAPA, Ed.08 |
| [RAMAN_RaiLED] | Dependability Analysis, RaiLED 1.1.4, 3BU 61000 0173 DUAPA, Ed.02 |
| [PSEMP_ELINT] | PMP and SEMP; ELEKTRA PV INT 2.10.0; 3BU 15000 9179 DPAPA, Ed.03 |

Table 1 : Referenced Documents

## Standards and Procedures

| Id | Reference |
| --- | --- |
| [THALES_FAILRATE] | Reliability Standard Failure Rate Factor; ANV: 1AA 00210 0001 BCZZA, Ed. 04; (THALES Standard) |
| [MIL_HDBK217F] | US MIL-HDBK 217F, Reliability prediction for electronic equipment |
| [IEC60050(192)] | IEC 60050-192:2015; International Electrotechnical Vocabulary (IEV) - Part 192: Dependability |
| [Birolini] | Reliability Engineering. Theory and Practice; Alessandro Birolini; ISBN: 354040287X |

Table 2 : Standards and Procedures

## Terms, Abbreviations and Acronyms

| Term | Meaning |
| --- | --- |
| Availability | Probability that an item is in a state to perform a required function under given conditions at a given instant of time, assuming that the required external resources are provided. [IEC60050(192)] A qualitative definition, focused on ability rather than on probability is also usual. The steady-state value of the point availability can be expressed in general by . [Birolini] |
| EBO2 | Einheitliche BedienOberfläche – the Human Machine Interface of ELEKTRA2 |
| CC | Central Controller |
| Failure rate | Limit for of the probability that an item will fail in the time interval , given that the item was new at and did not fail in the interval , divided by . [Birolini] |
| FiT | Failures in Time (unit: ) |
| Ground Benign (GB) | Nonmobile, temperature and humidity controlled environments readily accessible to maintenance; includes laboratory instruments and test equipment, medical electronic equipment, business and scientific computer complexes, and missiles and support equipment in ground silos. [MIL_HDBK217F] |
| Ground Fixed (GF) | Moderately controlled environments such as installation in permanent racks with adequate cooling air and possible installation in unheated buildings; includes permanent installations of air traffic control radars and communication facilities [MIL_HDBK217F]. |
| KVM | Keyboard, video and mouse |
| [MIL_HDBK217F] | The Military Handbook is published by the US Department of Defense, and provides mathematical models for electrical, electronic and electro-mechanical components. Each model calculates the component failure rate for given combinations of environmental conditions, component value and type, electrical stress conditions and manufactured quality level |
| MEN | Mikro Elektronik Gmbh Nürnberg |
| MEN-D6/D10 | Printed circuit board of the company MEN |
| MMI | Man Machine Interface |
| MTBF | The expectation of the operating time between failures. [IEC60050(192)]. is usually reserved for items with constant failure rate . In this case, is the expected value (mean) of the exponentially distributed item’s failure-free operating time. In particular , where is the given, fixed cumulative operating time and the total number of failures (failed items) during . [Birolini] |
| MTBM | Mean Time Between Maintenance |
| MTTM | Mean Time To Maintain |
| MTTR | The expectation of the time to restoration. [IEC60050(192)] is the expected value (mean) of an item’s repair time. It is obtained from the distribution function of the repair time as . For reliability or availability calculation a constant repair rate (i.e. exponentially distributed repair times with ) can often be used to get approximate results. [Birolini] |

Table 3 : Terms, Abbreviations and Acronyms

# System Overview

A description of the ELEKTRA 2.x system can be found in [EL2_SYS_ADD].

A description of the generic functions and the prediction of the failure behaviour of the ELEKTRA 2.x system can be found in the document [RAM_ANA_EL2].

In the following sections, subsystems of ELEKTRA PV INT 2.10.0 with respect to the operational behaviour will be analysed.

## RaiLED

The electronics of the RaiLed consists of two boards, the LED Control Board 4 and the LED Board 4 together with an external resistor kit.

The LED Control Board 4 controls and monitors the LED Board 4 (LED light source), its basic circuit applies to all colors and it serves as an interface to the lamp transformer. The LED Board 4 consists of a single-channel function module and a dual-channel monitoring module.

The following table is taken from [RAMAN_RaiLED] shows the FiT, resp. MTBF values of the RaiLed components.

Basis for the calculated failure rates was an environmental condition of ground fixed 40 °C.

For different colours the RaiLed is populated with either 7 or 9 LEDs. The failure rates for these two characteristics are listed in the table below.

| Component | Type | Fit | MTBF |
| --- | --- | --- | --- |
|  |  | GF40 | [y] |
| Load resistors + connector |  | 18 | 6 246,0 |
| LED Board 4 | G, W, B: 7 LEDs | 628 | 181,6 |
|  | R, Y: 9 LEDs | 800 | 142,6 |
| LED Control Board 4 |  | 1 520 | 75,1 |
| RaiLed 1.1.4 | G, W, B: 7 LEDs | 2 167 | 52,7 |
|  | R, Y: 9 LEDs | 2 339 | 48,8 |

Table 4: FiT and MTBF values of RaiLed 1.1.4

The following variants of Railed exist:

- RaiLed 20: As a replacement for the 12 V / 20 W signal lamps (Lens diameter 210mm).

- Railed 20 130: Like RaiLed 20 but with 130mm lens diameter.

- Railed 20 160: Like RaiLed 20 but with 160mm lens diameter.

A detailed calculation of all RaiLed variants can be found in [RAMAN_RaiLED].

All RAM related requirements for RaiLed are analysed in [RAMAN_RaiLED].

Note: The product RaiLed ist not part of the ELEKTRA 2 product version International (short PV INT 2.10.0).

## Hardware Interfaces

The hardware interfaces are the link for commands and messages between the computers of the element- and interface level and the external components. They are produced in printed circuit board technology and placed in interface cabinets, see [SYS_Description] for more details.

Interface controllers (IC1Ds) are used to communicate with all different types of interfaces. The IC1D is connected via TTP with the ELEKTRA EC to send commands to the trackside equipment and retrieve status information from there.

In comparison with e.g. a failure of the vital core system ELEKTRA CC, a failure of one element or a small group of elements has typically far less operational consequences. Interface subsystems and interface controllers are designed as non-redundant subsystems.

The failure rates of the interfaces are derived from the figures given in [RAM_ANA_EL2].

In the following sections interfaces to a point machine, signals and AzLM are given as example for EL2 hardware inferfaces.

### WEM-V2 Interfaces

WEM-V2 interfaces are used to send commands and retrieve status information from a point machine. Up to three WEM-V2 are mounted in a subrack and are controlled by one interface controller (IC1D). According to the relationship between the IC1D and WEM-V2 interfaces, we have following failure units with different severity level of consequences:

- IC1D unit with 3 WEM-V2: Three WEM fail due to a failure of the controlling IC1D,

- IC1D unit with 2 WEM-V2: Two WEM fail due to a failure of the controlling IC1D,

- IC1D unit with 1 WEM-V2: One WEM fails due to a failure of the controlling IC1D,

- WEM-V2: A failure of a WEM-V2 occurs.

| Subsystem | Logical equation | FiT =[10-9/h] GB 40 | MTBF [y] |
| --- | --- | --- | --- |
| WEM-V2 | WEM-V2 | 600 | 190,3 |
| IC1D+1 WEM-V2 | IC1D1W=ICBP2D+IC1D+WEM-V2+WEBG | 3 820 | 29,9 |
| IC1D+2 WEM-V2 | IC1D2W=ICBP2D+IC1D+2*WEM-V2+WEBG | 4 420 | 25,8 |
| IC1D+3 WEM-V2 | IC1D3W=ICBP2D+IC1D+3 *WEM-V2+WEBG | 5 020 | 22,7 |

Table 5: Failure rates with WEM-V2 Interfaces

### Signal Interfaces

Signal interfaces are used to send commands (signal aspects) and retrieve status information from a signal. Different types of signal interfaces are used according to the different signal types and aspects.

- Main / Distant Signal; Subsidiary / Shunting Signal

  - LAM2F (always present)

  - 1st LAM3F (if green lamp, yellow lamp2 is present)

  - 2nd LAM3F (if yellow lamp1 is present)

  - 3rd LAM3F (if subsidiary/shunting signal is present)

  - According to these combinations the back panel LABG6F supports the mounting of one LAM2F and up to three LAM3F and provides the connection to the interface controller IC1D.

- Stand Alone Distant Signal

  - LAM2F (yellow lamp; always present)

  - LAM3F (green lamp; always present)

  - According to this combination the back panel LABG4F supports the mounting of one LAM2F and one LAM3F and provides the connection to the interface controller IC1D.

- Stand Alone Shunting Signal

  - LAM1F (always present)

  - According to this arrangement the back panel LABG5F supports the mounting of one LAM1F and provides the connection to the interface controller IC1D.

According to the different signal types and different arrangements following failure units are to consider:

| Signal Type | Logical equation | FiT =[10-9/h] GB 40 | MTBF [y] |
| --- | --- | --- | --- |
| Stand Alone Shunting Signal | LAM1F | 740 | 154,3 |
| Main / Distant, Shunting / Subsidiary, Stand Alone Distant Signal | LAM2F | 1 100 | 103,8 |
|  | LAM3F | 1 150 | 99,3 |
| Main / Distant Signal | SigMD_0 = IC1D + ICBP2D + LAM2F + LABG6F | 4 320 | 26,4 |
|  | SigMD_1 = IC1D + ICBP2D + LAM2F + LAM3F + LABG6F | 5 470 | 20,9 |
|  | SigMD_2 = IC1D + ICBP2D + LAM2F + 2*LAM3F + LABG6F | 6 620 | 17,2 |
| Shunting / Subsidiary Signal | SigSS_1 = IC1D + ICBP2D + LAM2F + LAM3F + LABG6F | 5 470 | 20,9 |
| Stand Alone Distant Signal | SigSA_D = IC1D + ICBP2D + LAM2F + LAM3F + LABG4F | 5 470 | 20,9 |
| Stand Alone Shunting Signal | SigSA_S = IC1D + ICBP2D + LAM1F + LABG5F | 3 960 | 28,8 |

Table 6: Failure rates with Signal Interfaces

### AzLM Interfaces

AzLM interfaces (AWM) are used to retrieve relevant track occupied or clear indication from the AzLM system. Up to three AWMs are mounted in a subrack and are controlled by one interface controller (IC1D). According to the relationship between the IC1D and AWM interfaces, we have following failure units with different severity level of consequences:

- IC1D unit with 3 AWM: Three AWM fail due to a failure of the controlling IC1D,

- IC1D unit with 2 AWM: Two AWM fail due to a failure of the controlling IC1D,

- IC1D unit with 1 AWM: One AWM fails due to a failure of the controlling IC1D,

- AWM: A failure of an AWM occurs.

The logical equations in the table below respect besides the PBAs also failures of the backplane (ICBP2D), resp. subrack (AWBG) of the HW in use.

| Item | Logical equation | FiT =[10-9/h] GB 40 | MTBF [y] |
| --- | --- | --- | --- |
| AWM | AWM | 357 | 319,8 |
| IC1D+1 AWM | IC1D1W=ICBP2D+IC1D+AWM+AWBG | 3 577 | 31,9 |
| IC1D+2 AWM | IC1D2W=ICBP2D+IC1D+2*AWM+AWBG | 3 934 | 29,1 |
| IC1D+3 AWM | IC1D3W=ICBP2D+IC1D+3 *AWM+AWBG | 4 291 | 26,6 |

Table 7: Failure rates with AWM Interfaces

## New concept of remote control

The following chapters provide information about a new architectural specific of the ELEKTRA 2.x system which is a new concept of remote control for optimized communication between EBO2 and ELEKTRA CC.

The new concept of remote control and operation provides a new communication from different single stations to the ELEKTRA CC via ELEKTRA internal redundant LAN. A communication via RCUs to provide this type of control is no longer necessary.

As an addition to all types of communication that are supported by the generic application (see [SYS_Description]), the new concept of remote control enables a connection of an ELEKTRA CC (equipped with MEN-D6 or MEN-D10 boards) to remote EBO2s. For an ELEKTRA CC as well as for an EBO2 it is of no relevance if they are installed at the same place or at different places.

Figure 1 below, taken from the document [SSDD_Remote], shows the physical architecture of the new communication between EBO2 and ELEKTRA CC. To be able to control each ELEKTRA from each EBO2 the networks of the ELEKTRAs are connected switches of the ELEKTRA internal networks. This extension enables a communication from each EBO2 to each CC.

Figure 1: Physical architecture of new communication EBO2 – ELEKTRA (category 1)

This new communication provides the following improvements (compared to the existing communication):

- Each EBO2 is now directly connected to each ELEKTRA CC.

- If one CC fails, all other CCs are still available by each running EBO2.

- If an RCU-RCU connection fails, it does not interrupt the communication EBO2 – CC any more.

- As shown in Figure 1, two hardware paths for the new communication between EBO2 and ELEKTRA CC can be identified. Both paths will be analysed in the following sections.

- Additionally, a connection from each EBO2 to each CC can be done via connecting the switches of the ELEKTRA internal network, which is a Category 1 network, and connecting EBO2s on a Category 2 network to the ELEKTRA internal network using the “Cat. 1-Cat. 2 separation” network. For this connection, additional hot redundant switches are added to the hardware path. See Figure 2 and [SSDD_Remote] for a detailed description.

Figure 2: Physical architecture of new remote control concept related Communication EBO2 – ELEKTRA with Category 1 and Category 2 networks

- Table 8 lists the components/subsystems that are part of the new communication between EBO2 and ELEKTRA CC. The values are taken from the document [RAM_ANA_EL2].

| Component/ Subsystem | FiT GB 40 | MTBF [y] |
| --- | --- | --- |
| EBO2 | 10 000 | 11,4 |
| KVM-Switch | 2 000 | 57,1 |
| Switch | 2 200 | 51,9 |
| CC_D6 | 6 393 | 17,8 |
| CC_D10 | 4 700 | 24,2 |

Table 8: MTBF values components of new communication

A detailed description of each component/subsystem can be found in [SYS_Description], [ADD_EBO2] and [RAM_ANA_EL2].

As there is no dedicated hardware specified in the generic project for the “Cat. 1-Cat. 2 separation” component (Figure 2), in this analysis network switches in 1oo2 hot-redundancy mode are used as reference hardware, which is from RAM point of view also a recommendation for the use in the field.

The failure rate of these additional network switches is negligible when used in 1oo2 hot-redundancy configuration, therefore only the two communication paths shown in Figure 1 are to be considered in the following sections.

### Redundancy characteristics of the new concept of remote control

To achieve the required availability, different redundancy concepts are used in order to reflect the hierarchy of the ELEKTRA architecture and according to the impact of control of the subsystems in use.

- Switch is 1oo2 hot stand-by

- CC is 1oo2 warm stand-by

- EBO2 is either a

  - stand-alone workplace or equipped with

  - MMI-PC 1oo2 or equipped with complete

  - workplace redundancy (two or more EBO2)

The characteristics of hot stand-by and warm stand-by are as follows:

Hot stand-by

A single fault can be tolerated without any impact on operational availability. The switchover between redundant components is fully transparent and enables this way a high availability as well as a high reliability (MTBF) of the system.

Warm stand-by

A single fault can be tolerated, but there is typically a small impact on operational availability. Contrary to a cold stand-by mode, hardware is in power on state but a switchover to the redundant component is not fully transparent as operational processes on the redundant component may need to be started.

Since there is an operational impact the reliability is essentially the same as that of a non-redundant system. But the downtime is significantly smaller than in cold stand-by resulting in high availability figures.

The subsystem EBO2 has three different characteristics from a dependability point of view. These are

- Stand-alone: The EBO2 workplace consists of one MMI-PC together with keyboard, monitor and mouse (KVM). A failure of one of these components implies replacement with the corresponding spare-part.

- Redundancy 1oo2: The EBO2 consists of two redundant MMI-PCs together with keyboard, monitor and mouse. A switch-over unit (KVM-switch) can be applied in case of a fault of the active PC in order to switch over the connection of keyboard, mouse and monitor to the redundant PC.
The hardware PC is in hot stand-by mode (see [ADD_EBO2]) while the subsystem EBO2 is in warm stand-by mode.
If the KVM-switch fails, this component has to be replaced before work with this EBO2 can continue. If the KVM-switch fails and no spare is foreseen, it can also be removed from the EBO2 workplace and the MMI-PC can be directly connected with keyboard, mouse and monitor (without using a redundant stand-by MMI-PC).
The advantage of this system characteristic compared with the stand-alone EBO2 is the better availability. Although a failure of an MMI-PC may cause an operational interruption, the switch-over unit keeps the downtime short compared with the time needed for manual replacement so that operation can be continued quickly (see Table 13).

- Workplace redundancy: Two stand-alone EBO2 are available at the same location. One is active, while the other is in standby and ready for use in case the first EBO2 workplace fails. 
If two or more workplaces are up and running simultaneously with the user already logged in, even a hot stand-by redundancy is possible, depending on the distribution of responsibilities. If one workplace fails and its responsibilities are already active on the second workplace, the redundancy is hot stand-by. If certain actions have to be taken before control of the same responsibilities can be assumed, the redundancy is warm stand-by.

### New communication path 1

Each EBO2 residing on a station is connected to the CC of the local station. The hardware path of communication comprises EBO2, a local switch and the local ELEKTRA CC:

- EBO2 – Switch – CCD6

| Path 1 | Availability [%] | Unavailability [min/y] |
| --- | --- | --- |
| Λ1,1 = EBO2stand_alone + Switch1oo2 + CCD6_1oo2 | 99,996 | 21,0 |
| Λ1,2 = EBO21oo2 + Switch1oo2 + CCD6_1oo2 | 99,999 | 4,2 |
| Λ1,3 = EBO2WP_red + Switch1oo2 + CCD6_1oo2 | > 99,999 | < 0,1 |

Table 9: New communication hardware path 1 (MEN-D6)

Table 9 shows an availability of nearly 100% for hardware path 1 when applying the workplace redundancy.

- EBO2 – Switch – CCD10

| Path 1 | Availability [%] | Unavailability [min/y] |
| --- | --- | --- |
| Λ1,1 = EBO2stand_alone + Switch1oo2 + CCD10_1oo2 | 99,996 | 21,0 |
| Λ1,2 = EBO21oo2 + Switch1oo2 + CCD10_1oo2 | 99,999 | 4,2 |
| Λ1,3 = EBO2WP_red + Switch1oo2 + CCD10_1oo2 | > 99,999 | < 0,1 |

Table 10: New communication hardware path 1 (MEN-D10)

Table 11 shows an availability of nearly 100% for hardware path 1 when applying the workplace redundancy.

### New communication path 2

Each EBO2 residing on a station can be connected to an ELEKTRA CC of another station via ELEKTRA internal LAN. This hardware path of communication comprises EBO2, a local switch, a remote switch and a remote ELEKTRA CC:

- EBO2 – Switch – Switch - CCD6

| Path 2 | Availability [%] | Unavailability [min/y] |
| --- | --- | --- |
| Λ2,1 = EBO2stand_alone + Switch1oo2 + Switch1oo2 + CCD6_1oo2 | 99,996 | 21,0 |
| Λ2,2 = EBO21oo2 + Switch1oo2 + Switch1oo2 + CCD6_1oo2 | 99,998 | 9,5 |
| Λ2,3 = EBO2WP_red + Switch1oo2 + Switch1oo2 + CCD6_1oo2 | > 99,999 | < 0,1 |

Table 11: New communication hardware path 2 (MEN-D6)

- EBO2 – Switch – Switch - CCD10

| Path 2 | Availability [%] | Unavailability [min/y] |
| --- | --- | --- |
| Λ2,1 = EBO2stand_alone + Switch1oo2 + Switch1oo2 + CCD10_1oo2 | 99,996 | 21,0 |
| Λ2,2 = EBO21oo2 + Switch1oo2 + Switch1oo2 + CCD10_1oo2 | 99,998 | 9,5 |
| Λ2,3 = EBO2WP_red + Switch1oo2 + Switch1oo2 + CCD10_1oo2 | > 99,999 | < 0,1 |

Table 12: New communication hardware path 2 (MEN-D10)

Note the availability figures in Table 11 and Table 15 are practically the same as in the previous section since the influence of the additional network switch in 1oo2 hot-redundancy mode is negligible.

### Total availability of new concept of remote control

We consider the communication of the CC with a local EBO2 together with a remotely located EBO2. The total availability of the new concept of remote control results in:

|  | Availability [%] | Unavailability [min/y] |
| --- | --- | --- |
| ΛTotal ,1= Λ1,1+ Λ2,1 | 99,992 | 42,0 |
| ΛTotal ,2= Λ1,2+ Λ2,2 | 99,998 | 8,4 |
| ΛTotal ,3= Λ1,3+ Λ2,3 | > 99,999 | < 0,1 |

Table 13: Total availability new concept of remote control (MEN-D6)

|  | Availability [%] | Unavailability [min/y] |
| --- | --- | --- |
| ΛTotal ,1= Λ1,1+ Λ2,1 | 99,992 | 42,0 |
| ΛTotal ,2= Λ1,2+ Λ2,2 | 99,998 | 8,4 |
| ΛTotal ,3= Λ1,3+ Λ2,3 | > 99,999 | < 0,1 |

Table 14: Total availability new concept of remote control (MEN-D10)

As shown in Table 13 and Table 14 the total availability of the new concept of remote control between EBO2s and ELEKTRA CCs is in each characteristic significantly greater than the required 99,90% (see REQ-04 in section 4 below).

# RAM Requirements

The shown fulfilment of availability and reliability requirements of the system ELEKTRA 2 is analysed  [RAM_ANA_EL2].

## RaiLed Requirements

The fulfilment of availability and reliability requirements of the subsystem RaiLed is analysed and shown in [RAMAN_RaiLED].

Note: The product RaiLed ist not part of the ELEKTRA 2 product version International (short PV INT 2.10.0).

## Hardware Interfaces Requirements

No specific RAM related requirements are given for hardware interfaces of ELEKTRA 2, the fulfilment of availability and reliability requirements of the system ELEKTRA 2 is analysed and shown in [RAM_ANA_EL2].

## Remote Control Requirements

The RAM requirements for the new concept of remote control, listed in Table 15 are obtained from chapter 3 of the document [Pflichtenheft_MAV].

- REQ-01: MTBF new concept of remote control

| Funktion | Fehlerbeschreibung | MTBF [y] |
| --- | --- | --- |
| Befehl welcher auch | Irgendein Befehl kann nicht erteilt werden | 1 |
| Rückmeldung welcher auch | Irgendeine Rückmeldung ist fehlerhaft |  |
| eine, oder mehrere Ergänzungsfunktionen | Eine, oder mehrere Ergänzungsfunktionen sind nicht funktionsfähig |  |
| eine Gruppe der Befehle | Eine Gruppe der Befehle kann nicht erteilt werden | 3 |
| eine Gruppe der Rückmeldungen | Störung einer Gruppe der Rückmeldung |  |
| alle Sonderbedienungen | keiner der Sonderbedienungen kann erteilt werden |  |
| alle, zum Erteilen der Sonderbedienungen notwendige Rückmeldungen | Störung aller Rückmeldungen, die zum Erteilen der Sonderbedienungen notwendig sind |  |
| erschwertes Erteilen der Sonderbedienungen | keine der Sonderbedienungen kann erteilt werden |  |
| Stellwerkaufzeichnungen | keine der Sonderbedienungen kann erteilt werden, bis auf die Signalhaltstellung |  |
| Fahrstraßeneinstellung | keine der Fahrstraßen kann eingestellt werden | 9 |
| alle Funktionen | Ausfall des Gesamtsystems | 25 |

Table 15: MTBF requirements new concept of remote control

- REQ-02: Scheduled maintenance new concept of remote control

| Maintenance | MTBM [y] | MTTM [h] |
| --- | --- | --- |
| scheduled replacement of component | 5 | 1 |
| feature and safety check | 10 | 24 |

Table 16: Scheduled maintenance new concept of remote control

- REQ-03: The MTTR shall not exceed 4 hours.

- REQ-04: The availability of the system shall be 99,90% at a minimum.

### Fulfilment of RAM Requirements

- REQ-01: MTBF new concept of remote control

| Path 1 / Path 2 | MTBF [h] | MTBF [y] |
| --- | --- | --- |
| Λ1,1 = EBO2stand_alone + Switch1oo2 + CCD6_1oo2 | 61 002 | 7,0 |
| Λ2,1 = EBO2stand_alone + Switch1oo2 + Switch1oo2 + CCD6_1oo2 |  |  |
| Λ1,2 = EBO21oo2 + Switch1oo2 + CCD6_1oo2 | 54 369 | 6,2 |
| Λ2,2 = EBO21oo2 + Switch1oo2 + Switch1oo2 + CCD6_1oo2 |  |  |
| Λ1,3 = EBO2WP_red + Switch1oo2 + CCD6_1oo2 | 156 421 | 17,9 |
| Λ2,3 = EBO2WP_red + Switch1oo2 + Switch1oo2 + CCD6_1oo2 |  |  |

Table 17: MTBF values path 1 / path 2 (MEN-D6)

| Path 1 / Path 2 | MTBF [h] | MTBF [y] |
| --- | --- | --- |
| Λ1,1 = EBO2stand_alone + Switch1oo2 + CCD10_1oo2 | 68 027 | 7,8 |
| Λ2,1 = EBO2stand_alone + Switch1oo2 + Switch1oo2 + CCD10_1oo2 |  |  |
| Λ1,2 = EBO21oo2 + Switch1oo2 + CCD10_1oo2 | 59 880 | 6,8 |
| Λ2,2 = EBO21oo2 + Switch1oo2 + Switch1oo2 + CCD10_1oo2 |  |  |
| Λ1,3 = EBO2WP_red + Switch1oo2 + CCD10_1oo2 | 212 766 | 24,3 |
| Λ2,3 = EBO2WP_red + Switch1oo2 + Switch1oo2 + CCD10_1oo2 |  |  |

Table 18: MTBF values path 1 / path 2 (MEN-D10)

The MTBF-requirement in Table 15 specifies four different MTBF figures according to different severity levels.

Comparison with Table 17 and Table 18 gives:

  - The fulfilment of the first two classes with MTBF = 1 year and MTBF = 3 years is given in all architectural variants: EBO2 (stand-alone/1oo2/WP_red) and control via locally installed resp. remotely installed CC.

  - The fulfilment of the third class with MTBF = 9 years is given when applying the architectural variant with work-place redundancy.

  - The requirement of the fourth class with MTBF = 25 years is not fulfilled when using MEN-D6 boards and nearly fulfilled when using MEN-D10 boards. The warm standby CC1oo2 architecture is a limiting factor with a reliability of roughly MTBF = 17,9 years with MEN-D6 and MTBF = 24,3 years with MEN-D10. Nevertheless, it should be noted, that due to the warm-standby concept of the CC architecture a high availability is given as shown in Table 13 and Table 14.

Result: Requirement partially fulfilled

- REQ-02: Scheduled maintenance new concept of remote control

As stated in [EL_Maintenance] section 4.7, redundant components and reliability practices guarantee operational fitness. In case of errors like subassembly failure, full redundancy is restored by replacement with reserve material.

The cyclical employment of reserve material does not lead in and of itself to improved reliability. It is still up to the customer, however, to annually employ or replace reserve material.

Due to the various redundancy concepts described in chapter 3.3, operational availability of the system/ subsystems during replacement of components is always ensured.

According to [EL_Maintenance] there is no necessity for preventive maintenance actions. Besides that, a recommendation is given to exchange filter mats depending on dust accumulation. Since there are no scheduled maintenance actions obligatory the requirement is fulfilled.

Result:	Requirement fulfilled

- REQ-03: The MTTR shall not exceed 4 hours.

The FIT values in chapter 3.3 are taken from [RAM_ANA_EL2] and are calculated with an MTTR = 4 h which is even a conservative value in most cases. Calculations of system quantities are based on the constant failure rates of PBAs and devices and the average repair time (MTTR) of 8 hours. Availability results, which fulfil the requirements based upon this MTTR = 8 h fulfil the requirement MTTR = 4h a fortiori, because these calculations are more conservative than calculations based upon an MTTR = 4 h.

Please note that most corrective maintenance activities can be done in a significantly shorter amount of time than the conservatively assumed MTTR = 4 h.

Result:	Requirement fulfilled

- REQ-04: The availability of the system shall be 99,90% at a minimum

As shown in Table 13 and Table 14 the availability of the system is in each characteristic significantly greater than the required 99,90%.

Result:	 Requirement fulfilled

# END OF DOCUMENT

See [ARPR_WEM-V2]

See [ARPR_AWM]

If the same components are used for the Cat. 1-Cat. 2 separation as for the Cat-1 connection, about 8 FiT must be considered when using two switches in a 1oo2 hot-redundant configuration.