Note to the author:

Do NOT edit this page (cover).

It will be updated by GBMS team.

Note to the author:

Do NOT edit this page (cover).

It will be updated by GBMS team.

Template for PRC

G-TMP A01008 rev.04

Template for PRC

G-TMP A01008 rev.04

| GBMS 2.0 | COUNTRY |
| --- | --- |
|  | ALL |
| Document TYPE | LANGUAGE |
| TEMPLATE based on G-TMP A01008 rev.04 <applies to the new/updated G/L-PRC> | EN |

Note to the author: This template does not apply to Legacy GBMS. For further details, ask the GBMS Team.

Note to the author: This template does not apply to Legacy GBMS. For further details, ask the GBMS Team.

REVISION HISTORY

| Rev. | Date | Author | Approver | Authorizer | Revision Description |
| --- | --- | --- | --- | --- | --- |
| 00 | March 2026 | F. Addevico G. Guarino M. Maddalena | C. Maiorano | O. Hocheid | First Issue |

DOCUMENTS TREE

< This tree refers to the template’s “documents tree”. When defining the PRC, please make sure to replace this with the process specific “documents tree” considering the PRD at Level 1 (L1).

Do not translate document titles.

For clarification, refer to the GBMS Team. >

- G-TRN A01000 Management System Transition Rules

  - G-TMP A01008 Template for PRC

TABLE OF CONTENTS

1	SCOPE	5

1.1	Purpose	5

1.2	Applicability	5

1.3	Effective Date	5

2	TERMS	6

2.1	Acronyms and Abbreviations	6

2.2	Definitions	7

3	REFERENCES	8

3.1	Hitachi Corporate Reference Documents	8

3.2	GBMS Reference Documents	8

3.3	Applicable Standards and Regulations	8

4	DESCRIPTION	9

4.1	FRACAS Applicability	10

4.2	FRACAS Stakeholders	11

4.3	FRACAS Tool definition (FRA-10)	15

4.4	FRACAS Deliverables (FRA-11)	16

4.4.1	Project / Product RAM Monitoring and Demonstration Plan	16

4.4.2	Project / Product Reliability Growth Plan	17

4.4.3	Project / Product RAM Monitoring and Demonstration Report (FRA-07)	17

4.4.4	Project / Product Reliability Growth Report	18

4.5	FRACAS activities during Manufacturing phase	18

4.5.1	Set Up the Process (FRA-01)	18

4.5.2	Field Data Collection and analysis (FRA-02)	18

4.5.3	Data Processing (FRA-03)	20

4.5.4	Corrective Action System (CAS-01)	20

4.5.5	Failure Review Board (FRA-04)	21

4.6	FRACAS activities during T&C and O&M	21

4.6.1	Set Up the Process (FRA-05)	21

4.6.2	Field Data Collection (FRA-06)	22

4.6.3	RAM Failure assessment (FRA-07)	25

4.6.4	Data Processing (FRA-08)	26

4.6.5	Corrective Action System (CAS-02)	27

4.6.6	Failure Review Board (FRA-09)	28

5	RECORDS	28

6	ANNEXES	30

Annex 1 - Executive Summary	30

Annex 2 - Flow chart	35

# SCOPE

## Purpose

This document describes how to plan, evaluate, execute, monitor, and control the SRS System/Subsystem project and product FRACAS and Demonstration activities.

## Applicability

Refer to the table below for the applicability of this document.

| Legal Entities | All RSBU Legal Entities |
| --- | --- |
| Line of Business (LoB) | Signalling and Rail Solutions (SRS) |
| Project/Bid Type/Criteria | Applies to all projects (R&D, Customer, Products & Solutions Roadmap etc.) |
| Additional Criteria and Notes | See §4.1 |

## Effective Date

This document is applicable as soon as it is published.

# TERMS

## Acronyms and Abbreviations

| ATS | Automatic Train Supervision |
| --- | --- |
| CBTC | Communications-Bases Train Control |
| DWH | Data WareHouse |
| FLDC | Fleet Data Collector |
| FRACAS | Failure Reporting Analysis and Corrective Action System |
| FRB | Failure Review Board |
| GA | Generic Application |
| GP | Generic Product |
| HSTS | Hitachi rail STS |
| KPI | Key Performance Indicator |
| LCC | Life Cycle Cost |
| LRU | Line Replaceable Unit |
| MTBCF | Mean Time Between Critical Failure |
| MTBF | Mean Time Between Failures |
| MTBFF | Mean Time Between Functional Failures |
| MTBSAF | Mean Time Between Service Affecting Failures |
| MDT | Mean Down Time |
| MTTR | Mean Time To Replace |
| OBS | On-Board System |
| O&M | Operation & Maintenance |
| RAM | Reliability, Availability, Maintainability |
| RAMS | Reliability, Availability, Maintainability, Safety |
| RCA | Root Cause Analysis |
| R&D | Research & Development |
| SA | Specific Application |
| SA | Service Availability |
| SRS | Signalling & Rail Solutions |
| T&C | Testing & Commissioning |
| WPL | Work Package Leader |

## Definitions

# REFERENCES

## Hitachi Corporate Reference Documents

Unless otherwise noted, references apply to the latest approved revision.

| - | - |
| --- | --- |

## GBMS Reference Documents

Unless otherwise noted, references apply to the latest approved revision.

| PRD S06000 | SRS RAMS |
| --- | --- |
| PRC S06003 | RAM, FRACAS and LCC activities management |
| PRC XXX | Defect management |

## Applicable Standards and Regulations

| ISO 22163:2023 | Railway applications — Railway quality management system — ISO 9001:2015 and specific requirements for application in the railway sector |
| --- | --- |
| CEI EN 50126 | Railway applications - The specification and demonstration of Reliability, Availability, Maintainability and Safety (RAMS). |

# DESCRIPTION

This chapter details the MON-01 activities of PRD S06000.

This chapter details the RAM-HL_8 actions of PRC S06003.

The Failure Reporting Analysis and Corrective Action System (FRACAS) is a process used to collect and record data related to failures and their corrective actions. The main benefit of FRACAS process is to provide crucial information addressing the timely identification and correction of design, part problems and/or process errors.

The FRACAS process includes two main activities:

- Failure reporting (FRA);

- Corrective Action definition (CAS).

When we refer to FRACAS process, we may therefore refer to either one of the two or both the activities above listed at the same time.

The FRACAS process within SRS organization serves three main purposes:

- Ensure factory/field data collection;

- Identify, implement and follow up product/system/project improvements;

- Evaluate RAM performances of products/systems/project to validate the predictive RAM parameters as well as to ensure the projects’ requirements are met.

FRACAS activities may be required during different project phases. Despite the common aim of improving robustness of final delivery, FRACAS scope is slightly different according to the project phase. Also, specific RAM metrics are evaluated and different stakeholders are involved. Therefore, the process description during manufacturing phase (§4.5) is not the same as during T&C and O&M phases (§4.6).

FRACAS scope during Manufacturing phase is:

- Identify early products design issues

- Improve products manufacturing process and reduce early product failures;

- Identify suppliers weaknesses;

- Reduce early life problems of the final delivery.

FRACAS scope during T&C and O&M phases is:

- Identify product weaknesses (design and manufacturing);

- Ensure proper monitoring of products RAM performance;

- Identify and solve early system issues (either due to design, installation, commissioning, environment or other causes);

- Ensure achievement of performance targets since the early stages of final delivery commissioning;

- Monitor final delivery performance.

The FRACAS process applies to any project and shall be tailored on a case by case basis. In this document the following nomenclature is used:

- Project FRACAS: applicable to Turkey and Complex Signalling projects. It is mainly driven by contractual requirements. Purpose is to monitor and demonstrate project RAM and Performance KPIs (e.g. MTBSAF, SA, punctuality, etc.) as well as identify and follow-up defects;

- Product FRACAS: applicable to Single technology projects. It is mainly driven by GP, GA and SA internal needs. Purpose is to validate RAM parameters at LRU/Platform and GA level, and to improve the product development.

There even is a synergy between the two, as data from the field may come from the operation of a particular project, and the solution deployed on this project is by definition constituted of products.

The flowchart in Annex 2 represents the overall FRACAS process at product and project level as well as their dependency. Each activity is listed below and detailed in the next chapters:

- Set-up the FRACAS Process - FRA-01 (see §4.5.1) and FRA-05 (see §4.6.1);

- Field Data Collection - FRA-02 (see §4.5.2) and FRA-06 (see §4.6.2);

- RAM Failure Assessment - FRA-07 (see §4.6.3);

- Data Processing - FRA-03 (see §4.5.3) and FRA-08 (see §4.6.4);

- Corrective Action System - CAS-01 (see §4.5.4) and CAS-02 (see §4.6.5);

- Failure Review Board - FRA-04 (see §4.5.5) and FRA-09 (see §4.6.6);

- FRACAS Tool definition - FRA-10 (see §4.3);

- FRACAS Deliverables - FRA-11 (see §4.4).

## FRACAS Applicability

FRACAS is generally mandatory for all projects. There may be exceptions or deviations as described in the sections below.

The list below identifies a minimum set of conditions sufficient to deem the FRACAS process application as mandatory. Hence, if any of following conditions is true, FRACAS activities shall be performed:

- Request from customer to have a RAM performance follow-up;

- Contractual RAM requirements to be demonstrated;

- Specific clauses regarding hidden/systematic failures within the Request for Proposal (RFP) or within contractual terms & conditions;

- Presence in the scope of supply of a strategic SRS Organization product for which a follow-up must be performed;

- If the project uses any new product (make or buy), FRACAS activities must be performed until the product(s) maturity is reached and products performances (RAM, Safety, functional) are demonstrated;

- If Hitachi O&M team is in charge of warranty and/or post warranty maintenance.

## FRACAS Stakeholders

The management of Product/Project FRACAS activities during the monitoring period of each product/project is mainly undertaken by RAM team. Further stakeholders are involved in the FRACAS organization and may vary depending on project characteristics (e.g. RAM requirements, O&M organization, project organization and project phase).

Product RAM WPL

- Define the Product FRACAS process;

- Communicate and agree the FRACAS process internally with relevant teams;

- Support the FRACAS tool development;

- Establish the generic data framework;

- Prepare and issue the Product RAM Monitoring and Demonstration Plan;

- Collect data from relevant projects for product demonstration purposes;

- Review failure classification from different projects to align it to the failure classification;

- Evaluate Product RAM indicators based on filed data;

- Prepare and issue the Product Reliability Growth Plan;

- Communicate eventual deviation from predictive RAM indicators and/or non-compliance with product RAM targets;

- Identify defects affecting Product RAM performance;

- Identify and suggest action plans (RAM gap and performance);

- Follow-up action plans;

- Evaluate the impact of the action plans on RAM performance and reliability growth;

- Prepare and issue the Product RAM Monitoring and Demonstration Report;

- Prepare and issue the Product Reliability Growth Report;

- Present RAM indicators internally to relevant teams;

- Participate in the internal Failure Review Boards.

Project RAM WPL

- Support the deployment of the Product FRACAS Process, if necessary;

- Support the Product RAM WPL in the analysis of the RAM indicators;

- Define the Project FRACAS process;

- Communicate and agree the FRACAS process with project team;

- Support the FRACAS tool development;

- Establish the project data framework;

- Prepare and issue the Project RAM Monitoring and Demonstration Plan;

- Collect and checks field data;

- Perform RAM assessment on collected field data;

- Evaluate RAM Indicators based on field data;

- Prepare and issue the Project Reliability Growth Plan;

- Communicate eventual deviation from predictive RAM indicators and/or non-compliance with project RAM targets;

- Identify defects affecting Project RAM performance;

- Identify and suggest action plans (RAM gap and performance);

- Follow-up action plans;

- Evaluate the impact of the action plans on RAM performance and reliability growth;

- Prepare  and issue the Project RAM Monitoring and Demonstration Report;

- Prepare and issue the Project Reliability Growth Report;

- Present RAM indicators internally and towards the Customer;

- Organize and lead the Failure Review Boards involving the relevant stakeholders.

1st level Maintainer (HSTS O&M, Customer or third party)

- Provide data about event identification and details about 1st level maintenance intervention;

- Support the Product RAM WPL / Project RAM WPL in providing details on occurred failures and their resolution.

- Provide expertise to assess failures chargeability and determine failures root causes;

- Participate in the FRB.

2nd level Maintainer (HSTS O&M Technical Services, Customer or third party)

- Provide details about 2nd level maintenance intervention;

- Support the Product RAM WPL / Project RAM WPL in providing details on occurred failures and their resolution.

- Provide expertise to assess failures chargeability and determine failures root causes;

- Participate in the FRB.

3rd level Maintainer (HSTS Manufacturing, Repairing & Improvement or External Provider)

- Provide products repair data;

- Ensure expertise about failures root causes identification;

- Ensure products failures data collection during manufacturing;

- Lead products and manufacturing process failures investigation;

- Support corrective actions identification;

- Participate in the FRB.

RAMS Manager

- Accountable for FRACAS activities within the project;

- Ensure that RAM and Safety tasks related to FRACAS activities/deliverables are completed timely according to the project’s milestones;

- Responsible for escalating any issue that needs to be reported to the Project Manager;

- Participate in the FRB.

Safety WPL

- Provide expertise for assessing impact on safety aspects;

- Identify and communicate defects impacting Safety performance;

- Participate in the FRB.

Commissioning Manager

- During T&C phase, enforce data collection process on the field;

- During T&C phase, provide expertise to assess failures chargeability and determine failures root causes;

- Participate in the FRB.

Warranty/DNP Manger

- Ensure data collection on the field;

- Ensure follow up of warranty claims raised by the Customer together with the Project Manager;

- Perform follow-up on the Defect correction process;

- Accountable for defect resolution;

- Manage the R&R (Return & Repair) process;

- Participate in the FRB.

Project ENG WPLs /Product design referents

- Ensure expertise about failures root causes identification;

- Support the Defect correction process;

- Participate in the FRB.

Project Engineer

- Support the Product RAM WPL / Project RAM WPL in providing configuration data (PBS, etc.);

- Coordinate action plans with relevant Project ENG WPLs

- Participate in the FRB.

Project Manager

- Ensure follow up of warranty claims raised by the Customer together with the Warranty / DNP Manager;

- Support the Product RAM WPL / Project RAM WPL in the management of RAM-related defects arisen for the specific project;

- Support warranty activities in collaboration with O&M entity. In the frame of the FRACAS process facilitate the resolution of any issue/dispute.

- Participate in the FRB with the Client.

Customer/Operator

- Provide the operational data (service delay, trip lost, etc.);

- Participate in the FRB to validate RAM assessment.

Obsolescence referent

- Provide obsolescence statement for relevant products/technologies;

- Provide expertise to the obsolescence resolution;

- Support the identification of corrective actions;

- Participate in the FRB for Product FRACAS.

Development referent

- Ensure expertise about root cause analysis;

- Support the Defect correction process;

- Participate in the FRB for Product FRACAS.

Quality referent

- Support and follow-up the Defect correction process;

- Participate in the FRB for Product FRACAS.

The table below aims at identifying the main FRACAS stakeholders for each project phase. Mandatory and Optional stakeholders are identified, depending on project needs/requirements.

| Stakeholder | Project Phase |  |  |
| --- | --- | --- | --- |
|  | Manufacturing | T&C | O&M |
| Product RAM WPL | Mandatory | Mandatory | Mandatory |
| Project RAM WPL | Mandatory | Mandatory | Mandatory |
| 1st level Maintainer | N/A | Optional | Mandatory |
| 2nd level Maintainer | N/A | Optional | Mandatory |
| 3rd level Maintainer | Mandatory | Mandatory | Mandatory |
| Safety WPL | Mandatory | Mandatory | Mandatory |
| Commissioning Manager | N/A | Mandatory | N/A |
| Warranty/DNP Manager | N/A | N/A | Mandatory |
| Project ENG WPLs | Mandatory | Mandatory | Mandatory |
| Project Engineer | Mandatory | Mandatory | Mandatory |
| Project Manager | Mandatory | Mandatory | Mandatory |
| Customer/Operator | Optional | Optional | Mandatory |

Table 4-1: Project FRACAS process stakeholders

The table below aims at identifying the stakeholders involved in the Product FRACAS process and their mandatory/optional classification. The Product FRACAS is an internal process, therefore referents from relevant entities shall be assigned based on the involved technology (e.g. OBS, CBTC).

| Stakeholder | Product FRACAS |
| --- | --- |
| Product RAM WPL | Mandatory |
| Project RAM WPL | Optional |
| Safety WPL | Mandatory |
| Manufacturing referent | Mandatory |
| Development referent | Mandatory |
| Obsolescence referent | Optional |
| Quality referent | Mandatory |
| Product design referent | Mandatory |
| Maintenance referent | Optional |

Table 4-2: Product FRACAS process stakeholders

## FRACAS Tool definition (FRA-10)

Field data collected for different projects may have different levels of detail depending on the methodologies, formats, requests from customers and test team organization defined for the specific project. With the aim of providing a coherent set of data as well as a standardized format to be used for the product RAM monitoring and demonstration, it is recommended to use the same tool for failure data collection.

Unless different projects’ needs, HMAX SW tool shall be used as a default tool for FRACAS activities to ensure that a centralized tool is used across all countries for the above mentioned purposes. It offers the possibility to automatically import data already collected for projects on different tools linked to HMAX (e.g. Maximo). Also, an unified tool will manage interface with the factory to collect repair data and link to project failure data and configuration. It also supports the Corrective Action System (CAS) and allows the immediate computation of complex performance and RAM KPIs thanks to the automatic acquisition of performance and diagnostic data.

All stakeholders involved in the process can access to FRACAS tool with different authorization levels, according to their responsibilities (§4.2). The Project RAM WPL / Product RAM WPL shall provide the information needed for the RAM failure assessment by filling in the proper fields within HMAX tool.

The Project RAM WPL / Product RAM WPL shall be involved in the process of the FRACAS tool definition to ensure that all the necessary functions are executed, including: the correct computation of RAM and performance KPIs; the development of interfaces with other systems (e.g. interface with ATS to record and analyse the ATS alarms); the definition of failures’ database in line with project and product FRACAS activities. This task can be performed through the following macro activities:

- Definition of KPIs list, formulae and criteria:

The Project RAM WPL / Product RAM WPL shall define the list of Quality Criteria (QC), RAM and LCC KPIs, formulae and criteria for their computation;

- Definition of input parameters for KPIs computation and interface with external systems:

Static and/or dynamic inputs data may be needed for the KPIs computation, as well as interface with other systems (e.g. train temperature from FLDC/DWH; actual delay from ATS; Operator/Customer tools).

Project RAM WPL / Product RAM WPL shall define each and every system, module or database that shall interface with the tool.

- Definition of RAM dashboards:

Project RAM WPL / Product RAM WPL shall define the default representation of KPIs as well as the necessary filters to customize the plots. Also, trend of specific KPIs may be shown according to defined models (e.g. Reliability growth models, spare parts consumption models). In this case, the Project RAM WPL / Product RAM WPL shall provide all the necessary inputs and formulae.

- Verification of FRACAS tool functionalities:

Project RAM WPL / Product RAM WPL shall continuously cooperate with the tool development team to ensure the correct implementation and customization of the FRACAS process. This includes the review of the tool technical specification as well as the verification of implemented functionalities since the preliminary phases of tool development.

## FRACAS Deliverables (FRA-11)

### Project / Product RAM Monitoring and Demonstration Plan

The Project / Product RAM Monitoring and Demonstration Plan is a document that provides the guidelines of the activities to be performed during the FRACAS process, including the details of the RAM demonstration.

This section describes the minimum content of the document:

- Presentation of the general process;

- List of applicable RAM requirements;

- FRACAS organization chart;

- Identification of field data inputs;

- Description of FRACAS tool;

- Definition of RAM and performance KPIs;

- Definition of pass/fail criteria;

- Failure Classification Review process;

- Deliverables to be produced.

Project RAM WPL / Product RAM WPL shall produce this document when the FRACAS process has been defined.

### Project / Product Reliability Growth Plan

The Project / Product Reliability Growth Plan focuses on Corrective Action System (CAS) activities aimed at improving the system/product performance by the identification and resolution of issues occurring on field.

The minimum contents of the document are:

- Presentation of the general process;

- Identification of field data inputs;

- Description of methodology for evaluating Reliability and performance indicators;

- Definition of pass/fail criteria;

- Failure Review Board process;

- Deliverables to be produced.

Depending on project / product needs, contents of this document can be included in the Project / Product RAM Monitoring and Demonstration Plan.

### Project / Product RAM Monitoring and Demonstration Report (FRA-07)

The Project / Product RAM Monitoring and Demonstration Report will be generated using failure data collected as specified in §4.5.2/§4.6.2 and shall provide results obtained from the FRACAS activities. It shall contain as a minimum:

- Compliance status to applicable RAM requirements;

- List of monitored equipment;

- Operating hours;

- Failure distribution Pie charts for Status, Cause and Equipment type;

- Analysis of main contributing failures;

- Methodology of KPIs’ evaluation;

- Observed RAM performance vs time plots, including applicable targets (RAM and performance KPIs);

- Pareto graphs of key drivers in the current RAM indicators;

- Action plans necessary to solve any non-compliance.

Further to the above mentioned information the RAM Monitoring and Demonstration Report will include the updated data framework containing field data collected and related RAM assessment, used as input for the estimation of system/product reliability indicators. Also, contents of this document may vary depending on project/product needs.

This document may be used by the Project RAM WPL / Product RAM WPL to share results of the FRACAS activities with project/product team.

### Project / Product Reliability Growth Report

Project / Product Reliability Growth Report will be produced to give evidence of the enhancement of the reliability performance of the system/product and shall contain as a minimum:

- Detailed description of observed issues and related corrective actions;

- Status of corrective actions’ implementation;

- Issues’ distribution Pie charts for Status, Cause and Equipment type;

- Analysis of main contributing factors;

- Observed improvement of reliability performance.

Depending on project / product needs, contents of this document can be included in the Project / Product RAM Monitoring and Demonstration Report.

## FRACAS activities during Manufacturing phase

### Set Up the Process (FRA-01)

The Project/Product FRACAS process must be set up in a clear and stable manner. This means that the following features shall be established:

- Identification of stakeholders involved and their responsibilities;

- FRACAS tool to be used and possible development;

- Definition of main inputs for data collection and analysis;

- Definition of KPIs and criteria;

- Identification of necessary deliverables;

- Identification of system acceptance criteria.

The FRACAS process shall be reviewed and agreed by all stakeholders involved.

### Field Data Collection and analysis (FRA-02)

Among all the FRACAS activities, data collection is certainly that one which may represent the synergy between Project and Product FRACAS. Indeed, most data can be used for processing RAM indicators at both Project and Product levels.

Therefore, it is highly recommended to use a common data framework for field data collection at Project and Product levels. A data framework is a template to be filled with field data. The Project data framework may require additional information to meet the applicable contractual requirements.

Data framework applicable to Manufacturing phase is composed of three data types:

| Project phase | Data type | Description |
| --- | --- | --- |
| Manufacturing | Project/product Identification | Any data related to the specific product / project. |
|  | Event identification | Any data related to the event, that is to say the result of an abnormal observation or malfunction as seen by the manufacturer. |
|  | Corrective action | The set of information necessary to classify the nature of the defect and the corrective action identify to solve it. |

Table 4-3 Data type - Manufacturing

Depending on project/product Organization, data collection could be under the responsibility of:

- Hitachi Rail’s Manufacturing sites: in this case each site plant is responsible for collecting data related to its products;

- External Provider: Hitachi Rail has to ensure that Manufacturer does implement FRACAS process during Manufacturing. This is in charge of Procurement that manages the identification, selection, acceptance and follow up of external suppliers.

The following table shows the baseline data collection template to be used.

| TABLE | FIELDS | DESCRIPTION | FORMAT | SOURCE |
| --- | --- | --- | --- | --- |
| Project / Product identification | Project Name | Name of project (if applicable) | Free text | Data provided by the manufacturer(s). When several manufacturing sites are involved (most of the times), multiple entries have to be foreseen. |
|  | Site Plant | The name of the manufacturing plant | List: manufacturing sites |  |
|  | Product PN | Part number of the product manufacturing process concerned | List of Part Numbers of products from configuration database |  |
|  | Product batch | The ID of the manufacturing batch the product belongs to | Free text |  |
|  | Serial number | The serial number of the product concerned by the defect observed | Free text |  |
|  | Software version | Software version of the item concerned by the defect observed | Free text |  |
|  | HW version | Hardware version of the item concerned by the defect observed | Free text |  |
| Event identification | Event ID | Primary key using the identification number of each event (anomaly) observed. | Automated number |  |
|  | Event date & Time | Date and time of the event | DD/MM/YYYY hh:mm:ss |  |
|  | Manufacturing phase | Phase of the manufacturing process during which the event occurred | List: "Fabrication”; “Assembling”; “Inspection”; “Test” |  |
|  | Event description | Circumstances and consequences of the detected anomaly | Free text |  |
|  | Symptoms | How has the defect been identified. What the user sees, message from interface, symptoms, etc. | Free text |  |
|  | Event Cause | Information about the kind of root cause identified for the event. | List: "Product Design issue”; “Product manufacture issue”; “Manufacturing Process”; “External causes”; “Operator mishandling”; “Other”. |  |
| Corrective action | Defect? | May the occurred event be classified as defect? | List: “Yes; No” |  |
|  | Defect ID | Identifier of known (or new) defect | List of defects |  |
|  | Defect Solution | The envisaged solution to the defect found. | Free text |  |

Table 4-4 Baseline RAM Data collection template – Manufacturing

### Data Processing (FRA-03)

The failure events observed during manufacturing are analysed in order to:

- Identify any need for corrective actions;

- Identify the need for revising the RAM assessment of the concerned product;

- Identify the need for modifying/challenging suppliers.

Project RAM WPL / Product RAM WPL shall collect the evidence of failures/defects occurred during the production and the related corrective actions implemented in the production process and/or the product design. The relevant output shall be shared internally or with the Customer (if necessary).

Production yield after Burn-In and after Ambient Temperature tests, are generally used as KPI representing the percentage of efficiency.

### Corrective Action System (CAS-01)

One of the objectives of the FRACAS process is to improve the product performance by identifying and removing any possible defect observed.

A possible defect is identified whenever an issue is identified during manufacturing phase related to:

- Product design;

- Manufacturing process;

- External Provider selection.

Data collected during manufacturing process helps to identify the root cause of the issue. FRACAS process helps the manufacturer to track the activities done in order to establish the root cause of the problem.

For each defect identified, a detailed root cause analysis has to be investigated and a Corrective action has to be identified, evaluated and implemented.

A corrective action may entail a modification of the configuration baseline. These modifications shall be managed according to PRC XXX (DEFECT MANAGEMENT PROCESS – management of WI)

Once a corrective action is implemented, its effectiveness has to be assessed by monitoring the manufacturing process and related KPIs.

Project RAM WPL / Product RAM WPL shall follow-up the status of defects and monitor the product performance following the implementation of corrective action.  The goal is to enhance the product’s performance and reduce the likelihood of failures.

### Failure Review Board (FRA-04)

Periodical failure review board (FRB) shall be held to evaluate all failures observed during the manufacturing phase and implement the Corrective Action System (CAS) to identify and remove any possible defect.

Project RAM WPL / Product RAM WPL shall organize, lead and record the FRBs. FRB frequency and attendees may vary depending on project/product’s needs.

## FRACAS activities during T&C and O&M

### Set Up the Process (FRA-05)

The Project/Product FRACAS process must be set up in a clear and stable manner. This means that the following features shall be established:

- Identification of stakeholders involved and their responsibilities;

- FRACAS tool to be used and possible development;

- Definition of main inputs for data collection and analysis;

- Definition of RAM and performance KPIs, formulae and criteria;

- RAM assessment criteria, including the definition of failure categories and RAM chargeability;

- Identification of necessary deliverables;

- Identification of system acceptance criteria.

The Project RAM WPL/Product RAM WPL shall ensure that the FRACAS process is aligned to any other process that interface with the FRACAS process (e.g. fault management process, asset management process).

The FRACAS process shall be reviewed and agreed by all stakeholders involved.

At this stage a RAM Monitoring and Demonstration Plan for the project/product should be issued.

### Field Data Collection (FRA-06)

Among all the FRACAS activities, data collection is certainly that one which may represent the synergy between Project and Product FRACAS. Indeed, most data can be used for processing RAM indicators at both Project and Product levels.

Therefore, it is highly recommended to use a common data framework for field data collection at Project and Product levels. A data framework is a template to be filled with field data. The Project data framework may require additional information to meet the applicable contractual requirements.

The input fields of the data framework are organized into 6 families:

| Project phase | Data type | Description |
| --- | --- | --- |
| Test & Commissioning – Operation & Maintenance | Project Identification | Any data related to the specific project. |
|  | Event Identification | Any data related to the operational environment of the event, that is the result of an abnormal observation or malfunction as seen by the Customer and/or the 1st line of maintenance. |
|  | Intervention – 1st maintenance level | Any data related to first level of maintenance activities (field). |
|  | Intervention – 2nd maintenance level | Any data related to the second level of maintenance activities (workshop). |
|  | Repair – 3rd maintenance level | Any data related to the third level of maintenance activities (repairs). If repairs are external to HSTS, information is still mandatory. |
|  | RAM failure assessment | The classification of failures according to their chargeability (under HSTS responsibility). The Project RAM WPL/Product RAM WPL is responsible for this field. |

Table 4-5 Data type – T&C – O&M

What follows is the generic data framework applicable to T&C and O&M phases, that Product RAM WPL/Project RAM WPL can re-use and adapt for their needs:

| TABLE | FIELDS | DESCRIPTION | FORMAT | SOURCE |
| --- | --- | --- | --- | --- |
| Project Identification | Project Name | Name of project according to the tool configuration | Free text | Project data provided by the Operator or HSTS O&M department (depending on who is responsible for receiving alarms from field) |
|  | Project phase | Phase of the project during which the event occurred | List: "T&C”; “DCP”; “Out of Warranty" |  |
| Event Identification | Event ID | Primary key using the identification number of each event (anomaly) observed. | Automated number |  |
|  | Event date & Time | Date and time of the event | DD/MM/YYYY hh:mm:ss |  |
|  | Event description | Circumstances and consequences of the detected anomaly | Free text |  |
| Intervention – 1st maintenance level | Intervention ID | Primary key using the identification number of each intervention following an anomaly, knowing that one event may lead to several interventions. | Automated number | First line of maintenance data provided by the Client maintenance team or HSTS O&M department (depending on who is responsible for 1st line of maintenance) |
|  | Type of intervention | Type of intervention performed. | List: “Corrective replacement; Preventive replacement; Retrofit; Other – No replacement” |  |
|  | Failure consequence | Effect of occurred failure event | List: “No Impact; Loss of Redundancy; Service Affecting Failure” |  |
|  | Service Impact | Did the occurred failure event affect the train circulation? | List: “Yes; No” |  |
|  | Delay | Possible delay caused by the occurred failure event on train circulation | hh:mm:ss |  |
|  | Detection mean | How the occurred failure event is detected? | Free text |  |
|  | Subsystem description | Name of the subsystem on which intervention is performed. | List of subsystems from configuration database |  |
|  | LRU Part number | Part number of the failed line replaceable unit | List of LRU Part Description from configuration database |  |
|  | LRU description | Name of the failed line replaceable unit | Automatic field from configuration database |  |
|  | Location | Name of location where the failure event occurred | List of locations from configuration database |  |
|  | Central Post/ Peripheral Post / Trackside / Train | Classification of location where the failure event occurred | List of locations from configuration database |  |
|  | Train ID | Identification number of train where the failure event occurred | List of train ID from configuration database |  |
|  | Intervention action by Maintenance Staff | Description of the actions done by field maintenance during intervention. | Free text |  |
|  | HW Replacement? | Is a HW replacement needed? | List: "Yes; No" |  |
|  | Active maintenance time | Active corrective maintenance time required resolving a malfunction, including the time to diagnose the fault, isolate the fault, replacing or repairing the faulty part, and operationally control the replaced or repaired part to check its operational condition. | mm:ss |  |
|  | Down time | Time during which the system was unavailable (taking into account administrative and logistics delays + active maintenance time) | mm:ss |  |
|  | Root Cause | A preliminary description by the maintainer of the possible cause(s) that led to the event | List: “Wrong manipulation; External cause; Vandalism; Operator/Driver mistake; Maintainer mistake (No Hitachi); Maintainer mistake (Hitachi); Random HW/SW failure; Systematic failure; SW upgrade; No Fault Found (NFF); Other” |  |
|  | Event Status | Status of maintenance intervention | List: “Open; Closed” |  |
| Intervention – 2nd maintenance level | Failed LRU part description | Name of the failed shop replaceable unit | List of LRU Part Description from PBS | Second level of maintenance data provided by the Customer maintenance team or HSTS O&M department or third party (depending on who is responsible for 2nd level of maintenance) |
|  | LRU Part number out | Part number of the failed shop replaceable unit | List of LRU Part number from PBS |  |
|  | SRU Part number in | Part number of the new shop replaceable unit | List of SRU Part number from PBS |  |
|  | SRU Serial number out | Serial number of the failed shop replaceable unit | Free text |  |
|  | SRU Serial number in | Serial number of the new shop replaceable unit | Free text |  |
| Repair – 3rd maintenance level | Repairing Ticket ID | ID of repair ticket | Automated number | Repair data provided by the manufacturer. As this guideline applies to HSTS products this data is under HSTS responsibility. Repair data is in possession of HSTS repair site. |
|  | Repair details | Any particular comment on repair activity in manufacturing unit | Free text |  |
|  | Repair diagnosis | Defines the root cause for classification of the failure | Free text |  |
|  | Repair action | Describes particular actions for corrective maintenance in manufacturing unit | Free text |  |
|  | Repair unit | Manufacturing unit responsible for replacement of the failed item | Free text |  |
| RAM Failure assessment | Chargeable to Hitachi? | Is the occurred failure event liable to Hitachi? | List: “Yes; No” | RAM Failure assessment provided by the RAM WPL and reviewed by the Product RAM WPL |
|  | HW / SW failure | Nature of occurred failure event | List: “HW; SW” |  |
|  | Systematic failure | May the occurred failure event be classified as systematic? | List: “Yes; No” |  |
|  | Systematic failure ID | Identifier of known (or new) systematic failure | List of defects |  |
|  | Service affecting failure | Did the occurred failure event affect the train circulation? | List: “Yes; No” |  |
|  | Safety Impact | In case of Safety impact, which is the consequences caused by the failure event? | Free text |  |
|  | Remarks | Any relevant remark | Free text |  |
|  | RAM status | Which is the status of the failure event validation by RAM team? | List: ”Open; Under investigation; Closed” |  |
|  | Failure category | Failure category assessed by Product RAM WPL | List of failure categories |  |
|  | Apparent failure? | May the occurred failure event be classified as apparent failure? | List: “Yes; No” |  |
|  | Effective failure? | May the occurred failure event be classified as effective failure? | List: “Yes; No” |  |
|  | Random failure? | May the occurred failure event be classified as random failure? | List: “Yes; No” |  |

Table 4-6 Baseline RAM Data collection template – T&C – O&M

### RAM Failure assessment (FRA-07)

Field data collected shall be analysed by the Project RAM WPL in order to:

- Assess event’s liability based on project’s scope of work and warranty conditions;

- Assess RAM chargeability, based on failure root cause, contractual, operational and environmental conditions;

- Classify each failure according to applicable failure categories and the nature of the failure.

In case of a project, the contract often dictates how failures should be sentenced. Project RAM WPL is required to follow the contractual rules in addition to the minimum set of failure categorization.

The RAM Product Engineer will review the failure categorisation carried out by Project RAM WPL for each project, to be aligned with Product RAM purposes.

As a minimum, the following categories of failures are to be considered:

| Failure Category | Description |
| --- | --- |
| Apparent failures | The failures that the Client sees and may deem in the first place chargeable to the project. |
| Effective failures | Failures that actually are under project’s responsibility, after a finer analysis of apparent failures. They may come from software, hardware or services anomaly, according to the specific project assumptions. |
| Random hardware failures | The failures that originate from random hardware failures of equipment under project’s responsibility. |
| Software Failures | The failures that originate from software failures of equipment under project’s responsibility. These are the failures solved by a software upgrade or bug correction. |
| Recurrent/systematic failures | The failures that result in incompleteness, inadequacy, or imperfection due to a flaw in design, material, or workmanship, including manufacturing and installation. |

Table 4-7 Failure categories definition

### Data Processing (FRA-08)

Field data collected and assessed, will be used to calculate Project and Product RAM and performance KPIs.

In case of a project, the contract often dictates which KPIs shall be demonstrated. In this case the Project RAM WPL is required to follow the contractual rules, in addition to the transverse KPIs defined for Product and GA purposes.

Usual RAM and Performance KPIs requested by projects are:

- MTBSAF, MTBFF, MTBCF;

- MTTR, MDT, MTTRS;

- Service Availability;

- Technical availability;

- Service Punctuality;

- Trains availability;

- On-Time Running (OTR);

- Maximum Train Journey Time (MTJT);

- Quality Criteria Departure Time (QCDT);

- Quality Criteria Train Temperature (QCTT);

- Quality Criteria Door Movement (QCDM);

- Annual Labour cost, Annual Material cost.

RAM and performance KPIs may be calculated at different product and system levels.

The Product RAM WPL will use the product FRACAS database to calculate transverse KPIs, considering data from different projects.

Transverse KPIs are:

- MTBF apparent: it is calculated considering each and every failure event requiring a maintenance intervention;

- MTBF effective: it is calculated considering all failure events confirmed to be real failures, i.e. discarding No Fault Found (NFF) events;

- MTBF random: it is calculated considering only random HW failures, i.e. discarding NFF, SW and systematic failures.

This indicator is comparable to the corresponding hardware MTBF predicted during the development phase of the product.

Transverse KPIs can be calculated at LRU / Assembly / System level.

### Corrective Action System (CAS-02)

One of the objectives of the FRACAS process is to improve the system performance by identifying and removing any possible defect observed.

A possible defect is identified whenever one or more failures of these types occur:

- Recurrent/systematic failures;

- Safety critical failures;

- Service related failures;

- High monetary event;

- Any failure due to a flaw in design, material, or workmanship, including manufacturing and installation.

For each suspected defect, the Warranty/DNP Manager shall alert the ENG, RAM and Safety WPLs for their confirmation and preliminary analysis.

Once the defect is confirmed, a detailed root cause analysis has to be investigated and a Corrective action has to be identified, evaluated and implemented.

Responsible of RCA and Corrective action(s) shall be identified depending on the type of defect.

A corrective action may entail a modification of the configuration baseline. These modifications shall be managed according to PRC XXX (DEFECT MANAGEMENT PROCESS – management of WI)

Once a corrective action is implemented, Project RAM WPL / Product RAM WPL shall assess the effectiveness of the Corrective Action by monitoring the system in operation.

In addition, Project RAM WPL / Product RAM WPL shall follow-up the status of defects and monitor the system performance, as well as review the field data and compare it to the predicted RAM performance of affected items. This structured process allows collecting accurate and real-world measurements of RAM parameters with the aim of achieving the systems’ reliability growth. This analysis can be used as a practical benchmark for future design improvements. The goal is to enhance the system/product’s performance and reduce the likelihood of failures.

### Failure Review Board (FRA-09)

The objectives of the failure review board (FRB) are to:

- Classify and evaluate all failures observed during the applicable RAM Demonstration period;

- Supervise and follow-up the RAM performance monitoring process;

- Make decisions relative to in-service reliability and maintainability demonstration;

- Follow-up and validate corrective actions effectiveness;

- Arbitrate any dispute over failure responsibility.

The FRB shall also raise, agree, track, close and develop recommendations after understanding the contributing factors to prevent similar issues in future.

Prior to each FRB, additional technical meetings may be needed to facilitate the discussion in front of clients and reduce disputes.

Outputs of this activity shall be:

- The fault attribution.

- The identification of any new potential defect.

Project RAM WPL / Product RAM WPL shall organize, lead and record the FRBs. After each meeting, a MoM shall be produced to track the topics discussed. Also, intermediate RAM and performance KPIs may be requested to present during the meetings. FRB frequency and attendees may vary depending on project/product’s needs. All stakeholders involved in the Project/Product FRACAS process may be required to participate in the FRB.

# RECORDS

The table hereafter provides the list of records relating to this document.

< Please ensure consistency among actions description, records table, and input/output section >

| Description | Identification | Retention | Retention period | Storage |
| --- | --- | --- | --- | --- |
| < Title / Short description of the purpose of the record > | < Unambiguous custom code (not GBMS doc code) used to identify the record (e.g., REPORT_A1_YEAR_MONTH). > | < Unit/Role responsible for the retention and conformity with the applicable regulation of the record (e.g., Regional HR, Project Manager, etc.). > | < e.g., #nn years / months; Entire product lifecycle; End of warranty period; As per local regulation; … > | < Format, media and, if applicable, protection used to store the record in conformity with the applicable regulation (e.g., electronic document stored in PDM Tool. > |

# ANNEXES

## Annex 1 - Executive Summary

| INPUTS < Depending on the boundaries of the specific higher-level document (PRD/PRC) activity/action detailed by the current document, please provide the list of: - Applicable parent PRD/PRC input further refined, and/or - Deliverables produced by any other parent PRD activity linked to the current document, if applicable. > Note: Each input listed here must be detailed in at least one action. |  |  | OUTPUTS < Depending on the boundaries of the specific higher-level document (PRD/PRC) activity/action detailed by the current document, please provide the list of: - Applicable parent PRD/PRC output further refined, and/or - Deliverables produced by this document and required by any other parent PRD activity linked to the current document, if applicable. > Note: Each output listed here must be detailed in at least one action. |  |  |
| --- | --- | --- | --- | --- | --- |
| MEASURES Refer to KPI <XLIST> defined in KPI XYYZZZ. < Please replace XLIST with the KPI(s) allocated to the procedure, if any; otherwise, replace the entire statement with “None”. > |  |  |  |  |  |
| WORK ENVIRONMENT STANDARD < Define standard work environment (tools, infrastructures) that shall be used while performing actions. > < Please ensure consistency with the same section of the G-PRD; each tool listed here must be detailed in at least one action. > |  |  |  |  |  |
| R.A.C.I. |  |  |  |  |  |
| Action ID & Name | R = Responsible | A = Accountable |  | C = Consulted | I = Informed |
| FRA-01 Set Up the Process - Manufacturing | Product / Project RAM WPL | RAMS Manager |  | 3rd level Maintainer/ Manufacturing referent | Project ENG WPLs Project Engineer Project Manager Development referent Quality referent Product design referent Safety WPL |
| FRA-02 Field Data Collection - Manufacturing | Product / Project RAM WPL | RAMS Manager |  | 3rd level Maintainer/ Manufacturing referent Safety WPL | Project ENG WPLs Project Engineer Project Manager |
| FRA-03 Data Processing – Manufacturing | Product / Project RAM WPL | RAMS Manager |  | 3rd level Maintainer/ Manufacturing referent Safety WPL | Project ENG WPLs Project Engineer Project Manager |
| FRA-04 Failure Review Board - Manufacturing | Product / Project RAM WPL | RAMS Manager |  | Project Engineer Project ENG WPLs Safety WPL Development referent Quality referent Product design referent | Project Manager Obsolescence referent |
| FRA-05 Set Up the Process – T&C, O&M | Product / Project RAM WPL | RAMS Manager |  | Commissioning Manager Warranty/DNP Manager Project Engineer | Project Manager Project ENG WPL Customer/ Operator Manufacturing referent Development referent Quality referent Product design referent Safety WPL |
| FRA-06 Field Data Collection – T&C, O&M | Product / Project RAM WPL | RAMS Manager |  | 1st/2nd/3rd level Maintainer Commissioning Manager Warranty/DNP Manager Project Engineer Safety WPL | Project Manager Project ENG WPL Customer/ Operator Manufacturing referent Development referent Quality referent Product design referent |
| FRA-07 RAM Failure Assessment – T&C, O&M | Product / Project RAM WPL | RAMS Manager |  | 1st/2nd/3rd level Maintainer Commissioning Manager Warranty/DNP Manager Project Engineer | Project Manager Project ENG WPL Customer/ Operator Manufacturing referent Development referent Quality referent Product design referent |
| FRA-08 Data Processing – T&C, O&M | Product / Project RAM WPL | RAMS Manager |  | 1st/2nd/3rd level Maintainer Commissioning Manager Warranty/DNP Manager Project Engineer Project ENG WPLs Safety WPL | Project Manager Customer/ Operator Manufacturing referent Development referent Quality referent Product design referent |
| FRA-09 Failure Review Board – T&C, O&M | Product / Project RAM WPL | RAMS Manager |  | 1st/2nd/3rd level Maintainer Commissioning Manager Warranty/DNP Manager Project Engineer Project ENG WPLs Safety WPL | Project Manager Customer/ Operator Manufacturing referent Development referent Quality referent Product design referent |
| FRA-10 FRACAS Tool definition | Product / Project RAM WPL | Tool development team |  | 1st/2nd/3rd level Maintainer Commissioning Manager Warranty/DNP Manager | Project Engineer Project Manager Customer/ Operator |
| FRA-11 FRACAS deliverables | Product / Project RAM WPL | RAMS Manager |  | - | Project / Product Teams |
| CAS-01 Corrective Action System - Manufacturing | Product / Project RAM WPL | Manufacturing referent |  | Project ENG WPLs Product design referent Safety WPL | Project Engineer Project Manager Quality referent |
| CAS-02 Corrective Action System – T&C, O&M | Product / Project RAM WPL Warranty/DNP Manager Project ENG WPLs | Warranty/DNP Manager Project Manager |  | Project Engineer Development referent Product design referent Safety WPL | Quality referent |
| Note: “Head of” is not intended as the specific job title of an individual, rather it represents the link to the latest organisational structure. < R = Responsible: those who do the work to achieve the task; A = Accountable: the one ultimately answerable for the correct and thorough completion of the deliverable or task, and the one who delegates the work to those responsible. As a general rule, only one accountable role can be specified for each task or deliverable. In case of multiple accountable roles are required, each of them shall be accountable for the steps of the action in their scope. Use a “/” to separate them. Stating a Team or a Unit as accountable is not allowed. C = Consulted: those whose opinions are sought (typically subject matter experts) and with whom there is two-way communication; Do not include people already mentioned as “responsible” or “accountable”. I = Informed: those who are kept up-to-date on progress, and with whom there is just one-way communication. Do not include people already mentioned as “responsible”, “accountable”, or “consulted”. > |  |  |  |  |  |

## Annex 2 - Flow chart

< OPTIONAL >

< Rules for drafting the flowchart:

- Use MS Visio to draft the flow chart and copy it as an image in this document. MS Visio objects allowed for the flowchart are represented below.

- Relevant internal/external interfaces defined in the parent PRD/PRC document shall be reflected in the workflow.

- Dependant process (grey pentagon) shall be linked to at least one input/output item (see INPUTS and OUTPUTS section of this template).

- Dependant procedure actions/activities (white pentagon) shall be linked to at least one input/output item (see INPUTS and OUTPUTS section of this template). White pentagon can’t be used to represent procedures belonging to other processes.

- It is allowed a swim lanes diagram (i.e. a diagram that visually distinguishes job sharing and responsibilities (i.e. ‘R’ in the RACI table)) for the actions that make up an activity.

- Be consistent with the action defined within the document.

- Actions should have correlations (links) between each other.

- Assign to each action the same identifier defined in the related action paragraph.

Note: this page may also be formatted in landscape layout if required (see next page). >

< LANDSCAPE LAYOUT >

Project RAM WPLs shall analyse field data and perform a double failure classification: the first according to project failure categories and client requirements; the second has to be done according to the standard root causes defined in this guideline.